IPP Fax Project

IPP Fax transport requirements

| Revision | Date | Author | Notes |
| --- | --- | --- | --- |
| 1 | 10/16/2000 | Paul Moore, Peerless Systems Networking | Initial |
| 2 | 7/15/2003 | Gail Songer, Peerless |  |

This document captures the requirements for the transport component of IPP Fax. This document assumes that the reader is familiar with IPP 1.1.

Glossary

Sender – A piece of hardware and / or software that sends IPP fax documents to an IPP Fax receiver

Receiver – A piece of hardware and / or software that receives IPP Fax traffic.

Sending user – The human that initiates the transmission of an IPP Fax

Receiving user – The intended human recipient of an IPP Fax

Public access

The fundamental extension that needs to be made to IPP 1.1 is that there needs to be strictly defined behavior for anonymously accessed printers. This is a fundamental characteristic of PSTN fax that IPP does not provide. An administrator of a IPP Fax receiver needs to be able to make it publicly available on the Internet (or an intranet) but also needs to be informed of the identity of the sending user and equipment.

The typical IPP Fax machine will therefore be setup to not allow anonymous access via ‘plain’ IPP but will permit IPP Fax use. The printer might also be set up to allow regular secure IPP access.

Basic requirements met by IPP 1.1

- Synchronous and timely delivery

- Carried on the Internet or in-house

- Can be secure – both encryption and authentication are supported

Basic rules

.

- A receiver is specified by an ippfax: URL

- Data privacy must be ensured

- The receiver must supply strong authentication

- The sender may supply strong authentication; the receiver may require strong authentication.

IPP extensions

A sender must identify a job as being an IPP Fax Job.

- Anonymous access is allowed if the sender identifies a job as an IPP Fax job.

- A sender may indicate a return URI

- A receiver must support PDF/is. It may support other document formats