Printer Working Group (PWG)

Overview of the

PWG Semantic Model

May 29, 2002

Version 0.4

File: ftp://ftp.pwg.org/pub/pwg/Semantic_model/PWG-Semantic-Model-Overview.doc

Table of Contents

1	Model Overview	4

1.1	Conformance terminology	5

1.2	Typographic conventions	5

2	Action Summary	5

3	Data Classes	5

3.1	Printer object class	5

3.1.1	Printer Description Attributes	6

3.1.2	The “PrinterState” attribute and the Printer Life Cycle	7

3.1.3	"Job Template" Printer Attributes	8

3.2	Job object class	9

3.2.1	“Job Template” Job Attributes	9

3.2.2	Job Description Attributes	10

3.2.3	The “JobState” attribute and the Job Life Cycle	11

4	Actions	12

4.1	Job Creation and document submission Actions	12

4.1.1	PrintJob	13

4.1.2	PrintUri	14

4.1.3	CreateJob	14

4.1.4	SendDocument	15

4.1.5	SendUri	16

4.1.6	ValidateJob	16

4.2	Job Control Actions	16

4.2.1	CancelJob	16

4.2.2	HoldJob	17

4.2.3	ReleaseJob	17

4.2.4	RestartJob	17

4.3	Status and information Actions	17

4.3.1	GetJobs	17

4.3.2	GetPrinterAttributes	18

4.3.3	GetJobAttributes	18

4.4	Printer Control Actions	19

4.4.1	PausePrinter	19

4.4.2	ResumePrinter	19

4.4.3	PurgeJobs	19

5	References	19

6	Appendices	20

6.1	Changes to remove some IPP specific aspects	20

6.2	Summary of Job and Printer object attributes	20

6.3	Action Status Codes	29

7	Change Log	30

Table of Figures

Figure 1 Model Overview	4

Figure 2 Printer data class	6

Figure 3 Printer Description Attributes	7

Figure 4 - The “PrinterState” attribute and the Printer life cycle	8

Figure 5 Job data Class	9

Figure 6 Job Template Attributes	10

Figure 7 Job Description Attributes	11

Figure 8 - The “JobState” Job Description attribute and the Job object life cycle	12

Figure 9 Production instruction processing	13

Table of Tables

Table 1 - Summary of Actions	5

Table 2 - Integer syntaxes whose “xxxSupported” syntax isn’t RangeOfInteger	8

Table 3 - Optional Job Template Attributes	21

Table 4 - Mandatory Job Description Attributes	22

Table 5 - Optional Job Description Attributes	23

Table 6 - Mandatory Printer Description Attributes	25

Table 7 - Optional Printer Description Attributes	27

# Model Overview

The Printer Working Group (PWG) has defined a simplified printing model.  It represents printing in a client/server print paradigm.  The PWG model describes the server as a Printer object.  A Printer object may represent one or more physical Printers.  The other main object is the Job.  A Printer can contain zero or more Jobs and a Job is contained in only one Printer.  The PWG model contains methods that act upon these objects.

Figure 1 Model Overview

The objects are represented in the semantic model as data classes. The methods are represented as a set of actions that act upon those data classes.  The actions permit the creation and control of Jobs as well as the submission of a Job’s document content or URL reference to the document content.  Other actions allow the control of the Job or Printer.  There are also actions to query a Printer or Job to access their attributes or to obtain a list of jobs.

The model uses a number of terms with specific meaning for a printer.

Media: A sheet of paper, or other material, used for printing.

Impression:  Everything printed on a single side of a media.

Page: A logical entity that represents the information contained on a single side of a sheet of media.  Note that this the electronic form and that multiple pages can be rendered into a single impression through N-Up printing.

## Conformance terminology

The following conformance terminology is used in this document (same as in [RFC2911]):

- “must” - verb indicates a requirement for the subject of the sentence.

- “should” - verb indicate a recommendation for the subject of the sentence.

- “may” - verb indicates an implementation option for the subject of the sentence.

- “support” - the Printer performs as defined for the semantics of the operation or attribute.

## Typographic conventions

The following typographic conventions are uses in the document:

Bold indicates an action or attribute that must be supported by the Printer.

Italic indicated an action or attribute that may be supported by the Printer.

Square brackets ([]) indicate an attribute that a client may omit from an action request or a Printer may omit from an action response.  A default value is specified for each such attribute that the Printer must use when the client omits such an attribute from an action request.

# Action Summary

Table 1 summarizes the actions defined for the Job and Printer.  See section 4 for more details.

| Job Creation and Document submission | Job Control | Status and Information access | Printer Control |
| --- | --- | --- | --- |
| PrintJob | CancelJob | GetJobs | PausePrinter |
| PrintUri | HoldJob | GetPrinterAttributes | ResumePrinter |
| CreateJob | ReleaseJob | GetJobAttributes | PurgeJobs |
| SendDocument | RestartJob |  |  |
| SendURI |  |  |  |
| ValidateJob |  |  |  |

Table 1 - Summary of Actions

# Data Classes

This section describes the data classes in the PWG semantic model.  The classes are taken from the model and semantics of IPP [rfc2911].  See Appendix section 6.1 for some systematic changes from the IPP semantics to align with XML practice.

## Printer object class

The Printer data class is represented by a collection of attributes divided into two groups as shown in Figure 2:

Printer Description Attributes - shown in Figure 3 and described in section 3.1.1

“Job Template” Printer Attributes - described in section

Figure 2 Printer data class

### Printer Description Attributes

Figure 3 shows the Printer Description Attributes.  These attributes represent the processing state of the printer and information that describes the printer such as its make, where it’s located and its speed.  It also contains the “xxxSupported” Printer attributes that correspond to Operation Attribute that the client can supply in action requests.  The semantics of the attributes are summarized in Table 6 and Table 7 in Appendix section 6.2.

Figure 3 Printer Description Attributes

### The “PrinterState” attribute and the Printer Life Cycle

The “PrinterState” attribute is one of the most important Printer Description attributes.  Figure 4 shows the values of the “PrinterState” attribute and the Printer life cycle as affected by actions on the Printer and job processing.

Figure 4 - The “PrinterState” attribute and the Printer life cycle

### "Job Template" Printer Attributes

See section 3.2.1 below for the attributes that may comprise this group.  If a Job Template attribute (e.g. Media) is supported, the Printer must have an associated xxxSupported (e.g. MediaSupported) and xxxDefault (e.g. MediaDefault) “Job Template” Printer attribute.  There may be an associated xxxReady (e.g. MediaReady) “Job Template” Printer attribute.  By retrieving the “Job Template” Printer attributes, a Client can determine all the attributes and their values that may be used in constructing a Job Creation action.

#### xxxSupported Attributes

These attributes list all the currently configured valid values for the “xxx” Job Template Attributes.  Though the Printer is configured to support the feature, human intervention may be required to process the job (e.g. selected paper may have to be loaded into a tray).  The syntax for xxxSupported is multi-valued when an “xxx” attribute is a string.  When “xxx” is an integer, the syntax of the corresponding “xxxSupported” attribute is usually RangeOfInteger which indicates the minimum and maximum values supported by the Printer.  However, there are some exceptions as indicated in Table 2.

Table 2 - Integer syntaxes whose “xxxSupported” syntax isn’t RangeOfInteger

| “xxx” attribute name | “xxx” syntax | “xxxSupported” syntax |
| --- | --- | --- |
| JobPriority | Integer | Integer (Max value) |
| Copies | Integer | Integer (Max value) |
| PageRanges | RangeOfInteger (multivalued) | Boolean (are PageRanges supported) |

#### xxxDefault Attributes

These attributes give the default value for the associated production instruction if the Job Template Attribute of the job and the instructions embedded in the PDL are not supplied.  The syntax for the “xxxDefault” attribute is the same as the corresponding “xxx” Job Template Attribute.  The only exception is that the PageRanges attribute does not have a PageRangesDefault attribute.

#### xxxReady Attributes

These attributes give the features available without human intervention.  The syntax for a “xxxReady” attribute is the same as the corresponding “xxx” Job Template Attribute.

## Job object class

The Job data class is represented by a collection of attributes divided into two groups as shown in Figure 5:

“Job Template” Job Attributes - shown in Figure 6 and described in section 3.2.1

Job Description Attributes - shown in Figure 7 and described in section 3.2.2

Figure 5 Job data Class

### “Job Template” Job Attributes

Figure 6 shows the “Job Template” Job Attributes.  All Job Template Attributes are optional for a Printer to support.  However, a Printer should support each Job Template Attribute that represent a feature of the Printer.  The semantics of the attributes are summarized in Table 3 in Appendix section 6.2 for a brief description of each Printer Description attribute.

Figure 6 Job Template Attributes

### Job Description Attributes

Figure 7 shows the Job Description Attributes. The mandatory attributes are shown in a solid box.  The optional attributes are shown in a dotted box.  The semantics of the attributes are summarized in  and Table 5 in Appendix section 6.2.

Figure 7 Job Description Attributes

### The “JobState” attribute and the Job Life Cycle

The “JobState” attribute is one of the most important Job Description attributes.  Figure 8 shows the values of the “JobState” attribute and the Job life cycle as affected by actions on the Job, Printer, and job processing.

Figure 8 - The “JobState” Job Description attribute and the Job object life cycle

# Actions

Actions may contain a number of parameters.  The first parameter is always the Operation Attributes for the Action.  The Operation Attributes contains common information such as the target of the action (Job or Printer), a version number, or a sequence id to tie the request and response together.  Other information is Action-specific such as the name of the Job to be created or a filter that controls the information to be returned in a query.  The sections below describe the Operation Attributes and any other Action specific parameters.

The mandatory parameters and attributes for the Printer to support are in bold and the optional ones are in italics.  The optional ones for the Client to supply are indicated in square brackets ([]).

## Job Creation and document submission Actions

This section describes the Job Creation actions that create a Job and the ones that create Document Content.  The Job Creation actions are: PrintJob, PrintUri, and CreateJob.  The PrintJob action also submits the Document content.  The PrintUri action submits a URI reference to the Document content which the Printer then retrieves when needed at a later time.  The CreateJob action only creates the job and the Client must issue subsequent SendDocument and SendUri actions in order to submit document content or a URI reference, respectively, for a job.

Production instructions contained in the arguments of the Job Creation action (i.e., Operation and Job Template attributes) is combined with Printer supplied information to create a Job instance.

The last action in this section is ValidateJob.  This operation allows a Client to send a request with all the information to create a Job, except the document content.  The Printer does not create a Job but informs the client whether a CreateJob, PrintJob or PrintUri with the same information would have succeeded.  This is useful for allowing a Client to verify the production instructions before sending a large PrintJob request.

A concept that is important in the PWG model is a set of production instructions that can be applied to a print job.  Examples of these instructions include the number of Copies and the media to use.  These instructions are often referred to as a Job Ticket.  The Job Ticket is made up of the Operation Attributes (see section 3.2.2) and the “Job Template” Job attributes (see section 3.2.1) in a Job Creation operation.

Figure 9 Production instruction processing

In the real world, production instructions are also contained in the document content for a job.  Page Description Languages (PDL) such as PostScript® and PCL® often contain production instructions.   Some environments use a printer specific driver to generate the PDL stream based on feature selections made through a user interface.  Given that productions instructions can occur in both the PDL and in an associated Job, the PWG model allows a Printer to declare its capability to resolve this conflict.  The Printer’s attribute “PdlOverride” declares if an attempt will be made to override the instructions in the PDL with the instructions in the Job.

There is a wide variety of capabilities in Printers.  An instance of a Printer is to subject to changes in its configured capabilities.  An example would be an administrative change in the media the Printer supports or disabling two-sided printing.  Clients need not check the capabilities of a Printer before creating their Job Template Attributes and submitting a job.  Since this is a client/server paradigm, it is always possible that the capabilities could change after checking a Printer’s capabilities and before a Job is submitted.  On the other hand, a client may use the Printer’s configured capabilities to create their Job Template Attributes and submit a job.

The PWG model allows a client to control the Printer’s acceptance of a job submission based on the job request and the Printer’s current configured capabilities as follows.  When the client supplies a ‘true’ value for the “AttributeFidelity” Operation attribute, the Printer must reject the job unless the Printer supports all of the supplied Job Template attributes and values.  When the client supplies a ‘false’ value or omits the attribute, the Printer must accept the job submission and ignore or substitute attributes and values, respectively, that it does not support.  Note that the “AttributeFidelity” Operation attribute covers only the creation of the Job.  It is implementation specific how a Printer handles processing a job when the Printer encounters unsupported production instructions in the document content.

### PrintJob

([rfc2911] §3.2.1)  Submit a print job with only one document and supply the document content data.  If the Printer accepts the job, it creates the Job object and returns a unique “JobId” attribute for the Printer and a globally unique “JobUri” attribute.  The Printer also sets the corresponding Job Description attributes with these values.

PrintJobRequest(Operation Attributes, [Job Template Attributes], Document Data)

Operation Attributes:

PrinterUri(uri):  The target printer for the job

[AttributeFidelity(Boolean)]:  Allows a user to control the Printer’s acceptance of the job submission based on whether or not the Printer supports all the supplied Job Template attributes and values (see section 4.1).  Default = ‘false’.

[documentName(string)]:  Name for the document to be used in an implementation specific manner.

[Compression(string)]:  Compression algorithm used on the Document Data, if any.

[requestingUserName], [JobName], [DocumentFormat], [JobKOctets], [jobImpressions], [JobMediaSheets]:  see section 3.2.2.

[Job Template Attributes]:

Any Job Template Attribute (see section 3.2.1) or vendor/site specific extension.

Document data:  The document to print.

PrintJobResponse(Operation Attributes, [Unsupported Attributes], Job Attributes)

Operation Attributes :

statusCode: Results of the action (see Appendix section 6.3).

[statusMessage]: Localized text description of the status code.

[detailedStatusMessage]:  Text for detailed and technical information about the job.

[Unsupported Attributes]: any unsupported or conflicting attributes and or attribute values.  May be returned on success or failure.

Job Attributes:

JobUri, JobId, JobState, JobStateReasons, [JobStateMessage], [NumberOfInterveningJobs] See section 3.2.2.

### PrintUri

([rfc2911] §3.2.2)  Identical to the PrintJob operation (see section 4.1.1) except that a client supplies a URI reference to the document data.  The calling sequence is the same as PrintJob () except that the Operation Attributes in the request contains the “documentUri” attribute and the Document Data is omitted.

### CreateJob

([rfc2911] §3.2.4)  Similar to the PrintJob operation (see section 4.1.1), except that in the CreateJob request the Client does not supply Document Data.  In addition, the client does not supply any of the "documentName", "DocumentFormat", "Compression", or "documentUri" operation attributes.  The client follows this action with one or more SendDocument or SendUri actions to provide the Input Document content.  The client supplies a single set of Job Template attributes that the Printer applies to the Output Document(s) of the job.  The “MultipleDocumentHandling” Job Template attribute controls whether the Printer produces separate Output Documents or combines the Input Documents into a single Output Document (see section 4.1.3.1).

#### The “MultipleDocumentHandling” Job Template attribute

When a client submits a job with more than one Input Document, the “MultipleDocumentHandling” Job Template attribute allows the client to specify whether the Printer is to (1) produce corresponding separate Output Documents or (2) combine the Input Documents into a single Output Document.  For example, the ‘single-document’ and ‘single-document-new-sheet’ values allow the client to staple all of the Input Documents into a single Output Document, with the latter value forcing each Input Document to start on a new sheet (useful when doing two-sided printing).  When requesting multiple Copies, the ‘separate-document-uncollated-Copies’ value results in the Copies of each Input Document being together in an Output set, while the ‘separate-document-collated-Copies’ value keeps a copy of each Input Document together in an Output set.  For example, a job with Input Documents A, B, C and “Copies” = 2 will result in A, A, B, B, C, C or A, B, C, A, B, C, respectively.  If the Printer supports multiple documents per job, the Printer must support this Job Template attribute with at least one value.

### SendDocument

([rfc2911] §3.3.1)  Submits the entire Document Content for the next Input Document of a job created by a previous CreateJob action (see section 4.1.3).

SendDocumentRequest(Operation Attributes, Document Data)

Operation Attributes:

JobUri(uri) or (PrinterUri(uri) and jobId(integer)):  The target job.

[requestingUserName]:  see section 3.2.2.

[documentName(string)]:  Name for the document to be used in an implementation specific manner.

[Compression(string)]:  Compression algorithm used on the Document Data, if any.

[lastDocument(boolean)]:  'true' if this is the last Input Document for the Job.  Default = ‘false’.

Document data:  The document to print.

SendDocumentResponse(Operation Attributes, [Unsupported Attributes], Job Attributes)

Operation Attributes :

statusCode:  Results of the action (see Appendix section 6.3).

[statusMessage]:  Localized text description of the status code

[detailedStatusMessage]:  Text for detailed and technical information.

[Unsupported Attributes]:  any unsupported or conflicting attributes and or attribute values.  May be returned on success or failure.

Job Attributes:

JobUri, JobId, JobState, JobStateReasons, [JobStateMessage], [NumberOfInterveningJobs] See section .

### SendUri

([rfc2911] §3.3.2)  Identical to the SendDocument operation (see section 4.1.4) except that a client supplies a URI reference to the Document Content data, instead of supplying the document content.  The calling sequence is the same as SendDocument () except that the Operation Attributes in the request contains the “documentUri” attribute and the Document Data is omitted.

### ValidateJob

([rfc2911] §3.2.3)  This operation is used only to verify capabilities of a Printer object against whatever attributes are supplied by the client in the ValidateJob request. By using the ValidateJob action a client can validate that an identical PrintJob, PrintUri or CreateJob would be accepted.

The calling sequence is similar to PrintJob (see section 4.1.1) except the request does not contain the Document Data and the response does not contain the Job Attributes.

## Job Control Actions

This section describes the actions that allow a client to control a Job after it has been submitted:  CancelJob, HoldJob, ReleaseJob, and RestartJob.

### CancelJob

([rfc2911] §3.3.3)  Allows a client to cancel a Print Job from the time the Job is created up to the time it is completed, canceled, or aborted.

CancelJobRequest(Operation Attributes)

Operation Attributes:

JobUri(uri) or (PrinterUri(uri) and JobId(integer)):  The target job.

[requestingUserName]: see section 3.2.2.

[message(string)]:  Message from the Client to the Printer Operator.  Utilized in an implementation specific manner.

CancelJobResponse(Operation Attributes, [Unsupported Attributes])

Operation Attributes :

statusCode:  Results of the action (see Appendix section 6.3).

[statusMessage]: Localized text description of the status code.

[detailedstatusMessage]: Text for detailed and technical information about the job

[Unsupported Attributes]:  any unsupported or conflicting attributes and or attribute values.  May be returned on success or failure.

### HoldJob

([rfc2911] §3.3.5)  Allows a client to hold a pending Job in the Printer so that it is not eligible for scheduling.  The request calling sequence is similar to CancelJob (see section 4.2.1) except that the “jobHoldUntil” attribute may be in the “Operation Attributes”.  The response sequence is the same as CancelJob.

### ReleaseJob

([rfc2911] §3.3.6)  Release a previously held Job so that it is again eligible for scheduling.  The calling sequence is the same as CancelJob (see section 4.2.1).

### RestartJob

([rfc2911] §3.3.7)  Restart a job that is retained in the Printer after processing has completed.  The request calling sequence is similar to CancelJob except that the “jobHoldUntil” attribute may be in the “Operation Attributes”.  The response sequence is the same as CancelJob (see section 4.2.1).

## Status and information Actions

This section describes the actions that allow a client to obtain status and attributes of Jobs and PrinterS:  GetJobs, GetPrinterAttributes, and GetJobAttributes.

### GetJobs

([rfc2911] §3.3.4)  Retrieve the list of Jobs belonging to the Printer. The Client may supply a list of Job attribute and/or attribute group names. A group of Job attributes will be returned for each returned Job.

GetJobsRequest(Operation Attributes)

Operation Attributes:

PrinterUri(uri):  The target printer containing the jobs

[requestingUserName]: see section 3.2.2.

[requestedAttributes (string(multivalued))]:  set of Job  Attribute and/or Attribute Group names to be returned for each Job.  Default = ‘JobUri’ and ‘JobId’.

[whichJobs(string)]:Allows user to restrict jobs returned to completed or active/queued states. (keywords: completed, not-completed (Default)).

[myJobs(boolean)]:  Allows user to restrict jobs returned to just the user’s jobs or all jobs.  Default = ‘false’.

[limit(integer)]:  Sets maximum number of jobs to return.  Default = no limit.

GetJobsResponse(Operation Attributes, [Unsupported Attributes], Job Attributes)

Operation Attributes :

statusCode: Results of the action (see Appendix section 6.3).

[statusMessage]: Localized text description of the status code.

[detailedstatusMessage]: Text for detailed and technical information about the job.

Unsupported Attributes: any unsupported or conflicting attributes and or attribute values.  May be returned on success or failure.

Job Attributes(sequence of requested attributes/values (multivalued)): A list of jobs each containing the requested attributes

### GetPrinterAttributes

([rfc2911] §3.2.5)  Returns the values of the requested attributes and/or attribute groups of a Printer.

GetPrinterAttributesRequest(Operation Attributes)

Operation Attributes:

PrinterUri(uri):  The target printer

[requestingUserName]: see section 3.2.2.

[requestedAttributes (string(multivalued))]:  set of Printer Attribute and/or Attribute Group names to be returned.  Default = ‘all’.

[DocumentFormat(string)]:  Since some capabilities can be PDL specific, this optional attributes allows the Printer to return capabilities based on PDL if known to the Printer.

GetPrinterAttributesResponse(Operation Attributes, [Unsupported Attributes], Printer Attributes)

Operation Attributes :

statusCode: Results of the action (see Appendix section 6.3).

[statusMessage]: Localized text description of the status code.

[detailedstatusMessage]: Text for detailed and technical information about the Printer.

[Unsupported Attributes]: any unsupported or conflicting attributes and or attribute values.  May be returned on success or failure.

Printer Attributes(requested attributes/values (multivalued)): The requested attributes

### GetJobAttributes

([rfc2911] §3.3.4)  Returns the values of the requested attributes and/or attribute groups of a Job.

GetJobAttributesRequest(Operation Attributes)

Operation Attributes:

JobUri(uri) or (PrinterUri(uri) and JobId(integer)):  The target job

[requestingUserName]: see section 3.2.2.

[requested-attributes (string(multivalued))]:  set of Job Attribute and/or Attribute Group names to be returned for each Job.  Default = ‘all’.

GetJobAttributesResponse(Operation Attributes, [Unsupported Attributes], Job Attributes)

Operation Attributes :

statusCode: Results of the action (see Appendix section 6.3).

[statusMessage]: Localized text description of the status code.

[detailedstatusMessage]: Text for detailed and technical information about the job.

[Unsupported Attributes]: any unsupported or conflicting attributes and or attribute values.  May be returned on success or failure.

Job Attributes(requested attribute/values(multivalued)): The requested attributes and their values)

## Printer Control Actions

This section describes actions which allow a client to control a Printer and may require operator credentials:  PausePrinter, ResumePrinter, and PurgeJobs.

### PausePrinter

([rfc2911] §3.2.7)  Stops the Printer object from scheduling jobs.

PausePrinterRequest(Operation Attributes)

Operation Attributes:

PrinterUri(uri):  The target printer for the job

[requestingUserName]:  see section 3.2.2.

PausePrinterResponse(Operation Attributes, [Unsupported Attributes])

Operational Attributes :

statusCode:  Results of the action (see Appendix section 6.3).

[statusMessage]:  Localized text description of the status code.

[detailedStatusMessage]:  Text for detailed and technical information.

[Unsupported Attributes]: any unsupported or conflicting attributes and or attribute values.  May be returned on success or failure.

### ResumePrinter

([rfc2911] §3.2.8)  Resume the scheduling of Jobs in the Printer.  The calling sequence is the same as PausePrinter (see section 4.4.1).

### PurgeJobs

([rfc2911] §3.2.9)  Removes all jobs from the Printer, regardless of their state. .  The calling sequence is the same as PausePrinter (see section 4.4.1).

# References

[rfc2911] RFC 2566 “Internet Printing Protocol/1.0 Model and Semantics”, March 1999 and RFC 2911 “Internet Printing Protocol/1.1 Model and Semantics”, September 2000, T. Hastings, R. Herriot, R. Debry, S. Isaacson, P. Powell

[PWG5101.1] IEEE-ISTO 5101.1-2001 Media Standardized Names <work in progress>, ftp://ftp.pwg.org/pub/pwg/standards/pwg5101.1.pdf, .doc, .rtf for standardized names

[rfc2046] RFC 2046 "Multipurpose Internet Mail Extensions (MIME) Part Two: Media Types", November 1996, Freed, N. and N. Borenstein

[rfc2048] RFC 2048 "Multipurpose Internet Mail Extension (MIME) Part Four: Registration Procedures", November 1996, Freed, N., Klensin, J. and J. Postel

[rfc1123] RFC 1123 " Requirements for Internet Hosts -- Application and Support ", October 1989, Branden, R.

# Appendices

## Changes to remove some IPP specific aspects

This section lists some changes to remove some IPP specific aspects from the PWG Semantic Model.  Any attribute name containing “ipp” has had the “ipp” removed.  The IPP operation names have the hyphens removed to be the PWG action names and the operations supported are mixed keywords, not integer enum values.  All attributes names have had the first letter capitalized and the ‘-‘ character removed and the character following the ‘-‘ has been capitalized.  The keyword attribute values defined remain unchanged and are all lower case, except for the ones that specify other attributes names (which are changed to be the mixed case without hyphens).  ISSUE 03:  What about the case and hyphens in status code names (and removing the integer values)?  The term “object” is sometimes changed to “data class”.  ISSUE 04:  Why? and Why not done consistently?  The term “operation” has been changed to “action” to use the term more frequently used with XML.

The aspects of the model that deal with globalization (i.e. character set & language) have been removed.  Globalization will be addressed in a protocol specific binding of this semantic model. The Printer globalization attributes are charsetConfigured, charsetSupported, naturalLanguageConfigured, naturalLanguageSupported and generatedNaturalLanguageSupported.

The types of the attributes have been simplified.  All keyword, text, name, DateTime, uri, UriScheme, enum and mimeMediaType are represented by the simple string type.  The term “keyword” continues to be used for string values enumerated as part of the PWG Model.  The integer enums values are replaced by their associated keyword.  The “1setOf X” types are represented as the base type and the “Multivalued” field in the tables below set to “Yes”.  Integers and Boolean types remain the same.  Any applicable constraints placed on the attribute values has been noted in the tables below.

## Summary of Job and Printer object attributes

This appendix summarizes the attributes for the Job and Printer objects.  For each attribute, the tables contain the attribute name, whether the attribute is multi-valued, its syntax, constraints (MAX = 2**31-1, MIN = -2**31, and Maxlength = number of octets for strings), and a reference to the document whether the semantics of the attribute is completely specified:

- Table 3 - Optional Job Template Attributes (see section 3.2.1)

(see section 3.2.2)

- Table 5 - Optional Job Description Attributes (see section 3.2.2)

- Table 6 - Mandatory Printer Description Attributes (see section 3.1.1)

- Table 7 - Optional Printer Description Attributes (see section 3.1.1)

Currently, this Appendix only lists attributes that are defined in [rfc2911], not any of the twelve additional IPP extension documents (see http://www.pwg.org/ipp/index.html).

Table 3 - Optional Job Template Attributes

| Attribute Name |  |  | Multivalued |  |  |  |  |  | Syntax |  | constraint | reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Description (values) |  |  |  |  |  |  |  |  |  |  |  |
| JobPriority |  |  |  |  |  |  |  |  | Integer |  | 1:100 | [rfc2911] §4.2.1 |
|  | Priority for scheduling the Job. A higher value specifies a higher priority. |  |  |  |  |  |  |  |  |  |  |  |
| JobHoldUntil |  |  |  |  |  |  |  |  | String |  | Type3 keyword | [rfc2911] §4.2.2 |
|  | Specifies the named time period during which the Job must become a candidate for printing. (keywords: no-hold, indefinite, day-time, evening, night, weekend, second-shift, third-shift) |  |  |  |  |  |  |  |  |  |  |  |
| JobSheets |  |  |  |  |  |  |  |  | String |  | type3 keyword | [rfc2911] §4.2.3 |
|  | Specifies which job start/end sheet(s), will be printed with a job.. (example: none, standard) |  |  |  |  |  |  |  |  |  |  |  |
| MultipleDocumentHandling |  |  |  |  |  |  |  |  | String |  | type2 keyword | [rfc2911] §4.2.4 |
|  | Controls whether Input Documents in multi-document jobs are combined into a single Output Document or are kept as separate Output Documents (see section 4.1.3.1). Useful for application of Finishings and the placement of one or more print-stream pages into impressions and onto media sheets for multi-document Jobs. (keywords: single-document, separate-documents-uncollated-Copies, separate-documents-collated-Copies, single-document-new-sheet) |  |  |  |  |  |  |  |  |  |  |  |
| Copies |  |  |  |  |  |  |  |  | Integer |  | 1:MAX | [rfc2911] §4.2.5 |
|  | The number of copies of the Output Document(s) to be printed. |  |  |  |  |  |  |  |  |  |  |  |
| Finishings |  | Yes |  |  |  |  |  |  | String |  | type2 keyword | [rfc2911] §4.2.6 |
|  | Identifies the finishing that the Printer uses for each copy of each printed Output Document in the Job (example: none, staple, punch, cover, bind, saddle-stitch, edge-stitch, staple-top-left, staple-bottom-left, staple-top-right, staple-bottom-right, edge-stitch-left, edge-stitch-top, edge-stitch-right, edge-stitch-bottom, staple-dual-left, staple-dual-top, staple-dual-right, staple-dual-bottom) |  |  |  |  |  |  |  |  |  |  |  |
| PageRanges |  |  | Yes |  |  |  |  | rangeOfInteger |  |  | Ascending non-overlapping values | [rfc2911] §4.2.7 |
|  | Identifies the range(s) of pages that are to be printed by the Printer for each copy of each Output Document. |  |  |  |  |  |  |  |  |  |  |  |
| Sides |  |  |  |  |  |  |  |  |  | String | type2 keyword | [rfc2911] §4.2.8 |
|  | Indicates how an impression is to be placed upon the side(s) of the media (keyword: one-sided, two-sided-long-edge, two-sided-short-edge, two-sided-long-edge, tumble) |  |  |  |  |  |  |  |  |  |  |  |
| NumberUp |  |  |  |  |  |  |  |  | Integer |  | 1:MAX | [rfc2911] §4.2.9 |
|  | Indicates the number of pages in an impression. |  |  |  |  |  |  |  |  |  |  |  |
| OrientationRequested |  |  |  |  |  |  |  |  | String |  | type2 keyword | [rfc2911] §4.2.10 |
|  | The desired orientation for printed pages. (keywords: portrait, landscape, reverse-landscape, reverse-portrait) |  |  |  |  |  |  |  |  |  |  |  |
| Media |  |  |  |  |  |  |  |  | String |  | type3 keyword | [rfc2911] §4.2.11 |
|  | The medium that the Printer uses for all impressions of the Job. (example: “na-letter”. See [pwg5101.1]) |  |  |  |  |  |  |  |  |  |  |  |
| PrinterResolution |  |  |  |  |  |  |  |  | resolution |  |  | [rfc2911] §4.2.12 |
|  | The resolution that Printer uses for the Job in cross-feed and feed direction in units of dpi or dpc. |  |  |  |  |  |  |  |  |  |  |  |
| PrintQuality |  |  |  |  |  |  |  |  | String |  | type2 keyword | [rfc2911] §4.2.13 |
|  | The print quality that the Printer uses for the Job. (keyword: draft, normal, high) |  |  |  |  |  |  |  |  |  |  |  |

Table 4 - Mandatory Job Description Attributes

| Attribute Name |  | Multivalued |  | Syntax | constraint | reference |
| --- | --- | --- | --- | --- | --- | --- |
|  | Description (values) |  |  |  |  |  |
| JobUri |  |  |  | String | uri | [rfc2911] §4.3.1 |
|  | The Printer sets this to the URI for the job. (example: ipp://www.company.com/printer/jobs/22) |  |  |  |  |  |
| JobId |  |  |  | Integer | 1:MAX | [rfc2911] §4.3.2 |
|  | The Printer sets this to the ID of the job that is unique for the Printer. |  |  |  |  |  |
| JobPrinterUri |  |  |  | String | uri | [rfc2911] §4.3.3 |
|  | The Printer set this to the URI of Printer that created this Job. (example: ipp://www.company.com/printer) |  |  |  |  |  |
| JobName |  |  |  | String | Maxlength=127 | [rfc2911] §4.3.5 |
|  | The Printer sets this to the client-supplied end-user friendly name for the Job, else the Printer must generate a name from other information. (example: “license agreement memo”) |  |  |  |  |  |
| JobOriginatingUserName |  |  |  | String | Maxlength=1023 | [rfc2911] §4.3.6 |
|  | The Printer sets this attribute to the most authenticated printable name that it can obtain (example: “John Doe”, \authDomain\John Doe”) |  |  |  |  |  |
| JobState |  |  |  | String | Type1 keyword | [rfc2911] §4.3.7 |
|  | The current state of the job (see section 3.2.3). See also JobStateReasons attribute below. (keywords: pending, pending-held, processing, processing-stopped, canceled, aborted, completed) |  |  |  |  |  |
| JobStateReasons |  | Yes |  | String | type2 keyword | [rfc2911] §4.3.8 |
|  | Provides additional information about the job's current state. (keywords: none, job-incoming, job-data-insufficient, document-access-error, submission-interrupted, job-outgoing, job-hold-until-specified, resources-are-not-ready, printer-stopped-partly, printer-stopped, job-interpreting, job-queued, job-transforming, job-queued-for-marker, job-printing, job-canceled-by-user, job-canceled-by-operator, job-canceled-at-device, aborted-by-system, unsupported-compression, compression-error, unsupported-document-format, document-format-error, processing-to-stop-point, service-off-line, job-completed-successfully, job-completed-with-warnings, job-completed-with-errors, job-restartable, queued-in-device)) |  |  |  |  |  |
| TimeAtCreation |  |  |  | Integer | MIN:MAX | [rfc2911] §4.3.14.1 |
|  | The time at which the Job was created in “PrinterUpTime” seconds. |  |  |  |  |  |
| TimeAtProcessing |  |  |  | Integer | MIN:MAX | [rfc2911] §4.3.14.2 |
|  | The time at which the Job first began processing. |  |  |  |  |  |
| TimeAtCompleted |  |  |  | Integer | MIN:MAX | [rfc2911] §4.3.14.3 |
|  | The time at which the Job completed. |  |  |  |  |  |
| JobPrinterUpTime |  |  |  | Integer | 1:MAX | [rfc2911] §4.3.14.4 |
|  | The amount of time (in seconds) that the Printer has been up and running. See Printer attribute “PrinterUpTime” |  |  |  |  |  |

Table 5 - Optional Job Description Attributes

| Attribute Name |  | Multivalued |  |  |  |  |  | Syntax |  | constraint | reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Description (values) |  |  |  |  |  |  |  |  |  |  |
| JobMoreInfo |  |  |  |  |  |  |  | String |  | uri | [rfc2911] §4.3.4 |
|  | URI used to obtain information intended for end user consumption about this specific Job. (example: “http://www.company.com/printer/embededjobpage”) |  |  |  |  |  |  |  |  |  |  |
| JobStateMessage |  |  |  |  |  |  |  | String |  | Maxlength=127 | [rfc2911] §4.3.6 |
|  | Specifies information about the "JobState" and "jobStateReasons" attributes in human readable text. (example: “Job completed successfully with warnings") |  |  |  |  |  |  |  |  |  |  |
| JobDetailedStatusMessage |  |  | Yes |  |  |  |  | String |  | Maxlength=1023 | [rfc2911] §4.3.10 |
|  | Specifies additional detailed and technical information about the job. Intended for use by the system administrator or other experienced technical persons. (example: “PostScript error: stack overflow”) |  |  |  |  |  |  |  |  |  |  |
| JobDocumentAccessErrors |  |  | Yes |  |  |  |  | String |  | Maxlength=1023 | [rfc2911] §4.3.11 |
|  | Information about each document access error for this job encountered by the Printer. (example: “(404) http://www.company.com/pub/fileToPrint.pdf ”) |  |  |  |  |  |  |  |  |  |  |
| NumberOfDocuments |  |  |  |  |  |  |  | Integer |  | 0:MAX | [rfc2911] §4.3.12 |
|  | The number of documents in the job. |  |  |  |  |  |  |  |  |  |  |
| OutputDeviceAssigned |  |  |  |  |  |  |  | String |  | Maxlength=127 | [rfc2911] §4.3.13 |
|  | Identifies the output device to which the Printer has assigned this job (example: “Pete’s Printer”) |  |  |  |  |  |  |  |  |  |  |
| DateTimeAtCreation |  |  |  |  |  |  |  | String |  | DateTime [rfc1123] | [rfc2911] §4.3.14.5 |
|  | Indicates the date and time at which the Job was created . (example: Fri, 03 May 2002 08:49:37 GMT) |  |  |  |  |  |  |  |  |  |  |
| DateTimeAtProcessing |  |  |  |  |  |  |  |  | String | DateTime [rfc1123] | [rfc2911] §4.3.14.6 |
|  | Indicates the date and time at which the Job first began processing. (example: Fri, 03 May 2002 08:49:37 GMT) |  |  |  |  |  |  |  |  |  |  |
| DateTimeAtCompleted |  |  |  |  |  |  |  | String |  | DateTime [rfc1123] | [rfc2911] §4.3.14.7 |
|  | Indicates the date and time at which the Job completed. (example: Fri, 03 May 2002 08:49:37 GMT) |  |  |  |  |  |  |  |  |  |  |
| NumberOfInterveningJobs |  |  |  |  |  |  |  | Integer |  | 0:MAX | [rfc2911] §4.3.15 |
|  | The number of jobs that are "ahead" of this job assuming the current scheduled order. |  |  |  |  |  |  |  |  |  |  |
| JobMessageFromOperator |  |  |  |  |  |  |  | String |  | Maxlength=127 | [rfc2911] §4.3.16 |
|  | Message to the end user indicating the reasons for any management action taken on a job. (example: “Job canceled due to length”, “Pick job up in mailbox”) |  |  |  |  |  |  |  |  |  |  |
| JobKOctets |  |  |  |  |  |  |  | Integer |  | 0:MAX | [rfc2911] §4.3.17.1 |
|  | The total size of the Job’s document(s) in integral units of 1024 octets. |  |  |  |  |  |  |  |  |  |  |
| jobImpressions |  |  |  |  |  |  |  | Integer |  | 0:MAX | [rfc2911] §4.3.17.2 |
|  | The total size in number of impressions in all the Job’s document(s). |  |  |  |  |  |  |  |  |  |  |
| JobMediaSheets |  |  |  |  |  |  | Integer |  |  | 0:MAX | [rfc2911] §4.3.17.3 |
|  | The total number of media sheets to be produced for this job. |  |  |  |  |  |  |  |  |  |  |
| JobKOctetsProcessed |  |  |  |  |  |  | Integer |  |  | 0:MAX | [rfc2911] §4.3.18.1 |
|  | the total number of octets processed in integral units of 1024 octets so far. |  |  |  |  |  |  |  |  |  |  |
| jobImpressionsCompleted |  |  |  |  |  |  | Integer |  |  | 0:MAX | [rfc2911] §4.3.18.2 |
|  | The number of impressions completed for the job so far. |  |  |  |  |  |  |  |  |  |  |
| JobMediaSheetsCompleted |  |  |  |  |  |  |  | Integer |  | 0:MAX | [rfc2911] §4.3.18.3 |
|  | The media-sheets completed marking and stacking for the entire job so far. |  |  |  |  |  |  |  |  |  |  |

Table 6 - Mandatory Printer Description Attributes

| Attribute Name |  | Multivalued |  |  | Syntax | constraint | reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Description (values) |  |  |  |  |  |  |
| PrinterUriSupported |  | Yes |  |  | String | uri | [rfc2911] §4.4.1 |
|  | Contains at least one URI for the Printer object. The PrinterUriSupported, UriAuthenticationSupported and the UriSecuritySupported are parallel attributes. Each of these attributes must have the same cardinality. The “i”th value of each of these attributes describes the URI for the printer, the authentication mechanism used and the security method used. (example: ipp://www.company.com/printer) |  |  |  |  |  |  |
| UriAuthenticationSupported |  |  |  | Yes | String | type2 keyword | [rfc2911] §4.4.2 |
|  | The Client authentication mechanism that the Printer object uses to identify the user. See PrinterUriSupported for additiona information. (keywords: none, requesting-user-name, basic, digest and certificate) |  |  |  |  |  |  |
| UriSecuritySupported |  | Yes |  |  | String | type2 keyword | [rfc2911] §4.4.3 |
|  | Identifies the security mechanisms used for accessing the Printer object. See PrinterUriSupported for additiona information. (keywords: none, ssl3, tls) |  |  |  |  |  |  |
| PrinterName |  |  |  |  | String | Maxlength=127 | [rfc2911] §4.4.4 |
|  | The end-user friendly name of the Printer object. (example: “Pete’s Printer”) |  |  |  |  |  |  |
| PrinterState |  |  |  |  | String | type1 keyword | [rfc2911] §4.4.11 |
|  | Identifies the current state of the device (see section 3.1.2). See “PrinterStateReasons” below. (keywords: idle, processing, stopped) |  |  |  |  |  |  |
| PrinterStateReasons |  | Yes |  |  | String | type2 keyword | [rfc2911] §4.4.12 |
|  | Augments the "printer-state" attribute to give more detailed information about the Printer state. Each keyword value may have a suffix to indicate its level of severity. The three suffixes (levels) are: “Report” (least severe), “Warning”, and “Error” (most severe). Keywords without suffixes are assumed to be “Error” (most severe). See reference sor semantics of defined keywords. (keywords: other, none, media-needed, media-jam, moving-to-paused, paused, shutdown, connecting-to-device, timed-out, stopping, stopped-partly, toner-low, toner-empty, spool-area-full, cover-open, interlock-open, door-open, input-tray-missing, media-low, media-empty, output-tray-missing, output-area-almost-full, output-area-full, marker-supply-low, marker-supply-empty, marker-waste-almost-full, marker-waste-full, fuser-over-temp, fuser-under-temp, opc-near-eol, opc-life-over, developer-low, developer-empty, interpreter-resource-unavailabl) |  |  |  |  |  |  |
| VersionsSupported |  | Yes |  |  | String | type2 keyword | [rfc2911] §4.4.14 |
|  | Identifies the version(s) that this Printer supports, including major and minor versions. This attribute is Protocol specific in an implementation and the current keywords refer to the IPP Protocol and Model. (keywords: 1.0, 1.1) |  |  |  |  |  |  |
| OperationsSupported |  | Yes |  |  | String | type2 keyword | [rfc2911] §4.4.15 |
|  | The set of supported actions for the Printer and Job. (keywords: PrintJob, PrintUri, ValidateJob, CreateJob, SendDocument, SendUri, CancelJob, GetJobAttributes, GetJobs, GetPrinterAttributes, HoldJob, ReleaseJob, RestartJob, PausePrinter, ResumePrinter, PurgeJobs). |  |  |  |  |  |  |
| DocumentFormatDefault |  |  |  |  | String | MimeMediaType [rfc2046], [rfc2048] | [rfc2911] §4.4.21 |
|  | The document format (i.e. PDL) that the Printer has been configured to assume if the client does not specify a document format in any of the actions that supply document content for a Job. The value “application/octet-stream” has a special meaning. This value is used to indicate that a Printer is capable of auto-sensing the format of the document. (examples: “application/octet-stream”, “application/postscript”, “application/vnd.hp-PCL”, “text/plain; charset=utf-8”) |  |  |  |  |  |  |
| DocumentFormatSupported |  |  | Yes |  | String | mimeMediaTyp | [rfc2911] §4.4.22 |
|  | The set of document formats that the Printer. (examples: “application/octet-stream”, “application/postscript”, “application/vnd.hp-PCL”, “text/plain; charset=utf-8”) |  |  |  |  |  |  |
| PrinterIsAcceptingJobs |  |  |  |  | Boolean |  | [rfc2911] §4.4.23 |
|  | Indicates whether the printer is currently able to accept jobs. |  |  |  |  |  |  |
| QueuedJobCount |  |  |  |  | integer | 0:MAX | [rfc2911] §4.4.24 |
|  | The number of jobs that the Printer has accepted but has not yet completed. |  |  |  |  |  |  |
| PdlOverrideSupported |  |  |  |  | String | type2 keyword | [rfc2911] §4.4.28 |
|  | Expresses the ability of a Printer to either attempt to override a document’s production instructions with Job Template Attributes or not. (keywords: attempted, not-attempted) |  |  |  |  |  |  |
| PrinterUpTime |  |  |  |  | integer | 1:MAX | [rfc2911] §4.4.29 |
|  | The amount of time (in seconds) that a Printer has been up and running |  |  |  |  |  |  |
| CompressionSupported |  | Yes |  |  | String | Type3 keyword | [rfc2911] §4.4.32 |
|  | Identifies the set of supported Compression algorithms for document content. (keywords: none, deflate, gzip, compress) |  |  |  |  |  |  |

Table 7 - Optional Printer Description Attributes

| Attribute Name |  | Multivalued |  |  |  |  |  | Syntax |  | constraint | reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Description (values) |  |  |  |  |  |  |  |  |  |  |
| PrinterLocation |  |  |  |  |  |  |  | String |  | Maxlength=127 | [rfc2911] §4.4.5 |
|  | Identifies the location of the device. (example: Pete’s Office) |  |  |  |  |  |  |  |  |  |  |
| PrinterInfo |  |  |  |  |  |  |  | String |  | Maxlength=127 | [rfc2911] §4.4.6 |
|  | Descriptive information about this Printer object.(example: “Out of courtesy for others, please print only small (1-5 page) jobs at this printer") |  |  |  |  |  |  |  |  |  |  |
| PrinterMoreInfo |  |  |  |  |  |  |  | String |  | uri | [rfc2911] §4.4.7 |
|  | URI used to obtain information intended for end user consumption about this specific Printer. (example: “http://www.company.com/printer/embeddedwebpage”) |  |  |  |  |  |  |  |  |  |  |
| PrinterDriverInstaller |  |  |  |  |  |  |  | String |  | Uri | [rfc2911] §4.4.8 |
|  | Intended for consumption by automata to locate the driver installer for this Printer object. Note: This attribute has not been used by any known implementation. (example: “http://www.company.com/printer/installerProgram” |  |  |  |  |  |  |  |  |  |  |
| PrinterMakeAndModel |  |  |  |  |  |  |  | String |  | Maxlength=127 | [rfc2911] §4.4.9 |
|  | Identifies the make and model of the device. (example: “Xerox Phaser 7700”, “HP LaserJet 1000”, “Lexmark Optra Color 45”) |  |  |  |  |  |  |  |  |  |  |
| PrinterMoreInfoManufacturer |  |  |  |  |  |  |  | String |  | uri | [rfc2911] §4.4.10 |
|  | URI used to obtain more information for end user consumption about this type of device. (example: “http://www.xerox.com/go/xrx/template/012.jsp?Xcntry=USA&Xlang=en_US&prodID=7700”, “http://www.lexmark.com/US/products/overview/0,1224,MjQ5fDE=,00.html”) |  |  |  |  |  |  |  |  |  |  |
| PrinterStateMessage |  |  |  |  |  |  |  | String |  | Maxlength=1023 | [rfc2911] §4.4.13 |
|  | Information about the "printer- state" and "printer-state-reasons" attributes in human readable text. (example: “Printer stopped due to paper jam”) |  |  |  |  |  |  |  |  |  |  |
| MultipleDocumentJobsSupported |  |  |  |  |  |  |  |  | boolean |  | [rfc2911] §4.4.16 |
|  | Indicates whether the Printer supports more than one document per job, i.e., more than one SendDocument and/or SendUri request per job. A multi-document per job Printer must implement this attribute and have a value of ‘true’. A single document per job Printer may either not support this attribute or support it with a value of ‘false’. |  |  |  |  |  |  |  |  |  |  |
| PrinterMessageFromOperator |  |  |  |  |  |  |  | String |  | Maxlength=127 | [rfc2911] §4.4.25 |
|  | End user information for the printer. (example: “printer unavailable until 1pm due to preventive mainanance”) |  |  |  |  |  |  |  |  |  |  |
| ColorSupported |  |  |  |  |  |  |  | boolean |  |  | [rfc2911] §4.4.26 |
|  | Indicates if the device is capable of any type of color printing at all, including highlight color. |  |  |  |  |  |  |  |  |  |  |
| ReferenceUriSchemesSupported |  |  |  |  | Yes |  |  | String |  | UriScheme | [rfc2911] §4.4.27 |
|  | Which URI schemes are supported by the printer to retrieve documents This attribute must be supported if the Printer is capable of print by reference. (example: ftp, http) |  |  |  |  |  |  |  |  |  |  |
| printerCurrentTime |  |  |  |  |  |  |  | String |  | DateTime [rfc1123] | [rfc2911] §4.4.30 |
|  | Indicates the current date and time. (example: Fri, 03 May 2002 08:49:37 GMT) |  |  |  |  |  |  |  |  |  |  |
| MultipleOperationTimeOut |  |  |  |  |  |  |  | Integer |  | 1:MAX | [rfc2911] §4.4.31 |
|  | Identifies the minimum time (in seconds) that a multi-document per job Printer must wait between actions on an open job. The actions can add documents to the open Job or close the Job. Timeouts are handled in an implementation specific manner. Multi-document per job PrinterS must implement this attribute. The recommended value is greater than 60 and less than 240. |  |  |  |  |  |  |  |  |  |  |
| JobKOctetsSupported |  |  |  |  |  |  | RangOfInteger |  |  | 0:MAX | [rfc2911] §4.4.33 |
|  | Specifies the allowable upper and lower bounds of the total size per job in integral units of 1024 octets. |  |  |  |  |  |  |  |  |  |  |
| JobImpressionsSupported |  |  |  |  |  |  | RangOfInteger |  |  | 0:MAX | [rfc2911] §4.4.34 |
|  | Specifies the upper and lower bounds for the number of impressions allowed per job. |  |  |  |  |  |  |  |  |  |  |
| JobMediaSheetsSupported |  |  |  |  |  |  | RangOfInteger |  |  | 0:MAX | [rfc2911] §4.4.35 |
|  | Specifies the upper and lower bounds for the number of media sheets allowed per job. |  |  |  |  |  |  |  |  |  |  |
| PagesPerMinute |  |  |  |  |  |  |  | Integer |  | 0:MAX | [rfc2911] §4.4.36 |
|  | Specifies the nominal number of pages per minute which may be generated by this printer. |  |  |  |  |  |  |  |  |  |  |
| PagesPerMinuteColor |  |  |  |  |  |  |  | Integer |  | 0:MAX | [rfc2911] §4.4.37 |
|  | Specifies the nominal number of pages per minute which may be generated by this printer when printing color. |  |  |  |  |  |  |  |  |  |  |

## Action Status Codes

This Appendix lists the status codes that the Printer returns in each action response.

The following status codes are returned when the Printer accepts the action request and indicates some degree of success:

successful-ok - Action succeeded and no requested attribute were substituted or ignored.

successful-ok-ignored-or-substituted-attributes - Action succeeded but some unsupported attributes were ignored or substituted.

successful-ok-conflicting-attributes - Action succeeded but some attributes were conflicting and have been substituted or ignored.

The following status codes are returned when the Printer rejects the action indicating some error on the part of the Client:

client-error-bad-request - malformed syntax or constraint exceeded.

client-error-forbidden - The Printer understood the request, but is refusing to fulfill it for authentication and/or authorization reasons.  The client should not try again even with credentials.

client-error-not-authenticated - The request requires user authentication.  The client may try again with suitable authentication.

client-error-not-authorized - The requester is not authorized to perform the request.  The Client should not try again.

client-error-not-possible - The action cannot be performed, because of the state of the target object.

client-error-timeout - The client did not produce a subsequent request within the time that the Printer was prepared to wait.

client-error-not-found - The target object was not found.

client-error-gone - The target object is no longer available.

client-error-request-entity-too-large - The request and/or the Document Content is too large.

client-error-request-value-too-long - A attribute value in the request is longer than the Printer supports.

client-error-document-format-not-supported - The document format is not supported.

client-error-attributes-or-values-not-supported - An attribute and/or value is not supported and must be in order to carry out the request. The Printer must return the unsupported attributes or values in the Unsupported Attributes group.

client-error-uri-scheme-not-supported - The URI scheme is not supported.

client-error-charset-not-supported - The charset is not supported.

client-error-conflicting-attributes - Some supplied attributes are conflicting.  The Printer must return them in the Unsupported Attributes group.

client-error-compression-not-supported - The compression of the Document Content is not supported.

client-error-compression-error - An error occurred when uncompressing the Document Content.

client-error-document-format-error - An error occurred when interpreting the Document Content.

client-error-document-access-error - An error occurred when the Printer attempted to access the Document Content through the URI supplied.

The following status codes are returned when the Printer rejects the action indicating some error on the part of the Printer:

server-error-internal-error - An unexpected internal error occurred.

server-error-operation-not-supported - The Printer does not support the requested action.

server-error-service-unavailable - The Printer is unable to service the request at this time due to overloading or maintenance.  The client should try again later as per the “message” Operation attribute.

server-error-version-not-supported - The Printer doesn’t support the requested major version of the protocol and returns the closest version that it does support.

server-error-device-error - The Printer encountered a device error that causes it to be unable to accept a new request.  For example, a paper jam for a Printer that doesn’t spool and so cannot accept a new job submission until the jam is fixed.

server-error-temporary-error - A temporary error such as a buffer full write error, a memory overflow, or a disk full condition.

server-error-not-accepting-jobs - The Printer is not currently accepting jobs.  Its “PrinterIsAcceptingJobs” Printer Description attribute is ‘false’.

server-error-busy - A temporary error indicating that the Printer is too busy processing jobs and/or other requests.  A Client should try again later.

server-error-job-canceled - The job has been canceled by an operator or aborted by the system.  For example, while the Client is transmitting the Document Content to the Printer.

server-error-multiple-document-jobs-not-supported - The Printer doesn’t support multiple document jobs and the client attempted to supply a second SendDocument or SendUri request.  The Printer’s “MultipleDocumentJobsSupported” Printer Description attribute is ‘false’.

# Change Log

5/16/02	PJZ	original draft

5/23/02	TH	re-organize draft with comments from Melinda Grant

5/26/02	TH	detailed review of the draft

5/29/02	PJZ	Incorporated comments prior to initial release