Internet Printing Protocol Working Group                    Tom Hastings

INTERNET DRAFT                                                     Xerox

Expires 19 March 2000                                       Ira McDonald

High North

19 September 1999

Definition of Managed Objects for IPP Servers using SMIv2

<draft-ietf-ipp-server-mib-00.txt>

Status of this Memo

This document is an Internet-Draft and is in full conformance with

all provisions of Section 10 of RFC2026.  Internet-Drafts are working

documents of the Internet Engineering Task Force (IETF), its areas,

and its working groups.  Note that other groups may also distribute

working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months

and may be updated, replaced, or obsoleted by other documents at any

time.  It is inappropriate to use Internet-Drafts as reference

material or to cite them other than as "work in progress."

To view the list of Internet-Draft Shadow Directories, see

http://www.ietf.org/shadow.html.

Abstract

This document is a submission to the Internet Printing Protocol

Working Group of the Internet Engineering Task Force (IETF).

Comments should be submitted to the ipp@pwg.org mailing list.

This document defines the IPP Server MIB for monitoring IPP Printer

entities in printers, spoolers, gateways, etc.  The IPP Server MIB is

designed to be used in conjunction with the Internet Printing

Protocol [IPP-PRO].  The IPP Server MIB may also be used to send

job-level events via SNMP traps for jobs submitted via any print

protocol (not just IPP).

Hastings, McDonald            Expires 19 March 2000             [Page 1]

Internet Draft             IPP Server MIB              19 September 1999

Table of Contents

1.  Introduction ...............................................       3

1.1.  Terminology for Conformance ............................       3

2.  SNMP Network Management Framework ..........................       4

2.1.  SNMPv1 Network Management Framework ....................       4

2.1.1.  Object Definitions .................................       4

2.2.  SNMPv2 Network Management Framework ....................       4

2.3.  SNMPv3 Network Management Framework ....................       5

3.  Overview of IPP Server MIB .................................       7

3.1.  Structure of IPP Server MIB ............................       7

3.2.  Relationship to other MIBs .............................       8

3.3.  Registration for IPP Notifications .....................       8

3.3.1.  Registration via IPP ...............................       8

3.3.2.  Registration via SNMP ..............................       8

3.4.  Encoding Rules for IPP Notifications ...................       9

4.  Definition of IPP Server MIB ...............................      10

4.1.  IppPrinterState (textual convention) ...................      11

4.2.  IppJobState (textual convention) .......................      12

4.3.  IppJobCollationType (textual convention) ...............      12

4.4.  IppTriggerEvent (textual convention) ...................      13

4.5.  Printer Group (Mandatory) ..............................      13

4.6.  Printer URI Group (Mandatory) ..........................      17

4.7.  Event Group (Conditionally Mandatory) ..................      19

4.8.  Printer Basic Event Group (Conditionally Mandatory) ....      25

4.9.  Job Basic Event Group (Conditionally Mandatory) ........      28

4.10.  Job Status Event Group (Conditionally Mandatory) ......      29

4.11.  Job Progress Event Group (Conditionally Mandatory) ....      30

4.12.  Conformance Information ...............................      32

5.  IANA Considerations ........................................      37

6.  Internationalization Considerations ........................      37

7.  Security Considerations ....................................      37

8.  References .................................................      38

9.  Intellectual Property Notice ...............................      38

10.  Authors' Addresses ........................................      38

11.  Full Copyright Statement ..................................      39

Hastings, McDonald            Expires 19 March 2000             [Page 2]

Internet Draft             IPP Server MIB              19 September 1999

1.  Introduction

The IPP Server MIB supports passive monitoring (but not active

configuration) of IPP Printer entities, via client polling of SNMP

objects in the 'ippPrinterTable' (one row for each configured IPP

Printer).

The IPP Server MIB supports (optional) dynamic monitoring of events

from IPP Printer entities, via server generation of SNMP traps

specifying SNMP objects in the IPP Event group.  Persistent events

are NOT supported in this IPP Server MIB - unlike the the IETF

Printer MIB [RFC-1759] that defines an Alert table.

1.1.  Terminology for Conformance

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT",

"SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" in this

document are to be interpreted per [RFC-2119].

Hastings, McDonald            Expires 19 March 2000             [Page 3]

Internet Draft             IPP Server MIB              19 September 1999

2.  SNMP Network Management Framework

2.1.  SNMPv1 Network Management Framework

The SNMPv1 Network Management Framework consists of the following:

- RFC 1155 (STD 16) defines Version 1 of the Structure of Management

Information (SMIv1), the mechanisms used for describing and naming

objects for the purpose of management;

- RFC 1212 (STD 16) defines a more concise description mechanism

(Concise-SMIv1), that is wholly consistent with the SMIv1;

- RFC 1157 (STD 15) defines Version 1 of the Simple Network

Management Protocol (SNMPv1), the protocol used for network access

to managed objects, including the security model SNMPv1c

(Community-based Security with SNMPv1 PDUs).

- RFC 1215 defines a convention for defining Traps for use with the

SMIv1;

- RFC 1213 (STD 17) defines MIB-II, the core set of managed objects

for the Internet suite of protocols, including the managed objects

for SNMPv1 network management agents.

The Framework permits new objects to be defined for the purpose of

experimentation and evaluation.

2.1.1.  Object Definitions

Managed objects are accessed via a virtual information store, termed

the Management Information Base or MIB.  Objects in the MIB are

defined using the subset of Abstract Syntax Notation One (ASN.1)

defined in the SMI.  In particular, each object object type is named

by an OBJECT IDENTIFIER, an administratively assigned name.  The

object type together with an object instance serves to uniquely

identify a specific instantiation of the object.  For human

convenience, we often use a textual string, termed the descriptor, to

refer to the object type.

2.2.  SNMPv2 Network Management Framework

The SNMPv2 Network Management Framework consists of the following:

Hastings, McDonald            Expires 19 March 2000             [Page 4]

Internet Draft             IPP Server MIB              19 September 1999

- RFC 1902 (Draft Standard) defines Version 2 of the Structure of

Management Information (SMIv2), the mechanisms used for describing

and naming objects for the purpose of management

- note that RFC 1902 is now obsoleted by RFC 2578 (STD 58);

- RFC 1903 (Draft Standard) defines the SMIv2 Textual Conventions,

the mechanisms used for defining 'named object types' (refinements

of standard SMIv2 types), for use in defining managed objects

- note that RFC 1903 is now obsoleted by RFC 2579 (STD 58);

- RFC 1904 (Draft Standard) defines the SMIv2 Conformance, the

mechanisms used for defining 'conformance statements', for use in

specifying precise levels of implementation conformance for

management agents

- note that RFC 1904 is now obsoleted by RFC 2580 (STD 58);

- RFC 1905 (Draft Standard) defines Version 2 of the Simple Network

Management Protocol (SNMPv2), the protocol used for network access

to managed objects, without any security model (see RFC 1901 below)

- note that RFC 1905 PDUs are also used by SNMPv3 in RFC 2574;

- RFC 1906 (Draft Standard) defines the SNMPv2 Transport Mappings,

used by SNMPv2 'on-the-wire' over various protocol suites;

- RFC 1907 (Draft Standard) defines the SNMPv2 Agent MIB, the set of

managed objects for SNMPv2 network management agents

- note that RFC 1907 is now also used with SNMPv1 (RFC 1157) and

SNMPv3 (RFC 2574) agents.

The SNMPv2 Network Management Framework is augmented by the

following:

- RFC 1901 (Experimental) defines the security model SNMPv2c

(Community-based Security with SNMPv2 PDUs).

2.3.  SNMPv3 Network Management Framework

The SNMPv3 Network Management Framework consists of the following:

- RFC 2578 (STD 58) defines Version 2 of the Structure of Management

Information (SMIv2), the mechanisms used for describing and naming

objects for the purpose of management

- note that RFC 1902 is now obsoleted by RFC 2578 (STD 58);

- RFC 2579 (STD 58) defines the SMIv2 Textual Conventions, the

mechanisms used for defining 'named object types' (refinements of

standard SMIv2 types), for use in defining managed objects

- note that RFC 1903 is now obsoleted by RFC 2579 (STD 58);

Hastings, McDonald            Expires 19 March 2000             [Page 5]

Internet Draft             IPP Server MIB              19 September 1999

- RFC 2580 (STD 58) defines the SMIv2 Conformance, the mechanisms

used for defining 'conformance statements', for use in specifying

precise levels of implementation conformance for management agents

- note that RFC 1904 is now obsoleted by RFC 2580 (STD 58);

- RFC 2571 (Draft Standard) defines an architecture for describing

SNMP Network Management Frameworks, including the SNMP Framework

MIB;

- RFC 2572 (Draft Standard) defines message processing and

dispatching for SNMP, including the SNMP Message Processing MIB;

- RFC 2573 (Draft Standard) defines SNMP applications and dispatching

for SNMP, including the SNMP Target and Notification MIBs

(standardized trap subscription/registration) and SNMP Proxy MIB;

- RFC 2574 (Draft Standard) defines Version 3 of the Simple Network

Management Protocol (SNMPv3), the protocol used for network access

to managed objects, including the security model SNMPv3u

(User-based Security with SNMPv2 PDUs - see RFC 2575 below) and the

SNMP User-based Security MIB;

- RFC 2575 (Draft Standard) defines the View-based Access Control

Model for use with all versions of SNMP, including the SNMP

View-based Access Control MIB.

The SNMPv3 Network Management Framework is augmented by the

following:

- RFC 1907 (Draft Standard) defines the SNMPv2 Agent MIB, the set of

managed objects for SNMPv2 network management agents

- note that RFC 1907 is now also used with SNMPv1 (RFC 1157) and

SNMPv3 (RFC 2574) agents.

Hastings, McDonald            Expires 19 March 2000             [Page 6]

Internet Draft             IPP Server MIB              19 September 1999

3.  Overview of IPP Server MIB

3.1.  Structure of IPP Server MIB

The IPP Server MIB defines three object groups and four notification

groups:

- Printer Group (Mandatory)

The IPP Printer group defines zero (0) leaf objects, one (1) table

object, one (1) index object, and ten (10) columnar objects

(including 5 counters).

'ippPrinterTable' contains one entry for name, state, and counters

of each IPP Printer currently configured on this managed host

system.

- Printer URI Group (Mandatory)

The IPP Printer URI group defines zero (0) leaf objects, one (1)

table object, one (1) index object, and three (3) columnar objects.

'ippPrinterURITable' contains one entry for each URI supported of

each IPP Printer currently configured on this managed host system.

- Event Group (Conditionally Mandatory)

The IPP Event group defines twenty-one (21) leaf objects (including

5 counters), zero (0) table objects, zero (0) index objects, and

zero (0) columnar objects.

The IPP Event group contains objects used as variable-bindings in

one or more of the four notifications defined in the IPP Server

MIB.

- Printer Basic Event Group (Conditionally Mandatory)

The IPP Printer Basic Event group defines one (1) notification.

'ippPrinterBasicV2Event' corresponds to IPP 'printer-basic-event'.

- Job Basic Event Group (Conditionally Mandatory)

The IPP Job Basic Event group defines one (1) notification.

'ippJobBasicV2Event' corresponds to IPP 'job-basic-event'.

- Job Status Event Group (Conditionally Mandatory)

The IPP Job Status Event group defines one (1) notification.

'ippJobStatusV2Event' corresponds to IPP 'job-status-event'.

- Job Progress Event Group (Conditionally Mandatory)

The IPP Job Progress Event group defines one (1) notification.

'ippJobProgressV2Event' corresponds to IPP 'job-progress-event'.

Hastings, McDonald            Expires 19 March 2000             [Page 7]

Internet Draft             IPP Server MIB              19 September 1999

3.2.  Relationship to other MIBs

To be supplied - refer to related 'standards track' or 'experimental'

SNMP MIBs.

3.3.  Registration for IPP Notifications

IPP Clients may register for IPP Notifications delivered via SNMP by

either of the following methods:

3.3.1.  Registration via IPP

IPP Create-Subscription and Job creation (Create-Job, Print-Job,

Print-URI) operations MAY be used to create per-Printer or per-Job

IPP Subscription objects and MAY specify

'notify-recipient' = 'ipp-snmp://hostname[.port]'

3.3.2.  Registration via SNMP

SNMP Set-Request operations MAY be used to create appropriate rows in

the SNMP Target and Notification MIB tables defined in [RFC-2573] and

MAY specify:

'snmpTargetAddrTDomain' and 'snmpTargetAddrTAddress'

- transport domain and transport address (equivalent to URI)

'snmpTargetAddressTimeout' and 'snmpTargetAddrRetryCount'

- retry timeout and limit (for acknowledged notifications delivered

using SNMP Inform-Request rather than SNMP Trap)

'snmpTargetAddrStorageType'

- row persistence of the registration

'snmpNotifyType'

- notification using SNMP Trap (unacknowledged) or SNMP

Inform-Request (acknowledged)

'snmpTargetParamsEntry'

- notification security and SNMP protocol version

Hastings, McDonald            Expires 19 March 2000             [Page 8]

Internet Draft             IPP Server MIB              19 September 1999

3.4.  Encoding Rules for IPP Notifications

Note:  **** Encoding Rules for SNMP Packet Size Limits ****

Over some transport protocols, the variable-bindings of this SNMP

trap MAY not fit the MTU (maximum transmission unit) size.

Conforming IPP Server MIB implementations SHALL perform this

procedure to ensure that all variable-bindings of this SNMP trap are

always included in the generated SNMP trap packet:

1)  Truncate all strings, EXCEPT 'ippEventPrinterStateReasons'

(or 'ippEventJobStateReasons' for job-level SNMP traps),

to the reduced maximum size that is specified in their

corresponding OBJECT clauses in the 'ippMIBCompliance'

MODULE-COMPLIANCE macro in this IPP Server MIB.

1a) If all variable-bindings now fit within the MTU,

then exit this procedure and generate the SNMP trap.

2)  Truncate the next one of the following string objects to

the empty string (zero length in the ASN.1 BER encoding),

in the order listed:

- 'ippEventSubscriberUserName'

- 'ippEventSubscriberUserData'

- 'ippEventJobName'

2a) If all variable-bindings now fit within the MTU,

then exit this procedure and generate the SNMP trap.

2b) If all variable-bindings do NOT fit within the MTU,

then repeat step (2) for next string object.

3)  Truncate the 'ippEventPrinterStateReasons' string object

(or 'ippEventJobStateReasons' for job-level SNMP traps)

to the reduced maximum size that is specified in its

corresponding OBJECT clause in the 'ippMIBCompliance'

MODULE-COMPLIANCE macro in this IPP Server MIB, preserving

all possible state reasons which triggered this SNMP trap.

3a) If all variable-bindings now fit within the MTU,

then exit this procedure and generate the SNMP trap.

4)  Truncate the 'ippEventPrinterStateReasons' string object

(or 'ippEventJobStateReasons' for job-level SNMP traps) to

the empty string (zero length in the ASN.1 BER encoding).

4a) If all variable-bindings now fit within the MTU,

then exit this procedure and generate the SNMP trap.

4b) If all variable-bindings do NOT fit within the MTU,

then -> logic error (variable-bindings MUST now fit).

See the DESCRIPTION clauses of the four IPP Server MIB notifications,

where the above algorithm is included or referenced:

'ippPrinterBasicV2Event'

'ippJobBasicV2Event'

'ippJobStatusV2Event'

'ippJobProgressV2Event'

Hastings, McDonald            Expires 19 March 2000             [Page 9]

Internet Draft             IPP Server MIB              19 September 1999

4.  Definition of IPP Server MIB

IPP-SERVER-MIB DEFINITIONS ::= BEGIN

--  Module: Internet Printing Protocol Server MIB (work-in-progress)

--  Editor: Tom Hastings, Ira McDonald

--  File:   <draft-ietf-ipp-server-mib-00.mib>

--  Date:   19 September 1999

IMPORTS

MODULE-IDENTITY,

NOTIFICATION-TYPE,

OBJECT-IDENTITY,

OBJECT-TYPE,

Counter32,

Integer32,

experimental

FROM SNMPv2-SMI             -- IETF RFC 2578 (SMIv2)

TEXTUAL-CONVENTION,

DisplayString,

TruthValue

FROM SNMPv2-TC              -- IETF RFC 2579 (SMIv2)

MODULE-COMPLIANCE,

OBJECT-GROUP,

NOTIFICATION-GROUP

FROM SNMPv2-CONF            -- IETF RFC 2580 (SMIv2)

SnmpAdminString

FROM SNMP-FRAMEWORK-MIB;    -- IETF RFC 2571 (SNMPv3)

ippServerMIB MODULE-IDENTITY

LAST-UPDATED "9909190000Z"

ORGANIZATION "IETF Internet Printing Protocol (IPP) Working Group"

CONTACT-INFO "

Tom Hastings

Postal:     Xerox Corporation, MS 834-03E

701 S Aviation Blvd

El Segundo, CA  90245

Phone:      +1 310-333-6413

Email:      hastings@cp10.es.xerox.com

Ira McDonald

Postal:     High North Inc

221 Ridge Ave

Grand Marais, MI  49839

Phone:      +1 906-494-2434

Email:      imcdonal@sdsp.mc.xerox.com"

DESCRIPTION

"The MIB module for passive monitoring (but not configuration)

of IPP Printer entities, via client polling of SNMP objects.

Also, the MIB module for dynamic monitoring of specific events

Hastings, McDonald            Expires 19 March 2000            [Page 10]

Internet Draft             IPP Server MIB              19 September 1999

from IPP Printers and Jobs, via server generation of SNMP traps

(traps defined in this MIB are optional for conformance).

Scope:  An IPP Printer, for applicability of and conformance to

the IPP Server MIB, is an entity playing a request-receiver role

(and optional notification-generator role) in the IPP protocol

(an embedded printer, a spooler, an application gateway, or any

other IPP server-side entity).

See:    (For IPP attributes referenced in this IPP Server MIB)

[IPP-MOD] - IPP/1.0 Model and Semantics - RFC 2566.

[IPP-PRO] - IPP/1.0 Encoding and Transport - RFC 2565.

[IPP-NOT] - IPP Event Notification (work-in-progress).

[PWG-JOB] - PWG Job Monitoring MIB v1.0."

--  Revision History

REVISION

"9909190000Z"                   -- 19 September 1999

DESCRIPTION

"Version 0.3 - aligned w/ [IPP-NOT] - 10 September 1999."

REVISION

"9908300000Z"                   -- 30 August 1999

DESCRIPTION

"Version 0.2 - aligned w/ [IPP-NOT] - 25 August 1999."

REVISION

"9908160000Z"                   -- 16 August 1999

DESCRIPTION

"Version 0.1 - aligned w/ Notification Model - 11 August 1999."

REVISION

"9908090000Z"                   -- 9 August 1999

DESCRIPTION

"Version 0.0 - the initial version."

::= { experimental 9999 }           -- to be assigned

ippMIBObjects               OBJECT IDENTIFIER ::= { ippServerMIB 1 }

ippMIBNotifications         OBJECT IDENTIFIER ::= { ippServerMIB 2 }

ippMIBConformance           OBJECT IDENTIFIER ::= { ippServerMIB 3 }

--

--  Textual Conventions

--

IppPrinterState ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The current state of this IPP Printer.

See:    Section 4.4.10 'printer-state' in [IPP-MOD]."

SYNTAX      INTEGER {

other(1),

unknown(2),

idle(3),

Hastings, McDonald            Expires 19 March 2000            [Page 11]

Internet Draft             IPP Server MIB              19 September 1999

processing(4),

stopped(5)

}

IppJobState ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The current state of this IPP Job.

See:    Section 4.3.7 'job-state' in [IPP-MOD]."

SYNTAX      INTEGER {

other(1),

unknown(2),

pending(3),

pendingHeld(4),

processing(5),

processingStopped(6),

canceled(7),

aborted(8),

completed(9)

}

IppJobCollationType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The collation type (for Documents and Copies) of this IPP Job.

See:    'JmJobCollationTypeTC' and

'jobCollationType' in [PWG-JOB];

Section 7 'Notification Content' in [IPP-NOT]."

SYNTAX      INTEGER {

other(1),

unknown(2),

uncollatedSheets(3),    -- sheets within each document copy

-- are not collated: 1 1 ..., 2 2 ...,

-- No corresponding value of IPP

-- "multiple-document-handling"

collatedDocuments(4),   -- internal collated sheets,

-- documents: A, B, A, B, ...

-- Corresponds to IPP "multiple-

-- document-handling"='separate-

-- documents-collated-copies'

uncollatedDocuments(5)  -- internal collated sheets,

-- documents: A, A, ..., B, B, ...

-- Corresponds to IPP "multiple-

-- document-handling"='separate-

-- documents-uncollated-copies'

}

Hastings, McDonald            Expires 19 March 2000            [Page 12]

Internet Draft             IPP Server MIB              19 September 1999

IppTriggerEvent ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The trigger event type for this IPP event (SNMP trap).

See:    Section 5.2 'notify-events' and

Section 7 'Notification Content' in [IPP-NOT]."

SYNTAX      INTEGER {

other(1),

unknown(2),

none(3),                        -- SHOULD not be used

printerRestarted(101),          -- 'printer-restarted'

printerShutdown(102),           -- 'printer-shutdown'

printerStateChanged(103),       -- 'printer-state-changed'

printerMediaChanged(104),       -- 'printer-media-changed'

printerConfigChanged(105),      -- 'printer-config-changed'

printerQueueChanged(106),       -- 'printer-queue-changed'

printerNoLongerFull(107),       -- 'printer-no-longer-full'

printerAlmostIdle(108),         -- 'printer-almost-idle'

jobCreated(201),                -- 'job-created'

jobCompleted(202),              -- 'job-completed'

jobStateChanged(203),           -- 'job-state-changed'

jobConfigChanged(204),          -- 'job-config-changed'

jobPurged(205),                 -- 'job-purged'

jobProgress(206)                -- 'job-progress'

}

--  Printer Group (Mandatory)

--

--  Implementation of this group is mandatory for all systems which

--  implement the Internet Printing Protocol and this IPP Server MIB.

ippPrinter                  OBJECT IDENTIFIER ::= { ippMIBObjects 1 }

ippPrinterTable OBJECT-TYPE

SYNTAX      SEQUENCE OF IppPrinterEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A table of IPP Printers on this managed host system.

Conformance:  Rows in this conceptual table SHOULD be permanent

(preserved across hardware resets and reboots), although they

MAY be added, removed, or modified by system administrators

(by some means outside the scope of this IPP Server MIB).

See:    Section 4.4 'Printer Description Attributes' and

Section 2.1 'Printer Object' in [IPP-MOD]."

::= { ippPrinter 1 }

ippPrinterEntry OBJECT-TYPE

Hastings, McDonald            Expires 19 March 2000            [Page 13]

Internet Draft             IPP Server MIB              19 September 1999

SYNTAX      IppPrinterEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"An entry for an IPP Printer on this managed host system."

INDEX       { ippPrinterIndex }

::= { ippPrinterTable 1 }

IppPrinterEntry ::= SEQUENCE {

ippPrinterIndex                 Integer32 (1..2147483647),

ippPrinterNaturalLanguage       DisplayString (SIZE (0..63)),

ippPrinterName                  SnmpAdminString (SIZE (0..127)),

ippPrinterState                 IppPrinterState,

ippPrinterStateReasons          SnmpAdminString (SIZE (0..255)),

ippPrinterIsAcceptingJobs       TruthValue,

ippPrinterIncomingConnections   Counter32, -- client connections

ippPrinterIncomingRequests      Counter32, -- operation requests

ippPrinterOutgoingErrors        Counter32, -- error responses

ippPrinterOutgoingWarnings      Counter32, -- warning responses

ippPrinterOutgoingEvents        Counter32  -- notifications

}

--  Example:

--

--      ippPrinterIndex                 = 1

--      ippPrinterNaturalLanguage       = 'en-us'

--      ippPrinterName                  = 'marsupial'

--      ippPrinterState                 = 'idle(3)'

--      ippPrinterStateReasons          = 'paused'

--      ippPrinterIsAcceptingJobs       = 'true(1)'

--      ippPrinterIncomingConnections   = 2487

--      ippPrinterIncomingRequests      = 32456

--      ippPrinterOutgoingErrors        = 937

--      ippPrinterOutgoingWarnings      = 1236

--      ippPrinterOutgoingEvents        = 127778

ippPrinterIndex OBJECT-TYPE

SYNTAX      Integer32 (1..2147483647)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The local unique identifier of this 'ippPrinterTable' row."

::= { ippPrinterEntry 1 }

ippPrinterNaturalLanguage OBJECT-TYPE

SYNTAX      DisplayString (SIZE (0..63))        -- 63 in [IPP-MOD]

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The natural language configured on this IPP Printer,

specified as a language-tag per RFC 1766, using International

Hastings, McDonald            Expires 19 March 2000            [Page 14]

Internet Draft             IPP Server MIB              19 September 1999

Standard language codes (ISO 639) and country codes (ISO 3166).

In particular, the natural language of 'ippPrinterName'.

Conformance:  This printer natural language SHALL be lowercase

normalized, per [IPP-MOD].

See:    Section 4.4.16 'natural-language-configured' and

Section 4.1.8 'naturalLanguage' in [IPP-MOD];

Section 2 'The Language tag' in RFC 1766,

'Tags for the Identification of Languages';

Section 4 'Languages' in RFC 2277 / BCP 18,

'IETF Policy on Character Sets and Languages'."

--  DEFVAL intentionally omitted - natural language SHALL be valid

::= { ippPrinterEntry 2 }

ippPrinterName OBJECT-TYPE

SYNTAX      SnmpAdminString (SIZE (0..127))     -- 127 in [IPP-MOD]

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The administrative name configured on this IPP Printer.

See:    Section 4.4.4 'printer-name' and

Section 4.1.2 'name' in [IPP-MOD]."

DEFVAL      { ''H }                 -- no printer name

::= { ippPrinterEntry 3 }

ippPrinterState OBJECT-TYPE

SYNTAX      IppPrinterState

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The current state of this IPP Printer.

See:    Section 4.4.11 'printer-state' in [IPP-MOD]."

--  DEFVAL intentionally omitted - current state SHALL be valid

::= { ippPrinterEntry 4 }

ippPrinterStateReasons OBJECT-TYPE

SYNTAX      SnmpAdminString (SIZE (0..255))     -- multivalue in IPP

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The comma-separated list of state reasons on this IPP Printer.

These printer state reasons are English keywords, NOT localized.

Note:   The datatype 'SnmpAdminString' permits any future IPP

extension to 'keyword or name', allowing product-specific and/or

site-specific names (in locale of 'ippPrinterNaturalLanguage')

to be present in this object.

See:    Section 4.4.12 'printer-state-reasons' and

Section 4.1.3 'keyword' in [IPP-MOD]."

Hastings, McDonald            Expires 19 March 2000            [Page 15]

Internet Draft             IPP Server MIB              19 September 1999

--  DEFVAL intentionally omitted - state reasons SHALL be valid

::= { ippPrinterEntry 5 }

ippPrinterIsAcceptingJobs OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The current mode for new end-user jobs on this IPP Printer.

'true'  - this IPP Printer is currently accepting new jobs

'false' - this IPP Printer is NOT currently accepting new jobs

See:    Section 4.4.23 'printer-is-accepting-jobs' and

Section 4.1.11 'boolean' in [IPP-MOD]."

DEFVAL      { true }                -- printer is accepting jobs

::= { ippPrinterEntry 6 }

ippPrinterIncomingConnections OBJECT-TYPE

SYNTAX      Counter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The total incoming connections received by this IPP Printer."

::= { ippPrinterEntry 7 }

ippPrinterIncomingRequests OBJECT-TYPE

SYNTAX      Counter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The total incoming requests received by this IPP Printer."

::= { ippPrinterEntry 8 }

ippPrinterOutgoingErrors OBJECT-TYPE

SYNTAX      Counter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The total outgoing error responses sent by this IPP Printer."

::= { ippPrinterEntry 9 }

ippPrinterOutgoingWarnings OBJECT-TYPE

SYNTAX      Counter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The total outgoing warning responses sent by this IPP Printer."

::= { ippPrinterEntry 10 }

ippPrinterOutgoingEvents OBJECT-TYPE

SYNTAX      Counter32

MAX-ACCESS  read-only

Hastings, McDonald            Expires 19 March 2000            [Page 16]

Internet Draft             IPP Server MIB              19 September 1999

STATUS      current

DESCRIPTION

"The total outgoing events sent by this IPP Printer."

::= { ippPrinterEntry 11 }

--  Printer URI Group (Mandatory)

--

--  Implementation of this group is mandatory for all systems which

--  implement the Internet Printing Protocol and this IPP Server MIB.

ippPrinterURI               OBJECT IDENTIFIER ::= { ippMIBObjects 2 }

ippPrinterURITable OBJECT-TYPE

SYNTAX      SEQUENCE OF IppPrinterURIEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A table of IPP Printer URI on this managed host system.

Conformance:  Rows in this conceptual table SHOULD be permanent

(preserved across hardware resets and reboots), although they

MAY be added, removed, or modified by system administrators

(by some means outside the scope of this IPP Server MIB).

See:    Section 4.4 'Printer Description Attributes' and

Section 2.1 'Printer Object' in [IPP-MOD]."

::= { ippPrinterURI 1 }

ippPrinterURIEntry OBJECT-TYPE

SYNTAX      IppPrinterURIEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"An entry for an IPP Printer URI on this managed host system."

INDEX       { ippPrinterIndex,

ippPrinterURIIndex }

::= { ippPrinterURITable 1 }

IppPrinterURIEntry ::= SEQUENCE {

ippPrinterURIIndex              Integer32 (1..2147483647),

ippPrinterURIString             SnmpAdminString (SIZE (0..255)),

ippPrinterURIAuthentication     SnmpAdminString (SIZE (0..63)),

ippPrinterURISecurity           SnmpAdminString (SIZE (0..63))

}

--  Example:

--

--      ippPrinterURIIndex              = 1

--      ippPrinterURIString             = 'ipp://wombat.com/printer'

--      ippPrinterURIAuthentication     = 'digest' (RFC 2617)

--      ippPrinterURISecurity           = 'tls' (RFC 2246)

Hastings, McDonald            Expires 19 March 2000            [Page 17]

Internet Draft             IPP Server MIB              19 September 1999

ippPrinterURIIndex OBJECT-TYPE

SYNTAX      Integer32 (1..2147483647)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The local unique identifier of this 'ippPrinterURITable' row,

when prefixed by 'ippPrinterIndex'."

::= { ippPrinterURIEntry 1 }

ippPrinterURIString OBJECT-TYPE

SYNTAX      SnmpAdminString (SIZE (0..255))     -- 1023 in [IPP-MOD]

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A printer URI supported on this IPP Printer.

This URI conforms to the syntax specified in RFC 2396.

Note:   The datatype 'SnmpAdminString' permits any future IPP

extension to native UTF-8, allowing international URI

(without using the hexadecimal escapes specified in RFC 2396)

to be present in this object.

See:    Section 4.4.1 'printer-uri-supported' and

Section 4.1.5 'uri' in [IPP-MOD];

Section 5.10 'printer-uri' in [IPP-NOT]."

--  DEFVAL intentionally omitted - printer URI SHALL be valid

::= { ippPrinterURIEntry 2 }

ippPrinterURIAuthentication OBJECT-TYPE

SYNTAX      SnmpAdminString (SIZE (0..63))      -- 255 in [IPP-MOD]

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The authentication mode supported for this IPP Printer URI.

This authentication mode is an English keyword, NOT localized.

Note:   The datatype 'SnmpAdminString' permits any future IPP

extension to 'keyword or name', allowing product-specific and/or

site-specific names (in locale of 'ippPrinterNaturalLanguage')

to be present in this object.

See:    Section 4.4.2 'uri-authentication-supported' and

Section 4.1.3 'keyword' in [IPP-MOD]."

--  DEFVAL intentionally omitted - authentication SHALL be valid

::= { ippPrinterURIEntry 3 }

ippPrinterURISecurity OBJECT-TYPE

SYNTAX      SnmpAdminString (SIZE (0..63))      -- 255 in [IPP-MOD]

MAX-ACCESS  read-only

STATUS      current

Hastings, McDonald            Expires 19 March 2000            [Page 18]

Internet Draft             IPP Server MIB              19 September 1999

DESCRIPTION

"The security mode supported on this IPP Printer URI.

This security mode is an English keyword, NOT localized.

Note:   The datatype 'SnmpAdminString' permits any future IPP

extension to 'keyword or name', allowing product-specific and/or

site-specific names (in locale of 'ippPrinterNaturalLanguage')

to be present in this object.

See:    Section 4.4.3 'uri-security-supported' and

Section 4.1.3 'keyword' in [IPP-MOD]."

--  DEFVAL intentionally omitted - security SHALL be valid

::= { ippPrinterURIEntry 4 }

--  Event Group (Conditionally Mandatory)

--

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which deliver any IPP events via SNMP.

ippEvent                    OBJECT IDENTIFIER ::= { ippMIBObjects 3 }

--  Example:

--

--      ippEventVersionNumber           = '1.0'

--      ippEventRequestID               = 35

--      ippEventNaturalLanguage         = 'en-us'

--      ippEventPrinterIndex            = 1

--      ippEventPrinterURIIndex         = 2

--      ippEventJobID                   = 267

--      ippEventJobName                 = 'Project schedule'

--      ippEventTriggerEvent            = 'jobStateChanged(203)'

--      ippEventSubscriptionID          = 457

--      ippEventSubscriberUserName      = 'thastings'

--      ippEventSubscriberUserData      = '1234'

--      ippEventPrinterState            = 'processing(4)'

--      ippEventPrinterStateReasons     = 'media-need'

--      ippEventPrinterIsAcceptingJobs  = 'true(1)'

--      ippEventJobState                = 'processing(5)'

--      ippEventJobStateReasons         = 'job-printing'

--      ippEventJobKOctetsCompleted     = 327

--      ippEventImpressionsCompleted    = 264

--      ippEventMediaSheetsCompleted    = 132

--      ippEventJobCollationType        = 'uncollatedSheets(3)'

--      ippEventSheetCompletedCopyNum   = 1

--      ippEventSheetCompletedDocNum    = 1

--      ippEventImpressionsInterpreted  = 286

--      ippEventImpressionsCompletedCC  = 264

ippEventVersionNumber OBJECT-TYPE

SYNTAX      DisplayString (SIZE (0..10))        -- 255 in [IPP-MOD]

MAX-ACCESS  read-only

Hastings, McDonald            Expires 19 March 2000            [Page 19]

Internet Draft             IPP Server MIB              19 September 1999

STATUS      current

DESCRIPTION

"The IPP version number used to encode this IPP event.

See:    Section 4.4.14 'ipp-versions-supported' in [IPP-MOD];

Section 3.3 'Version-number' in [IPP-PRO]."

--  DEFVAL intentionally omitted - version number SHALL be valid

::= { ippEvent 1 }

ippEventRequestID OBJECT-TYPE

SYNTAX      Integer32 (0..2147483647)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The request identifier (sequence number) for this IPP event

or zero (if not applicable, because this is a unique event).

See:    Section 3.6 'Request-id' of [IPP-PRO];

Section 5.7 'request-id' of [IPP-NOT]."

DEFVAL      { 0 }                   -- no request ID

::= { ippEvent 2 }

ippEventNaturalLanguage OBJECT-TYPE

SYNTAX      DisplayString (SIZE (0..63))        -- 63 in [IPP-MOD]

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The natural language of text string attributes in this event,

specified as a language-tag per RFC 1766, using International

Standard language codes (ISO 639) and country codes (ISO 3166).

In particular, the natural language of 'ippEventJobName'

(when present in the variable-bindings of this event).

Conformance:  This event natural language SHALL be lowercase

normalized, per [IPP-MOD].

See:    Section 4.4.16 'natural-language-configured' and

Section 4.1.8 'naturalLanguage' in [IPP-MOD];

Section 5.6 'attributes-natural-language' in [IPP-NOT];

Section 2 'The Language tag' in RFC 1766,

'Tags for the Identification of Languages';

Section 4 'Languages' in RFC 2277 / BCP 18,

'IETF Policy on Character Sets and Languages'."

--  DEFVAL intentionally omitted - natural language SHALL be valid

::= { ippEvent 3 }

ippEventPrinterIndex OBJECT-TYPE

SYNTAX      Integer32 (0..2147483647)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of 'ippPrinterIndex' for this IPP event

or zero (if not applicable, because no printer is associated).

Hastings, McDonald            Expires 19 March 2000            [Page 20]

Internet Draft             IPP Server MIB              19 September 1999

See:    'ippPrinterIndex' in this IPP Server MIB."

DEFVAL      { 0 }                   -- no printer associated

::= { ippEvent 4 }

ippEventPrinterURIIndex OBJECT-TYPE

SYNTAX      Integer32 (0..2147483647)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of 'ippPrinterURIIndex' for this IPP event

or zero (if not applicable, because no printer is associated).

See:    'ippPrinterURIIndex' in this IPP Server MIB."

DEFVAL      { 0 }                   -- no printer associated

::= { ippEvent 5 }

ippEventJobID OBJECT-TYPE

SYNTAX      Integer32 (0..2147483647)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The job identifier for this IPP event

or zero (if not applicable, because no job is associated).

See:    Section 4.3.2 'job-id' of [IPP-MOD]."

DEFVAL      { 0 }                   -- no job identifier

::= { ippEvent 6 }

ippEventJobName OBJECT-TYPE

SYNTAX      SnmpAdminString (SIZE (0..255))     -- 1023 in [IPP-MOD]

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The name submitted by owner at creation time of this IPP Job.

See:    Section 4.3.5 'job-name' and

Section 4.1.2 'name' in [IPP-MOD]."

DEFVAL      { ''H }                 -- no job name

::= { ippEvent 7 }

ippEventTriggerEvent OBJECT-TYPE

SYNTAX      IppTriggerEvent

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The trigger event type for this IPP event (SNMP trap).

See:    Section 5.2 'notify-events' and

Section 7 'Notification Content' in [IPP-NOT]."

--  DEFVAL intentionally omitted - trigger event SHALL be valid

::= { ippEvent 8 }

ippEventSubscriptionID OBJECT-TYPE

SYNTAX      Integer32 (0..2147483647)

Hastings, McDonald            Expires 19 March 2000            [Page 21]

Internet Draft             IPP Server MIB              19 September 1999

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The subscription identifier for this IPP event

or zero (if not applicable, because this is a unique event).

See:    Section 5.8 'subscription-id' and

Section 7 'Notification Content' in [IPP-NOT]."

DEFVAL      { 0 }                   -- no subscription ID

::= { ippEvent 9 }

ippEventSubscriberUserName OBJECT-TYPE

SYNTAX      SnmpAdminString (SIZE (0..255))     -- 1023 in [IPP-MOD]

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The subscriber user name associated with this IPP event.

See:    Section 4.3.6 'job-originating-user-name' in [IPP-MOD];

Section 5.11 'subscriber-user-name' and

Section 7 'Notification Content' in [IPP-NOT]."

DEFVAL      { ''H }                 -- no subscriber user name

::= { ippEvent 10 }

ippEventSubscriberUserData OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE (0..63))         -- 63 in [IPP-NOT]

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The subscriber user data associated with this IPP event.

See:    Section 5.4 'subscriber-user-data' and

Section 7 'Notification Content' in [IPP-NOT]."

DEFVAL      { ''H }                 -- no subscriber user data

::= { ippEvent 11 }

ippEventPrinterState OBJECT-TYPE

SYNTAX      IppPrinterState

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The current state of this IPP Printer.

See:    Section 4.4.11 'printer-state' in [IPP-MOD]."

--  DEFVAL intentionally omitted - current state SHALL be valid

::= { ippEvent 12 }

ippEventPrinterStateReasons OBJECT-TYPE

SYNTAX      SnmpAdminString (SIZE (0..255))     -- multivalue in IPP

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The comma-separated list of state reasons on this IPP Printer.

These printer state reasons are English keywords, NOT localized.

Hastings, McDonald            Expires 19 March 2000            [Page 22]

Internet Draft             IPP Server MIB              19 September 1999

Note:   The datatype 'SnmpAdminString' permits any future IPP

extension to 'keyword or name', allowing product-specific and/or

site-specific names (in locale of 'ippEventNaturalLanguage')

to be present in this object.

See:    Section 4.4.12 'printer-state-reasons' and

Section 4.1.3 'keyword' in [IPP-MOD]."

--  DEFVAL intentionally omitted - state reasons SHALL be valid

::= { ippEvent 13 }

ippEventPrinterIsAcceptingJobs OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The current mode for new end-user jobs on this IPP Printer.

'true'  - this IPP Printer is currently accepting new jobs

'false' - this IPP Printer is NOT currently accepting new jobs

See:    Section 4.4.23 'printer-is-accepting-jobs' and

Section 4.1.11 'boolean' in [IPP-MOD]."

DEFVAL      { true }                -- printer is accepting jobs

::= { ippEvent 14 }

ippEventJobState OBJECT-TYPE

SYNTAX      IppJobState

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The current state of this IPP Job.

See:    Section 4.3.7 'job-state' in [IPP-MOD]."

--  DEFVAL intentionally omitted - current state SHALL be valid

::= { ippEvent 15 }

ippEventJobStateReasons OBJECT-TYPE

SYNTAX      SnmpAdminString (SIZE (0..255))     -- multivalue in IPP

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The comma-separated list of state reasons on this IPP Job.

These job state reasons are English keywords, NOT localized.

Note:   The datatype 'SnmpAdminString' permits any future IPP

extension to 'keyword or name', allowing product-specific and/or

site-specific names (in locale of 'ippEventNaturalLanguage')

to be present in this object.

See:    Section 4.3.8 'job-state-reasons' and

Section 4.1.3 'keyword' in [IPP-MOD]."

--  DEFVAL intentionally omitted - state reasons SHALL be valid

::= { ippEvent 16 }

Hastings, McDonald            Expires 19 March 2000            [Page 23]

Internet Draft             IPP Server MIB              19 September 1999

ippEventJobKOctetsProcessed OBJECT-TYPE

SYNTAX      Counter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The total number of octets processed in K octets, rounded up.

See:    Section 4.3.18.1 'job-k-octets-processed' and

Section 4.1.12 'integer' in [IPP-MOD]."

--  DEFVAL omitted per SMIv2            - should be initialized to zero

::= { ippEvent 17 }

ippEventImpressionsCompleted OBJECT-TYPE

SYNTAX      Counter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The total number of impressions completed for this IPP Job.

See:    Section 4.3.18.2 'job-impressions-completed' and

Section 4.1.12 'integer' in [IPP-MOD]."

--  DEFVAL omitted per SMIv2            - should be initialized to zero

::= { ippEvent 18 }

ippEventMediaSheetsCompleted OBJECT-TYPE

SYNTAX      Counter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The total number of media sheets completed for this IPP Job.

See:    Section 4.3.18.3 'job-media-sheets-completed' and

Section 4.1.12 'integer' in [IPP-MOD]."

--  DEFVAL omitted per SMIv2            - should be initialized to zero

::= { ippEvent 19 }

ippEventJobCollationType OBJECT-TYPE

SYNTAX      IppJobCollationType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The collation type (for Documents and Copies) of this IPP Job.

See:    'JmJobCollationTypeTC' and

'jobCollationType' in [PWG-JOB];

Section 7 'Notification Content' in [IPP-NOT]."

--  DEFVAL intentionally omitted - job collation type SHALL be valid

::= { ippEvent 20 }

ippEventSheetCompletedCopyNum OBJECT-TYPE

SYNTAX      Integer32 (0..2147483647)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

Hastings, McDonald            Expires 19 March 2000            [Page 24]

Internet Draft             IPP Server MIB              19 September 1999

"The ordinal number of the Copy currently being stacked for

the current Document in this IPP Job.

See:    'sheetCompletedCopyNumber' in [PWG-JOB];

Section 7 'Notification Content' in [IPP-NOT]."

--  DEFVAL intentionally omitted - copy number SHALL be valid

::= { ippEvent 21 }

ippEventSheetCompletedDocNum OBJECT-TYPE

SYNTAX      Integer32 (0..2147483647)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The ordinal number of the Document currently being stacked for

this IPP Job.

See:    'sheetCompletedDocumentNumber' in [PWG-JOB];

Section 7 'Notification Content' in [IPP-NOT]."

--  DEFVAL intentionally omitted - document number SHALL be valid

::= { ippEvent 22 }

ippEventImpressionsInterpreted OBJECT-TYPE

SYNTAX      Counter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The total number of impressions interpreted for this IPP Job.

See:    'impressionsInterpreted' in [PWG-JOB];

Section 7 'Notification Content' in [IPP-NOT]."

--  DEFVAL omitted per SMIv2            - should be initialized to zero

::= { ippEvent 23 }

ippEventImpressionsCompletedCC OBJECT-TYPE

SYNTAX      Counter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The total number of impressions completed for the current Copy

of the current Document in this IPP Job.

See:    'impressionsCompletedCurrentCopy' in [PWG-JOB];

Section 7 'Notification Content' in [IPP-NOT]."

--  DEFVAL omitted per SMIv2            - should be initialized to zero

::= { ippEvent 24 }

--  Printer Basic Event Group (Conditionally Mandatory)

--

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which deliver this IPP event via SNMP.

ippPrinterBasicV1Enterprise OBJECT-IDENTITY

STATUS      current

DESCRIPTION

"The value of the enterprise-specific OID in an SNMPv1 trap

Hastings, McDonald            Expires 19 March 2000            [Page 25]

Internet Draft             IPP Server MIB              19 September 1999

for a 'printer-basic-event' generated by this managed system."

::= { ippMIBNotifications 1 }

ippPrinterBasicV2EventPrefix

OBJECT IDENTIFIER ::= { ippPrinterBasicV1Enterprise 0 }

ippPrinterBasicV2Event NOTIFICATION-TYPE

OBJECTS {

ippEventVersionNumber,

ippEventRequestID,

ippEventNaturalLanguage,

ippEventPrinterIndex,

ippEventPrinterURIIndex,

ippEventJobID,                  -- per-job subscription events

ippEventJobName,                -- per-job subscription events

ippEventTriggerEvent,

ippEventSubscriptionID,

ippEventSubscriberUserName,

ippEventSubscriberUserData,

ippEventPrinterState,

ippEventPrinterStateReasons,

ippEventPrinterIsAcceptingJobs

}

STATUS      current

DESCRIPTION

"This SMIv2 trap corresponds to the IPP 'printer-basic-event'.

This trap is sent whenever requested by a prior subscription for

the included printer-level 'ippEventTriggerEvent', which may be:

- 'printer-restarted'

- 'printer-shutdown'

- 'printer-state-changed'

- 'printer-media-changed'

- 'printer-config-changed'

- 'printer-queue-changed'

- 'printer-no-longer-full'

- 'printer-almost-idle'

- or other registered IPP 'printer-basic-event' trigger

Additional variable-bindings MAY be appended to this trap:

- IPP Printers, which support 'printer-current-time', SHALL add

'hrSystemDate' from the IETF Host Resources MIB (RFC 1514).

- IPP Printers MAY add any other useful object from any MIB.

The IPP Notification attribute 'printer-uri' has been

included by reference (not by value) in these variable-bindings

(via 'ippEventPrinterIndex' and 'ippEventPrinterURIIndex').

The IPP Notification attribute 'printer-name' has been

included by reference (not by value) in these variable-bindings

(via 'ippEventPrinterIndex').

Hastings, McDonald            Expires 19 March 2000            [Page 26]

Internet Draft             IPP Server MIB              19 September 1999

The IPP Notification attribute 'attributes-charset' has been

omitted from these variable bindings, since the UTF-8 string

type 'SnmpAdminString' from RFC 2571 (SNMPv3 Framework MIB)

is used in this IPP Server MIB, for compliance with RFC 2277

(IETF Policy on Character Sets and Languages, January 1998).

IPP Printer implementations MUST support UTF-8, per [IPP-MOD].

The IPP Notification attribute 'trigger-time' has been

omitted from these variable-bindings, since the mandatory object

'sysUpTime' from the System group of IETF MIB-II (RFC 1213)

MUST be included in SNMPv1, SNMPv2, and SNMPv3 traps (either

in the envelope in SNMPv1 or as a fixed binding in SNMPv2/v3).

Note:   **** Encoding Rules for SNMP Packet Size Limits ****

Over some transport protocols, the variable-bindings of this

SNMP trap MAY not fit the MTU (maximum transmission unit) size.

Conforming IPP Server MIB implementations SHALL perform this

procedure to ensure that all variable-bindings of this SNMP trap

are always included in the generated SNMP trap packet:

1)  Truncate all strings, EXCEPT 'ippEventPrinterStateReasons',

to the reduced maximum size that is specified in their

corresponding OBJECT clauses in the 'ippMIBCompliance'

MODULE-COMPLIANCE macro in this IPP Server MIB.

1a) If all variable-bindings now fit within the MTU,

then exit this procedure and generate the SNMP trap.

2)  Truncate the next one of the following string objects to

the empty string (zero length in the ASN.1 BER encoding),

in the order listed:

- 'ippEventSubscriberUserName'

- 'ippEventSubscriberUserData'

- 'ippEventJobName'

2a) If all variable-bindings now fit within the MTU,

then exit this procedure and generate the SNMP trap.

2b) If all variable-bindings do NOT fit within the MTU,

then repeat step (2) for next string object.

3)  Truncate the 'ippEventPrinterStateReasons' string object

to the reduced maximum size that is specified in its

corresponding OBJECT clause in the 'ippMIBCompliance'

MODULE-COMPLIANCE macro in this IPP Server MIB, preserving

all possible state reasons which triggered this SNMP trap.

3a) If all variable-bindings now fit within the MTU,

then exit this procedure and generate the SNMP trap.

4)  Truncate the 'ippEventPrinterStateReasons' string object to

the empty string (zero length in the ASN.1 BER encoding).

4a) If all variable-bindings now fit within the MTU,

then exit this procedure and generate the SNMP trap.

4b) If all variable-bindings do NOT fit within the MTU,

then -> logic error (variable-bindings MUST now fit)."

::= { ippPrinterBasicV2EventPrefix 1 }

Hastings, McDonald            Expires 19 March 2000            [Page 27]

Internet Draft             IPP Server MIB              19 September 1999

--  Job Basic Event Group (Conditionally Mandatory)

--

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which deliver this IPP event via SNMP.

ippJobBasicV1Enterprise OBJECT-IDENTITY

STATUS      current

DESCRIPTION

"The value of the enterprise-specific OID in an SNMPv1 trap

for a 'job-basic-event' generated by this managed system."

::= { ippMIBNotifications 2 }

ippJobBasicV2EventPrefix

OBJECT IDENTIFIER ::= { ippJobBasicV1Enterprise 0 }

ippJobBasicV2Event NOTIFICATION-TYPE

OBJECTS {

ippEventVersionNumber,

ippEventRequestID,

ippEventNaturalLanguage,

ippEventPrinterIndex,

ippEventPrinterURIIndex,

ippEventJobID,

ippEventJobName,

ippEventTriggerEvent,

ippEventSubscriptionID,

ippEventSubscriberUserName,

ippEventSubscriberUserData,

ippEventJobState,

ippEventJobStateReasons

}

STATUS      current

DESCRIPTION

"This SMIv2 trap corresponds to the IPP 'job-basic-event'.

This trap is sent whenever requested by a prior subscription for

the included job-level 'ippEventTriggerEvent', which may be:

- 'job-created'

- 'job-config-changed'

- or other registered IPP 'job-basic-event' trigger

Additional variable-bindings MAY be appended to this trap:

- IPP Printers, which support 'printer-current-time', SHALL add

'hrSystemDate' from the IETF Host Resources MIB (RFC 1514).

- IPP Printers MAY add any other useful object from any MIB.

See:    DESCRIPTION clause of 'ippPrinterBasicV2Event' trap for

information about (apparently) omitted IPP attributes.

Note:   **** Encoding Rules for SNMP Packet Size Limits ****

Hastings, McDonald            Expires 19 March 2000            [Page 28]

Internet Draft             IPP Server MIB              19 September 1999

Over some transport protocols, the variable-bindings of this

SNMP trap MAY not fit the MTU (maximum transmission unit) size.

Conforming IPP Server MIB implementations SHALL perform this

procedure to ensure that all variable-bindings of this SNMP trap

are always included in the generated SNMP trap packet:

1)  Truncate all strings, EXCEPT 'ippEventJobStateReasons',

to the reduced maximum size that is specified in their

corresponding OBJECT clauses in the 'ippMIBCompliance'

MODULE-COMPLIANCE macro in this IPP Server MIB.

1a) If all variable-bindings now fit within the MTU,

then exit this procedure and generate the SNMP trap.

2)  Truncate the next one of the following string objects to

the empty string (zero length in the ASN.1 BER encoding),

in the order listed:

- 'ippEventSubscriberUserName'

- 'ippEventSubscriberUserData'

- 'ippEventJobName'

2a) If all variable-bindings now fit within the MTU,

then exit this procedure and generate the SNMP trap.

2b) If all variable-bindings do NOT fit within the MTU,

then repeat step (2) for next string object.

3)  Truncate the 'ippEventJobStateReasons' string object

to the reduced maximum size that is specified in its

corresponding OBJECT clause in the 'ippMIBCompliance'

MODULE-COMPLIANCE macro in this IPP Server MIB, preserving

all possible state reasons which triggered this SNMP trap.

3a) If all variable-bindings now fit within the MTU,

then exit this procedure and generate the SNMP trap.

4)  Truncate the 'ippEventJobStateReasons' string object to

the empty string (zero length in the ASN.1 BER encoding).

4a) If all variable-bindings now fit within the MTU,

then exit this procedure and generate the SNMP trap.

4b) If all variable-bindings do NOT fit within the MTU,

then -> logic error (variable-bindings MUST now fit)."

::= { ippJobBasicV2EventPrefix 1 }

--  Job Status Event Group (Conditionally Mandatory)

--

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which deliver this IPP event via SNMP.

ippJobStatusV1Enterprise OBJECT-IDENTITY

STATUS      current

DESCRIPTION

"The value of the enterprise-specific OID in an SNMPv1 trap

for a 'job-status-event' generated by this managed system."

::= { ippMIBNotifications 3 }

ippJobStatusV2EventPrefix

OBJECT IDENTIFIER ::= { ippJobStatusV1Enterprise 0 }

Hastings, McDonald            Expires 19 March 2000            [Page 29]

Internet Draft             IPP Server MIB              19 September 1999

ippJobStatusV2Event NOTIFICATION-TYPE

OBJECTS {

ippEventVersionNumber,

ippEventRequestID,

ippEventNaturalLanguage,

ippEventPrinterIndex,

ippEventPrinterURIIndex,

ippEventJobID,

ippEventJobName,

ippEventTriggerEvent,

ippEventSubscriptionID,

ippEventSubscriberUserName,

ippEventSubscriberUserData,

ippEventJobState,

ippEventJobStateReasons,

ippEventJobKOctetsProcessed,

ippEventImpressionsCompleted,

ippEventMediaSheetsCompleted

}

STATUS      current

DESCRIPTION

"This SMIv2 trap corresponds to the IPP 'job-status-event'.

This trap is sent whenever requested by a prior subscription for

the included job-level 'ippEventTriggerEvent', which may be:

- 'job-completed'

- 'job-state-changed'

- 'job-purged'

- or other registered IPP 'job-status-event' trigger

Additional variable-bindings MAY be appended to this trap:

- IPP Printers, which support 'printer-current-time', SHALL add

'hrSystemDate' from the IETF Host Resources MIB (RFC 1514).

- IPP Printers MAY add any other useful object from any MIB.

See:    DESCRIPTION clause of 'ippPrinterBasicV2Event' trap for

information about (apparently) omitted IPP attributes.

See:    DESCRIPTION clause of 'ippJobBasicV2Event' trap for

required encoding rules for SNMP packet size limits."

::= { ippJobStatusV2EventPrefix 1 }

--  Job Progress Event Group (Conditionally Mandatory)

--

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which deliver this IPP event via SNMP.

ippJobProgressV1Enterprise OBJECT-IDENTITY

STATUS      current

DESCRIPTION

Hastings, McDonald            Expires 19 March 2000            [Page 30]

Internet Draft             IPP Server MIB              19 September 1999

"The value of the enterprise-specific OID in an SNMPv1 trap

for a 'job-progress-event' generated by this managed system."

::= { ippMIBNotifications 4 }

ippJobProgressV2EventPrefix

OBJECT IDENTIFIER ::= { ippJobProgressV1Enterprise 0 }

ippJobProgressV2Event NOTIFICATION-TYPE

OBJECTS {

ippEventVersionNumber,

ippEventRequestID,

ippEventNaturalLanguage,

ippEventPrinterIndex,

ippEventPrinterURIIndex,

ippEventJobID,

ippEventJobName,

ippEventTriggerEvent,

ippEventSubscriptionID,

ippEventJobKOctetsProcessed,

ippEventImpressionsCompleted,

ippEventMediaSheetsCompleted,

ippEventJobCollationType,

ippEventSheetCompletedCopyNum,

ippEventSheetCompletedDocNum,

ippEventImpressionsInterpreted,

ippEventImpressionsCompletedCC

}

STATUS      current

DESCRIPTION

"This SMIv2 trap corresponds to the IPP 'job-progress-event'.

This trap is sent whenever requested by a prior subscription for

the included job-level 'ippEventTriggerEvent', which may be:

- 'job-progress'

- or other registered IPP 'job-progress-event' trigger

Additional variable-bindings MAY be appended to this trap:

- IPP Printers, which support 'printer-current-time', SHALL add

'hrSystemDate' from the IETF Host Resources MIB (RFC 1514).

- IPP Printers MAY add any other useful object from any MIB.

See:    DESCRIPTION clause of 'ippPrinterBasicV2Event' trap for

information about (apparently) omitted IPP attributes.

See:    DESCRIPTION clause of 'ippJobBasicV2Event' trap for

required encoding rules for SNMP packet size limits."

::= { ippJobProgressV2EventPrefix 1 }

Hastings, McDonald            Expires 19 March 2000            [Page 31]

Internet Draft             IPP Server MIB              19 September 1999

--  Conformance Information

--

--  with Mandatory and Optional Conformance Groups

ippMIBGroups        OBJECT IDENTIFIER ::= { ippMIBConformance 2 }

ippMIBCompliance MODULE-COMPLIANCE

STATUS      current

DESCRIPTION

"The compliance statements for SNMP management agents that

implement the Internet Printing Protocol MIB."

MODULE -- this module

MANDATORY-GROUPS {

ippPrinterGroup,

ippPrinterURIGroup

}

GROUP       ippEventGroup

DESCRIPTION

"Implementation of this group is conditionally mandatory;

mandatory for systems which deliver any IPP events via SNMP."

GROUP       ippPrinterBasicEventGroup

DESCRIPTION

"Implementation of this group is conditionally mandatory;

mandatory for systems which deliver this IPP event via SNMP."

GROUP       ippJobBasicEventGroup

DESCRIPTION

"Implementation of this group is conditionally mandatory;

mandatory for systems which deliver this IPP event via SNMP."

GROUP       ippJobStatusEventGroup

DESCRIPTION

"Implementation of this group is conditionally mandatory;

mandatory for systems which deliver this IPP event via SNMP."

GROUP       ippJobProgressEventGroup

DESCRIPTION

"Implementation of this group is conditionally mandatory;

mandatory for systems which deliver this IPP event via SNMP."

OBJECT      ippPrinterNaturalLanguage

SYNTAX      DisplayString (SIZE (0..5))         -- 63 in [IPP-MOD]

DESCRIPTION

"It is conformant to implement this object as a shorter string;

only 5 octets maximum string length need be supported."

OBJECT      ippPrinterName

SYNTAX      SnmpAdminString (SIZE (0..31))      -- 127 in [IPP-MOD]

DESCRIPTION

Hastings, McDonald            Expires 19 March 2000            [Page 32]

Internet Draft             IPP Server MIB              19 September 1999

"It is conformant to implement this object as a shorter string;

only 31 octets maximum string length need be supported."

OBJECT      ippPrinterStateReasons

SYNTAX      SnmpAdminString (SIZE (0..63))      -- multivalue in IPP

DESCRIPTION

"It is conformant to implement this object as a shorter string;

only 63 octets maximum string length need be supported."

OBJECT      ippPrinterURIString

SYNTAX      SnmpAdminString (SIZE (0..63))      -- 1023 in [IPP-MOD]

DESCRIPTION

"It is conformant to implement this object as a shorter string;

only 63 octets maximum string length need be supported."

OBJECT      ippPrinterURIAuthentication

SYNTAX      SnmpAdminString (SIZE (0..31))      -- 255 in [IPP-MOD]

DESCRIPTION

"It is conformant to implement this object as a shorter string;

only 31 octets maximum string length need be supported."

OBJECT      ippPrinterURISecurity

SYNTAX      SnmpAdminString (SIZE (0..31))      -- 255 in [IPP-MOD]

DESCRIPTION

"It is conformant to implement this object as a shorter string;

only 31 octets maximum string length need be supported."

OBJECT      ippEventVersionNumber

SYNTAX      DisplayString (SIZE (0..5))         -- 255 in [IPP-MOD]

DESCRIPTION

"It is conformant to implement this object as a shorter string;

only 5 octets maximum string length need be supported."

OBJECT      ippEventNaturalLanguage

SYNTAX      DisplayString (SIZE (0..5))         -- 63 in [IPP-MOD]

DESCRIPTION

"It is conformant to implement this object as a shorter string;

only 5 octets maximum string length need be supported."

OBJECT      ippEventJobName

SYNTAX      SnmpAdminString (SIZE (0..31))      -- 1023 in [IPP-MOD]

DESCRIPTION

"It is conformant to implement this object as a shorter string;

only 31 octets maximum string length need be supported."

OBJECT      ippEventJobStateReasons

SYNTAX      SnmpAdminString (SIZE (0..63))      -- multivalue in IPP

DESCRIPTION

"It is conformant to implement this object as a shorter string;

only 63 octets maximum string length need be supported."

Hastings, McDonald            Expires 19 March 2000            [Page 33]

Internet Draft             IPP Server MIB              19 September 1999

OBJECT      ippEventSubscriberUserName

SYNTAX      SnmpAdminString (SIZE (0..31))      -- 1023 in [IPP-MOD]

DESCRIPTION

"It is conformant to implement this object as a shorter string;

only 31 octets maximum string length need be supported."

OBJECT      ippEventSubscriberUserData

SYNTAX      SnmpAdminString (SIZE (0..31))      -- 1023 in [IPP-MOD]

DESCRIPTION

"It is conformant to implement this object as a shorter string;

only 31 octets maximum string length need be supported."

::= { ippMIBConformance 1 }

--  Conformance Groups

--

--  with Required Objects for all Implementations

ippPrinterGroup OBJECT-GROUP

OBJECTS {

ippPrinterNaturalLanguage,

ippPrinterName,

ippPrinterState,

ippPrinterStateReasons,

ippPrinterIsAcceptingJobs,

ippPrinterIncomingConnections,

ippPrinterIncomingRequests,

ippPrinterOutgoingErrors,

ippPrinterOutgoingWarnings,

ippPrinterOutgoingEvents

}

STATUS      current

DESCRIPTION

"IPP Printer group - printer state and counters"

::= { ippMIBGroups 1 }

ippPrinterURIGroup OBJECT-GROUP

OBJECTS {

ippPrinterURIString,

ippPrinterURIAuthentication,

ippPrinterURISecurity

}

STATUS      current

DESCRIPTION

"IPP Printer URI group - printer URI, authentication, security"

::= { ippMIBGroups 2 }

ippEventGroup OBJECT-GROUP

OBJECTS {

ippEventVersionNumber,

ippEventNaturalLanguage,

Hastings, McDonald            Expires 19 March 2000            [Page 34]

Internet Draft             IPP Server MIB              19 September 1999

ippEventRequestID,

ippEventPrinterIndex,

ippEventPrinterURIIndex,

ippEventJobID,

ippEventJobName,

ippEventTriggerEvent,

ippEventSubscriptionID,

ippEventSubscriberUserName,

ippEventSubscriberUserData,

ippEventPrinterState,

ippEventPrinterStateReasons,

ippEventPrinterIsAcceptingJobs,

ippEventJobState,

ippEventJobStateReasons,

ippEventJobKOctetsProcessed,

ippEventImpressionsCompleted,

ippEventMediaSheetsCompleted,

ippEventJobCollationType,

ippEventSheetCompletedCopyNum,

ippEventSheetCompletedDocNum,

ippEventImpressionsInterpreted,

ippEventImpressionsCompletedCC

}

STATUS      current

DESCRIPTION

"IPP Event group - printer and job event attributes"

::= { ippMIBGroups 3 }

ippPrinterBasicEventGroup NOTIFICATION-GROUP

NOTIFICATIONS {

ippPrinterBasicV2Event

}

STATUS      current

DESCRIPTION

"IPP Printer Basic Event group - 'printer-basic-event'"

::= { ippMIBGroups 4 }

ippJobBasicEventGroup NOTIFICATION-GROUP

NOTIFICATIONS {

ippJobBasicV2Event

}

STATUS      current

DESCRIPTION

"IPP Job Basic Event group - 'job-basic-event'"

::= { ippMIBGroups 5 }

ippJobStatusEventGroup NOTIFICATION-GROUP

NOTIFICATIONS {

ippJobStatusV2Event

}

STATUS      current

Hastings, McDonald            Expires 19 March 2000            [Page 35]

Internet Draft             IPP Server MIB              19 September 1999

DESCRIPTION

"IPP Job Status Event group - 'job-status-event'"

::= { ippMIBGroups 6 }

ippJobProgressEventGroup NOTIFICATION-GROUP

NOTIFICATIONS {

ippJobProgressV2Event

}

STATUS      current

DESCRIPTION

"IPP Job Progress Event group - 'job-progress-event'"

::= { ippMIBGroups 7 }

END

Hastings, McDonald            Expires 19 March 2000            [Page 36]

Internet Draft             IPP Server MIB              19 September 1999

5.  IANA Considerations

None.

6.  Internationalization Considerations

All text and name string objects defined in the IPP Server MIB are of

type 'SnmpAdminString' (UTF-8 stream-encoded ISO 10646/Unicode)

imported from the SNMP Framework MIB defined in [RFC-2571].  The

natural language of all of these text and name string objects is

specified by either 'ippPrinterNaturalLanguage' or

'ippEventNaturalLanguage' (see below).

The Printer group defines the 'ippPrinterNaturalLanguage' object that

reports the currently configured natural language on the IPP Printer

as a language-tag [RFC-1766], using International Standard language

codes [ISO-639] and country codes [ISO-3166].

The Event group defines the 'ippEventNaturalLanguage' object that

reports the subscriber requested natural language in each IPP trap as

a language-tag [RFC-1766], using International Standard language

codes [ISO-639] and country codes [ISO-3166].

7.  Security Considerations

The IPP Server MIB defines only 'read-only' objects.  It is suitable

for use with any version of SNMP, as no update security is required

(because no configuration updates are supported).

No sensitive information is available in IPP Server MIB objects or in

the bindings of IPP Server MIB traps.

Hastings, McDonald            Expires 19 March 2000            [Page 37]

Internet Draft             IPP Server MIB              19 September 1999

8.  References

To be supplied.

9.  Intellectual Property Notice

The IETF takes no position regarding the validity or scope of any

intellectual property or other rights that might be claimed to

pertain to the implementation or use of the technology described in

this document or the extent to which any license under such rights

might or might not be available; neither does it represent that it

has made any effort to identify any such rights.  Information on the

IETF's procedures with respect to rights in standards-track and

standards-related documentation can be found in BCP-11.  Copies of

claims of rights made available for publication and any assurances of

licenses to be made available, or the result of an attempt made to

obtain a general license or permission for the use of such

proprietary rights by implementers or users of this specification can

be obtained from the IETF Secretariat."

The IETF invites any interested party to bring to its attention any

copyrights, patents or patent applications, or other proprietary

rights which may cover technology that may be required to practice

this standard.  Please address the information to the IETF Executive

Director.

10.  Authors' Addresses

Tom Hastings

Xerox Corporation

701 S Aviation Blvd, MS 834-03E

El Segundo, CA  90245

Phone: +1 310-333-6413

Email: hastings@cp10.es.xerox.com

Ira McDonald

High North Inc

221 Ridge Ave

Grand Marais, MI  49839

Phone: +1 906-494-2434 or +1 906-494-2697

Email: imcdonal@sdsp.mc.xerox.com

Hastings, McDonald            Expires 19 March 2000            [Page 38]

Internet Draft             IPP Server MIB              19 September 1999

11.  Full Copyright Statement

Copyright (C) The Internet Society (1999).  All Rights Reserved.

This document and translations of it may be copied and furnished to

others, and derivative works that comment on or otherwise explain it

or assist in its implementation may be prepared, copied, published

and distributed, in whole or in part, without restriction of any

kind, provided that the above copyright notice and this paragraph are

included on all such copies and derivative works.  However, this

document itself may not be modified in any way, such as by removing

the copyright notice or references to the Internet Society or other

Internet organizations, except as needed for the purpose of

developing Internet standards in which case the procedures for

copyrights defined in the Internet Standards process must be

followed, or as required to translate it into languages other than

English.

The limited permissions granted above are perpetual and will not be

revoked by the Internet Society or its successors or assigns.

This document and the information contained herein is provided on an

"AS IS" basis and THE INTERNET SOCIETY AND THE INTERNET ENGINEERING

TASK FORCE DISCLAIMS ALL WARRANTIES, EXPRESS OR IMPLIED, INCLUDING

BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF THE INFORMATION

HEREIN WILL NOT INFRINGE ANY RIGHTS OR ANY IMPLIED WARRANTIES OF

MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

Hastings, McDonald            Expires 19 March 2000            [Page 39]