Internet Printing Protocol Working Group                  Robert Herriot

INTERNET DRAFT                                                Consultant

<draft-ietf-ipp-url-scheme-05.txt>                          Ira McDonald

Updates: RFC 2910                                         High North Inc

[Target Category: Standards Track]                           20 May 2002

Expires 20 November 2002

Internet Printing Protocol/1.1:

IPP URL Scheme

<draft-ietf-ipp-url-scheme-05.txt>

Copyright (C) The Internet Society (2002). All Rights Reserved.

Status of this Memo

This document is an Internet-Draft and is in full conformance with

all provisions of Section 10 of RFC2026.  Internet-Drafts are working

documents of the Internet Engineering Task Force (IETF), its areas,

and its working groups.  Note that other groups may also distribute

working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months

and may be updated, replaced, or obsoleted by other documents at any

time.  It is inappropriate to use Internet-Drafts as reference

material or to cite them other than as "work in progress."

To view the list of Internet-Draft Shadow Directories, see

http://www.ietf.org/shadow.html.

Abstract

This memo defines the "ipp" URL (Uniform Resource Locator) scheme.

This memo updates IPP/1.1: Encoding and Transport (RFC 2910), by

expanding and clarifying Section 5 'IPP URL Scheme' of RFC 2910.  An

"ipp" URL is used to specify the network location of a print service

that supports the IPP Protocol (RFC 2910), or of a network resource

(for example, a print job) managed by such a print service.

Herriot, McDonald           Expires 20 November 2002            [Page 1]

Internet Draft                IPP URL Scheme                 20 May 2002

Table of Contents

1.  Introduction ...............................................       3

2.  Terminology ................................................       5

2.1.  Conformance Terminology ................................       5

2.2.  Model Terminology ......................................       5

3.  IPP Model for Printers and Jobs ............................       6

4.  IPP URL Scheme .............................................       7

4.1.  IPP URL Scheme Applicability ...........................       7

4.2.  IPP URL Scheme Associated Port .........................       7

4.3.  IPP URL Scheme Associated MIME Type ....................       7

4.4.  IPP URL Scheme Character Encoding ......................       8

4.5.  IPP URL Scheme Syntax ..................................       8

4.6.  IPP URL Examples .......................................       8

4.6.1.  IPP Printer URL Examples ...........................       9

4.6.2.  IPP Job URL Examples ...............................       9

4.7.  IPP URL Comparisons ....................................      10

5.  Conformance Requirements ...................................      11

5.1.  IPP Client Conformance Requirements ....................      11

5.2.  IPP Printer Conformance Requirements ...................      11

6.  IANA Considerations ........................................      13

7.  Internationalization Considerations ........................      13

8.  Security Considerations ....................................      13

9.  Normative References .......................................      15

10.  Informative References ....................................      15

11.  Acknowledgments ...........................................      16

12.  Authors' Addresses ........................................      16

13.  Full Copyright Statement ..................................      17

14.  Appendix A - Registration of "ipp" URL Scheme .............      17

15.  Appendix X - Change History ...............................      20

Herriot, McDonald           Expires 20 November 2002            [Page 2]

Internet Draft                IPP URL Scheme                 20 May 2002

1.  Introduction

This memo conforms to all of the requirements in Registration

Procedures for URL Scheme Names [RFC2717].  This memo also follows

all of the recommendations in Guidelines for new URL Schemes

[RFC2718].

See section 1 'Introduction' of [RFC2911] and section 1

'Introduction' of [RFC3196] for overview information about IPP.  See

section 10 'Description of the Base IPP Documents of [RFC3196] for a

full description of the IPP document set.

This memo updates IPP/1.1: Encoding and Transport (RFC 2910), by

expanding and clarifying Section 5 'IPP URL Scheme' of RFC 2910, but

does not define any new parameters or other new extensions to the

syntax of IPP URLs.

The IPP URL scheme defined in this document is based on the ABNF for

the HTTP URL scheme defined in HTTP [RFC2616], which in turn is

derived from the URI Generic Syntax [RFC2396] and further updated for

IPv6 by [RFC2732].  An IPP URL is transformed into an HTTP URL

according to the rules specified in section 5 of IPP Protocol

[RFC2910].

This document defines IPP URL scheme applicability, associated port

(631), associated MIME type ("application/ipp"), character encoding,

and syntax.

This document is laid out as follows:

- Section 2 defines the terminology used throughout the document.

- Section 3 supplies references to the IPP Printer and IPP Job object

model defined in IPP Model [RFC2911].

- Section 4 specifies the IPP URL scheme.

- Section 5 specifies the conformance requirements for IPP Clients

and IPP Printers that claim conformance to this document.

- Sections 6, 7, and 8 specify IANA, internationalization, and

security considerations.

- Sections 9, 10, 11, 12, and 13 specify normative references,

informative references, acknowledgements, authors' addresses, and

full IETF copyright statement.

- Section 14 (Appendix A) is a completed registration template for

the IPP URL Scheme (see section 6.0 of [RFC2717]).

Herriot, McDonald           Expires 20 November 2002            [Page 3]

Internet Draft                IPP URL Scheme                 20 May 2002

[Ed Note:  Section 15, which should be removed before publication as

an RFC, contains a complete change history of this IPP URL document.]

Herriot, McDonald           Expires 20 November 2002            [Page 4]

Internet Draft                IPP URL Scheme                 20 May 2002

2.  Terminology

This specification document uses the terminology defined in this

section.

2.1.  Conformance Terminology

The uppercase terms "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL

NOT" "SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" in

this document are to be interpreted as described in [RFC2119].  These

terms are used to specify conformance requirements for all

implementations (both print clients and print services) of this

specification.

2.2.  Model Terminology

See section 12.2 'Model Terminology' in IPP Model [RFC2911].

Herriot, McDonald           Expires 20 November 2002            [Page 5]

Internet Draft                IPP URL Scheme                 20 May 2002

3.  IPP Model for Printers and Jobs

See section 2 'IPP Objects', section 2.1 'Printer Object', and

section 2.2 'Job Object' in [RFC2911] for a full description of the

IPP object model and terminology.

In this document, "IPP Client" means the software (on some hardware

platform) that submits, monitors, and/or manages print jobs via the

IPP Protocol [RFC2910] to a print spooler, print gateway, or physical

printing device.

In this document, "IPP Printer object" means the software (on some

hardware platform) that receives print jobs and/or printer/job

operations via the IPP Protocol [RFC2910] from an "IPP Client".

In this document, "IPP Printer" is a synonym for "IPP Printer

object".

In this document, "IPP Job object" means the set of attributes and

documents for one print job instantiated on an "IPP Printer".

In this document, "IPP Job" is a synonym for "IPP Job object".

In this document, "IPP URL" means a URL with the "ipp" scheme.

Note:  In this document, "IPP URL" is a synonym for "ipp-URL" (in

section 4 'IPP URL Scheme' of this document) and "ipp-URL" (in

section 5 'IPP URL Scheme' of [RFC2910]).

Herriot, McDonald           Expires 20 November 2002            [Page 6]

Internet Draft                IPP URL Scheme                 20 May 2002

4.  IPP URL Scheme

4.1.  IPP URL Scheme Applicability

The "ipp" URL scheme MUST only be used to specify absolute URLs

(relative IPP URLs are not allowed) for IPP print services and their

associated network resources.  The "ipp" URL scheme MUST only be used

to specify the use of the abstract protocol defined in IPP Model

[RFC2911] over an HTTP [RFC2616] transport, as defined in IPP

Protocol [RFC2910].  Any other transport binding for the abstract

protocol defined in IPP Model [RFC2911] would require a different URL

scheme.

The "ipp" URL scheme allows an IPP client to choose an appropriate

IPP print service (for example, from a directory).  The IPP client

can establish an HTTP connection to the specified IPP print service.

The IPP client can send IPP protocol requests (for example, a

'Print-Job' request) and receive IPP protocol responses over that

HTTP connection.

4.2.  IPP URL Scheme Associated Port

All IPP URLs which do NOT explicitly specify a port MUST be resolved

to IANA-assigned well-known port 631, as registered in

[IANA-PORTREG].

See:  IANA Port Numbers Registry [IANA-PORTREG].

See:  IPP Protocol [RFC2910].

4.3.  IPP URL Scheme Associated MIME Type

All IPP URLs MUST be used to specify network print services which

support the "application/ipp" MIME media type as registered in

[IANA-MIMEREG] for IPP protocol requests and responses.

See:  IANA MIME Media Types Registry [IANA-MIMEREG].

See:  IPP Protocol [RFC2910].

Herriot, McDonald           Expires 20 November 2002            [Page 7]

Internet Draft                IPP URL Scheme                 20 May 2002

4.4.  IPP URL Scheme Character Encoding

IPP URLs MUST use [RFC2396] encoding, as do their equivalent HTTP

URLs.  Characters other than those in the "reserved" and "unsafe"

sets [RFC2396] are equivalent to their ""%" HEX HEX" encoding.

4.5.  IPP URL Scheme Syntax

The abstract protocol defined in IPP Model [RFC2911] places a limit

of 1023 octets (NOT characters) on the length of a URI (see section

4.1.5 'uri' in [RFC2911]).

Note:  IPP Printers ought to be cautious about depending on URI

lengths above 255 bytes, because some older client implementations

might not properly support these lengths.

IPP URLs MUST be represented in absolute form.  Absolute URLs MUST

always begin with a scheme name followed by a colon.  For definitive

information on URL syntax and semantics, see "Uniform Resource

Identifiers (URI): Generic Syntax and Semantics" [RFC2396].  This

specification adopts the definitions of "host", "port", "abs_path",

and "query" from [RFC2396], as updated for IPv6 by [RFC2732].

The IPP URL scheme syntax in ABNF is as follows:

ipp-URL = "ipp:" "//" host [ ":" port ] [ abs_path [ "?" query ]]

If the port is empty or not given, port 631 is assumed.  The

semantics are that the identified resource (see section 5.1.2 of

[RFC2616]) is located at the IPP print service listening for HTTP

connections on that port of that host, and the Request-URI for the

identified resource is 'abs_path'.

If the 'abs_path' is not present in the URL, it MUST be given as "/"

when used as a Request-URI for a resource (see section 5.1.2 of

[RFC2616]).

4.6.  IPP URL Examples

Note:  Literal IPv4 or IPv6 addresses SHOULD NOT be used in IPP URLs.

Herriot, McDonald           Expires 20 November 2002            [Page 8]

Internet Draft                IPP URL Scheme                 20 May 2002

4.6.1.  IPP Printer URL Examples

The following are examples of well-formed IPP URLs for IPP Printers

(for example, to be used as protocol elements in 'printer-uri'

operation attributes of 'Print-Job' request messages):

ipp://abc.com

ipp://abc.com/printer

ipp://abc.com/printer/tiger

ipp://abc.com/printer/fox

ipp://abc.com/printer/tiger/bob

ipp://abc.com/printer/tiger/ira

Each of the above URLs are well-formed URLs for IPP Printers and each

would reference a logically different IPP Printer, even though some

of those IPP Printers might share the same host system.  The 'bob' or

'ira' last path components might represent two different physical

printer devices, while 'tiger' might represent some grouping of IPP

Printers (for example, a load-balancing spooler).  Or the 'bob' and

'ira' last path components might represent separate human recipients

on the same physical printer device (for example, a physical printer

supporting two job queues).  In either case, both 'bob' and 'ira'

would behave as different and independent IPP Printers.

The following are examples of well-formed IPP URLs for IPP Printers

with (optional) ports and paths:

ipp://abc.com

ipp://abc.com/~smith/printer

ipp://abc.com:631/~smith/printer

The first and second IPP URLs above MUST be resolved to port 631

(IANA assigned well-known port for IPP).  The second and third IPP

URLs above are equivalent (see section 4.7 below).

4.6.2.  IPP Job URL Examples

The following are examples of well-formed IPP URLs for IPP Jobs (for

example, to be used as protocol elements in 'job-uri' attributes of

'Print-Job' response messages):

ipp://abc.com/printer/123

ipp://abc.com/printer/tiger/job123

IPP Job URLs are valid and meaningful only until Job completion and

possibly an implementation defined optional period of persistence

after Job completion (see IPP Model [RFC2911]).

Herriot, McDonald           Expires 20 November 2002            [Page 9]

Internet Draft                IPP URL Scheme                 20 May 2002

Ambiguously, section 4.3.1 'job-uri' of IPP Model [RFC2911] states

that:

"the precise format of a Job URI is implementation dependent."

Thus, the relationship between the value of the "printer-uri"

operation attribute used in a 'Print-Job' request and the value of

the "job-uri" attribute returned in the corresponding 'Print-Job'

response is implementation dependent.  Also, section 4.3.3

'job-printer-uri' of IPP Model [RFC2911] states that the

'job-printer-uri' attribute of a Job object:

"permits a client to identify the Printer object that created this

Job object when only the Job object's URI is available to the

client."

However, the above statement is false, because the transform from an

IPP Job URL to the corresponding IPP Printer URL is unspecified in

either IPP Model [RFC2911] or IPP Protocol [RFC2910].

IPP Printers that conform to this specification SHOULD only generate

IPP Job URLs (for example, in the "job-uri" attribute in a

'Print-Job' response) by appending exactly one path component to the

corresponding IPP Printer URL (for interoperability).

4.7.  IPP URL Comparisons

When comparing two IPP URLs to decide if they match or not, an IPP

Client MUST use the same rules as those defined for HTTP URI

comparisons in [RFC2616], with the sole following exception:

- A port that is empty or not given MUST be treated as equivalent to

the well-known port for that IPP URL (port 631);

See:  Section 3.2.3 'URI Comparison' in [RFC2616].

Herriot, McDonald           Expires 20 November 2002           [Page 10]

Internet Draft                IPP URL Scheme                 20 May 2002

5.  Conformance Requirements

5.1.  IPP Client Conformance Requirements

IPP Clients that conform to this specification:

a) MUST only send IPP protocol connections to the port specified in

each given IPP URL (if present) or otherwise to IANA assigned

well-known port 631;

b) MUST only send IPP URLs used as protocol elements in outgoing IPP

protocol request messages (for example, in the "printer-uri"

operation attribute in a 'Print-Job' request) that conform to the

ABNF specified in section 4.5 'IPP URL Scheme Syntax' of this

document;

c) MUST only convert IPP URLs to their corresponding HTTP URL forms

according to the rules in section 5 'IPP URL Scheme' in [RFC2910].

5.2.  IPP Printer Conformance Requirements

IPP Printers that conform to this specification:

a) MUST listen for incoming IPP protocol connections on IANA-assigned

well-known port 631, unless explicitly configured by system

administrators or site policies;

b) SHOULD NOT listen for incoming IPP protocol connections on any

other port, unless explicitly configured by system administrators

or site policies;

c) SHOULD only accept IPP URLs used as protocol elements in incoming

IPP protocol request messages (for example, in the "printer-uri"

operation attribute in a 'Print-Job' request) that conform to the

ABNF specified in section 4.5 'IPP URL Scheme Syntax' of this

document;

d) SHOULD only send IPP URLs used as protocol elements in outgoing

IPP protocol response messages (for example, in the "job-uri"

attribute in a 'Print-Job' response) that conform to the ABNF

specified in section 4.5 'IPP URL Scheme Syntax' of this document;

e) SHOULD only generate IPP Job URLs (for example, in the "job-uri"

attribute in a 'Print-Job' response) by appending exactly one path

component to the corresponding IPP Printer URL (for

Herriot, McDonald           Expires 20 November 2002           [Page 11]

Internet Draft                IPP URL Scheme                 20 May 2002

interoperability);

f) SHOULD NOT use literal IPv6 or IPv4 addresses in configured or

locally generated IPP URLs.

Herriot, McDonald           Expires 20 November 2002           [Page 12]

Internet Draft                IPP URL Scheme                 20 May 2002

6.  IANA Considerations

This IPP URL Scheme specification does not introduce any additional

IANA considerations, beyond those described in [RFC2910] and

[RFC2911].

See:  Section 6 'IANA Considerations' in [RFC2910]

See:  Section 6 'IANA Considerations' in [RFC2911].

7.  Internationalization Considerations

This IPP URL Scheme specification does not introduce any additional

internationalization considerations, beyond those described in

[RFC2910] and [RFC2911].

See:  Section 7 'Internationalization Considerations' in [RFC2910].

See:  Section 7 'Internationalization Considerations' in [RFC2911].

8.  Security Considerations

This IPP URL Scheme specification does not introduce any additional

security considerations, beyond those described in [RFC2910] and

[RFC2911], except the following:

a) An IPP URL might be faked to point to a rogue IPP print service,

thus collecting confidential document contents from IPP clients.

Server authentication mechanisms and security mechanisms specified

in the IPP Protocol [RFC2910] are sufficient to address this

threat.

b) An IPP URL might be used to access an IPP print service by an

unauthorized IPP client.  Client authentication mechanisms and

security mechanisms specified in the IPP Protocol [RFC2910] are

sufficient to address this threat.

c) An IPP URL might be used to access an IPP print service at a print

protocol application layer gateway (for example, an IPP to LPD

gateway [RFC2569]) causing silent compromise of IPP security

mechanisms.  There is no practical defense against this threat by

a client system.  System administrators should avoid such

compromising configurations.

d) An IPP URL does not have parameters to specify the required client

authentication mechanism (for example, 'certificate' as defined in

section 4.4.2 'uri-authentication-supported' of IPP Model

Herriot, McDonald           Expires 20 November 2002           [Page 13]

Internet Draft                IPP URL Scheme                 20 May 2002

[RFC2911]) and required security mechanism (for example, 'tls' as

defined in section 4.4.3 'uri-security-supported' of IPP Model

[RFC2911]).  Service discovery or directory protocols might be

used to discover the required client authentication and security

mechanisms associated with given IPP URLs.

Historical Note:  During the development of this document,

consideration was given to the addition of standard IPP URL

parameters for the client authentication and security mechanisms.

However, based on a strong IETF IPP Working Group concensus, no

parameters were added to the "ipp" URL scheme as originally defined

in IPP Protocol [RFC2910] in September 2000, for reasons of backwards

compatibility with the many currently shipping implementations of

IPP/1.1.

See:  Section 8 'Security Considerations' in [RFC2910].

See:  Section 8 'Security Considerations' in [RFC2911].

Herriot, McDonald           Expires 20 November 2002           [Page 14]

Internet Draft                IPP URL Scheme                 20 May 2002

9.  Normative References

[RFC2234] D. Crocker, P. Overell.  Augmented BNF for Syntax

Specifications: ABNF, RFC 2234, November 1997.

[RFC2396] T. Berners-Lee, R. Fielding, L. Masinter.  Uniform Resource

Identifiers (URI): Generic Syntax, RFC 2396, August 1998.

[RFC2616] R. Fielding, J. Gettys, J. Mogul, H. Frystyk, L. Masinter,

P. Leach, T. Berners-Lee.  Hypertext Transfer Protocol -- HTTP/1.1,

RFC 2616, June 1999.

[RFC2732] R. Hinden,B. Carpenter, L. Masinter.  Format for Literal

IPv6 Addresses in URL's, RFC 2732, December 1999.

[RFC2910] R. Herriot, S. Butler, P. Moore, R. Turner, J. Wenn.

IPP/1.1 Encoding and Transport [IPP Protocol], RFC 2910, September

2000.

[RFC2911] T. Hastings, R. Herriot, R. deBry, S. Isaacson, P. Powell.

IPP/1.1 Model and Semantics [IPP Model], RFC 2911, September 2000.

[US-ASCII] Coded Character Set -- 7-bit American Standard Code for

Information Interchange, ANSI X3.4-1986.

10.  Informative References

See:  Section 10 'References' in [RFC2910].

[IANA-MIMEREG] IANA MIME Media Types Registry.

ftp://ftp.iana.org/in-notes/iana/assignments/media-types/...

[IANA-PORTREG] IANA Port Numbers Registry.

ftp://ftp.iana.org/in-notes/iana/assignments/port-numbers

[RFC2569] R. Herriot, T. Hastings, N. Jacobs, J. Martin.  Mapping

between LPD and IPP Protocols, RFC 2569, April 1999.

[RFC2717] R. Petke, I. King.  Registration Procedures for URL Scheme

Names, RFC 2717, November 1999.

[RFC2718] L. Masinter, H. Alvestrand, D. Zigmond, R. Petke.

Guidelines for new URL Schemes, RFC 2718, November 1999.

[RFC3196] T. Hastings, C. Manros, P. Zehler, C. Kugler, H. Holst.

Internet Printing Protocol/1.1: Implementor's Guide, RFC 3196,

November 2001.

Herriot, McDonald           Expires 20 November 2002           [Page 15]

Internet Draft                IPP URL Scheme                 20 May 2002

11.  Acknowledgments

This document is a product of the Internet Printing Protocol Working

Group of the Internet Engineering Task Force (IETF).

Thanks to Pat Fleming (IBM), Tom Hastings (Xerox), Harry Lewis (IBM),

Hugo Parra (Novell), Don Wright (Lexmark), and all the members of the

IETF IPP WG.

Section 5 'IPP URL Scheme' in IPP Protocol [RFC2910] was the primary

input to this IPP URL Scheme specification.

12.  Authors' Addresses

Robert Herriot

Consultant

706 Colorado Ave

Palo Alto, CA  94303

Phone: +1 650-327-4466

Fax:   +1 650-327-4466

Email: bob@herriot.com

Ira McDonald

High North Inc

221 Ridge Ave

Grand Marais, MI  49839

Phone: +1 906-494-2434

Email: imcdonald@sharplabs.com

Usage questions and comments on this IPP URL Scheme should be sent

directly to the editors at their above addresses (and to the IPP

mailing list, if you are a subscriber - see below).

IPP Web Page:      http://www.pwg.org/ipp/

IPP Mailing List:  ipp@pwg.org

To subscribe to the IPP mailing list, send the following email:

1) send it to majordomo@pwg.org

2) leave the subject line blank

3) put the following two lines in the message body:

subscribe ipp

Herriot, McDonald           Expires 20 November 2002           [Page 16]

Internet Draft                IPP URL Scheme                 20 May 2002

end

Implementers of this specification are encouraged to join the IPP

Mailing List in order to participate in any discussions of

clarification issues and comments.  In order to reduce spam the

mailing list rejects mail from non-subscribers, so you must subscribe

to the mailing list in order to send a question or comment to the IPP

mailing list.

13.  Full Copyright Statement

Copyright (C) The Internet Society (2002).  All Rights Reserved.

This document and translations of it may be copied and furnished to

others, and derivative works that comment on or otherwise explain it

or assist in its implementation may be prepared, copied, published

and distributed, in whole or in part, without restriction of any

kind, provided that the above copyright notice and this paragraph are

included on all such copies and derivative works.  However, this

document itself may not be modified in any way, such as by removing

the copyright notice or references to the Internet Society or other

Internet organizations, except as needed for the purpose of

developing Internet standards in which case the procedures for

copyrights defined in the Internet Standards process must be

followed, or as required to translate it into languages other than

English.

The limited permissions granted above are perpetual and will not be

revoked by the Internet Society or its successors or assigns.

This document and the information contained herein is provided on an

"AS IS" basis and THE INTERNET SOCIETY AND THE INTERNET ENGINEERING

TASK FORCE DISCLAIMS ALL WARRANTIES, EXPRESS OR IMPLIED, INCLUDING

BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF THE INFORMATION

HEREIN WILL NOT INFRINGE ANY RIGHTS OR ANY IMPLIED WARRANTIES OF

MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

14.  Appendix A - Registration of "ipp" URL Scheme

Note:  The following registration obsoletes section 5 'IPP URL

Scheme' of IPP Protocol [RFC2911].

URL Scheme Name:  ipp

URL Scheme Syntax:

ipp-URL = "ipp:" "//" host [ ":" port ] [ abs_path [ "?" query ]]

Herriot, McDonald           Expires 20 November 2002           [Page 17]

Internet Draft                IPP URL Scheme                 20 May 2002

Character Encoding Considerations:

IPP URLs MUST use [RFC2396] encoding, as do their equivalent HTTP

URLs.  Characters other than those in the "reserved" and "unsafe"

sets [RFC2396] are equivalent to their ""%" HEX HEX" encoding.

Intended Usage:

The intended usage of the "ipp" URL scheme is COMMON.

An "ipp" URL is used to specify the network location of a print

service that supports the IPP Protocol [RFC2910], or of a network

resource (for example, a print job) managed by such a print

service.  An IPP client can choose to establish an HTTP connection

to the specified print service for transmission of IPP protocol

requests (for example, IPP print job submission requests).

Applications or Protocols which use this URL scheme:

See:  Section 5 'IPP URL Scheme' in IPP Protocol [RFC2910].

Interoperability Considerations:

See:  Section 9 'Interoperability with IPP/1.0 Implementations' in

IPP Protocol [RFC2910].

Security Considerations:

See:  Section 8 'Security Considerations' in IPP Protocol

[RFC2910].

Relevant Publications:

[RFC2910] R. Herriot, S. Butler, P. Moore, R. Turner, J. Wenn.

IPP/1.1 Encoding and Transport [IPP Protocol], RFC 2910, September

2000.

[RFC2616] R. Fielding, J. Gettys, J. Mogul, H. Frystyk,

L. Masinter, P. Leach, T. Berners-Lee.  Hypertext Transfer

Protocol -- HTTP/1.1, RFC 2616, June 1999.

[RFCnnnn] R. Herriot, I. McDonald.  IPP/1.1:  IPP URL Scheme, RFC

nnnn, mmmm yyyy.

[Ed Note:  The above should be replaced with the correct RFC

number and date when this document is published as an RFC.]

Person & email address to contact for further information:

Herriot, McDonald           Expires 20 November 2002           [Page 18]

Internet Draft                IPP URL Scheme                 20 May 2002

Robert Herriot

Consultant

706 Colorado Ave

Palo Alto, CA  94303

Phone: +1 650-327-4466

Fax:   +1 650-327-4466

Email: bob@herriot.com

or

Ira McDonald

High North Inc

221 Ridge Ave

Grand Marais, MI  49839

Phone: +1 906-494-2434

Email: imcdonald@sharplabs.com

Herriot, McDonald           Expires 20 November 2002           [Page 19]

Internet Draft                IPP URL Scheme                 20 May 2002

15.  Appendix X - Change History

[To be deleted before RFC publication]

20 May 2002 - draft-ietf-ipp-url-scheme-05.txt

- expanded IPP in title of document, per RFC Editor;

- revised 'References' section, separating into 'Normative

References' and 'Informative References' sections, per RFC Editor;

- revised 'Abstract' section, to emphasize that this document updates

RFC 2910, per Ned Freed;

- revised several sections, deleting repeated paragraphs and

references to RFC 2373 "IPv6 Addressing Architecture", per Ned

Freed;

- revised 'IPP URL Examples' section, adding examples of IPP URLs

used to uniquely identify IPP Jobs, per Ned Freed;

- revised 'IPP URL Examples' section, deleting both IPv4 and IPv6

literal address examples, per Ned Freed;

- revised 'Conformance Requirements' section, to specify requirements

both for IPP URLs used for the network location of IPP print

services and for IPP URLs used as protocol elements in IPP protocol

requests and responses, per Ned Freed;

- revised 'Security Considerations' section, to discuss security

threats associated with the use of IPP URLs (see section 2.4 of

[RFC2718]), per Ned Freed;

- added 'Appendix A - Registration of "ipp" URL Scheme' with

completed form (see section 6.0 'Registration Template' of

[RFC2717]).

10 January 2002 - draft-ietf-ipp-url-scheme-04.txt

- final edits after IESG 'last call' comments;

- revised all titles in sections 4.x to remove redundant prefix of

'IPP URL Scheme', for readability;

- revised 'Abstract', section 1 'Introduction', section 4.1

'Applicability and Intended Usage', section 4.5 'IPP URL Scheme

Syntax', and section 6 'IANA Considerations', to explicitly state

that the "ipp" URL scheme is intended for IANA registration in the

IETF URL scheme tree;

- revised section 4.5 'IPP URL Scheme Syntax', to delete references

to unused ABNF components from [RFC2396];

- revised section 11 'Authors' Addresses', to update contact info for

both editors and to add the IPP Web page and mailing list

subscription info;

- moved 'Appendix X - Change History' to back of document, to

facilitate final edits for RFC publication (including deletion of

change history);

2 April 2001 - draft-ietf-ipp-url-scheme-03.txt

- final edits after IETF IPP WG 'last call' comments;

Herriot, McDonald           Expires 20 November 2002           [Page 20]

Internet Draft                IPP URL Scheme                 20 May 2002

- revised 'Abstract' and section 1 'Introduction' to remove

references to ISSUE's and request for comments to the 'ipp@pwg.org'

mailing list, in preparation for publication as an RFC;

- revised section 4.5 'IPP URL Scheme Syntax' to delete all

references to HTTP proxy behavior (which IPP does NOT specify), per

request of Don Wright;

- revised section 4.6 'IPP URL Examples' to remove note discouraging

the use of literal IP addresses in URLs, to remove dependency on

Informational [RFC1900];

- revised section 4.7 'IPP URL Comparisons' to specify the use of

rules defined in section 3.2.3 'URI Comparison' in [RFC2616], with

the sole exception that an empty port MUST be treated as equivalent

to the IPP well-known port 631, per request of Don Wright;

- revised section 9 'References' to delete all unused references;

- revised section 11 'Authors' Addresses' to add the address of the

IPP WG mailing list for usage questions and comments;

13 February 2001 - draft-ietf-ipp-url-scheme-02.txt

- revised section 3 'IPP Model for Printers and Jobs' and section 4.5

'IPP URL Scheme Syntax' to add notes stating that "IPP URL" (in

this document) is a synonym for "ipp-URL" in [RFC2910], per request

of Bob Herriot;

- revised section 4.5 'IPP URL Scheme Syntax' to correct typo that

showed "http:" rather than "ipp:" in the one-line ABNF, per request

of Tom Hastings;

- revised section 4.6 'IPP URL Examples' to add a note discouraging

the use of literal IP addresses in URLs, per [RFC2616] and

[RFC1900];

5 February 2001 - draft-ietf-ipp-url-scheme-01.txt

- revised section 4.1 'IPP URL Applicability and Intended Usage' to

clarify that a given IPP URL MAY identify an IPP Printer object or

an IPP Job object, per request of Tom Hastings;

- revised section 4.5 'IPP URL Scheme Syntax' to define IPP URLs

consistently with section 3.2.2 'http URL' of HTTP [RFC2616], per

request of Tom Hastings;

- revised section 4.5 'IPP URL Scheme Syntax' to clarify that IPP

URLs may reference IPP Printer objects, IPP Job objects, or

(possibly other future) IPP objects, per request of Bob Herriot;

- added section 4.6 'IPP URL Examples' to supply meaningful examples

of IPP URLs with host names, IPv4 addresses, and IPv6 addresses,

per request of Tom Hastings;

- added section 4.7 'IPP URL Comparisons' to define IPP URL

comparisons consistently with section 3.3 'URI Comparison' of HTTP

[RFC2616], per request of Tom Hastings;

- revised section 5.1 'Conformance Requirements for IPP Clients' to

clarify that an IPP Client MUST convert IPP URLs to their

corresponding HTTP URL forms according to section 5 'IPP URL

Scheme' in [RFC2910], per request of Tom Hastings and Bob Herriot;

- revised section 5.1 'Conformance Requirements for IPP Clients' and

section 5.2 'Conformance Requirements for IPP Printers' to clarify

Herriot, McDonald           Expires 20 November 2002           [Page 21]

Internet Draft                IPP URL Scheme                 20 May 2002

that IPP Clients and IPP Printers SHOULD interoperate with IPP/1.0

systems according to section 9 'Interoperability with IPP/1.0

Implementations' in [RFC2910], per request of Carl Kugler;

- revised section 5.2 'Conformance Requirements for IPP Printers' to

clarify that an IPP Printer MUST listen on (IANA assigned

well-known) port 631, unless explicitly configured otherwise, per

request of Michael Sweet;

- revised section 5.2 'Conformance Requirements for IPP Printers' to

clarify that an IPP Printer SHOULD NOT listen on ports other than

(IANA assigned well-known) port 631, unless explicitly configured,

per request of Don Wright;

- revised section 6 'IANA Considerations' to clarify that the sole

purpose of the entire document is IANA registration of the "ipp"

URL scheme;

- deleted Appendix A 'Registration of IPP Port' as unnecessary (port

is already registered);

- deleted Appendix B 'Registration of MIME "application/ipp" as

unnecessary (MIME registry has recently caught up to RFC 2910);

11 January 2001 - draft-ietf-ipp-url-scheme-00.txt

- initial version - simple "ipp" URL scheme without parameters or

query part (consistent with existing and IPP/1.1 implementations);

- added Appendix A 'Registration of IPP Port' (placeholder) for

updated IANA registration of port 631 with references to IPP/1.1;

- added Appendix B 'Registration of MIME "application/ipp"' with

updated IANA registration for IPP MIME type with references to both

IPP/1.0 and IPP/1.1;

Herriot, McDonald           Expires 20 November 2002           [Page 22]