Proposed Internet-Draft	S. Isaacson

D. Taylor

Novell, Inc.

M. MacKay

Novell, Inc.

P. Zehler

Xerox Corporation

T. Hastings

Xerox Corporation

C. Manros

Xerox Corporation

R. Herriot

Sun Microsystems

November 11, 1996

Internet Printing Protocol

Version 0.9, November 11, 1996

NOTE: Because this document has been extensively changed from its original form, it has many rough spots which will need further editing. At this time, the reader should read it for major concepts.

Status of this Memo

This document is a working version of a protocol specification.  It will eventually become an Internet-Draft by following well defined IETF procedures.  At that time, the following paragraphs must be included:

This document is an Internet-Draft.  Internet-Drafts are working documents of the Internet Engineering Task Force (IETF), its areas, and its working groups.  Note that other groups may also distribute working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months and may be updated, replaced, or obsoleted by other documents at any time.  It is inappropriate to use Internet-Drafts as reference material or to cite them other than as ``work in progress.''

To learn the current status of any Internet-Draft, please check the ``1id-abstracts.txt'' listing contained in the Internet-Drafts Shadow Directories on ftp.is.co.za (Africa), nic.nordu.net (Europe), munnari.oz.au (Pacific Rim), ds.internic.net (US East Coast), or ftp.isi.edu (US West Coast).

Abstract

This Internet-Draft specifies an Internet Printing Protocol (IPP) based on HTTP [still to be ratified by the IPP group].  protocol for the Internet. This protocol is heavily influence by  the semantic operations and attributes defined in ISO/IEC 10175 Document Printing Application (DPA) parts 1 and 3.  It also incorporates some of the implementation and interoperability lessons learned from other printing related standards such as POSIX System Administration - Part 4 (POSIX 1378.4) and X/Open A Printing System Interoperability Specification(PSIS).

IPP is defined as a set of abstract data types and operations. The operations are implemented using a protocol that is HTTP based [still to be ratified by the IPP group].

The IPP protocol initially covers only user operations on basic print service objects, but will cover management operation as soon as possible. Authentication and some access control will be required for the CancelJob operation. Additional access Control, Device Management, and Service Management will be added to the protocol as soon as possible. Some monitoring and management is possible through other protocols. The SNMP Printer MIB [1] is an example of one of these.  In the areas where there are no existing standards, many are being worked in other distributed service forums (management, security, etc.).  As these services become more standardized, this document (and hence the protocol) can be updated to reflect the integration and relationships with these other standards.

Table of Contents

1. Introduction	5

2.	Distributed Printing	6

2.1 Components	6

2.2 Objects	6

2.2.1 Printer	7

2.2.2 Job	9

2.2.3 Document	9

2.2.4 Initial Value Job	9

2.2.5 Initial Value Document	9

2.3 Object Relationships	9

2.4 Use of Naming and Directory Services	9

2.4.1 Status	10

2.4.2 Resolution	11

2.4.4 Color Supported	11

2.4.5 Maximum Speed	11

2.4.6 Maximum Speed Units	11

2.4.7 Plug and Play Device Id	11

2.4.8 Model	11

2.4.9 Manufacturer	11

2.4.10 Type	12

2.4.11 PDLs Supported	12

2.4.12 Sides Supported	12

2.5 OIDs	12

3.	Internet Printing Model	12

3.1 Object Instances 	12

3.2 Limits and Defaults	13

3.3 List Object Attribute Scoping Rules	13

4. Operations	13

4.1 Common Data Structures	13

4.1.1 XDR	13

4.1.2 ASN.1	16

4.2 Errors	17

4.2.1 ASN.1	17

5. Binding and Unbinding	19

5.1 Bind Operation	19

5.1.1 Bind Argument	19

5.1.2 Bind Result	21

5.2 Unbind Operation	22

5.2.1 Unbind Argument	22

5.2.2 Unbind Result	22

6. User Operations	22

6.1 Print Operation	22

6.1.1  Print Argument	22

6.1.2  Print Result	24

6.2 Cancel Job Operation	25

6.2.1  Cancel Job Argument	25

6.2.2  Cancel Job Result	26

6.3 List Object Attributes Operation	26

6.3.1  List Object Attributes Argument	27

6.3.2  List Object Attributes Result	30

6.4 Modify Job Operation	31

6.4.1  Modify Job Argument	31

6.4.2  Modify Job Result	32

6.5 Resubmit Job Operation	33

6.5.1 Resubmit Job Argument	33

6.5.2 Resubmit Job Result	34

7. Object Attributes	35

7.1 Job Attributes	36

7.1.1 Job Informational Attributes	37

7.1.2 Printer Selection Attributes	39

7.1.3 Job Status Attributes	39

7.1.4 Job Results Handling Attributes	43

7.1.5 Job Event Handling Attributes	44

7.1.6 Job Scheduling Instructions Attributes	45

7.2 Document Attributes	47

7.2.1 Document Description Attributes	47

7.2.2 Document Production Instruction Attributes	48

7.2.3 Document Characteristics Attributes	52

7.2.4 Document Status Attributes	54

7.3 Operation Attributes	54

7.4 Printer Attributes	54

7.4.1 printer-name	56

7.4.2 printer-state	56

7.4.3 message	56

7.4.4 printer-initial-value-job	56

7.4.5 printer-initial-value-document	57

7.4.6 fonts-supported	58

7.4.7 fonts-ready	58

7.4.8 media-supported	58

7.4.9 media-ready	58

7.4.10 printer-associated-printers	58

7.4.11 document-formats-supported	58

7.4.12 numbers-up-supported	59

7.4.13 finishings-supported	59

7.4.14 sides-supported	59

7.4.15 job-sheets-supported	59

7.4.16 document-sheets-supported	60

7.4.17 maximum-copies-supported	60

7.4.18 notification-delivery-methods-supported	61

7.4.19 physical-printers-supported	61

7.4.20 Logical-printers-supported	61

7.4.21 events-supported	61

7.4.22 transfer-methods-supported	61

7.4.23 locales-supported	61

7.4.24 multiple-documents-supported	61

7.4.28 cancel-individual-document-supported	62

7.4.29 modify-individual-document-supported	62

7.5 Initial Value Job Attributes	62

7.6 Initial Value Document Attributes	62

7.7 Relationship to ISO/IEC 10175 Conformance Levels	62

8.  Security Considerations	63

9.  References	63

10.  Author's Address	63

1. Introduction

This document is a Proposed Internet-Draft. It is a specification for a protocol that can be used for distributed printing on the Internet.  This protocol, Internet Printing Protocol (IPP)  is heavily influence by the printing model introduced in the Document Printing Application (ISO/IEC 10175 DPA) standard.  The DPA model describes a distributed printing service made up of cooperating networked entities.  DPA also identifies user and administrative roles and operations.  These ideas and concepts, when unified with other Internet protocols and services, realizes a distributed print service for the Internet.

This document assumes a distributed computing environment where print service users (clients, applications, drivers, etc.) cooperate and interact with print service providers (servers, printers, gateways, etc.) to realize the print service.

The actual protocol is yet HTTP based [still to be ratified by the IPP group].

2.	Distributed Printing

The distributed printing service is defined as a collection of coordinating and cooperating entities in a distributed computing environment. The model assumed by this protocol is potentially an n-tier client/server model, but the model will be optimized for the normal cases of a 1-tier model(client to printer) and a 2-tier (client to server to printer). Users of the 1-tier and 2-tier models should not be aware of any extra complexity to support 3 or more tiers

A client is able to access the services offered by a server by invoking one or more operations associated with the server. Each operation has associated arguments and results.  The arguments provide additional data which is passed from the client to the server.  The results return the status and outcome of the desired operation back to the client from the server.

2.1 Components

In the distributed printing service the entities or components are:

-	One or more humans or agents acting on behalf of humans.  Humans (or their agents) act in the role of Users, Operators, Managers, or Administrators.

-	One or more clients.  Clients are computer network nodes with which end users interact in order to manipulate the distributed print service.  A client implements the IPP protocol.

-	One or more print service providers (servers).  An instance of a print service provider implements the IPP protocol by receiving and responding to IPP operations.  A print service provider is also a “client” of yet another instance of a print service provider. Some of these clients use the IPP protocol; others use some other protocol. The last print service provider in the n-tier chain is a “client” to a print engine. There are several different types of print service providers which are defined later in this document.

This LDPA specification only defines the operation used by Users.  The operations used by Operators, Managers, and Administrators may be added if there is time.

2.2 Objects

To accomplish the action(s) requested via an operation, the print service provider manages and manipulates data objects.  These are simply convenient collections of data that may represent other objects (real life or computer system) elsewhere.  A client supplies arguments in the form of attribute values for some of these objects.  A server informs the client of the status or outcome of an operation by also providing attribute values for the objects involved in the operations.  These objects are not encapsulations of both data and behavior as in other object oriented models, but are simple collections of attribute/value pairs. [We may try to fix this in our new design, but it’s not high priority.]

The objects which are relevant to this protocol are:

- Printer (contains server, queue and printer concepts)

- Job  (contains job and document concepts, a document object may

be added in the future)

- Job Template (contains Initial Value Job and Initial Value

Document)

IPP defines the operations that interact with and affect the real-life objects represented by the protocol’s object definition.

2.2.1 Printer

[Note: it is not clear when ‘Printer’ refers to the hardware and when it refers to a software printer object.]

This document shall use the following terms:

- 	Output Device: printer hardware,

- 	Print Server: a program that augments one or more Output Devices.

-	Printer: the software realization of a printer implemented in an Output Device or a Printer Server.

One of the most significant components within the distributed printing service is a Printer. A Printer object is a composition of some of the functionality that has traditionally been tied to other components within the printing system. A Printer can support the functionality of spooling, job management, device management, server, as well as more traditional device components.

A Printer can be in one of two authorization modes:

Public Access: The Printer is not restricted with any access control checks. The authorization allows anyone. The Printer uses a simple, name only (no password or credential) form of binding. [What does this last sentence mean?]

ISSUE: does Public access really mean no authentication. I

would expect not. Otherwise, a person can cancel anyone’s jobs.

Controlled Access: The Printer may have some restrictions based on some authentication and authorization scheme.  The Printer uses some form of credential based binding. . [What does this last sentence mean?]

A Printer object represents an instance of a print service provider which implements the IPP protocol.  This allows the Printer to provide a common interface for all types of disparate and diverse physical devices or as well as a gateway interface for other non-Internet based printing systems.

To a print service user, a Printer has the “looks and feel” of a any typical physical printer.   Jobs are submitted to and managed at the Printer.  The Printer can accept or reject submitted jobs based on job attributes which are sent along with the print job.  The Printer tracks all jobs that have been submitted to it.  The Printer can be modified to indicated a corresponding behavior change at the device level (either manually or automatically).  In the Controlled Access mode, the Printer has an identity with a security or credential service.

An object that an end-user views as a Printer can be implemented with either of the following configurations.

-	an Output Device which supports the IPP protocol. This Output Device may or may not have a job queue, and if it has a job queue it may be of very limited size. An administrator configures this Printer to receive jobs directly from a client.

-	a Print Server and one or more downstream Output Devices. The Print Server supports incoming IPP protocol and uses either IPP or some other protocol to communicate with downstream Output Devices. A Print Server augments downstream Output Devices by supporting a large job queue. An administrator configures the downstream Output Devices to receive jobs from the Print Server. When there is only one downstream Output Device, the Printer object in the Print Server has the same values as those in the downstream Output Device. Note that if the downstream Output Device supports the IPP protocol, then its Printer Object and the Print Server’s Printer Object are identical, attribute for attribute. When there are two or more downstream Output Devices, the Printer object in the Print Server has the union of values in the downstream Output Devices.

The print system shall also support gateways. Such gateways shall translate incoming IPP protocol to some other protocol and shall translate some other protocol to IPP protocol. Gateways shall pass operations through with minimal delay. Further description of them is beyond the scope of this document.

ISSUE: where does file conversion occur when there is no associated application, e.g. text or HTML to PostScript. To keep the model simple, the Printer defined above should only queue and print jobs. Conversion of text, HTML  should occur within some other server which forwards the job to a Printer.

The following describes the allowed chains of printer types using RFC 822 syntax:

normal usage:

ipp-end-user [PrintServer] OutputDevice

outbound gateway:

ipp-end-user Gateway AnotherPrintSystem

inbound gateway:

AnotherPrintSystem Gateway [PrintServer] OutputDevice

Although other arrangements of these printer types is possible, they are not useful.

0 Printer Configurations

The following questions may arise:

1. How can I submit a job to a queue and have that job be routed to any available physical printer?

In the

2. How can I achieve both “fan out” and “fan in” of client to Printers and from Printers to physical print devices?

For “fan in”, an administrator or a user creates a name space entry which maps a name to a printer object and a job template.  This job template provides the default values to a client when a client creates a job object. In this model, the client does the work of defaulting, or alternatively, the server (possibly the Printer receiving a URL request) returns a HTML form with potential values and default values all filled in. The print service adds no additional default values for unspecified attributes, but an Output Device takes its own default action for unspecified attributes.

This model currently contains a mechanism for defaulting in two places: during GUI initialization in the client and during printing of the job in the Output Device. There may be a need to associated defaulting (i.e. a job template) with a Printer Object so that an administrator can enforce value that are left undefined.  This is currently an open issue that will not be resolved for version 1.0.

For “fan out”, an administrator sets up a Print Server to fan out to one or more downstream Output Devices  for load balancing and reliability.

3. How can a Printer be configured to service multiple spooling  Printers?

The system administrator establishes whether an end-user has direct access to an Output Device or indirect access via a Print Server. If multiple  Print Servers  are used to feed the same Output Device , their schedulers are not coordinated. Although this configuration is possible, it is not recommended.

2.2.2 Job

A Job object is used to model a job.  A job can consist of one document.  There are certain job attributes that pertain to the running and scheduling of the entire job (all documents).  There are other job attributes that define global behavior or defaults for all contained documents.

Still other job attributes pertain to each document within a job. There are no separate document objects, no attributes that pertain to one document in a job but not to others, except for the attribute that specifies the location of the documents.

In future versions, jobs will be able to contain more than one document, and documents will become separate object with attributes that override corresponding job attributes.

2.2.4 Job Template

An Job Template object is used to model job defaults.  These are essentially job attributes that a client uses to initialize a newly created job object.

2.3 Object Relationships

Instances of objects within the system have relationships which must be maintained persistently along with the persistent storage of the objects themselves.

An instance of a print service provider is a Printer.  The Printer is represented via a Printer object.  A Printer can contain zero, one, or more Job objects.  A Job object contains one or more Documents. The following relationships are examples:

•	“Document object D1 belongs to Job object J1”, or

•	“Job object J1 belongs to Printer object P1”.

2.4 Use of Naming and Directory Services

[ There is more work that need to be done to define the name service in an HTTP context. A name is a URL and is probably resolved at Print Servers and Output Devices.]

Any distributed service uses some sort of naming and/or directory service for (X.500, NDS, DCE naming, DNS).  It is outside the scope of this protocol to define which name service to use or what the protocol is for using that name service, but the following discussion helps to clarify how the name service is intended to be used.

To distributed printing system, the instances of print service providers are represented by objects of type Printer.  These same instances are also registered to the name/directory service.  There is one entry in the name service for each Printer.

That is, instances of print service providers are represented to IPP as Printer objects.  These objects represent their real-life counterparts, the print service provider (software, hardware, or firmware). However, for directory lookup, there is an entry in the naming service that also represents the Printer.

It is important to remember that a Printer object represents the current status and configuration information of a certain print service provider.  The Printer object contains attributes and values that describe the characteristics and capabilities of the print device.  However, a few of the most important attributes from the Printer object are duplicated in the entry in the directory.  These attributes are used for filtered directory lookups. The results of these searches enable a user to select an appropriate printer. It is the responsibility of the Printer itself to keep these attributes consistent and accurate.  This requirement frees the directory or some directory agent from continually polling registered entities for configuration changes.

The following attributes are in the directory entry:

Fully Distinguished Name (the name within the directory’s

name space)

Description

Location

Owner

Address

Status

Resolution

Color Supported

Maximum Speed

Maximum Speed Units

Device Id

Model

Manufacturer

Type

PDLs Supported

Sides Supported

The final set of name service entry attributes needs to be finalized and rationalized with the PSIS name service recommendations [6] as well as implementation experience.

2.4.1 Status

[ Such a dynamic value seem like it could be a problem in some name service entries.]

The printer status field in the directory entry is really a “summary” attribute of the true printer state.  The following mapping takes place between the Printer Status attribute in the directory entry and the printer-state attribute in the Printer object:

“Not Connected”

STATE_NOT_CONNECTED

STATE_PAUSED_NOT_CONNECTED

“Shutdown”

STATE_SHUTDOWN

“Active”

STATE_IDLE

STATE_PAUSED

STATE_PRINTING

“Stopped”

STATE_STOPPED

STATE_PAUSED_STOPPED

Even though the Printer  may not be up and running, the directory entry still exists in the directory.  In this case, the directory entry represents the fact that it may begin running at some future time.

2.4.2 Resolution

This is a single valued, maximum resolution in either the horizontal or vertical direction of the print device in dpi.

2.4.4 Color Supported

This is a BOOLEAN for either yes, color printing is supported, or no color printing is not supported.

2.4.5 Maximum Speed

This is the maximum speed of the printer in the units defined in Maximum Speed Units

2.4.6 Maximum Speed Units

This is the units of the maximum speed rating of the print device.  This can be: pages per minute, sheets per minutes, characters per second, etc.

2.4.7 Plug and Play Device Id

This attribute can be used for automatic driver download and other automatic configuration tasks.

2.4.8 Model

This is a simple text string defined by the manufacturer.

2.4.9 Manufacturer

This is a simple text string defined by the manufacturer.  There is no registration, and there is a possibility of overlap, but the goal is to keep this simple, not too complex.

2.4.10 Type

This is the printing mechanism of the print device: laser, ink jet, thermal, etc.

2.4.11 PDLs Supported

This is a list of all of the page description languages (PDLs) that the printer and/or its interpreter(s) support.

2.4.12 Sides Supported

This is either a 1 or a 2 to indicate the maximum number of sides on which the printer can automatically print.

2.5 OIDs

[OIDs do not belong in this model until we establish the protocol]

3.	Internet Printing Model

3.1 Object Instances

All instances of all objects have an identifier attribute that makes them unique so that they can be unambiguously referenced.  In the object-oriented model, these are the globally unique object references which are created by factories or constructors.

The following objects have the following mandatory identifier attributes:

Object				Identifier						Containing Object

Printer				printer-name					None

Job					job-identifier					Printer

Job Template		job-template-name				None

3.2 Limits and Defaults

This IPP specification does not include any mechanism for specifying for enforcing “limits” or any other kinds constraints.  However, defaults are achieved through the implementation of Job Template.

3.3

-

-

-

-

-

4. Operations

IPP defines the following end user operations:

The following symbols are used in the tables below:

P		perform the operation directly

PF		perform the operation; forward to Output Device sometimes

UA		unsupported in an Output Device unless it supports queuing

U		unsupported operation

| Operation | Print Server | Output Device |
| --- | --- | --- |
| - Print | PF | P |
| - Cancel Job | PF | P |
| - Get Attributes | PF | P |
| - |  |  |
| - Get Jobs | PF | P |

Lower priority (version 2) end user operations are:

| Operation | Print Server | Output Device |
| --- | --- | --- |
| - Modify Job | P | UA |
| - Resubmit Job | P | UA |

Management operations are (we want these in version 1.0):

| Operation | Print Server | Output Device |
| --- | --- | --- |
| - Clean Queue | PF | UA |
| - Disallow Queuing | P | UA |
| - Allow Queuing | P | UA |
| - Pause Printing | P | P |
| - Resume Printing | P | P |
| - Promote Job | PF | UA |
| - Shutdown Printer | P | P |
| - Startup Printer | P | P |
| - Create Printer | P | U |
| - Delete Printer | P | U |
| - Set Attribute | P | P |
| - Get Local Attributes | P | P |

4.1 Common Data Structures

This section describes the common data structures that are used by two or more operations.

-

-

------------

------------

--------------------

------------

--

--

--

--

----

----

---

----

-

-	-----

----

----

---

----

-

-

-

-

-

-

-

----

--

----

-

----

----

----

----

----

----

-

--

--

--

--

-

-

-

-

--

--

-

-

--

-

---

--

--

----

---

--

--

----

-

-

-

-

--

-

-

-

---

-

-

-

-

-

--

---

-

-

-

-

-

-

--

-

-

-

-

-

-

--

-

-								-

-

-

-

--

--

--

----

---

--

--

---

--

-

--

--

-

-

-

-

-

--

--

--

--

-

-

-

--

----

--

---

--

----

-

-

----

-

-

-

-

-

--

6. User Operations

6.1 Print Operation

When an end-user uses GUI to submit a job, the GUI client gets an HTML form from the default printer.  If the end-user changes the selected printer, the GUI client gets the HTML form from that printer. The HTML form comes with the values supported by the printer and it is initialized by the values from the job template associated with the named printer.

[Further work needs to done to define the above concept.]

6.1.1  Print Argument

[We should be trying to create a protocol where the entire job is incorporated into a single transmission.  This eliminates the need for Add Document and Close Job.]

The following abstract data types are part of the Print Argument:

|  | . |
| --- | --- |
| Create Job | the Job Id is returned in the Print Results. |
| Printer Name |  |
| Job and DocumentAttributes |  |
| All Document Contents |  |
|  | . |
| Common Arguments | Common to all three forms of Print Argument |

-

-----

-

-

----

-

--

---

---

----

---

---

---

-

-

--

-

--

-----

------

---

-

-

-

-

--

-

-

-

-

-

6.1.2  Print Result

The following abstract data types are part of the Print Result:

| Job Id | Used for all other operations on this Job. |
| --- | --- |
| Server State | Optional state information about the Print Service Provider |
| Message | Optional message |
| Job Status | Job state information |
| Errors | Optional Error Information |

-

-----			-

-----

-

----

-

-----

------

-----

-----

---

-

---

---

6.2 Cancel Job Operation

6.2.1  Cancel Job Argument

The following abstract data types are part of the Cancel Job Argument:

|  | . |
| --- | --- |
| Job Id | The identifier of the job to be canceled. |
| Document Number | Optional document number of the document to cancel within a given job. [ probably not supported] |
| Message | Optional message to the operator. |
| Retention Period | Optional period for retaining the cancelled job. |
| Common Arguments |  |

-

-

--

---

-

-------

-

-

Result

The following abstract data types are part of the Cancel Job Result:

| Job Status | Optional Job status information |
| --- | --- |
| Errors | Optional Error Information |

----

6.3 Get

6.3.1  Get  Attributes Argument

The following abstract data types are part of the Get Attributes  Argument:

|  | . |
| --- | --- |
| SPECIFICATION |  |
| Selector | A job or printer name URL (the class is implicit in the object named) |
| Requested Attributes | A set of attributes in which the requestor is interested |
|  | [ I would like to get rid of this if possible] |
| Common Arguments |  |

6.3.1  Get Jobs Argument

The following abstract data types are part of the Get Jobs  Argument:

| Selector | A printer name |
| --- | --- |
| Filtering | A lightweight filtering mechanism, such as all jobs versus a particular user’s jobs. |
| Requested Attributes | A set of job attributes in which the requestor is interested |
| Common Arguments |  |

-

---

-----

-

-

-

---

-

---

-

---

--

--

--

--

--

--

--

-

--

-

--

---

----

----

-

---

-----

---

-

----

-

-

-

-

-

-

-

-

--

-

-

-

--

-

--

-

----

--

--

---

----

-

-

-

-

6.3.2  Get Attributes and Get Jobs Result

The following abstract data types are part of the List Object Attributes Result:

| Result Attributes | Attribute set containing the returned results. |
| --- | --- |
| Errors | Optional Error Information |

---

-

-

-

-

-

----

-

-

-

6.4 Modify Job Operation

6.4.1  Modify Job Argument

The following abstract data types are part of the Modify Job Argument (the attributes that can be modified may be severely restricted):

| Job Id | Which job to modify. |
| --- | --- |
|  | [There are no document attributes to modify.] |
| Job Attributes | Attribute set for Job attributes. Only replacement is possible; the GUI fetches the value and then sets a new one. |
| Message | Optional Message. |
| Common Arguments |  |

-

--

-----

--------

------

------

----

---

---

--

--------

------

---

-

-----

----

-

-

-

--

-

-

--

---

--

-

-------

-

6.4.2  Modify Job Result

The following abstract data types are part of the Modify Job Result:

| Modify Status | Modify result attributes. |
| --- | --- |
| Errors | Optional Error Information |

----

6.5 Resubmit Job Operation

6.5.1 Resubmit Job Argument

The following abstract data types are part of the Resubmit Argument:

|  | . |
| --- | --- |
| Destination Printer Name | Optional name of the destination printer. |
| Operation | MOVE or COPY |
| Job Set | A set of jobs to move or copy. Each entry in the set has: Job Id, Document Number, Job attributes, and Document attributes. |
| Message | Optional Message |
| Common Arguments |  |

-

------

----

-

-

6.5.2 Resubmit Job Result

The following abstract data types are part of the Resubmit Job Result:

| Resubmit Job Set | A set of jobs that were resubmitted. Each element in the set has: Old Job Id, New Job Id, and an attribute set with info about the results of the move or copy. |
| --- | --- |
| Errors | Optional Error Information |

-

-----

--

----

--

-

---

--

7. Object Attributes

This section describes the attributes and their associated values that are part of the LDPA protocol. The list below shows the objects and their attributes that are included within the scope of this protocol:

Job Attributes

Job Informational Attributes (set by client)

job-originator (an authenticated value)

job-name

job-originating-host

Job Informational Attributes (set by Printer)

job-identifier

job-identifier-on-output-device (used by operator)

Printer Selection Attributes (set by client)

printer-name-requested

output-device-requested

Job Status Attributes (set by Printer)

current-job-state

printer-assigned  [ let’s keep it simple ]

submission-time

job-message-from-administrator

completion-time

job-state-reasons

impressions-completed

media-sheets-completed

Job sheet  Attributes (set by client)

job-sheets

Job Event Handling Attributes (set by client)

notification-profile (two classes of events, delivery

methods other than email are a problem with internet)

Job Scheduling Instructions Attributes (set by client)

job-hold

job-priority

Job-print-off-peak

job-retention-period

Document Description Attribute

document-format

Document Production Instruction Attributes (set by client)

document-format

medium-select

number-up

finishing

sides

copies

printer-resolution-select

print-quality

page-select

Attributes for Conversion of Text Files (set by client)

width

length

left-margin

right-margin

top-margin

bottom-margin

repeated-tab-stops

header-text

footer-text

number-pages

default-font

default-character-set

content-orientation

Job Resource  Attributes (set by

process which produces PDL file; for use in scheduling)

document-format-used

fonts-used

character-sets-used

media-used

sides-used

print-quality-used

finishing-used

printer-resolution-used

total-job-octets

job-impression-count

job-media-sheet-count

Document Contents (one per document)

number-of-documents

document-content (actual contents or a path reference)

Operation Attributes

operation-locale

default-delivery-addresses

Printer Attributes (Print Servers and Output Devices)

printer-name

printer-location

printer-model

printer-types

printer-state

printer-state-message

message

notification-profile

access-control-list

fonts-supported

font-substitutions

media-supported

document-formats-supported

numbers-up-supported

finishings-supported

sides-supported

print-qualities-supported

maximum-printer-speed

printer-resolutions-supported

delivery-methods-supported

character-sets-supported

job-sheets-supported

maximum-copies-supported

maximum-job-octets

maximum-job-retention-period

maximum-job-priority

maximum-impressions

maximum-media-sheets

off-peak-times

notification-delivery-methods-supported

downstream-printers

events-supported

locales-supported

locale

sheet-count

printer-timeout-period

Job Template (attributes from the following sections of Job)

Job sheet  Attributes

Job Event Handling Attributes

Job Scheduling Instructions Attributes

Document Production Instruction Attributes

Attributes for Conversion of Text Files

In the following sections, most of the text has been taken word for word from ISO/IEC 10175 DPA (Final, June 1996).

7.1 Job Attributes

A job object contains a set of job attributes and one or more document.  The server shall create a printable job object in response to a client that invokes one or more Print abstract-operations.  A client shall use a job template associated with the selected printer in order to initialize the job. ----

--

There is a table for each attribute that shows its: name, syntax, multi or single valuedness (S or M), and any relevant notes.

7.1.1 Job Informational Attributes

These attributes provide information to identify a print-job.

The client may specify job-information attributes in:

a)	Print:  all, except id-att-job-identifier

b)	ModifyJob: all, except id-att-job-identifier, id-att-job-owner

c)	ListObjectAttributes:  all

7.1.1.1 job-identifier

| job-identifier | jobIdentifierSyntax | S |  |
| --- | --- | --- | --- |

This attribute provides the job-identifier for this job on the server.  The server shall generate a job-identifier value that is unique on that server, but need not be unique across the distributed environment.

The value of the job-identifier attribute shall be returned by the server as part of the PrintResult in the first Print abstract-operation for the job.  The client shall pass its value as part of the argument in subsequent abstract-operations for the same job.

7.1.1.1 job-identifier-on-output-device

This attribute holds the job-identifier assigned by an Output Device. It provides a way for a Print Server to relates its copy of a job with a job on the Output Device.

7.1.1.2 job-originator

| job-originator | Text | S |  |
| --- | --- | --- | --- |

This attribute specifies the name of the person submitting the print job.  The Print Server or Output Device shall set this attribute to the most authentic name that it can obtain from the client.

[ We may choose to call this job-owner, but it should still be the most authenticated name of the user ]

---

-

----

-					-----

----

----				-

----				-

----

----

-				-----

-		-----

----

----

----

----

----			-

----

----

-				-----

----

----

----

----

----

----

----

--

-

----

---

7.1.1.3 TBD

7.1.1.4 job-name

| job-name | simpleNameSyntax | S |  |
| --- | --- | --- | --- |

This attribute supplies a human readable string for the print-job. This string is used for naming the print-job in human-readable "free-form" fashion.

This attribute is intended for enabling a user or the user's application to convey a job name that may be printed on a start sheet, returned in a Get-Attributes  result, or used in notification or logging messages.

If the client does not specify this attribute, A Print Server or Output Device shall set it to the name of the file of the first document in the job. -

- job-originating-host

The client sets this attribute to contain the host name from which the job is submitted.

- number-of-documents

The client sets this attribute to be the number of document being submitted.

7.1.2 Printer Selection Attributes

These attributes provide information to help select a particular printer.  If more than one printer-selection attribute is specified, the server shall select a printer that meets all of the criteria.

The client may specify printer-selection attributes in:

a)	Print:  all, except the value of printer-name-requested (which shall be passed as an explicit parameter of the first PrintArgument, rather than as an attribute)

b)	ModifyJob:  all,  except printer-name-requested

c)	ListObjectAttributes:  all

7.1.2.1 printer-name-requested

| printer-name-requested | simpleNameSyntax | S |  |
| --- | --- | --- | --- |

This attribute identifies the printer to be used for printing the job.  ---

-----

7.1.2.1 output-device-requested

| output-device-requested | simpleNameSyntax | S |  |
| --- | --- | --- | --- |

This attribute identifies the Output Device to be used for printing the job. This attribute has significance when the printer-name-requested references a Print Server and that Print Server has two or more downstream Output Devices.

7.1.3 Job Status Attributes

These attributes specify the job status before, during and after the processing of the print-job by the server.  The server shall create the job object with these attributes (if implemented) and shall assign appropriate values to each such job-status attribute.

The client may specify job-status attributes in:

a)	Print:  none

b)	ModifyJob:  none

c)	GetAttributes and GetJobs:  all

7.1.3.1 current-job-state

| current-job-state | objectIdentifierSyntax | S |  |
| --- | --- | --- | --- |

This attribute identifies the current state of the job (pending, printing, held, etc.).

The following job state standard values are defined: id-val-job-state-unknown, id-val-job-state-pre-processing, id-val-job-state-held, id-val-job-state-pending, id-val-job-state-processing, --------id-val-job-state-terminating, id-val-job-state-retained, id-val-job-state-completed

The IPP protocol supports all values for job states, but printers are not required to generate all job states, only those which are appropriate for the particular implementation.

------

7.1.3.2 printer-assigned

| printer-assigned | simpleNameSeqSyntax | S |  |
| --- | --- | --- | --- |

This attribute identifies the Output Device to which this job has been assigned, if any.

When the job is first submitted and the printer has not yet assigned any printers to the job, the value shall be empty.

If the Print Server  has assigned a printer to the job, the value  shall contain the assigned Output Device .

--

An empty value shall be returned if this attribute is supported, but this job has not yet been assigned to any Output Devices .

----

-----

The printers-assigned value shall not be the same as the printer requested by the user.-----

The value of the job's printers-assigned attribute shall remain after the job has completed, so that users can determine the Output Device on which the job was printed.

7.1.3.3 submission-time

| submission-time | TimeSyntax | S |  |
| --- | --- | --- | --- |

This attribute indicates the time at which the this job was accepted by the printer.  If the printer does not support the notion of time, the attribute is not stored as part of the job object.

--

--

---

--

----

--

-

7.1.3.5 job-message-from-administrator

| job-message-from-administrator | simpleNameSyntax | S |  |
| --- | --- | --- | --- |

This attribute provides a message from an operator, system administrator or ‘intelligent' process to indicate to the user the reasons for modification or other management action taken on a job.

7.1.3.6 completion-time

| completion-time | TimeSyntax | S |  |
| --- | --- | --- | --- |

This attribute indicates the time at which this job completed.  Providing this time is useful for jobs which are retained after printing.

7.1.3.7 job-state-reasons

| job-state-reasons | enum | M |  |
| --- | --- | --- | --- |

This attribute identifies the reason or reasons that the job is in the state that it is in (e.g., held, terminating, retained, completed, etc.).  The printer shall indicate the particular reason(s) by setting the value of the job-state-reasons attribute.  It is valid for the printer to set the value of the job-state-reasons attribute to the empty set.

The following standard values are defined: id-val-reasons-documents-needed, id-val-reasons-job-hold-set, id-val-reasons-job-print-after-specified, id-val-reasons-required-resources-not-ready, id-val-reasons-successful completion, id-val-reasons-completed-with-warnings, id-val-reasons-completed-with-errors, id-val-reasons-cancelled-by-user, id-val-reasons-cancelled-by-operator, id-val-reasons-aborted-by-system, id-val-reasons-logfile-pending, id-val-reasons-logfile-transferring

7.1.3.8 number-of-documents

| number-of-documents | cardinalSyntax | S |  |
| --- | --- | --- | --- |

This attribute indicates the number of documents in the job. ----

-

----

| -- |  |  |  |
| --- | --- | --- | --- |

--

7.1.3.9 impressions-completed

| impressions-completed | cardinalSyntax | S |  |
| --- | --- | --- | --- |

This attribute contains the number of impressions completed by the Output Device.

7.1.3.9 media-sheets-completed

| media-sheets-completed | cardinalSyntax | S |  |
| --- | --- | --- | --- |

This attribute contains the number of media-sheets completed by the Output Device

7.1.4 Job sheet  Attributes

These attributes specify the actions to be undertaken for printing of job sheets. .

The client may specify job-results-handling attributes in:

a)	Print:  all

b)	ModifyJob:  all

c)	ListObjectAttributes:  all

7.1.4.1 job-sheets

| job-sheets | enum | S |  |
| --- | --- | --- | --- |

This attribute determines what type of job-sheets the Output Device should print with the job.  The possible enum values are: none, default and custom. ‘None’ means that no job sheets should be printed.  ‘Default’ means that the job sheets defined by an administrator should be used. ‘Custom’ means that additional attributes, not defined in this document, determined whether start sheets, end sheets or slip sheets should be in the document

--

---

------

-

7.1.5 Job Event Handling Attributes

7.1.5.1 notification-profile

| notification-profile | eventHandlingProfileSyntax | M |  |
| --- | --- | --- | --- |

This attribute is a specification of events about which the user and/or designate are to be notified. In addition, this attribute specifies how the event notifications are to be delivered.

This attribute will support three events classes: job-completion only, job-problems-only, and job-completion-and-problems.

This attribute will support only one delivery method, namely email.  This method of notification is quite deficient for timely notification to an end-user who receives a lot of email, but there are no other choices.  The internet community needs to solve this problem, perhaps with an extremely-urgent email.

7.1.6 Job Scheduling Instructions Attributes

These attributes provide additional hints for the scheduling of a print-job. How a print-service uses this information in scheduling jobs is implementation-specific.

The client may specify job-scheduling-instruction attributes in:

a)	Print:  all

b)	ModifyJob:  all

c)	ListObjectAttributes:  all

7.1.6.1 job-hold

| job-hold | booleanSyntax | S |  |
| --- | --- | --- | --- |

This attribute specifies whether the print-job is a candidate for scheduling for printing or not, when the server would otherwise place the job in the pending or processing states

When the value is FALSE, the printer shall not hold the job from being scheduled for printing, unless there are other reasons (see the current-job-state and the job-state-reasons job-status attributes).

When the value is TRUE, the printer shall place the job in the held state and add the job-hold-set value to the job's job-state-reasons attribute and shall not schedule the print-job for printing.  If the job enters the held state because its job-hold attribute was TRUE, a client shall reset the job's job-hold attribute to FALSE by means of the ModifyJob operation before the printer can schedule the job for printing.  When the value is set to FALSE as a result of the ModifyJob operation, the printer shall remove the job-hold-set value from the job-state-reasons attribute and, if no other reasons remain, shall change the job's current-job-state to pending so that the job becomes a candidate for being scheduled on printer(s).

7.1.6.2 job-priority

| priority | prioritySyntax | S |  |
| --- | --- | --- | --- |

This attribute specifies a priority for scheduling the print-job. It is used by servers that employ a priority-based scheduling algorithm.

A higher value specifies a higher priority. The value 1 is defined to indicate the lowest possible priority (a job which a priority-based scheduling algorithm shall pass over in favour of higher priority jobs). The value 100 is defined to indicate the highest possible priority. Priority is expected to be evenly or ‘normally' distributed across this range. The mapping of vendor-defined priority over this range is implementation-specific. The omission of this attribute implies that the user places no constraints concerning priority on the scheduling of the print-job.

An operator can modify a job to have any priority. An end-user is restricted to the value of printer attribute maximum-priority-end-user.

7.1.6.3 job-print-off-peak

| job-print-after | enumSyntax | S |  |
| --- | --- | --- | --- |

This attribute specifies whether or not a job should print during off-peak hours. If this attribute is present, it contains a value with which an administrator has associated allowable print times.

-

---------------

--

7.1.6.4 job-retention-period

| job-retention-period | deltaTimeSyntax | S |  |
| --- | --- | --- | --- |

Delta time provides an integer value for a period of elapsed time, measured in seconds.

This attribute specifies the minimum period of time following the completion of job processing and printing that the server shall keep job attributes, document attributes, and document data.  The server may keep these attributes and data longer than the value of the job-retention-period attribute.

Job-retention-period specifies a lower bound on how long job attributes, document attributes and document data shall be retained by a server after printing has completed, whilst job-discard-time sets an upper bound on retention of the job and document attributes independent of whether the job is ever scheduled for, starts or completes printing.

In addition to providing status information to a user after a job has completed printing, the job-retention-period also provides the mechanism for retaining job's document data after it has been printed, so that the job may be printed again, possibly with modified attributes, such as the job-copies component of the job-results attribute.

NOTE - The mechanism to reprint the job is outside the scope of this part of ISO/IEC 10175; part 3 of this International Standard (in preparation) includes a Resubmit abstract-operation to enable this function.

7.2 Document Attributes

This section discusses attribute that pertain to documents,  including the document-content attribute which specifies the document data. --------

--

7.2.1 Document Description Attributes

These attributes identify the intended document format.-

The client may specify document-description attributes in:

a)	Print:  all----

b)	ModifyJob:  none------

c)	ListObjectAttributes:  all---

7.2.1.1 document-format

| document-format | docFormatSyntax | S |  |
| --- | --- | --- | --- |

This client specifies this attribute if the included documents are not yet in a format acceptable to an Output Device. This attribute specifies the format that the Printer shall translate the documents to in order to print.

Note: this attribute is rarely needed.

-----

-----

----

-----

-----

------

-

------

7.2.2 Document Production Instruction Attributes

These attributes provide information that affect the rendering and finishing of the document and are referred to as document production instructions (DPI).  DPI may also be contained in the document to be printed.

After the information from these attributes has been folded into the document data, they are no longer relevant and can be discarded from a job.  The resource attributes indicate from printer features a document needs in order to print correctly.

If there is a conflict between the value of one of these attributes, and a corresponding parameter found in the document (either implicit or explicit), the value of the attribute shall take precedence over the document parameter.

---

The client may specify document production-instruction attributes in:

a)	Print:  all

b)	ModifyJob:  all

c)	ListObjectAttributes:  all

--

7.2.2.2 medium-select

| -medium-select | nameSyntax | S |  |
| --- | --- | --- | --- |

This attribute identifies the medium that the Printer r shall use for all  pages of the document regardless of what media are specified within the document.

The values for medium include medium-names, medium-sizes, input-trays and electronic forms so that one attribute specifies the media.

Standard values are defined: TBD

--

-

--

--

--

----

--

-----

--

----

--

7.2.2.3 number-up

| number-up | cardinalOrNameOrOidSyntax | S |  |
| --- | --- | --- | --- |

A CardinalSyntax allows attribute values that can specify either a Cardinal or an OID (that normally names a Cardinal).

This attribute specifies the number of source page-images to impose upon a single instance of a selected medium. The attribute can be specified either by a number directly or by naming an imposition object which specifies some particular number-up imposition.

In general, only certain numeric values are valid for this attribute, depending upon the server and printer implementations to which the print-request is directed.  A value of 0 or none shall suppress any server default number up, if any.

This attribute primarily controls the translation, scaling and rotation of page images, but a site may choose to add embellishments, such as borders to each logical page. A site may even choose to add an attribute to control the presence or characteristics of such embellishments.

The following standard values are defined: id-val-generic-none, id-val-imposition-simple-1-up, id-val-imposition-simple-2-up, id-val-imposition-simple-4-up.

NOTE – The value 0 or none specifies that no convenience imposition functions shall be performed; 0 or none is needed to suppress any special number-up operation because a value of 1 for some sites may cause the server to alter the placement, or size of the page image, or to add embellishments, such as borders or to rotate the page depending on content-orientation.

The server may support three values for number-up besides 0 (and id-val-generic-none), namely 1 (and id-val-imposition-1-up), 2 (and id-val-imposition-simple-2-up) and 4 (and id-val-imposition-simple-4-up), which this document will reference by the respective names of 0-up, 1-up, 2-up and 4-up, henceforth.  These 1-up, 2-up and 4-up values provide a simple means for users to request the printing of compact documents of a temporary or informal nature.

7.2.2.4 finishing

| finishing | enum | S |  |
| --- | --- | --- | --- |

This attribute identifies a sequence of one or more finishing-processes to be applied to each copy of the printed document.

Finishing encompasses the operations that may be applied to the media output of a print-job.  Examples include stapling, saddle-stitching, hole-drilling, binding with tape, etc.

This attribute allows the requester to specify one or more individual finishing processes may be specified in the finishing attribute.  Each of the individual processes is specified by including the required parameters for each of the individual finishing processes in the finishing attribute.

Standard values for this attribute are defined: TBD.

7.2.2.5 sides

| sides | enum | S |  |
| --- | --- | --- | --- |

This attribute specifies whether the document should be printed in one of three ways: 1-sided (simplex), 2-sided-flip-on-long-edge (duplex), 2-sided-flip-on-short-edge (tumble),

7.2.2.6 copyies-

| copies- | cardinalSyntax | S |  |
| --- | --- | --- | --- |

This attribute specifies the number of copies of the job to be printed.

A value of 1 for copies- shall generate a single human perceptible copy of the electronic document.

--

--

-

-

7.2.2.1 printer-resolution-select

| printer-resolution-select | cardinalSyntax | S |  |
| --- | --- | --- | --- |

This attribute specifies the resolution that the Printer should use.

7.2.2.1 print-quality

| print-quality | enum | S |  |
| --- | --- | --- | --- |

This attribute specifies the print quality that the Printer should use.

7.2.2.1 page-select

| page-select | integerRange | S |  |
| --- | --- | --- | --- |

This attribute specifies the pages in the document that the Printer should use. This attribute is unlikely to be useful for jobs with more than one document.

7.2.2.1 files-are-one-document (for the future 2 or more documents)

| files-are-one-document | booleanSyntax | S |  |
| --- | --- | --- | --- |

This attribute is relevant only if a job consists of two or more files. It controls finishing operations, and job-sheet placement.

If the files for the job are a and b  and this attribute is true, then files a and b are treated as a single document for finishing operations. Also, there will be no slip sheets between files a and b. If more than one copy is made, the ordering must be  a, b, a, b, …. The attribute files-are-interleaved is ignored.

If the files for the job are a and b  and this attribute is false or unspecified, then each file is treated as a single document for finishing operations. Also, a client may specify that a slip sheet be between files a and b. If more than one copy is made, and the attribute files-are-interleaved false or unspecified, the ordering is a, a, b, b, …. If more than one copy is made, and the attribute files-are-interleaved true, the ordering is a, b, a, b, ….

7.2.2.1 files-are-interleaved (for the future 2 or more documents)

| files-are-interleaved | booleanSyntax | S |  |
| --- | --- | --- | --- |

This attribute is used in conjunction with files-are-one-document (q.v.).

7.2.2 Attributes for Conversion of Text Files (set by client)

The attributes in this section specify formatting for text documents. If any of these attributes is not specified, a Printer shall uses its own defaults.

7.2.2.1 width

| width | cardinalSyntax | S |  |
| --- | --- | --- | --- |

This attribute specifies the media width for the document.

7.2.2.1 length

| length | cardinalSyntax | S |  |
| --- | --- | --- | --- |

This attribute specifies the media length for the document.

7.2.2.1 left-margin

| left-margin | cardinalSyntax | S |  |
| --- | --- | --- | --- |

This attribute specifies the left-margin for the document.

7.2.2.1 right-margin

| right-margin | cardinalSyntax | S |  |
| --- | --- | --- | --- |

This attribute specifies the right-margin for the document.

7.2.2.1 top-margin

| top-margin | cardinalSyntax | S |  |
| --- | --- | --- | --- |

This attribute specifies the top-margin for the document.

7.2.2.1 bottom-margin

| bottom-margin | cardinalSyntax | S |  |
| --- | --- | --- | --- |

This attribute specifies the bottom-margin for the document.

7.2.2.1 repeated-tab-stops

| repeated-tab-stops | cardinalSyntax | S |  |
| --- | --- | --- | --- |

This attribute specifies the tab stops for the document.

7.2.2.1 header-text

| header-text | Text | S |  |
| --- | --- | --- | --- |

This attribute specifies the header text for the document.

7.2.2.1 footer-text

| footer-text | Text | S |  |
| --- | --- | --- | --- |

This attribute specifies the footer text for the document.

7.2.2.1 number-pages

| number-pages | booleanSyntax | S |  |
| --- | --- | --- | --- |

This attribute specifies that the pages should be numbered in the document.

7.2.2.1 default-font

| default-font | nameSyntax | S |  |
| --- | --- | --- | --- |

This attribute specifies the font to use for all text in the document.

7.2.2.1 default-character-set

| default-character-set | enum | S |  |
| --- | --- | --- | --- |

This attribute specifies the code-set in which the document is encoded.

7.2.2.1 content-orientation

| content-orientation | enum | S |  |
| --- | --- | --- | --- |

This attribute specifies the orientation of the document: landscape or portrait.

7.2.3 Job Resource Attributes

This group of attributes describes the resources needed to print the job.

The values provided by these attributes are intended to assist the print-server in validating and scheduling the print-job. Providing these attributes independent of the document allows the server to schedule a job or to validate the resources required to print the document without interpreting the contents of the document. This provides the opportunity for a server to support a broad set of document formats yet still support fast efficient scheduling and validation of each job.  --

The values of these attributes are hints to the server about production instructions and resources needed to print a document, but the printer does not use these attributes during the actual printing of a document.  It is the duty of the process that translates the document to the printer’s PDL to provide these values. If such values are lacking, the Printer shall assume that the document doesn’t ask for any resources that are unavailable. Such value may be missing if the translation process fails to provides such values, or if no translation occurs (e.g. the document is a PostScript document.

---

-

The client may specify document-characteristic attributes in:

a)	Print:  all (translation process, not the end-user)

b)	ModifyJob:  none

c)	ListObjectAttributes:  all

7.2.3.1 document-format-used

| document-format-used | docFormatSyntax | S |  |
| --- | --- | --- | --- |

This attribute identifies the document format needed to print this job.

This attribute identifies the overall print document format used for the document.  It consists of three elements, a document-format, a document-format-variants and a document-format-version. The latter two elements are optional.

[The format needs to be simplified, but I’ll leave in the text below for now.]

The document-format element identifies a particular family of document formats, of which there may exist several versions or variants.  The document-format-variants and document-format-version elements identify a specific instance of a document format. The variant refers to a particular functional subset of a format. For example, the format PostScript has variants of level 1 and level 2, and the format PCL has several variants, including PCL4 and PCL5.

The version distinguishes among successive releases of the same basic format and variant. For example, successive versions of Xerox Interpress include versions 2.0, 2.1, 3.0, 3.1, etc.

The document-format-variants element consists of a single text string.  If it is necessary to identify more than one variant, the respective variant values shall all be contained in the document-format-variants element, separated from one another by commas.

If the client omits the document-format-variants or document-format-version elements, the server may supply a format-specific default.

Proprietary values for the document-format, document-format-variants, and document-format-version elements are assigned by the owners of those formats.

7.2.3.1 fonts-used

| fonts-used | fontReferenceSyntax | M |  |
| --- | --- | --- | --- |

This attribute identifies the font resources used  in the job.

7.2.3.1 character-sets-used

| character-set-used | enums | S |  |
| --- | --- | --- | --- |

This attribute identifies the character-set used in the document. This attribute is relevant only for files that are not in ASCII, such as text files and possibly PCL files. PostScript files are always ASCII.

7.2.3.2 media-used

| media-used | nameSyntax | M |  |
| --- | --- | --- | --- |

This attribute identifies the media, input-trays or electronic forms needed to print the job.

--------

Standard values for this attribute are defined: TBD

---

This attribute is intended for scheduling and validation. The server uses this attribute with the printer attributes media-supported for validation and media-ready for scheduling.

7.2.3.2 sides-used

| sides-used | enum | S |  |
| --- | --- | --- | --- |

This attribute specifies whether a job needs simplex, duplex or tumble printing.

7.2.3.2 output-bin-used

| output-bin-used | enum | S |  |
| --- | --- | --- | --- |

This attribute specifies what output bins the job needs.

7.2.3.2 print-quality-used

| print-quality-used | enum | S |  |
| --- | --- | --- | --- |

This attribute specifies what print quality the job needs.

7.2.3.2 finishing-used

| finishing-used | enum | S |  |
| --- | --- | --- | --- |

This attribute specifies what finishing the job needs.

7.2.3.2 printer-resolution-used

| printer-resolution-used | cardinalSyntax | S |  |
| --- | --- | --- | --- |

This attribute specifies what resolution the job needs. This attribute is the first of three that a client can use to specify the size of a job.

7.2.3.2 total-job-octets

| total-job-octets | nameSyntax | S |  |
| --- | --- | --- | --- |

This attribute specifies the total size of the job in octets.

7.2.3.2 job-impression-count

| job-impression-count | nameSyntax | S |  |
| --- | --- | --- | --- |

This attribute specifies the total size of the job in impressions.

7.2.3.2 job-media-sheet-count

| job-media-sheet-count | nameSyntax | S |  |
| --- | --- | --- | --- |

This attribute specifies the total size of the job in media-sheets.

7.2.4 Document Contents (one per document)

7.2.1 Document Description Attributes

These attributes name and reference the individual documents in a job.

The client may specify document-description attributes in:

a)	Print:  all, except document-type, transfer-method, and document-content shall be passed as explicit parameters of the Print abstract-operation and shall not be passed as attributes.

b)	ModifyJob:  none

c)	ListObjectAttributes:  all, except id-att-document-content

7.2.1.1 number-of-documents

| number-of-documents | cardinalSyntax | S |  |
| --- | --- | --- | --- |

This attribute specifies the number of documents in the job.

7.2.1.2 document-content

| document-content | documentContentSyntax | S |  |
| --- | --- | --- | --- |

This attribute is a sequence with one element for each document in the job.  Each element contains the following fields:

- document-name: the name of the document specified by the end-

user.

- file-reference: the path to the current location of the file

- URL: the URL of the file.

- contents: actual contents of the file.

Of the last three fields, only one should be present at any time.

-

-

---

----

7.3 Operation Attributes

TBD

7.4 Printer Attributes

A printer object may represent either a Print Server or Output Device .

A Printer Object in an Output Device  contains a set of printer object attributes that represent an Output Device capable of rendering a document in visible form.  Examples include electronic and electro-mechanical printers such as laser printers, ink-jet printers, and various kinds of impact printers, but may include other types of output devices such as microfiche imagers and plotters as well.

A Printer Object in a Print Server  contains a set of printer object attributes that are the union of the Printer objects in the downstream Output Devices.  This object txtends the capabilities of an Output Device.  For example, an administrator might define a single Print Server to represent all of the Output Devices of the same type and capability in a single location, associated with a particular server.  A user/client would normally send a print-job to a Print Server , and allow the Print Server to assign the job to a particular Output Device  based on the relative load and availability of the printers under its control, thus providing a load balancing service.  However, nothing  precludes a user/client from sending a print-job to an Output Device .  Such a restriction is up to the policy of the system administrator and the access control that the administrator specifies.

--

--------

----

------

The attributes defined in this subclause provide information about a particular Print Server or Output Device ;  all of the attributes apply to Print Servers and Output Devices .

7.4.1 printer-name

| printer-name | simpleNameSyntax | S |  |
| --- | --- | --- | --- |

This attribute uniquely identifies the printer.

7.4.2 printer-state

| printer-state | objectIdentifierSyntax | S |  |
| --- | --- | --- | --- |

This attribute identifies the current state of the printer.  The LDPA protocol support all values for printer states, however printers are not required to generate all the printer states, only those which are appropriate for the particular implementation.

The following standard values are defined: id-val-printer-state-unknown, id-val-printer-state-idle, id-val-printer-state-printing, id-val-printer-state-needs-attention, id-val-printer-state-paused, id-val-printer-state-shutdown, id-val-printer-state-job-start-wait, id-val-printer-state-job-end-wait, id-val-printer-state-job-password-wait, id-val-printer-state-needs-key-operator,

id-val-printer-state-connecting-to-printer,

id-val-printer-state-timed-out

7.4.2 printer-state-reasons

| printer-state-reasons | enum | S |  |
| --- | --- | --- | --- |

This attributes specifies reasons for being in a printer-state.

7.4.3 message

| message | messageSyntax | S |  |
| --- | --- | --- | --- |

This attribute provides a message from an operator, system administrator or ‘intelligent' process to indicate to the user the reasons for modification or other management action taken on a job.

------------------

-----

----

---------------

------

----

----

7.4.6 fonts-supported

| fonts-supported | fontReferenceSyntax | M |  |
| --- | --- | --- | --- |

This attribute identifies the font resources supported by this printer and indicates the state of readiness for each font.

7.4.8 media-supported

| media-supported | nameOrOidSyntax | M |  |
| --- | --- | --- | --- |

This attribute identifies the media, media-sizes, input trays, and electronic forms supported by this printer, and indicates the state of readiness for each resource. There may be just two states: ready and needs-installing, or there may be a third state: needs-purchasing.

.

7.4.11 document-formats-supported

| document-formats-supported | docFormatSyntax | M |  |
| --- | --- | --- | --- |

This attribute identifies the document-formats, including the document-format-variants and document-format-versions, supported by the Output Device and the server software collectively. This set includes both the formats that are native to the Output Device and those formats that the server software can translate to one that is native to the Output Device. From the client's point of view, this set contains all formats in which documents can be submitted to this printer.

Proprietary document format identifiers, variants, and versions are assigned by the owners of those formats.

7.4.12 numbers-up-supported

| numbers-up-supported | numbersUpSupportedSyntax | S |  |
| --- | --- | --- | --- |

This attribute identifies the number-up values and imposition objects supported by this printer.  The cardinal-range is an alternative (shorthand) way of specifying consecutive cardinal-values.

There are no standard values defined.

7.4.13 finishings-supported

| finishings-supported | nameOrOidSyntax | S |  |
| --- | --- | --- | --- |

This attribute identifies the per-document finishing objects supported by this printer, that is the server-installed finishing objects that may be used as values of the finishing document attribute.

NOTE: What are the values of this attribute since we have no Finishing objects.

7.4.14 sides-supported

| sides-supported | sidesSyntax | M |  |
| --- | --- | --- | --- |

This attribute indicates the values of the sides attribute supported by this printer, i.e., the different numbers of surfaces of a medium that can be imaged by this printer.

7.4.15 job-sheets-supported

| job-sheets-supported | nameOrOidSyntax | M |  |
| --- | --- | --- | --- |

This attribute identifies the auxiliary-sheet-s values supported by this printer.

To allow no job sheets, the system administrator shall include the value id-val-generic-none as a value for this attribute. The client specifies that there are no job sheets by using the value id-val-generic-none as the value of the job-sheets attribute.

If the job-sheets attribute is not specified or contains a value which the printer does not support, and the job-sheets value is non-compulsory (so that the server accepts the job), then the server may select from among the values of this attribute.  The server shall not select the value id-val-generic-none unless it is the only value specified for the job-sheets-supported attribute.

NOTE – It is preferable for the server to produce some job auxiliary-sheet, even if not the desired one, rather than produce none at all.

7.4.16 document-sheets-supported

| document-sheets-supported | nameOrOidSyntax | M |  |
| --- | --- | --- | --- |

This attribute identifies the auxiliary-sheets values supported by this printer.

To allow no document sheets, the system administrator shall include the value id-val-generic-none as a value for this attribute. The client specifies that there are no document sheets by using the value id-val-generic-none as the value of the document-sheets attribute.

If the document-sheets attribute is not specified or contains a value which the printer does not support, and the document-sheets value is non-compulsory (so that the server accepts the job), then the server may select from among the values of this attribute. The server shall not select the value id-val-generic-none unless it is the only value specified for the document-sheets-supported attribute.

NOTE – It is preferable for the server to produce some job auxiliary-sheet, even if not the desired one, rather than produce none at all.

7.4.17 maximum-copies-supported

| maximum-copies-supported | cardinalSyntax | S |  |
| --- | --- | --- | --- |

This attribute indicates the maximum number of copies of a document that can be rendered by this printer in a single print-job.

A server shall ensure that neither a document's copy-count attribute nor any single job-copies element of a ResultsProfile exceeds the value specified in this attribute.  A server may ensure that for each document the product of the document's copy-count and the sum of all job-copies in all result-sets does not exceed this value.

A value of 0 shall indicate there is no limit on the maximum number of document copies for this printer.

7.4.18 notification-delivery-methods-supported

| notification-delivery-methods-supported | TBD | S |  |
| --- | --- | --- | --- |

7.4.19 physical-printers-supported

| physical-printers-supported | distinguishedNameStringSyntax | M |  |
| --- | --- | --- | --- |

This attribute identifies the physical printers (printer's realization attribute is either physical or logical-and-physical) supported by this server.

7.4.20 Logical-printers-supported

| logical-printers-supported | distinguishedNameStringSyntax | M |  |
| --- | --- | --- | --- |

This attribute identifies the logical printers (printer's realization attribute is either logical or logical-and-physical) supported by this server.

7.4.21 events-supported

| events-supported | objectIdentifierSyntax | S |  |
| --- | --- | --- | --- |

This attribute identifies the event types and event classes supported by this printer.

7.4.22 transfer-methods-supported

| transfer-methods-supported | objectIdentifierSyntax | M |  |
| --- | --- | --- | --- |

This attribute identifies the transfer-methods supported by this server.

7.4.23 locales-supported

TBD

7.4.24 multiple-documents-supported

| multiple-documents-supported | booleanSyntax | S |  |
| --- | --- | --- | --- |

This attribute indicates whether this object (printer or server) is capable of processing and printing multiple documents per job.

This printershall not support any operation involving multiple documents unless this attribute has the value TRUE.  In spite of this requirement, it is still a printer driver implementation option of whether to support modifying and/or cancelling individual documents within a multi-document job or not.

7.4.28 cancel-individual-document-supported

| cancel-individual-document-supported | booleanSyntax | S |  |
| --- | --- | --- | --- |

This attribute indicates whether this object (printer or server) is capable of cancelling the printing of individual documents within a multiple document job.

7.4.29 modify-individual-document-supported

| modify-individual-document-supported | booleanSyntax | S |  |
| --- | --- | --- | --- |

This attribute indicates whether the server is capable of modifying the print-request parameters for individual documents within a multiple document job.

7.5 Initial Value Job Attributes

The attributes for an Initial Value Job object can be any of the Job object attributes defined in section 7.1.

7.6 Initial Value Document Attributes

The attributes for an Initial Value Document object can be any of the Document object attributes defined in section 7.2.

7.7 Relationship to ISO/IEC 10175 Conformance Levels

In ISO/IEC 10175 DPA Appendix E, three Conformance Levels are defined.  For levels 1 and 2, an additional set of attributes for multiple-document job support are defined.  These additional levels are indicated by the letter M.  Thus, level 2M indicates support for a basic set of operations and attributes with additional support for multiple-document jobs.  The scope of LDPA is essentially the same as level 2M as defined by DPA.

LDPA is explicitly designed to be extensible.  This means that in addition to the attributes defined in this specification, specific implementation instances may support not only the basic protocol as defined in this specification, but might add vendor specific extentions.

Also, for the core set of attributes listed in this specification, it is not required that a conforming server support all (standard) values of all supported attributes.  For example, it is not required that a printer implement all finishing methods indicated by the standard values.

The explicit requirement of the term “supported”, with respect to one of the attributes that deal with printer functions or resources, is that the server shall recognize the attribute and those values that are supported, and shall be able to respond to a query about which values that printer does, in fact, support.

8.  Security Considerations

This protocol does not identify any new security mechanisms. The authentication mechanisms (as well as extentions) built into the RPC infrastructure are recommended.  Also, the Bind operation described in section 5 supports the notion of authentication via simple or credential based arguments.

9.  References

[1]	Smith, R., Wright, F., Hastings, T., Zilles, S., and Gyllenskog, J., "Printer MIB", RFC 1759, March 1995.

[2]	Srinivasan, R., “RPC: Remote Procedure Call Protocol Specification Version 2", RFC 1831, August 1995.

[3]	Srinivasan, R., “XDR: External Data Representation Standard", RFC 1832, August 1995.

[4]	Postel, J., “Instructions to RFC Authors”, RFC 1543, October 1993.

[5]	ISO/IEC 10175 Document Printing Application (DPA)

[6]	Herriot, R. (editor),  X/Open A Printing System Interoperability Specification (PSIS), August 1995.

[7]	Kirk, M. (editor), POSIX System Administration -- Part 4: Printing Interfaces, POSIX 1387.4 D8, 1994.

[8]	ISO 8824 “Abstract Syntax Notation One ASN.1"

[9]	ISO 8825 “Basic Encoding Rules BER”

[10]	McLaughlin, L. III, (editor), “Line Printer Daemon Protocol” RFC 1179, August 1990.

10.  Author's Address

Scott A. Isaacson

Novell, Inc.

122 E 1700 S

Provo, UT   84606

Phone: 801-861-7366

Fax:   801-861-4025

EMail: scott_isaacson@novell.com

F. Devon Taylor

Novell, Inc.

122 E 1700 S

Provo, UT   84606

Phone: 801-861-7179

Fax:   801-861-4025

EMail: devon_taylor@novell.com

Mike MacKay

Novell, Inc.

2180 Fortune Dr.

San Jose, CA   95131

Phone: 408-577-6368

Fax:   408-577-5151

EMail: mmackay@novell.com

Peter Zehler

Xerox Corporation

800 Phillips Rd. M/S 111-01X

Webster, NY 14580

Phone: 716-265-8755

Fax:   716-265-8792

EMail: peter_zehler@wb.xerox.com

Carl-Uno Manros

Xerox Corporation

701 S. Aviation Blvd.

El Segundo, CA   90245

Phone: 310-333-8273

Fax:   310-333-5514

EMail: cmanros@cp10.es.xerox.com

Tom Hastings

Xerox Corporation

701 S. Aviation Blvd.

El Segundo, CA   90245

Phone: 310-333-6413

Fax:   310-333-5514

EMail: hastings@cp10.es.xerox.com

Robert Herriot

Sun Microsystems Inc.

2550 Garcia Ave., MPK-17

Mountain View, CA 94043

Phone: 415-786-8995

Fax:	 415-786-

Email: robert.herriot@eng.sun.com