Contribution from Keith Carter

January 24, 1997

--------------------- Directory Subgroup: Issues ----------------------------------

1.  Should LDAP be included in the Directory Schema?

Proposal:  Remove section “2.3 Directory Entries Using LDAP” to decouple

the schema from a specific directory protocol.

2.  The use of “objective” attributes vs. “subjective” attributes still

needs to be resolved.  For example, for Maximum Print Quality is it

better to have values like “high”, “medium”, “low” or to have explicit,

quantified, measurable values?  Some considerations are:  end-users

don’t often know what explicit objective values are or what they really

mean and they want to depend on an administrator to define what is

“high” quality printing and what is “low” quality, especially since

today’s objective values that equate to “high” are tomorrow’s objective

values that equate to “medium”.  On the other hand, some end-users

demand the control and power explicit values can give them when they do

filtered searching.  For example, they know and appreciate the

difference between 20ppm and 23 ppm printers.

Proposal:  We need to focus on the primary “customer” of the Directory

Schema who is the end-user selecting printer objects.  Based on my

interaction with customers, most end-users want to search for printer

objects using standard values “low”, “medium” and “high” for quality,

cost and speed.  I realize there are power end-users who want to make a

selection based on explicit values, but these end-users can always query

for a list of printer objects using the basic 3 values of the directory

attribute of interest and then query each printer object in the list for

its specific value.  However, I expect most of the time the power

end-users will know the explicit capabilities of printer objects.  So,

we should stick with “high”, “medium” and “low” for these attributes.

Another option is to provide a mechanism to specify explicit values for

these attributes if that is really necessary.  I invite Jim Walker of

DAZEL and Scott Isaacson of Novell to share their “real world”

experience since I understand both DAZEL and Netware 4.x/NDPS also allow

end-users to search for printer objects based on attributes.

3.  Is the cost per page tied to the media it is printed on?  Do we need

to define some standard for determining the cost per page?

Proposal:  Please read the proposal for issue 2 which basically states

not to put this level of granularity in the cost attribute of a printer

object entry in the directory.  If this is required for accounting

applications, then an explicit value on cost should be stored in the

printer object.  Furthermore, based on the series of email responses on

this issue, I’m convinced that it will be complex to define values for

the cost attribute more granular than “high”, “medium” and “low”.

Finally, a URL reference for more information on the cost of a printer

seems like a printer object attribute since this does not allow a user

to select a printer object based solely on the attributes in its

directory entry.

4.  Is information that is as variable as media-ready stored in the Name

Service?

Proposal:  No.  This requires an implementation to synch up the physical

attributes of a printer object with its printer object entry in the

directory which I don’t believe we should require of an implementation.

If there is a tidal wave of support for media-ready, then we could make

this attribute optional and indicate that an administrator may specify

the values so as not to require implementations to synchronize printer

objects and their entries in the directory.  Scott Isaacson and Jim

Walker, what does your experience tell you about putting such attributes

in printer object entries in the directory?

5.  How is “near my hotel” done?  Is there a required format for printer

location in the schema?

Proposal:  The Directory Schema has an attribute called Location.  Per

the current definition in the Directory Schema document, this field is a

free form string that can contain site-specific location information.

This seems sufficient and keeps things straightforward for an

implementation.

6.  The client should be able to ask either the printer or the Name

Service for the location of the driver/driver installer.

Proposal:  Put the location of the driver/driver installer in the

printer object - not in the directory.  This location could change based

on the printer(s) assigned to a printer object so putting this in the

directory opens the door to synchronization issues in keeping a printer

object entry in the directory current with its printer object.

Furthermore, I do not believe that this attribute is required by an

end-user to select a printer object.

7.  Should Device Id be in the Directory Schema?  If so, is this the

IEEE 1284 device id, the Object Identifier as used in the Host Resource

MIB hrDeviceId object, or some other identifier?

Proposal:  Remove Device Id from the Directory Schema.  Make and Model

should be sufficient in terms of locating printer object entry(ies) that

use a specific device.  Once an end-user selects a printer object, an

IPP application can query the printer object to get its device id so the

application can select the printer driver for the printer object.

8.  We must specify which attributes are “mandatory” and which are

“optional”.  LDAP uses the terms “must” and “may” to identify attributes

that “must” appear and attributes that “may” appear in a given entry in

the directory.

Proposal:  TBD.

9.  What should be stated about security?

Proposal:  Replace section “3. Security Considerations” with a statement

like the following:

This document does not identify specific security mechanisms used by a

directory service to control access to printer object entries in the

directory.  Security is left to the implementation of the directory

service.

---------------------------- End of Directory Issues ---------------------------