Working Draft, June 9, 2003	Important information to review is highlighted like this.

Maturity Level:  Interim	There is 1 ISSUE highlighted like this.

The Printer Working Group (PWG)

Standard for
The Internet Printing Protocol (IPP):

Job Extensions

Abstract: This IPP specification extends the Job semantics of the IPP Model and Semantics [rfc2911] object model.  This specification defines some new Operation attributes for use in Job Creation and Document Creation operations.  The Printer copies these Operation attributes to the corresponding Job Description attributes, which the clients may query.  The Document Creation Operation attributes describe the Document Content and permit the Printer to reject requests that it cannot process correctly.  Some corresponding "xxx-default" and "xxx-supported" Printer attributes are defined.  This specification defines some Job Template attributes that apply to a multi-document Job as a whole.  This specification also defines some additional values for the "job-state-reasons" Job Description attribute.  Each of the attributes defined in this specification are independent of each other and are OPTIONAL for a Printer to support.

Working Draft, June 4, 2003

The Printer Working Group (PWG)
Standard for

The Internet Printing Protocol (IPP):

Job Extensions

This version of the PWG Proposed Standard is available electronically at:

ftp://ftp.pwg.org/pub/pwg/ipp/new_JOBX/wd-ippjobx10-20030604.pdf, .doc, .rtf

This document is a Working Draft for an IEEE-ISTO PWG Candidate Standard.  For a definition of a "PWG Candidate Standard" and its transition to a "PWG Standard", see:  ftp://ftp.pwg.org/pub/pwg/general/pwg-process.pdf.  After approval by the PWG (by a Last Call) to transition a PWG Working Draft to a PWG Candidate Standard, the resulting PWG Candidate Standard will be available electronically at: ftp://ftp.pwg.org/pub/pwg/cs/.  After approval by the PWG (by a Last Call) to transition a PWG Candidate Standard to a PWG Standard, the resulting PWG Standard will be available electronically at: ftp://ftp.pwg.org/pub/pwg/standards/.

Copyright (C) 2003, IEEE ISTO. All rights reserved.

This document may be copied and furnished to others, and derivative works that comment on, or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice, this paragraph and the title of the Document as referenced below are included on all such copies and derivative works.  However, this document itself may not be modified in any way, such as by removing the copyright notice or references to the IEEE-ISTO and the Printer Working Group, a program of the IEEE-ISTO.

Title:  The Printer Working Group Standard for the Internet Printing Protocol (IPP): Job and Document Operation extensions

The IEEE-ISTO and the Printer Working Group DISCLAIM ANY AND ALL WARRANTIES, WHETHER EXPRESS OR IMPLIED INCLUDING (WITHOUT LIMITATION) ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

The Printer Working Group, a program of the IEEE-ISTO, reserves the right to make changes to the document without further notice.  The document may be updated, replaced or made obsolete by other documents at any time.

The IEEE-ISTO takes no position regarding the validity or scope of any intellectual property or other rights that might be claimed to pertain to the implementation or use of the technology described in this document or the extent to which any license under such rights might or might not be available; neither does it represent that it has made any effort to identify any such rights.

The IEEE-ISTO invites any interested party to bring to its attention any copyrights, patents, or patent applications, or other proprietary rights which may cover technology that may be required to implement the contents of this document. The IEEE-ISTO and its programs shall not be responsible for identifying patents for which a license may be required by a document and/or IEEE-ISTO Industry Group Standard or for conducting inquiries into the legal validity or scope of those patents that are brought to its attention. Inquiries may be submitted to the IEEE-ISTO by e-mail at:

ieee-isto@ieee.org.

The Printer Working Group acknowledges that the IEEE-ISTO (acting itself or through its designees) is, and shall at all times, be the sole entity that may authorize the use of certification marks, trademarks, or other special designations to indicate compliance with these materials.

Use of this document is wholly voluntary.  The existence of this document does not imply that there are no other ways to produce, test, measure, purchase, market, or provide other goods and services related to its scope.

About the IEEE-ISTO

The IEEE-ISTO is a not-for-profit corporation offering industry groups an innovative and flexible operational forum and support services.  The IEEE-ISTO provides a forum not only to develop standards, but also to facilitate activities that support the implementation and acceptance of standards in the marketplace.  The organization is affiliated with the IEEE (http://www.ieee.org/) and the IEEE Standards Association (http://standards.ieee.org/).

For additional information regarding the IEEE-ISTO and its industry programs visit http://www.ieee-isto.org.

About the IEEE-ISTO PWG

The Printer Working Group (or PWG) is a Program of the IEEE Industry Standards and Technology Organization (ISTO) with member organizations including printer manufacturers, print server developers, operating system providers, network operating systems providers, network connectivity vendors, and print management application developers.  The group is chartered to make printers and the applications and operating systems supporting them work together better.  All references to the PWG in this document implicitly mean “The Printer Working Group, a Program of the IEEE ISTO.” In order to meet this objective, the PWG will document the results of their work as open standards that define print related protocols, interfaces, procedures and conventions. Printer manufacturers and vendors of printer related software will benefit from the interoperability provided by voluntary conformance to these standards.

In general, a PWG standard is a specification that is stable, well understood, and is technically competent, has multiple, independent and interoperable implementations with substantial operational experience, and enjoys significant public support.

For additional information regarding the Printer Working Group visit:  http://www.pwg.org

Contact information:

IPP Web Page:  http://www.pwg.org/ipp/

IPP Mailing List:  ipp@pwg.org

To subscribe to the ipp mailing list, send the following email:

1) send it to majordomo@pwg.org

2) leave the subject line blank

3) put the following two lines in the message body:

subscribe ipp

end

Implementers of this specification are encouraged to join the IPP Mailing List in order to participate in any discussions of clarifications or review of registration proposals for additional names.  Requests for additional extensions, for inclusion in this specification, should be sent to the IPP Mailing list for consideration.  In order to reduce spam the mailing list rejects mail from non-subscribers, so you must subscribe to the mailing list in order to send a question or comment to the mailing list.

Contents

1	Introduction and Summary	8

1.1 Scope	8

2	Terminology	9

2.1 Conformance Terminology	9

2.2 Other Terminology	10

3	Operation Attributes for Job Creation and Document Creation operations	10

3.1 Operation attributes for Job Creation operations only	13

3.1.1 ipp-attribute-fidelity (boolean) ([rfc2911] section 3.2.1 and 15)	13

3.1.1.1 Why there are no “ipp-attribute-fidelity-default” and “ipp-attribute-fidelity-supported” attributes	13

3.1.2 job-mandatory-attributes (1setOf type2 keyword)	13

3.1.2.1 Why there are no “job-mandatory-attributes-default” and “job-mandatory-attributes-supported” Printer Description attributes	14

3.1.3 output-device-requested (name(127))	14

3.1.3.1 output-device-default (name(MAX))	15

3.2 Operation attributes for both Job and Document Creation operations	15

3.2.1 document-charset (charset)	15

3.2.2 document-digital-signature (type2 keyword)	16

3.2.3 document-format (mimeMediaType)	17

3.2.4 document-format-details (1setOf collection)	17

3.2.4.1 document-source-application-name (name(MAX))	19

3.2.4.2 document-source-application-version (text(127))	19

3.2.4.3 document-source-os-name (name(40))	19

3.2.4.4 document-source-os-version (text(40))	19

3.2.4.5 document-format (mimeMediaType)	20

3.2.4.6 document-format-device-id (text(127))	20

3.2.4.7 document-format-version (text(127))	20

3.2.4.8 document-natural-language (1setOf naturalLanguage)	20

3.2.5 document-format-version (text(127))	20

3.2.5.1 Why no "document-format-version-default"	21

3.2.6 document-message (text(MAX))	21

3.2.7 document-name (name(MAX))	21

3.2.8 document-natural-language (naturalLanguage)	21

4	Job Template attributes	22

4.1 Job Template attributes defined for Job-wide effect in multi-document jobs	22

4.1.1 job-copies (integer(1:MAX))	23

4.1.1.1 job-copies-default (integer(1:MAX))	23

4.1.1.2 job-copies-supported (rangeOfInteger(1:MAX))	23

4.1.2 job-cover-back (collection)	23

4.1.2.1 job-cover-back-default (collection)	23

4.1.2.2 job-cover-back-supported (1setOf type2 keyword)	23

4.1.3 job-cover-front (collection)	23

4.1.3.1 job-cover-front-default (collection)	24

4.1.3.2 job-cover-front-supported (1setOf type2 keyword)	24

4.1.4 job-finishings (1setOf type2 enum)	24

4.1.4.1 job-finishings-default (1setOf type2 enum)	24

4.1.4.2 job-finishings-ready (1setOf type2 enum)	24

4.1.4.3 job-finishings-supported (1setOf type2 keyword)	24

4.1.5 job-finishings-col (1setOf collection)	24

4.1.5.1 job-finishings-col-default (collection)	24

4.1.5.2 job-finishings-col-ready (collection)	25

4.1.5.3 job-finishings-col-supported (1setOf type2 keyword)	25

5	Job Description attributes	25

5.1 Job Description attributes that describe the Job as a whole	25

5.1.1 errors-count (integer(0:MAX))	26

5.1.2 ipp-attribute-fidelity (boolean)	26

5.1.3 job-mandatory-attributes (1setOf type2 keyword)	26

5.1.4 output-device-requested (name(127))	26

5.1.5 warnings-count (integer(0:MAX))	27

5.2 Job Description attributes that are defaults for the Document(s) in the Job	27

5.2.1 compression (type3 keyword)	27

5.2.2 document-charset (charset)	27

5.2.3 document-digital-signature (type2 keyword)	27

5.2.4 document-format (mimeMediaType)	27

5.2.5 document-format-details (1setOf collection)	27

5.2.6 document-format-version (text(127))	27

5.2.7 document-message (text(MAX))	28

5.2.8 document-name (name(MAX))	28

5.2.9 document-natural-language (naturalLanguage)	28

6	New values to existing attributes	28

6.1 New job-state-reasons(1setOf type2 keyword) Values	28

6.2 New media (type 3 keyword | name(MAX)) Values	29

7	New Printer Description attributes	30

7.1 document-charset-default (charset)	30

7.2 document-charset-supported (1setOf charset)	30

7.3 document-digital-signature-default (type2 keyword)	31

7.4 document-digital-signature-supported (1setOf type2 keyword)	31

7.5 document-format-details-default (collection)	31

7.6 document-format-details-supported (1setOf type2 keyword)	31

7.7 document-format-version-default (text(127))	31

7.8 document-format-version-supported (1setOf text(127))	32

7.9 document-natural-language-default (naturalLanguage)	32

7.10 document-natural-language-supported (1setOf naturalLanguage)	32

7.11 output-device-default (name(127))	32

7.12 output-device-supported (1setOf name(127))	32

8	Conformance Requirements	32

8.1 Printer conformance requirements	32

8.2 Client conformance requirements	33

9	Normative References	33

10	Informative References	34

11	IANA Considerations	36

11.1 Attribute Registrations	36

11.2 Keyword attribute values	38

11.3 Enum Attribute Values	39

12	Internationalization Considerations	39

13	Security Considerations	39

14	Contributors	39

15	Acknowledgments	39

16	Author’s Address	39

17	Annex A: Additional requirements for a revision of the PWG IEEE/ISTO 5101.1 Candidate Standard [pwg5101.1] to add the new ‘choice_xxx’	40

18	Annex B: Change Log (informative - to be removed before publication)	40

Table of Tables

Table 1 - Summary of Job Creation Operation attributes	8

Table 2 - Summary of Job and Document Creation Operation attributes	9

Table 3 - Summary of Job Template Attributes	9

Table 4 - Operation, Default, and Supported Attributes	11

Table 5 - Operation Attributes supplied in Job and Document Creation Requests	12

Table 6 - "document-format-details" member Operation attributes	18

Table 7 - Job Template attributes	22

Table 9 - Job Description attributes that describe the Job as a whole	25

Table 10 - New "job-state-reasons" attribute values	29

Table 11 - New Printer Description attributes	30

Table 12 - Keyword values of "document-format-details-supported Printer attribute	31

# Introduction and Summary

This IPP specification extends the Job semantics of the IPP Model and Semantics [rfc2911] object model.  This specification defines some new Operation attributes for use in Job Creation and Document Creation operations.  The Printer copies these Operation attributes to the corresponding Job Description attributes, which the clients may query.  The Document Creation Operation attributes describe the Document Content and permit the Printer to reject requests that it cannot process correctly.  Some corresponding "xxx-default" and "xxx-supported" Printer attributes are defined.  This specification defines some Job Template attributes that apply to a multi-document Job as a whole.  This specification also defines some additional values for the "job-state-reasons" Job Description attribute.  Each of the attributes defined in this specification are independent of each other and are OPTIONAL for a Printer to support.

The intent is to propose this specification as an IPP extension and which will be referenced as part of the PWG Semantic Model.  The purpose for these extensions is so that the print industry can have a common semantic specification for use in IPP, the PWG Semantic Model [pwg-sm], the PWG Print Service Interface (PSI) project [pwg-psi], the PWG IPP Fax (IPPFAX) project [ippfax], and the Free Standards Group Job Ticket API [fsg-jt].  Note:  There are a number of footnotes that relate this IPP specification to the PWG Semantic model and other PWG projects, such as the Print Service Interface (PSI) [pwg-psi].

## Scope

Table 1 summarizes the Operation attributes defined in this specification for use with Job Creation operations.  The Printer copies these Operation attributes to corresponding Job Description attributes defined in this specification.

Table 1 - Summary of Job Creation Operation attributes

| Operation attribute | Printer support | Description |
| --- | --- | --- |
| ipp-attribute-fidelity (boolean) | MUST | Allows the client to indicate whether or not the Printer MUST support all Job Template attributes supplied in Job Creation operations or MUST reject the job. Defined in [rfc2911] section 3.2.1.2 and 15. |
| job-mandatory-attributes (1setOf type2 keyword) | MAY | Allows the client to list the Job Template attributes that the Printer MUST support when supplied in Job Creation operations or MUST reject the job. |
| output-device-requested (name(127)) | MAY | Allows the client to indicate a preference for output devices, for a Printer that supports more than one output device (see fanout in [rfc2911] section 2.1). |

Table 2 summarizes the Operation attributes defined in this specification for use with Job Creation operations and Send-Document and Send-URI operations.  These Operation attributes allow the client to supply descriptions of the Document Content to the Printer about individual Document Contents at request time.  When supplied in a Job Creation request, these Operation attributes provide defaults for the Document or Documents of the Job that the Documents inherit.  When supplied in a Document Creation operation, they describe the Document explicitly.  This specification does not define corresponding Document Description attributes, since this specification does not include or depend on a Document object.

Table 2 - Summary of Job and Document Creation Operation attributes

| Operation attribute | Printer support | Description |
| --- | --- | --- |
| document-charset (charset) | MAY | The charset of the Document content |
| document-digital-signature (type2 keyword) | MAY | The type of digital signature, if any used in the Document Content. |
| document-format-details (1setOf collection) | MAY | The details of the Document Content, including when its a package of files (e.g., zip, multipart/related). The details include information, such as the application and the operating system that created the document, the intended device type (when the format is device-dependent), and the natural languages of the document. |
| document-format-version (text(127)) | MAY | The version of the document format. |
| document-format-natural-language (naturalLanguage) | MAY | The primary natural language of the Document Content. |
| job-mandatory-attributes (1setOf type2 keywords) | MAY | The Job Template attributes that the client is requesting that the Printer MUST support or reject the Job. |
| output-device-requested (name(127) | MAY | The output device that the client is requesting for a Printer that represents multiple output devices. |

Table 3 summarizes the Job Template attributes defined in this specification for use with Job Creation operations.  These Job Template attributes are defined to apply to the Job as a whole, independent of the value of the "multiple-document-handling" Job Template attribute ([rfc2911] section 4.2.4) whose 'separate-documents-collated-copies' and 'separate-documents-uncollated-copies' which cause the Printer to produce separate Output Documents.

Table 3 - Summary of Job Template Attributes

| Job Template attribute | Printer support | Description |
| --- | --- | --- |
| job-copies (integer(1:MAX)) | MAY | The number of copies of the entire Job. |
| job-cover-back (collection) | MAY | The cover for the back of the entire Job. |
| job-cover-front (collection) | MAY | The cover for the front of the entire Job. |
| job-finishings (1setOf type2 enum) | MAY | The finishing for the entire Job. |
| job-finishings-col (1setOf collection) | MAY | The finishing characteristics for the entire Job. |

# Terminology

This section defines terminology used throughout this document.

## Conformance Terminology

Capitalized terms, such as MUST, MUST NOT, REQUIRED, SHOULD, SHOULD NOT, MAY, and OPTIONAL, have special meaning relating to conformance as defined in RFC 2119 [rfc2119].

## Other Terminology

This document uses the same terminology as [rfc2911], such as “client”, “Printer”, “attribute”, “attribute value”, “keyword”, “Job Template attribute”, “Operation attribute”, “operation”, “request”, “response”, and “support” (see [rfc2911] section 12.2.3) with the same meaning.  In addition, the following terms are defined for use in this document:

Document Creation operation - operations that accept Document Content.  Specifically: Print-Job, Print-URI, Send-Document and Send-URI.  Note:  Print-Job and Print-URI are both Job Creation and Document Creation operations.

Finished Document - The result that the Printer produces on Sheets after the operations of printing, folding, cutting, finishing, etc. are completed.

Job Attribute - an Operation attribute or a Job Template attribute that applies to the Job (object) as a whole.  For example, “job-name” is an Operation attribute and “job-priority” is a Job Template attribute.

Job Creation operation - operations that create Job objects, specifically: Print-Job, Print-URI, and Create-Job as defined in [rfc2911].

Operation attribute - an attribute supplied by a client in the Operation Attribute group of a request in order to affect the behavior of the operation or an attribute returned by the Printer in the Operation Attributes group in an operation response in order to report the results of the operation.  For some Operation attributes, the Printer copies the values supplied by the client to corresponding Job Description attributes.

Page - a synonym for the term “print-stream page” as defined in [rfc2911] section 12.2.4.  A Page is a page according to the definition of a page in the language used to express the document data (PDL).  Usually a Page contains a single page as intended by the author of the source document.  However, if the application that generates the PDL places multiple author pages on a single Page, they are still considered a single Page as far as the Printer and this specification is concerned.

Sheet - the unit of media that a printer puts marks on.  A Sheet is the most basic unit of output from a Printer. A printer may mark on one side or on both sides of a Sheet.

# Operation Attributes for Job Creation and Document Creation operations

This section defines Operation attributes for use in Job Creation operations and/or Document Creation operations and as such describe characteristics of the Job and Document, rather than instructions to the Printer.  For some of these Operation attributes there are corresponding “xxx-default” and “xxx-supported” Printer Description attributes (see section 7).

Table 4 lists the Operation attributes and their attribute syntaxes supplied in Job Creation and Document Creation operations and the attribute syntaxes of their corresponding "xxx-default" and "xxx-supported" Printer Description attributes, if any.  A "-" indicates that there isn't an "xxx-default" or "xxx-supported" Printer Description attribute.

Table 4 - Operation, Default, and Supported Attributes

| Operation attribute: attribute name "xxx" | Operation attribute syntax | Printer: Default Value attribute syntax "xxx-default" | Printer: Supported Values attribute syntax "xxx-supported" |
| --- | --- | --- | --- |
| document-charset | charset | charset | 1setOf charset |
| document-digital-signature | type2 keyword | type2 keyword | 1setOf type2 keyword |
| document-format | mimeMediaType | mimeMediaType | 1setOf mimeMediaType |
| document-format-details | 1setOf collection | 1setOf collection | 1setOf type2 keyword |
| document-source-application-name | name(MAX) | name(MAX) | - |
| document-source-application-version | text(127) | text(127) | - |
| document-source-os-name | name(40) | name(40) | - |
| document-source-os-version | text(40) | text(40) | - |
| document-format | mimeMediaType | mimeMediaType | - |
| document-format-device-id | text(127) | text(127) | - |
| document-format-version | text(127) | text(127) | - |
| document-natural-language | naturalLanguage | naturalLanguage | - |
| document-format-version | text(127) | text(127) | 1setOf text(127) |
| document-natural-language | naturalLanguage | naturalLanguage | 1setOf naturalLanguage |
| ipp-attribute-fidelity | boolean | - | - |
| job-mandatory-attributes | 1setOf type2 keyword | - | - |
| output-device-requested | name(127) | name(127) | 1setOf name(127) |

Table 5 lists the Operation attributes that clients supply in Job and Document Creation operations and whether the Printer copies (indicated by “”) the supplied value to the corresponding Job Description attribute.  The following statements apply to all Operation attributes for Job and Document Creation operations and are not repeated in each Operation attribute Description:

If the Printer doesn’t support the “xxx” Operation attribute, the Printer MUST ignore the “xxx” attribute (as with any OPTIONAL Operation attribute) and return the “xxx” attribute with the supplied value as unsupported in the Unsupported Attributes group as specified in [rfc2911] section 3.1.7.

If the Printer supports the “xxx” Operation attribute, the Printer MUST also:

- support the corresponding "xxx-default" and "xxx-supported" Printer Description attributes as well, if defined (see Table 4 and section 7).  If the Printer supports either of the “xxx-default” or “xxx-supported” Printer Description attributes, then the Printer MUST also support the corresponding Operation attributes.

- validate the values supplied by the client in the "xxx" Operation attribute against the values indicated in the Printer's corresponding "xxx-supported" Printer Description attribute (see section 7).  If the values do not validate, the Printer MUST perform one of the actions indicated for that Operation attribute, unless explicitly specified otherwise in the Description, such as (1) reject the request, (2) accept the request and either (a) hold the job for operator intervention or (b) print the Job ignoring the attribute.  In all of these cases, the Printer MUST return the Operation attribute with the unsupported value in the Unsupported Attributes group in the response (see [rfc2911] section 3.1.7).

| Legend: |  |
| --- | --- |
| “Creation operations” | column indicates which operations the Operation attribute can be supplied in: PJ=Print-Job, PU=Print-URI, CJ=Create-Job, SD=Send-Document, and SU=Send-URI. |
| “y” | the Operation attribute is defined for use with the indicated operations. |
| “y” | the Operation attribute is defined for use with the indicated operations and has a corresponding Job Description attribute (see section 5) ) defined to which the Printer MUST support and copy this Operation attribute, if it supports the Operation attribute. See Table 4 for whether there are corresponding “xxx-default” and “xxx-supported” Printer Description attributes defined. ISSUE 01: OK to make corresponding Job Description attributes that are default Document Description attributes as indicated below with y highlighted in yellow in the CJ and PJ,PU columns? |
| “-” | the Operation attribute is not defined for the indicated operation. |
| “Conformance” | column indicates the conformance requirements for what the client supplies in a request and the Printer supports in a request. For (indented) member attributes, a MUST means if the collection attribute is supplied or supported. |
| “Section” | column indicates where the Operation attribute is defined in this spec. |

Table 5 - Operation Attributes supplied in Job and Document Creation Requests

| Operation attribute name for Job and Document Creation requests | Attribute Syntax | Creation operations |  |  | Conformance |  | Section |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CJ | PJ, PU | SD, SU | Client supply | Printer support |  |
| document-charset | charset | y | y | y | MAY | MAY | §3.2.1 |
| document-digital-signature | type2 keyword | y | y | y | MAY | MAY | §3.2.2 |
| document-format | mimeMediaType | y | y | y | MAY | MUST | §3.2.3 |
| document-format-details | 1setOf collection | y | y | y | MAY | MAY | §3.2.4 |
| document-source-application-name | name(MAX) | y | y | y | MAY | MAY | §3.2.4.1 |
| document-source-application-version | text(127) | y | y | y | MAY | MAY | §3.2.4.2 |
| document-source-os-name | name(40) | y | y | y | MAY | MAY | §3.2.4.3 |
| document-source-os-version | text(40) | y | y | y | MAY | MAY | §3.2.4.4 |
| document-format | mimeMediaType | y | y | y | MUST | MUST | §3.2.4.5 |
| document-format-device-id | text(127) | y | y | y | MAY | MAY | §3.2.4.6 |
| document-format-version | text(127) | y | y | y | SHOULD | MUST | §3.2.4.7 |
| document-natural-language | naturalLanguage | y | y | y | MAY | MAY | §3.2.4.8 |
| document-format-version | text(127) | y | y | y | MAY | MAY | §3.2.5 |
| document-message | text(MAX) | y | y | y | MAY | MAY | §3.2.6 |
| document-name | name(MAX) | y | y | y | MAY | MUST | §3.2.7 |
| document-natural-language | naturalLanguage | y | y | y | MAY | MAY | §3.2.8 |
| ipp-attribute-fidelity | boolean | y | y | - | MAY | MUST | §3.1.1, |
| job-mandatory-attributes | 1setOf type2 keyword | y | y | - | MAY | MAY | §3.1.2 |
| output-device-requested | name(127) | y | y | - | MAY | MAY | §3.1.3 |

## Operation attributes for Job Creation operations only

This section defines additional operation attributes that the client MAY supply in Job Creation operations (Print-Job, Print-URI, and Create-Job) and Validate-Job, but not in Send-Document or Send-URI operations.

### ipp-attribute-fidelity (boolean) ([rfc2911] section 3.2.1 and 15)

This REQUIRED Operation attribute specifies the policy for the Printer to use (see [rfc2911] §3.2.1.1) when the client supplies unsupported Job Template attributes or values in a Job Creation operation.  The value ‘true’ indicates that total fidelity to client-supplied Job Template attributes and values is required for this Job Creation operation.  If the client supplies the value ‘false’ or omits the attribute, then the Printer MUST accept the Job Creation operation.

The Printer MUST copy this attribute to the corresponding (new) Job Description attribute (see section 5.1.2).

#### Why there are no “ipp-attribute-fidelity-default” and “ipp-attribute-fidelity-supported” attributes

Because [rfc2911] defines the default value to be ‘false’ there is no point to define an “ipp-attribute-fidelity-default” Printer Description attribute.  Because [rfc2911] REQUIRES a Printer to support the “ipp-attribute-fidelity” Operation attribute there is no need for an “ipp-attribute-fidelity-supported” Printer Description attribute.

### job-mandatory-attributes (1setOf type2 keyword)

This OPTIONAL Operation attribute permits the client to identify which Job Template attributes the Printer MUST support in this Job Creation request in order to accept the Job.  The Printer MUST reject this Job Creation operation if the client supplies an unsupported Job Template attribute or value in the request that is also identified in this list.  The client MAY supply the “job-mandatory-attributes” Operation attribute in a Job Creation operation or Validate-Job operation and MUST NOT supply this attribute in any other operation.  If the Printer supports the "job-mandatory-attributes" Operation attribute, the Printer MUST support this attribute in all Job Creation operations that it supports and in the Validate-Job operation.

If the client supplies the “job-mandatory-attributes” Operation attribute and the ‘ipp-attribute-fidelity” Operation attribute with the ‘true’ value, the Printer MUST ignore the “job-mandatory-attributes”, since the client is requesting that all of the supplied Job Template attributes in the Job Creation request MUST be supported in order for the Printer to accept the Job.

The Printer MUST copy this attribute to the corresponding “job-mandatory-attributes” Job Description attribute (see section 5.1.3) for auditing and trouble-shooting purposes.

The values of the “job-mandatory-attributes” attribute are the keyword attribute names that identify Job Template attributes; for example, ‘job-hold-until’, ‘job-finishings’ (see section 4.1.4), ‘copies’ and ‘media’.  In order to identify a member attribute of a collection attribute, the keyword names of both the collection attribute and the member attribute are given separated by a period (.) character.  For example, to make the “media” attribute of the “cover-front” collection attribute mandatory, supply the keyword value: ‘cover-front.media’.

The Printer MUST accept any keyword values for the “job-mandatory-attributes” attribute itself whether or not the corresponding Job Template attributes are supported by the Printer and MUST NOT return any unsupported keywords in the Unsupported Attributes Group in the response.  If the client supplies this "job-mandatory-attributes" Operation attribute in a Job Creation operation with values that represent Job Template attributes that the Printer doesn't support, the Printer MUST otherwise accept the attribute anyway.  The Printer MUST reject this Job Creation only if the client actually supplies any of the identified Job Template attributes that the Printer doesn't support.  Because the Printer doesn't reject when the "job-mandatory-attributes" Operation attribute has unsupported attribute keyword names, the client MAY supply all of the attribute keyword names that the client supports without having to tailor the list to the particular Printer to which the client is submitting the Job Creation operation, thereby, simplifying client use of this attribute.

Note:  The “job-mandatory-attributes” has nothing to do with whether the Printer overrides the  PDL with the identified Job Template attributes (see [rfc2911] sections 4.4.28 and 15).

#### Why there are no “job-mandatory-attributes-default” and “job-mandatory-attributes-supported” Printer Description attributes

By definition, omission of the “job-mandatory-attributes” Operation attribute (and the “ipp-attribute-fidelity” Operation attribute with a ‘false’ value or omitted) means that the client is requesting that none of the supplied Job Template attributes need to be supported in order to accept the Job, i.e., the Printer MUST NOT reject the job because of any unsupported Job Template attributes.  So the default is an empty list, so there is no point to have a “job-mandatory-attributes-default” Printer attribute.

By definition, the client can supply any Job Template attribute in the value list of the “job-mandatory-attributes” Operation attribute, whether or not the Printer actually supports that Job Template attribute.  Therefore, it is not possible to have a “job-mandatory-attributes-supported” list, since all possible keywords would have to be expressed.

Note: the client MAY query the Printer’s “job-creation-attributes-supported” (see [ippsave] section 7.1) and “document-creation-attributes-supported” (see [ippdoc] section 10.1) to see which Job Template and Document Template attributes the Printer actually supports.

### output-device-requested (name(127))

This OPTIONAL Operation attribute specifies the name of the target output device for a Printer that fans-out to multiple devices (see [rfc2911] section 2.1).  This allows the client to control which output device the Printer selects.  If this attribute is omitted, the Printer selects whichever output device seems best for the job in an implementation dependent manner.  The client can discover which output devices the Printer supports by querying the Printer's "output-device-requested-supported" Printer attribute (see section 7.12).

If the Printer supports this Operation attribute, the Printer MUST support it only on Job Creation operations, not subsequent Document Creation operations for this Job.

If the Printer supports this Operation attribute, the Printer MUST support the "output-device-assigned" Job Description attribute with the same values (see [rfc2911] section 4.3.13).

If a client supplies this “output-device-requested” Operation attribute with a value that the Printer doesn’t support, the Printer MUST perform one of the following actions depending on implementation:  (1) reject the request OR (2) accept the request and either (a) put the Job in the 'pending-held' state or (b) process the job ignoring the attribute.  See section above for more details.

#### output-device-default (name(MAX))

This OPTIONAL Printer Description attribute specifies the default value for the Job’s “output-device-assigned” Job Description attribute.  A Printer MAY have a dynamic scheduling algorithm to assign jobs to output devices, in which case the “output-device-default” Printer attribute is not configured.  If the Printer does not have a dynamic scheduling algorithm to assign Jobs to output devices, the Printer copies the value of the “output-device-default” Printer attribute to the Job’s “output-device-assigned” attribute when the client omits the “output-device-requested” Operation attribute.

## Operation attributes for both Job and Document Creation operations

This section defines the semantics for the Operation attributes that a client MAY supply in a Job Creation operation (Print-Job, Print-URI, Create-Job) or a Document Creation operation (Print-Job, Print-URI, Send-Document and Send-URI).  Note: Print-Job and Print-URI are both Job Creation and Document Creation operations.

### document-charset (charset)

This OPTIONAL Operation attribute identifies the charset of the Document Content.  This Operation attribute is intended to be supported by Printers that support a document-format in which the charset may be ambiguous in the Document content, such as ‘application/vnd.hp-PCL’ where the charset escape sequence MAY be omitted from the data.

If a client supplies the “document-charset” Operation attribute in a Document Creation operation with a value that the Printer doesn’t support, the Printer MUST reject the job and return the status code: ‘client-error-charset-not-supported’ (see [rfc2911] section 3.1.4.14), since the interpretation of the content will be mis-understood by the Printer..

The standard values for these attributes are Internet Media types (sometimes called MIME types).  For further details see the description of the 'mimeMediaType' attribute syntax in [rfc2911] section 4.1.9.

The Printer MUST support this attribute if it supports a document format for which the Document Content can be ambiguous with respect to the charset being used to encode the data.  For example, the ‘application/vnd.hp-PCL’ MAY omit the charset escape sequence.  Although [rfc2854] strongly RECOMMENDS that ‘text/html’, include a “; charset=charset’ parameter in the MIME type itself (that is, explicitly in the value of the “document-format” attribute), often the charset= parameter is not present in the MIME type and the Printer MUST assume a charset.  Thus a Printer that supports the text/html MIME type MUST support the “document-charset” operation attributes.  Also many text files use the various 8-bit ISO 8859 charsets in which the lower half is US-ASCII and the upper half is various Latin sets (about 8 or 9), Greek, Cyrillic, Hebrew, and Arabic.  Shift JIS is another example where the left half is US-ASCII, but the right half can be one of a number of things.  But if the data doesn't contain the charset escape sequences, this attribute can help the Printer know what the charset is in the Document.  Such document formats are called “charset-ambiguous” document formats.  Finally, even though [rfc2046] REQUIRES the “; charset=” MIME type parameter if the file contains other than US-ASCII, in practice, the “; charset=” MIME type parameter is often omitted, thus making ‘text/plain’ a charset-ambiguous document format.  See the discussion in [rfc2911] section 4.1.9 and [rfc2046].

Printer MAY also check the Document content while processing to see that the document data meets the coding requirements for the supplied charset.  If the data contains something that is outside the charset encoding, the Printer MUST either (1) hold the job or (2) abort, and SHOULD NOT continue printing square boxes for encoded characters that are outside the charset supplied by the client.

Many other document formats, such as ‘application/postscript’ and ‘application/pdf’, are not charset-ambiguous, so that Printers that support only such formats MAY omit support of the “charset” Operation/ Description attributes.

If the Printer supports the "document-charset" Operation attribute, it MUST also support the corresponding "document-charset-default" and "document-charset-supported" Printer Description attributes as well and vice versa (see sections 7.1 and 7.2).

### document-digital-signature (type2 keyword)

This OPTIONAL Operation attribute identifies the technology of digital signature in the Document content.

If a client supplies this “document-digital-signature” Operation attribute in a Document Creation operation with a value that the Printer doesn’t support, the Printer MUST perform one of the following actions depending on implementation:  (1) reject the request OR (2) accept the request, set the Job's "job-state-reasons" attribute to 'digital-signature-type-not-supported' (see section 6.1), and either (a) put the Job in the 'pending-held' state or (b) process the job ignoring the attribute.  See section 3 above for more details.

If the Printer supports this attribute and the value supplied by the client, the Printer MAY verify the signature according to the rule for that signature format and “document-format”.  If the signature does not verify, then the Printer MUST perform one of the following actions depending on implementation (which MAY depend on the “document-format” and a site-established policy):

- ignore the signature (and MAY indicate on the printed output some how) and MUST set the Job's "job-state-reasons" Job Description attribute to the 'digital-signature-did-not-verify' value (see section 6.1).

- put the job on hold and wait for human intervention and MUST set the Job's "job-state-reasons" Job Description attribute to the 'job-digital-signature-wait' and 'digital-signature-did-not-verify' values (see section 6.1), OR

- abort the job depending on implementation and/or site configuration and MUST set the Job's "job-state-reasons" Job Description attribute to the 'job-aborted-by-system' and 'digital-signature-did-not-verify' values (see section 6.1).

Different PDLs have different techniques of embedding the signature.  If the Printer supports the "document-digital-signature" attribute and the supplied “document-format”, the Printer MUST accept the document.  However, if the Printer either doesn’t support the value of the “document-digital-signature’ Operation attribute or doesn’t support the technique used by the supplied “document-format” attribute, the Printer MUST either (1) ignore the signature or (2) put the job on hold and wait for human intervention, depending on implementation.  If the Printer doesn’t support the supplied value, the Printer MUST return the “document-digital-signature” attribute with its value as unsupported in the Unsupported attributes group as specified in [rfc2911] section 3.1.7.

The following standard keyword values are defined:

'dss':  Digital Signature Standard (DSS) [dss].

‘none’:  There is not a digital signature in the Document content.  If this attribute is supported, the ‘none’ value MUST be implemented but MAY be removed by the administrator..

'pgp':  PGP Message Exchange Formats [rfc1991].

‘smime':  Enhanced Security Services for S/MIME [rfc2634].

'xmldsig'  XML Digital Signature [xmldsig].

### document-format (mimeMediaType)

This REQUIRED Operation attribute (see [rfc2911] section 3.2.1.1) indicates the document format of the supplied Document Content.  If a client supplies the “document-format” Operation attribute with a value that the Printer doesn’t support, the Printer MUST reject the job and return the status code: ‘client-error-document-format-not-supported’ (see [rfc2911] section 3.1.4.11), since the Printer cannot process the data.

The standard values for this attribute are Internet Media types (sometimes called MIME types).  For some MIME types, such as ‘text/plain’ [rfc2046], the “; charset=” parameter is defined which specifies the charset of the Document content.  The Printer MUST support the charset MIME parameter as well as the “document-charset” Operation attribute (section 3.2.1) if the Printer supports any text MIME type, such as ‘text/plain’ or ‘text/html’.  For further details see the description of the 'mimeMediaType' attribute syntax in [rfc2911] section 4.1.9 and [rfc2046] and the “document-charset” Operation attribute (section 3.2.1).

Many printer languages in the Printer MIB do not have a MIME type registered with IANA.  It is the intent of the PWG to registers all of the Printer MIB v2 XXX Printer Languages as 'application/vnd.PWG-XXX'.  If a Printer Language already has a registered MIME Media type or gets a registration in the future, then that MIME Type MUST be used in preference to the 'application/vnd.PWG-XXX' MIME Type.  For example, PostScript has the MIME Media type: 'application/postscript' and PCL has the MIME Media Type: 'application/vnd.HP-PCL, so those MIME Media types MUST be used instead of 'application/vnd.PWG-PS' and 'application/vnd.PWG-PCL'.  For example, the NEC NPDL printer language from the Printer MIB does not have a MIME type registered with IANA.  Therefore, the “document-format” = ‘application/vnd.PWG-NPDL’ MUST be used.

Note:  CIP4 did not agree to register MIME types for other organizations and standards.  Instead, CIP4 will just add values to the flat file that aren’t MIME types.  The values will not start with any of the MIME type roots: application/, audio/, image/, message/, model/, multipart/, test/, or video.

ACTION ITEM (Tom and Ira): Propose a format for the file.

See also the “document-format-version” Operation attribute in section 3.2.5 and the “document-format-details” Operation attribute in section 3.2.4.

### document-format-details (1setOf collection)

This OPTIONAL Operation attribute supplied in Document Creation operations summarizes the details of the Document content, including any files contained in the Document.  This Operation attribute is intended to be supported by Printers that support a packaging document format, such as ‘application/zip’ or ‘multipart/related’, in order to detail the contents.  However, this Operation attribute MAY be used with any document format.  See Table 6 for a list of the member attributes along with the client and Printer conformance requirements.

All member attributes, if supplied, MUST have contents.  The value MUST NOT be omitted.

If a Printer supports a packaging MIME type, such as ‘multipart/related’ or ‘application/zip’, the Printer MUST support this “document-format-details” Operation attribute.  If a Printer does not support a packaging MIME type, it MAY omit support of the “document-format-details” Operation attribute and Description attribute.  The purpose of the “document-format-details” Operation attribute is to describe the details of the document format or formats in the supplied Document, including the packaging format itself, so that the Printer knows what to expect.

If a client supplies the “document-format-details” Operation attribute in a Document Creation request with member attributes and/or values that the Printer doesn’t support, the Printer MUST perform one of the following actions depending on implementation:  (1) reject the request OR (2) accept the request and either (a) put the Job in the 'pending-held' state or (b) process the job ignoring the attribute.  See section 3.2 above for more details.  However, if the  “document-charset” or the “document-format” member attributes are the ones in question, the Printer MUST reject the request, same as for the “document-charset” and “document-format” Operation attributes (see sections 3.2.1 and 3.2.3) and return the 'client-error-charset-not-supported'  (see [rfc2911] section 13.1.4.16) or the  ‘client-error-document-format-not-supported’ (see [rfc2911] section 13.1.4.11), respectively.

For a packaging format, the “document-format-details” attribute does not provide means to associate each collection value with particular document in the archive file, so it does not provide a “manifest” of the packaging.  One of the packaging collection values MAY describe the packaging format itself, in which case that collection value MUST be the first collection value.  The remaining collection values provides the summary of the details of the contained files in any order.

There MUST NOT be any duplicate collection values, that is, no collection values with all the same member attribute values;  it’s a set, not a sequence.  So 100 PostScript files with the same details in a .zip file:

(1) MAY have one collection value with the “document-format” member attribute = ‘application/zip’ MIME type value and MAY have additional member attributes that give details about the .zip file itself and

(2) MUST have one collection value containing a “document-format” member attribute = ‘application/postscript’ and MAY have additional member attributes that give the details about the PostScript files.

If a file in a packaging file is itself a packaging file, the single “document-format-details” (1setOf collection) Operation attribute SHOULD contain the flattened distinct collection values for all files at all nested levels.  Therefore, the “document-format-details” collection attribute is not recursively defined and MUST NOT contain the “document-format-details” attribute as a member attribute.

Table 6 - "document-format-details" member Operation attributes

| Member Attribute Name | Attribute Syntax | Client supplies | Printer Support | Member Attribute Section | Top Level Attribute Section |
| --- | --- | --- | --- | --- | --- |
| document-source-application-name | name(MAX) | MAY | MAY | 3.2.4.1 | N/A |
| document-source-application-version | text(127) | MAY | MAY | 3.2.4.2 | N/A |
| document-source-os-name | name(40) | MAY | MAY | 3.2.4.3 | N/A |
| document-source-os-version | text(40) | MAY | MAY | 3.2.4.4 | N/A |
| document-format | mimeMediaType | MUST | MUST | 3.2.4.5 | 3.2.3 |
| document-format-device-id | text(127) | MAY | MAY | 3.2.4.6 | N/A |
| document-format-version | text(127) | SHOULD | MUST | 3.2.4.7 | 3.2.5 |
| document-natural-language | 1setOf naturalLanguage | MAY | MAY | 3.2.4.8 | 3.2.8 |

The client can determine which member attributes of the “document-format-details” are supporter by querying the Printer’s “document-format-details-supported” (1setOf type2 keyword) Printer Description attribute (see section 7.6).

Table 6 shows that three of the member attributes also have comparable top level Operation attributes defined with the same names.  See the sections listed in the rightmost column.  These three high level attributes have corresponding “xxx-default” and “xxx-supported” Printer Description attributes.  These values are also for use with the corresponding three member attributes.  The remaining member attributes do not have any corresponding “xxx-default” and “xxx-supported” Printer Description attribute.

If a client supplies this “document-format-details” Operation attribute with a member attribute with a value that the Printer doesn’t support, the Printer MUST perform one of the following actions:  (1) reject the request OR (2) accept the request and either (a) put the Job in the 'pending-held' state or (b) process the job ignoring the attribute.  See section 3 above for more details.

#### document-source-application-name (name(MAX))

This OPTIONAL member Operation attribute identifies the application that generated the document.  The value MUST be all lowercase in order to facilitate matching.  The version number MUST NOT be included (see the "document-source-application-version" attribute).  Examples: “photoshop”, “microsoft word”, “unknown”.

#### document-source-application-version (text(127))

This OPTIONAL member Operation attribute identifies the version number of the application that created the document.  The value MUST be all lowercase in order to facilitate matching.  The intent of this attribute is for purposes of affecting the interpreting by the Printer for any formats for which the source application version might have different semantics.  Examples:

“8.1 (4331)” for Winzip, “5.0.5 10/26/2001” for Acrobat, “2000 (9.0.4119 sr-1)” for Microsoft Word

If the client omits this member attribute, that matches with any version.  Similarly, the Printer’s member attribute MAY be omitted or be a zero length string to indicate any.

#### document-source-os-name (name(40))

This OPTIONAL member Operation attribute identifies the name of the operating system on which the document was generated.  Valid values are the operating system names defined in the IANA Operating System name registry [os-names] with the version number portion removed (see the “document-source-os-version” attribute).  IANA Operating System Names consist of up to 40 uppercase US-ASCII letters, hyphen (“-“), period (“.”), and slash (“/”) characters.  The value MUST be all lowercase in order to facilitate matching.

Possible example IANA OS Registry values: ‘linux’, ‘macos’, ‘sun-os’, ‘unix’, “unknown”, ‘WINDOWS‘.

ACTION ITEM:  Get Sun to register SOLARIS with IANA.

#### document-source-os-version (text(40))

This OPTIONAL member Operation attribute identifies the version of the operating system on which the document was generated.  Valid values include the version portion of any of the operating system names defined in the IANA Registry [os-names].  The value MUST be all lowercase in order to facilitate matching.  The value MUST NOT include the name portion of the registered OS name (see “document-source-os-name” attribute).

Possible example values for the indicated “document-source-os-name” value are:

for ‘linux’:  ‘1.0’, ‘1.2’, ‘2,0’, ‘2.2’, ‘2.4’

for ‘unix’:  ‘bsd’, ‘v’, ‘v.1’, ‘v.2’, ‘v.3’, ‘pc’

for ‘windows’:  ‘95’, ‘98’, ‘ce’, ‘nt’, ‘nt-2’, ‘nt-3’, ‘nt-3.5’, ‘nt-3.51’, ‘nt-4’, ‘nt-5’ (windows 2000),

ACTION ITEM:  Get Microsoft to register NT-5.1 (maybe NT-5.2) with IANA.

#### document-format (mimeMediaType)

This REQUIRED member Operation attribute identifies the document format (see [rfc2911] §3.2.1.1) of the collection value of which it is a member.  The client MUST supply and the Printer MUST support this member attribute in each collection value.  The values for this member attribute are Internet Media types (sometimes called MIME types) and have the same values and semantics as for the top level “document-format” Operation attribute with the same name (see section 3.2.3 for the semantics).

#### document-format-device-id (text(127))

This OPTIONAL member Operation attribute identifies the type of device for which the document was formatted, including manufacturer and model.  This attribute is intended to identify document formats that are not portable, e.g., PDLs that are device dependent.  The value of this variable MUST exactly match the IEEE 1284-2000 Device ID string (see [IEEE1284] clause 6), except the length field MUST NOT be specified.  See the Microsoft Universal Plug and Play [upnp] section 2.2.6 DeviceId parameter for details and examples.  Here is an example showing only the required fields for a PostScript document:

MANUFACTURER:ACME Co.;COMMAND SET:PS;MODEL:LaserBeam 9;

#### document-format-version (text(127))

This REQUIRED member Operation attribute identifies the level or version of the document format identified by the “document-format” member attribute.  The client MAY supply and the Printer MUST support this member attribute if the Printer supports the “document-format-details” Operation attribute.  The semantics and supported values of this “document-format-version” member attribute MUST be the same as for the top level “document-format-version” Operation attribute with the same name (see section 3.2.5 for the semantics).

#### document-natural-language (1setOf naturalLanguage)

This OPTIONAL member Operation attribute specifies the natural language(s) of the collection value of which it is a member.  The values for this attribute are natural language codes (with optional country codes) and have the same values and semantics as for the top level “document-natural-language” (naturalLanguage) operation attribute with the same name (see section 3.2.8 for the semantics).  If the document contains multiple natural languages, then the first value MUST be the primary language of the document.

### document-format-version (text(127))

This OPTIONAL Operation attribute identifies the level or version of the document format of the Document content identified by the “document-format” Operation attribute (section 3.2.3).

If the client supplies this “document-format-version” Operation attribute, the client MUST also supply the “document-format” Operation attribute (section 3.2.3), so that the Printer does not need to parse the “document-format-version” string in order to determine the document format and version.

If a client supplies this “document-format-version” Operation attribute with a value that the Printer doesn’t support as indicated by the Printer's "document-format-version-supported" attribute (see section 7.8), the Printer MUST perform one of the following actions, depending on implementation:  (1) reject the request OR (2) accept the request and either (a) put the Job in the 'pending-held' state or (b) process the job ignoring the attribute.  See section 3 above for more details.

The defined values are taken from the Printer MIB [rfc1759] langTC values (after removing the “lang”) used in the prtInterpreterLangFamily and the prtInterpreterLangLevel attributes (SNMP calls them “objects”) separated by the “/” character.  If there is no version, just the langTC prefix without the “/” is used.  Note: the Printer MIB prtInterpreterLangVersion attribute is not used, since no standard values are given in the Printer MIB.

The short first field before the “/” character are unique, so that the version values are self-identifying so that all the versions for all document formats can be combined into a single list for the Printer’s “document-format-version-supported” Printer attribute.  The first field also tends to be the acronym for the document format and often is also the file name extension registered with IANA.  The Printer MIB shows the first field in uppercase for readability.  However, the Printer MUST perform case-insensitive matching of supplied values with supported values.  If the common acronym includes a “/”, such as TIFF/IT, the “/” is changed to “-“ so that the first field ends with the first “/.”

Example values:

‘PS/3’:  For Postscript level 3 [rfc1759].

‘PCL/5e’:  For PCL 5e [rfc1759].

‘PDF/1.4’:  For PDF version 1.4 [pdf]

‘PDF/X-1a:2001’:  For PDF/X-1a [iso15930]

‘DCS/2.0’  For Document Color Separation (DCS), version 2.0.  [dcs2.0]

‘TIFF-IT/FP:1998’:  TIFF/IT [iso12639] - Full Page - baseline.

‘TIFF-IT/CT:1998’:  TIFF/IT [iso12639] - Continuous Tone picture data - baseline

‘TIFF-IT/FP/P1:1998’:  TIFF/IT [iso12639] - Full Page - profile 1

‘TIFF-IT/CT/P1:1998’:  TIFF/IT [iso12639] - Continuous Tone picture data - profile 1

‘Automatic’:  auto sense the format [rfc1759].  Is use in combination with the ‘application/octet-stream’ (see [rfc2911] section 4.1.9.1) value of the “document-format” attribute (section 3.2.3).

#### Why no "document-format-version-default"

There is no “document-format-version-default” since to meaningful, the default would have to depend on the “document-format”.  Currently, there are no “xxx-default” attributes whose values depend on the value of other attributes.  Thus an implementation is free to supply its own defaults the depend on the “document-format”.

### document-message (text(MAX))

This OPTIONAL Operation attribute contains a message from the user to the operator about the Document.

### document-name (name(MAX))

The Printer MUST support this Operation attribute in Print-Job, Print-URI, Send-Document, and Send-URI operations (see [rfc2911 section 3.2.1.1) and MAY support it in the Create-Job operation.  For the Print-Job, Print-URI, and Create-Job operations, the Printer MUST copy the value to the corresponding Job Description attribute.

### document-natural-language (naturalLanguage)

This OPTIONAL Operation attribute specifies the primary natural language of the Document content (see [rfc2911] section 3.2.1.1 and section 4.1.8).

If the client supplies this "document-natural-language" Operation attribute with a value that the Printer doesn’t support, the Printer MUST perform one of the following actions:  (1) reject the request OR (2) accept the request and either (a) put the Job in the 'pending-held' state or (b) process the job ignoring the attribute.  See section 3 above for more details.

The values MAY include region as well as the language according to [rfc2911 section 4.1.8.  If the document contains more than one language, the value is the first or primary language of the document.  The Printer MAY use this value to select fonts or other Globalization processing.  As indicated in [rfc2911] section 4.1.8, example include:

'en':  for English

'en-us': for US English

'fr':  for French

'de':  for German

# Job Template attributes

Table 7 lists the new Job Template attributes defined in this specification.  These Job Template attributes can be supplied in the Job Creation operations: Print-Job, Print-URI, and Create-Job operations and the Validate-Job operation.

Job Template attributes are OPTIONAL for a Printer to support and for a client to supply in a Job Creation request.  If a Printer supports a Job Template attribute, then the Printer MUST copy the supplied attribute to the Job object, so that a client MAY query the attributes in subsequent Get-Job-Attributes/Get-Jobs operations (see [rfc2911] section 4.2).  In addition, as in [rfc2911], for each “xxx” Job Template attribute supported, the Printer MUST support the corresponding “xxx-default” and “xxx-supported” Printer attributes, if defined.

Table 7 - Job Template attributes

| Job Template attribute name "xxx" | Attribute Syntax | Printer: Default Value attribute syntax "xxx-default" | Printer: Supported Values attribute syntax "xxx-supported" |
| --- | --- | --- | --- |
| job-copies | integer(1:MAX) | integer(1:MAX) | rangeOfInteger(1:MAX) |
| job-cover-back | collection | collection | 1setOf type2 keyword |
| cover-type | type2 keyword | - | 1setOf type2 keyword |
| media | type3 keyword ¦ name(MAX) | - | 1setOf (type3 keyword ¦ name(MAX)) |
| media-col | collection | - | 1setOf type2 keyword |
| member attributes [pwg5100.3] | See [pwg5100.3] | - | - |
| job-cover-front | collection | collection | 1setOf type2 keyword |
| member attributes - same as job-cover-back | see job-cover-back |  |  |
| job-finishings | 1setOf type2 enum | 1setOf type2 enumError: Reference source not found | 1setOf type2 enum |
| job-finishings-col | collection | collection | 1setOf type2 keyword |
| finishing-template | name(MAX) | - | 1setOf name(MAX) |
| stitching | collection |  | 1setOf type2 keyword |
| stitching-reference-edge | type2 keyword | - | 1setOf type2 keyword |
| stitching-offset | integer (0:MAX) | - | 1setOf (integer(0:MAX) ¦ rangeOfInteger(0:MAX)) |
| stitching-locations | 1setOf integer(0:MAX) | - | 1setOf (integer(0:MAX) ¦ rangeOfInteger(0:MAX)) |

## Job Template attributes defined for Job-wide effect in multi-document jobs

This section defines additional Job Template attributes that have Job-wide effect and so are intended for use in the creation of multi-document Jobs.  These Job Template attributes have the "job-" prefix to indicate their Job wide scope.  When Clients supply these job-wide Job Template attributes, Client MUST supply these attributes in Job Creation and Validate-Job operations only.  Clients MUST NOT supply these Job Template attributes as Document Template attributes in the Send-Document and Send-URI operations ([ippdoc]) or in any other operations.  Clients MUST NOT supply these attributes as member attributes of the "overrides" Job Template attribute [ippOverride].

### job-copies (integer(1:MAX))

This OPTIONAL Job Template attribute specifies the number of job copies, i.e., with all of the Documents taken together.  Its interpretation is independent of the value of the “multiple-document-handling” Job Template attribute.  See the “copies” Job Template attribute defined in [rfc2911] section 4.2.5 which switches between job copies and document copies depending on the value of the “multiple-document-handling” Job Template attribute ([rfc2911] section 4.2.4).  The “job-copies” Job Template attribute is equivalent to “copies” with “multiple-document-handling” = ‘single-document-new-sheet’.

#### job-copies-default (integer(1:MAX))

This OPTIONAL Job Template Printer attribute specifies the default number of copies for the job as a whole.  The Printer MUST support this attribute if it supports the "job-copies" Job Template attribute.

#### job-copies-supported (rangeOfInteger(1:MAX))

This OPTIONAL Job Template Printer attribute specified the supported range of the number of copies for the job as a whole.  The Printer MUST support this attribute if it supports the "job-copies" Job Template attribute.

### job-cover-back (collection)

This OPTIONAL Job Template attribute specifies the cover for the back of the job, i.e., with all of the Documents taken together and is independent of the value of the “multiple-document-handling” Job Template attribute.  The member attributes are the same as “cover-back” (see [pwg5100.3] section 3.1) and with the same conformance requirements.  However, the “cover-back” Job Template attribute switches between a job back cover and a document back cover depending on the value of the “multiple-document-handling” Job Template attribute ([rfc2911] section 4.2.4).  The “job-cover-back” Job Template attribute is equivalent to “cover-back” with “multiple-document-handling” = ‘single-document-new-sheet’.

#### job-cover-back-default (collection)

This OPTIONAL Job Template Printer attribute specifies the default the cover for the back of the job.  The Printer MUST support this attribute if it supports the "job-cover-back" Job Template attribute.

#### job-cover-back-supported (1setOf type2 keyword)

This OPTIONAL Job Template Printer attribute specified the supported member attributes of the "job-cover-back" Job Template collection attributes.  The Printer MUST support this attribute if it supports the "job-cover-back" Job Template attribute.

### job-cover-front (collection)

This OPTIONAL Job Template attribute specifies the cover for the front of the job, i.e., with all of the Documents taken together and is independent of the value of the “multiple-document-handling” Job Template attribute.  The member attributes are the same as the “cover-front” Job Template attribute (see [pwg5100.3] section 3.2) and with the same conformance requirements.  However, the “cover-front” Job Template attribute switches between a job front cover and a document front cover depending on the value of the “multiple-document-handling” Job Template attribute ([rfc2911] section 4.2.4).  The “job-cover-front” Job Template attribute is equivalent to “cover-front” with “multiple-document-handling” = ‘single-document-new-sheet’.

#### job-cover-front-default (collection)

This OPTIONAL Job Template Printer attribute specifies the default the cover for the front of the job.  The Printer MUST support this attribute if it supports the "job-cover-front" Job Template attribute.

#### job-cover-front-supported (1setOf type2 keyword)

This OPTIONAL Job Template Printer attribute specified the supported member attributes of the "job-cover-front" Job Template attributes.  The Printer MUST support this attribute if it supports the "job-cover-front" Job Template collection attribute.

### job-finishings (1setOf type2 enum)

This OPTIONAL Job Template attribute specifies the job finishings, i.e., with all of the Documents taken together.  Its interpretation is independent of the value of the “multiple-document-handling” Job Template attribute.  See the “finishings” Job Template attribute defined in [rfc2911] section 4.2.6 which switches between job finishings and document finishings depending on the value of the “multiple-document-handling” Job Template attribute ([rfc2911] section 4.2.4).  The “job-finishings” Job Template attribute is equivalent to “finishings” with “multiple-document-handling” = ‘single-document-new-sheet’.

#### job-finishings-default (1setOf type2 enum)

This OPTIONAL Job Template Printer attribute specifies the default job finishings.  The Printer MUST support this attribute if it supports the "job-finishings" Job Template attribute.

#### job-finishings-ready (1setOf type2 enum)

This OPTIONAL Job Template Printer attribute specifies the ready job finishings, that is, the job finishings that maybe used without operation intervention.  The Printer MUST support this attribute if it supports the "job-finishings" Job Template attribute.

#### job-finishings-supported (1setOf type2 keyword)

This OPTIONAL Job Template Printer attribute specified the supported job finishings values.  The Printer MUST support this attribute if it supports the "job-finishings" Job Template attribute.

### job-finishings-col (1setOf collection)

This OPTIONAL Job Template attribute specifies the job finishings collection, i.e., with all of the Documents taken together.  Its interpretation is independent of the value of the “multiple-document-handling” Job Template attribute.  See the “finishings-col” Job Template attribute defined in [pwg5100.3] section 3.2 which switches between job finishings collection and document finishings collection depending on the value of the “multiple-document-handling” Job Template attribute ([rfc2911] section 4.2.4).  The “job-finishings-col” Job Template attribute is equivalent to “finishings-col” with “multiple-document-handling” = ‘single-document-new-sheet’.

#### job-finishings-col-default (collection)

This OPTIONAL Job Template Printer attribute specifies the default job finishings collection.  The Printer MUST support this attribute if it supports the "job-finishings-col" Job Template attribute.

#### job-finishings-col-ready (collection)

This OPTIONAL Job Template Printer attribute specifies the ready job finishings collection, that is, the job finishings collection that maybe used without operation intervention.  The Printer MUST support this attribute if it supports the "job-finishings-col" Job Template attribute.

#### job-finishings-col-supported (1setOf type2 keyword)

This OPTIONAL Job Template Printer attribute specified the supported member attributes of the "job-finishings-col" Job Template collection attribute.  The Printer MUST support this attribute if it supports the "job-finishings-col" Job Template attribute.

# Job Description attributes

This section defines the new Job Description defined in this IPP specification.  Job Description attributes are Job object attributes that are filled in by the Printer.  Some Job Description attribute values are supplied by the client in the Job operation using corresponding Operation attributes as indicated with “y”.  The Printer MUST NOT support modification of any of these Job Description attributes using the Set-Job-Attributes operation ([rfc3380]).

This section divides Job Descriptions into two subsections:  Section 5.1 contains Job Description attributes that describe the Job as a whole and Section 5.2 contains Job Description attributes that are defaults for the Document(s) in the Job.  The following legend is used in the tables in both sections:

| Legend: |  |
| --- | --- |
| IPP attribute syntax | the attribute syntax (see [rfc2911] section 4.1). |
| Set by | indicates whether the value is set by the client or Printer in the Job Creation operation: client - means that the Printer MUST populate this Job Description attribute by copying from the corresponding Operation attribute that the client MAY supply in the Job Creation operation. See Table 5 for the corresponding Operation attribute. Printer - means that the Printer sets the value based on the state of the Printer. |
| Printer conformance | indicates MUST or MAY for Printer conformance. |
| Reference | references to the sub-section in this document if there is significant information there. Those subsections that are only references to other specifications contain only that reference in the Reference column. |

## Job Description attributes that describe the Job as a whole

This section defines those Job Description attributes that describe the Job as a whole.  See section 5.2 below for those Job Description attributes that are defaults for the Document or Documents in the Job.

Table 8 - Job Description attributes that describe the Job as a whole

| Job Description attribute name | IPP Attribute Syntax | Set by | Printer conformance | Reference |
| --- | --- | --- | --- | --- |
| errors-count | integer(0:MAX) | Printer | MAY | [this spec] §5.1.1 |
| ipp-attribute-fidelity | boolean | client | MAY | [this spec] §5.1.2 |
| job-mandatory-attributes | 1setOf type2 keyword | client | MAY | [this spec] §5.1.3 |
| output-device-requested | name(127) | client | MAY | [this spec] §5.1.4 |
| output-device-assigned | name(127) | Printer | MAY | [rfc2911] §4.3.7.2 |
| warnings-count | integer(0:MAX) | Printer | MAY | [this spec] §5.1.5 |

Most of the existing “job-state” and “job-state-reasons” Job Description attribute values apply equally well to individual documents in a multi-document job (with removal of the ‘job-’ prefix for those that apply to both Job and Document).  Furthermore, the job states are completely service-type neutral, so they can apply to scanning. faxing, formatting processes as well.  The job state reasons have a number of reasons that are print service specific, but most can apply to other types of service as well.  The reason for separating state from reasons, is so that the states can be the fundamental states that most implementations have, while the state reasons can vary depending on the sophistication of the system.  The states are the ones that clients are most likely to base actions upon, and the state reasons are more information, both for programs and people.

### errors-count (integer(0:MAX))

This OPTIONAL Job Description attribute counts the number of errors encountered processing the job as a whole.  An error where some portion of the printed output is not present, such as a glyph encountered which isn't in the Printer's font or an image in a page was in a format that the Printer didn't support, though the Printer did support the overall document format.

### ipp-attribute-fidelity (boolean)

This OPTIONAL Job Description attribute indicates the value of the corresponding “ipp-attribute-fidelity” Operation attribute ([rfc2911] section 3.2.1.1) that the client supplied in the Job Creation operation.  This Job Description attribute has no effect on the processing of the job once the Job Creation operation has been accepted.  However, this attribute does provide an audit capability to the user and service desk for comparing unexpected printed results with what was submitted.

### job-mandatory-attributes (1setOf type2 keyword)

This OPTIONAL Job Description attribute indicates which Job Template attributes that the client required that the Printer Support in order to accept the Job Creation request.  The Printer populates this attribute with the value of the corresponding “job-mandatory-attributes” Operation attribute (see section 3.1.2) supplied by the client in the Job Creation operation.  This Job Description attribute has no effect on the processing of the job once the Job Creation operation has been acceptedError: Reference source not found.   However, this Job Description attribute does provide an audit capability of the Job object in the Job History (see [rfc2911] section 4.3.7.2) to the user and service desk for comparing unexpected printed results with what was submitted.  See section  3.1.2 for additional details of the semantics of this “job-mandatory-attributes” Operation attribute that are not repeated here.

### output-device-requested (name(127))

This OPTIONAL Job Description attribute indicates the name of the output device that the client has supplied in the "output-device-requested" Operation attribute (see section 3.1.3), if any, in the Job Creation operation.  The Printer sets the Job's "output-device-assigned" Job Description attribute (see [rfc2911] section ) to the name of the actual device assigned.

### warnings-count (integer(0:MAX))

This OPTIONAL Job Description attribute counts the number of warnings encountered processing the job as a whole.  A warning is where no information is lost, but the appearance of the printed output MAY not be as expected.

## Job Description attributes that are defaults for the Document(s) in the Job

This section defines those Job Description attributes that provide defaults for the Document Or Documents in the Job.  See section 5.1 above for those Job Description attributes that describe the Job as a whole.

### compression (type3 keyword)

This REQUIRED Job Description attribute specifies the default compression used to represent the document(s) in the Job.  The Printer sets this Job Description attribute from the corresponding Operation attribute (see [rfc2911] §3.2.1.1) supplied by the client in the Job Creation operation.

### document-charset (charset)

This OPTIONAL Job Description attribute specifies the default charset used to represent this document (see section 3.2.1).  The Printer sets this Job Description attribute from the corresponding Operation attribute supplied by the client in the Job Creation operation.

### document-digital-signature (type2 keyword)

This OPTIONAL Job attribute specifies the default digital signature technology used in the Document content.  The Printer sets this Job Description attribute from the corresponding “document-digital-signature” Operation attribute (see section 3.2.2 for the semantics), if supplied by the client in the Job Creation operation.  If a Printer supports the “document-digital-signature” Operation attribute, the Printer MUST support this “document-digital-signature” Job Description attribute.

### document-format (mimeMediaType)

This REQUIRED Job Description attribute specified the default document format of the Document Object content.  The Printer sets this Job Description attribute from the corresponding “document-format” Operation attribute (see [rfc2911] section 3.2.1.1 and [this spec] section 3.2.3 for the semantics), if supplied by the client in the Job Creation operation.

### document-format-details (1setOf collection)

This OPTIONAL Job Description attribute specifies the default details of the document format(s) of the Document Object content.  The Printer sets this Job Description attribute from the corresponding “document-format-details” Operation attribute (see section 3.2.4 for the semantics), if supplied by the client in the Job Creation operation.  If a Printer supports the “document-format-details” Operation attribute, the Printer MUST support this “document-format-details” Job Description attribute.

### document-format-version (text(127))

This OPTIONAL Job Description attribute specifies the default version of the document format of the Document Object content.  The Printer sets this Job Description attribute from the corresponding “document-format-version” Operation attribute (see section 3.2.5 for the semantics), if supplied by the client in the Job Creation operation (see section ).  If a Printer supports the “document-format-version” Operation attribute, the Printer MUST support this “document-format-version” Job Description attribute.

### document-message (text(MAX))

This OPTIONAL Job Description contains either (1) the default  message from the user to the operator about the Document(s) or (2) a message from the operator, system administrator, or "intelligent" process to indicate to the end user the reasons for modification or other management action taken on the Document.  The Printer populates this Job Description attribute from the “document-message” (text(MAX) Operation attribute value (see section 3.2.6) supplied by the client in the Job Creation operation.

In addition, the user and the operator can set the value of this Job Description attribute explicitly using the Set-Job-Attributes operation (see [rfc3380]).

### document-name (name(MAX))

This REQUIRED Job Description attribute specifies the default name of the document (see [rfc2911] §3.2.1.1).  The Printer sets this Job Description attribute from the corresponding Operation attribute (see section 3.2.7) supplied by the client in the Job Creation operation.

### document-natural-language (naturalLanguage)

This OPTIONAL Job Description attribute specifies the default primary natural language of the Document content (see [rfc2911] section 3.2.1.1).  The Printer sets this Job Description attribute from the corresponding Operation attribute (see section 3.2.8) supplied by the client in the Job Creation operation.

# New values to existing attributes

This section defines new values for existing attributes.

## New job-state-reasons(1setOf type2 keyword) Values

The REQUIRED “job-state-reasons” Job Description attributes ([rfc2911] section 4.3.8) provide additional information about the job's current state, i.e., information that augments the value of the job's "job-state" attribute.

These new values MAY be used with any job state for which the reason makes sense.  When implemented, the Printer MUST return these values when the reason applies and MUST NOT return them when the reason no longer applies whether the value of the Job's "job-state" attribute changed or not.  When the Job does not have any reasons for being in its current state, the value of the Job's "job-state-reasons" attribute MUST be 'none'.

Note: While values cannot be added to the “job-state” attributes without impacting deployed clients that take actions upon receiving "job-state" values, it is the intent that additional "job-state-reasons" values can be defined and registered without impacting such deployed clients.  In other words, the "job-state-reasons" attribute are intended to be extensible.

Table 9 lists the new keyword values for the “job-state-reasons” attribute defined in this specification as indicated in the References column.  In Table 9 the keywords are listed in alphabetic order, while in their descriptions following Table 9, they are listed in the most likely order of occurrence to help understanding.

Table 9 - New "job-state-reasons" attribute values

| IPP Job Description attribute name | IPP Attribute Syntax plus Values | Printer conformance | Reference |
| --- | --- | --- | --- |
| job-state-reasons | 1setOf type2 keyword | MUST | [rfc2911] §4.3.8 |
|  | digital-signature-did-not-verify | MAY | [this spec] §6.1 |
|  | digital-signature-type-not-supported | MAY | [this spec] §6.1 |
|  | errors-detected | MAY | [this spec] §6.1 |
|  | job-digital-signature-wait | MAY | [this spec] §6.1 |
|  | warnings-detected | MAY | [this spec] §6.1 |

For ease of understanding, the following value descriptions are presented in the order in which the reasons are likely to occur (if implemented):

'digital-signature-type-not-supported: One or more documents in the Job contain a digital signature type that is not supported.  Implementation or site policy determines the state of the Job once this condition is detected.

'job-digital-signature-wait': The Job is currently being held because the Job was submitted with a “document-digital-signature” attribute value that is not supported.

'digital-signature-did-not-verify': One or more documents in the Job contains a digital signature type that is supported (see section 3.2.2), but the signature did not verify when the Printer attempted to verify the signature.

'errors-detected' : If the Printer supports this value, the Printer MUST add this value to “job-state-reasons” when the Printer generates the first error message. That is, a single occurrence of this value is present in the “job-state-reasons” Job Description attribute if the Printer has generated one or more errors.  An error indicates loss of information, such as a page not imaged.  See also the ‘warnings-detected’ value.

'warnings-detected' : If the Printer supports this value, the Printer MUST add this value to “job-state-reasons” when the Printer generates the first warning message. That is, a single occurrence of this value is present in the “job-state-reasons” Job Description attribute if the Printer has generated one or more warnings.  A warning indicates something did not go as expected, but there was no loss of information.  For example, a font substitution.  See also the ‘errors-detected’ value.

## New media (type 3 keyword | name(MAX)) Values

This section specifies one new value for the "media" Job Template attribute:

'choice_iso_a4_210x297mm_na_letter_8.5x11in':  The client has explicitly allowed a choice of media for the Job.  The Printer MUST choose physical media of size na_letter_8.5x11in (215.9 x 279.4 mm) or iso_a4_210x297mm.  The Printer MUST NOT choose physical media of any other size.  The Printer MAY scale the image to fit, but any scaling MUST be isomorphic scaling and without image content loss, or overflow to additional sheets (so-called tiling), or truncate (and MUST notify the sending user and receiving user by implementation means if truncation is performed, since that is loss of information).

Rationale:  If printing by reference, the size of the document isn’t known by the submitter.

See Annex A (section 17) for additional requirements for a revision of the PWG IEEE/ISTO 5101.1 Candidate Standard [pwg5101.1] to include the new 'choice_iso_a4_210x297mm_na_letter_8.5x11in' and other choice values.

# New Printer Description attributes

This section lists the new Printer Description attributes defined in this document.  These Printer attributes are “xxx-supported” and “xxx-default” Printer attributes which indicate the default and supported values that the Printer supports..  The Printer MUST support these Printers attributes if the Printer supports the corresponding “xxx” Operation attributes (see section 3) that the client can supply in a Job Creation or Document Creation request.

The column labels are:

| Column Heading |  |
| --- | --- |
| “Attribute Syntax” | the attribute syntax (see [rfc2911] section 4.1). Many “xxx-default” and “xxx-supported” attributes have the ‘collection’ syntax which consists of a number of member attributes. These member attributes are not listed in this table when they are the same as the corresponding “xxx” attribute in Table 7. Only those few Printer attribute collection attributes that do not have a corresponding Job Template attributes show the member attributes in this table. |
| “Reference” | the section which defines the semantics of the attribute. |

Table 10 - New Printer Description attributes

| Printer Description attribute name | Attribute Syntax |  | Reference |
| --- | --- | --- | --- |
| document-charset-default | charset |  | [this-spec] §7.1 |
| document-charset-supported | 1setOf charset |  | [this-spec] §7.2 |
| document-digital-signature-default | type2 keyword |  | [this-spec] §7.3 |
| document-digital-signature-supported | 1setOf type2 keyword |  | [this-spec] §7.4 |
| document-format-details-default | collection |  | [this-spec] §7.5 |
| document-format-details-supported | 1setOf type2 keyword |  | [this-spec] §7.6 |
| document-format-version-default | text(127) |  | [this-spec] §7.7 |
| document-format-version-supported | 1setOf text(127) |  | [this-spec] §7.8 |
| document-natural-language-default | naturalLanguage |  | [this-spec] §7.9 |
| document-natural-language-supported | 1setOf naturalLanguage |  | [this-spec] §7.10 |
| output-device-requested-supported | 1setOf name(127) |  | [this spec] §7.12 |

## document-charset-default (charset)

This OPTIONAL Printer Description attribute indicates the default charset that the Printer will assume for charset-ambiguous document formats when the client omits the “document-charset” Operation attribute (section 3.2.1).  The Printer MUST support this attribute if it supports the corresponding “document-charset” Operation attribute (see section 3.2.1)

## document-charset-supported (1setOf charset)

This OPTIONAL Printer Description attribute indicates the charsets that the Printer supports in Document Content, that is, supports as values of the “document-charset” Operation attribute (section 3.2.1).  The Printer MUST support this attribute if it supports the corresponding “document-charset” Operation attribute (see section 3.2.1).

## document-digital-signature-default (type2 keyword)

This OPTIONAL Printer Description attribute indicates the default digital signature, if any, that the Printer will assume when the client omits the “document-digital-signature” Operation attribute (section 3.2.2).  The Printer MUST support this attribute if it supports the “document-digital-signature” Operation attribute.

## document-digital-signature-supported (1setOf type2 keyword)

This OPTIONAL Printer Description attribute indicates the digital signature technologies that the Printer supports in Document Contents, that is, supports as values of the “document-digital-signature” Operation attribute (section 3.2.2).  The Printer MUST support this attribute if it supports the “document-digital-signature” Operation attribute.

## document-format-details-default (collection)

This OPTIONAL Printer Description attribute indicates the default document format details that the Printer will assume when the client omits the “document-format-details” Operation attribute (section §3.2.4).  The member attributes are the same as “document-format-details” (see section 3.2.4). The Printer MUST support this attribute, if it supports the “document-format-details” Operation attribute (see section 3.2.4).

## document-format-details-supported (1setOf type2 keyword)

This OPTIONAL Printer Description attribute lists the type2 keyword names of the member attributes of “document-format-details” that the Printer supports.  The Printer MUST support this “document-format-details” (1setOf type2 keyword) Printer Description attribute, if the Printer supports the “document-format-details” (1setOf collection) Operation attribute (see section 3.2.4).  Table 11 lists the type2 keywords defined for use in the “document-format-details-supported” Printer attribute.

Table 11 - Keyword values of "document-format-details-supported Printer attribute

| Type2 keyword values | Printer conformance |
| --- | --- |
| document-source-application-name | MAY |
| document-source-application-version | MAY |
| document-source-os-name | MAY |
| document-source-os-version | MAY |
| document-format | MUST |
| document-format-device-id | MAY |
| document-format-version | MUST |
| document-natural-language | MAY |

## document-format-version-default (text(127))

This OPTIONAL Printer Description attribute indicates the default document format version that the Printer will assume when the client omits the “document-format-version” Operation attribute (section 3.2.5).  The Printer MUST support this attribute if it supports the “document-format-version” Operation attribute.

## document-format-version-supported (1setOf text(127))

This OPTIONAL Printer Description attribute indicates the document format versions that the Printer supports for Document Contents, that is, supports as values of the “document-format-version” Operation attribute (section 3.2.5).  The Printer MUST support this attribute if it supports the “document-format-version” Operation attribute.

## document-natural-language-default (naturalLanguage)

This OPTIONAL Printer Description attribute indicates the default document natural language that the Printer will assume for Document Content when the client omits the “document-natural-language” Operation attribute ([rfc2911] section 3.2.1.1).  The Printer MUST support this attribute if it supports the “document-natural-language” Operation attribute.

## document-natural-language-supported (1setOf naturalLanguage)

This OPTIONAL Printer Description attribute lists the natural languages supported in any of the supported document formats, that is, it is the union of the natural languages.  The Printer MUST support this attribute if it supports the “document-natural-language” Operation attribute ([rfc2911] section 3.2.1.1).

## output-device-default (name(127))

This OPTIONAL Printer Description attribute is the default for both the “output-device-requested” Job Description and the “output-device-assigned” Job Description attribute.  A Printer MAY support the “output-device-default” Printer attribute if the Printer’s scheduling algorithm has a default output device.  On the other hand, a Printer MAY have a dynamic scheduling algorithm in which case this attribute is not supported.

## output-device-supported (1setOf name(127))

This OPTIONAL Printer Description attributes lists the names of output devices that the Printer supports.  The Printer MUST support this attribute if it supports the "output-device-requested" Operation attribute (see section 3.1.3).

# Conformance Requirements

This specification contains a number of independent OPTIONAL attributes.  This section summarizes the conformance requirements for Printers and clients.

## Printer conformance requirements

In order to claim conformance to this specification, a Vendor MUST explicitly identify the attributes and values supported by the Printer implementation in any such claim of conformance.  In addition, the Vendor's Printer implementation:

- MUST meet the conformance requirements for Printers specified in [rfc2911].

- MAY support any OPTIONAL Operation attribute according to section 3 and the corresponding Job Description attributes in section 5 and corresponding Printer attributes in section 7.

- MAY support any Job Template attribute according to section 4.

- MAY support any of the "job-state-reasons" values according to section 6.

## Client conformance requirements

In order to claim conformance to this specification, a Vendor MUST explicitly identify the attributes and values supported by the client implementation in any such claim of conformance.  In addition, the Vendor's client:

- MUST meet the conformance requirements for clients specified in [rfc2911]

- MAY support any Operation attribute according to section 3 and the corresponding Job Description attributes in section 5 and corresponding Printer attributes in section 7.

- MAY support any Job Template attribute according to section 4.

# Normative References

[dcs2.0]

Document Color Separation (DCS), version 2.0.  Available at:  http://www.npes.org/standards/Tools/DCS20Spec.pdf

[dss]

U.S. Department of Commerce, "Digital Signature Standard (DDS)", Federal Information Processing Standards Publication 186-1 (FIPS PUB 186-1), December 15, 1998.

[ippdoc]

Hastings, T., and P. Zehler, "Internet Printing Protocol (IPP): Document Object", October 14, 2002, ftp://ftp.pwg.org/pub/pwg/ipp/new_DOC/wd-ippdoc10-20030519.doc, .pdf, .rtf  <work in progress>

[ippOverride]

Zehler, P., Herriot, R., and K. Ocke,  "Internet Printing Protocol (IPP): Page Overrides", May 27, 2003, ftp://ftp.pwg.org/pub/pwg/ipp/new_EXC/wd-ippOverride10-20030527.doc, .pdf, .rtf,  <work in progress>

[ippjobx] - this specification

Hastings, T., and P. Zehler, “Internet Printing Protocol (IPP): Job Extensions”, May 30, 2003, ftp://ftp.pwg.org/pub/pwg/ipp/new_JOBX/wd-ippjobx10-20030530.doc, .pdf, .rtf,  <work in progress>.

[iso12639]

ISO 12639:1998 Graphic technology -- Prepress digital data exchange -- Tag image file format for image technology (TIFF/IT), 1998.

[iso15930]

ISO 15930-1:2001 Graphic technology -- Prepress digital data exchange -- Use of PDF -- Part 1: Complete exchange using CMYK data (PDF/X-1 and PDF/X-1a), 2001.

[pdf]

“Adobe Portable Document Format”,  Version 1.4, PDF Reference, Third Edition, Adobe Systems Incorporated, http://www.adobe.com, December 2001.

[pwg5100.4]

Herriot, R., Ocke, K., "Internet Printing Protocol (IPP): Override Attributes for Documents and Pages", IEEE-ISTO 5100.4:2001, February 7, 2001, ftp://ftp.pwg.org/pub/pwg/standards/pwg5100.4.pdf. - being obsoleted by (1) a new Page Overrides specification which removes the "document-overrides" Job Template attribute and Operation attribute in favor of using the Document Object [ippdoc] and (2)  a clarified definition of “pages-per-subset” Job Template attribute [ippfutures].

[pwg5100.3]

Ocke, K., Hastings, T., "Internet Printing Protocol (IPP): Production Printing Attributes - Set1", IEEE-ISTO 5100.3-2001, February 12, 2001, ftp://ftp.pwg.org/pub/pwg/standards/pwg5100.3.pdf.

[rfc1991]

D. Atkins, W. Stallings, P. Zimmermann, "PGP Message Exchange Formats", RFC 1991, August, 1996.

[rfc2046]

Freed, N., and N. Borenstein, “Multipurpose Internet Mail Extensions (MIME) Part Two: Media Types”, RFC 2046, November 1996.

[rfc2634]

P. Hoffman, "Enhanced Security Services for S/MIME", RFC 2634, June 1999.

[rfc2854]

Connolly, D., and L. Masinter, “The 'text/html' Media Type”, RFC 2854, June 2000.

[rfc2911]

R. deBry, T. Hastings, R. Herriot, S. Isaacson, P. Powell, "Internet Printing Protocol/1.1: Model and Semantics", RFC 2911,  September 2000.

[rfc3380]

Hastings, T., Herriot, R., Kugler, C., and H. Lewis, "Internet Printing Protocol (IPP): Job and Printer Set Operations", RFC 3380, September 2002.

[rfc3382]

deBry, R., Hastings, T., Herriot, R., Ocke, K., and P. Zehler, "Internet Printing Protocol (IPP): The ‘collection’ attribute syntax", RFC 3382, September 2002.

[upnp]

Microsoft, “Basic PrintBasic:1.0 Service Template Version 1.01 For Universal Plug and Play Version 1.0”

[xmldsig]

D. Eastlake, J. Reagle, D. Solo "XML-Signature Syntax and Processing", <draft-ietf-xmldsig-core-11.txt>, October 31, 2000.

# Informative References

[fsg-jt]

Free Software Group, Open Printing Job Ticket API, Visit: http://www.freestandards.org/openprinting/

[ippact]

Carney, D., and H. Lewis, “Internet Printing Protocol (IPP), “-actual” attributes extension”, work in progress, January 31, 2003, ftp://ftp.pwg.org/pub/pwg/ipp/new_ACT/pwg-ipp-actual-attrs-v04-030131.pdf

[ippfax]

Moore, P., Songer, G., Hastings, T., McDonald, Pulera, and M. Joel, “IPP FAX Protocol”, July 26, 2001, ftp://ftp.pwg.org/pub/pwg/QUALDOCS/pwg-ifx-ippfax-P16-030423.pdf.

[ippfutures]

Work in progress to contain attributes not needed immediately.  No draft is currently available.

[pwg-psi]

“Print Service Interface (PSI)”, work in progress to become a PWG IEEE-ISTO standard.  Visit http://www.pwg.org/.

[pwg-sm]

Zehler, P., and S. Albright, “PWG Semantic Model”, work in progress to become a PWG IEEE-ISTO standard, version 0.11, September 27, 2002.  ftp://ftp.pwg.org/pub/pwg/Semantic-Model/PWG-Semantic-Model.pdf

[rfc2565]

Herriot, R., Butler, S., Moore, P., and R.  Turner, "Internet Printing Protocol/1.0: Encoding and Transport", RFC 2565, April 1999.

[rfc2566]

R.  deBry, T.  Hastings, R.  Herriot, S.  Isaacson, and P.  Powell, "Internet Printing Protocol/1.0: Model and Semantics", RFC 2566, April 1999.

[rfc2567]

Wright, D., "Design Goals for an Internet Printing Protocol", RFC 2567, April 1999.

[rfc2568]

Zilles, S., "Rationale for the Structure and Model and Protocol for the Internet Printing Protocol", RFC 2568, April 1999.

[rfc2569]

Herriot, R., Hastings, T., Jacobs, N., Martin, J., "Mapping between LPD and IPP Protocols", RFC 2569, April 1999.

[rfc3196]

Hastings, T., Manros, C., Zehler, P., Kugler, C., and H. Holst, “Internet Printing Protocol/1.1: Implementer’s Guide”, RFC 3196, November 2001.

[rfc3239]

Kugler, C., Lewis, H. and T. Hastings, “Internet Printing Protocol (IPP): Requirements for Job, Printer, and Device Administrative Operations”, RFC 3239, February 2002.

# IANA Considerations

This section contains the registration information for IANA to add to the various IPP Registries according to the procedures defined in RFC 2911 [rfc2911] section 6 to cover the definitions in this document.  The resulting registrations will be published in the http://www.iana.org/assignments/ipp-registrations registry.

Note to the PWG Editor:  Replace the References below to [this spec]  with the IEEE-ISTO standard number  for the appropriate document, so that it accurately reflects the content of the information for the IANA Registry.

## Attribute Registrations

The following table lists all the attributes defined in this specification.  These are to be registered according to the procedures in RFC 2911 [rfc2911] section 6.2.  If the attribute has a separate sub-section definition in this specification, that sub-section is listed as the Reference.  This table contains the reference to another specification that completely defines each attribute.

Attribute Name (attribute syntax)                Reference    Section

----------------------------------               ----------   --------

Job Description attributes:

compression (type3 keyword)                      [this spec]  5.2.1

document-charset (charset)                       [this spec]  5.2.2

document-digital-signature (type2 keyword)       [this spec]  5.2.3

document-format (mimeMediaType)                  [this spec]  5.2.4

document-format-details (1setOf collection)      [this spec]  5.2.5

document-source-application-name (name(MAX))   [this spec]   5.2.5

document-source-application-version (text(127))

[this spec]   5.2.5

document-source-os-name (name(40))             [this spec]   5.2.5

document-source-os-version (text(40))          [this spec]   5.2.5

document-format (mimeMediaType)                [this spec]   5.2.5

document-format-device-id (text(127))          [this spec]   5.2.5

document-format-version (text(127)             [this spec]   5.2.5

document-natural-language (1setOf naturalLanguage)

[this spec]   5.2.5

document-format-version (text(127))              [this spec]  5.2.6

document-message (text(MAX))                     [this spec]  5.2.7

document-name (name(MAX))                        [this spec]  5.2.8

document-natural-language (naturalLanguage)      [this spec]  5.2.9

errors-count (integer(0:MAX))                    [this spec]  5.1.1

job-attribute-fidelity (boolean)                 [this spec]  5.1.2

job-mandatory-attributes (1setOf type2 keyword)  [this spec]  5.1.3

output-device-requested(name(127))               [this spec]  5.1.4

warnings-count (integer(0:MAX))                  [this spec]  5.1.5

Job Template attributes:

job-copies (integer(1:MAX))                      [this spec]  4.1.1

job-copies-default (integer(1:MAX))              [this spec]  4.1.1.1

job-copies-supported (rangeOfInteger(1:MAX))     [this spec]  4.1.1.2

job-cover-back (collection)                      [this spec]  4.1.2

<member attributes are the same as “cover-back”[pwg5100.3]  3.1>

job-cover-back-default (collection)              [this spec]  4.1.2.1

<member attributes are the same as “cover-back”[pwg5100.3]  3.1>

job-cover-back-supported (1setOf type2 keyword)  [this spec]  4.1.2.2

job-cover-front (collection)                     [this spec]  4.1.3

<member attributes are the same as “cover-front”[pwg5100.3] 3.1>

job-cover-front-default (collection)             [this spec]  4.1.3.1

<member attributes are the same as “cover-front”[pwg5100.3] 3.1>

job-cover-front-supported (1setOf type2 keyword) [this spec]  4.1.3.2

job-finishings (1setOf type2 enum)               [this spec]  4.1.4

job-finishings-default (1setOf type2 enum)       [this spec]  4.1.4.1

job-finishings-ready (1setOf type2 enum)         [this spec]  4.1.4.2

job-finishings-supported (1setOf type2 keyword)  [this spec]  4.1.4.3

job-finishings-col (1setOf collection)           [this spec]  4.1.5

<member attributes are the same as “finishings-col”[pwg5100.3] 3.2>

job-finishings-col-default (1setOf type2 enum)   [this spec]  4.1.5.1

job-finishings-col-ready (1setOf type2 enum)     [this spec]  4.1.5.2

job-finishings-col-supported (1setOf type2 keyword)

[this spec]  4.1.5.3

Printer Description attributes:

document-charset-default (charset)                    [this spec]  7.1

document-charset-supported (1setOf charset)           [this spec]  7.2

document-digital-signature-default (type2 keyword)    [this spec]  7.3

document-digital-signature-supported (1setOf type2 keyword)

[this spec]  7.4

document-format-details-default (collection)          [this spec]  7.5

<member attributes are the same as “document-format-details”

[this spec] 3.2.4>

document-format-details-supported (1setOf type2 keyword)

[this spec]  7.6

document-format-version-default (text(127))           [this spec]  7.7

document-format-version-supported (1setOf text(127))  [this spec]  7.8

document-natural-language-default (naturalLanguage)   [this spec]  7.9

document-natural-language-supported (1setOf naturalLanguage)

[this spec]  7.10

output-device-requested-supported (1setOf name(127))  [this spec]  7.12

Operation attributes:

document-charset (charset)                       [this spec]   3.2.1

document-digital-signature (type2 keyword)       [this spec]   3.2.2

document-format-details (1setOf collection)      [this spec]   3.2.4

document-source-application-name (name(MAX))   [this spec]   3.2.4.1

document-source-application-version (text(127))

[this spec]   3.2.4.2

document-source-os-name (name(40))             [this spec]   3.2.4.3

document-source-os-version (text(40))          [this spec]   3.2.4.4

document-format (mimeMediaType)                [this spec]   3.2.4.5

document-format-device-id (text(127))          [this spec]   3.2.4.6

document-format-version (text(127)             [this spec]   3.2.4.7

document-natural-language (1setOf naturalLanguage)

[this spec]   3.2.4.8

document-format-version (text(127))              [this spec]   3.2.5

document-natural-language (naturalLanguage)      [this spec]   3.2.8

job-mandatory-attributes (1setOf type2 keyword)  [this spec]   3.1.2

output-device-requested (name(127))              [this spec]   3.1.3

## Keyword attribute values

document-digital-signature (type2 keyword)       [this spec]   3.2.2

document-digital-signature-default (type2 keyword)

[this spec]   7.3

document-digital-signature-supported (1setOf type2 keyword)

[this spec]   7.4

dss                                            [this spec]   3.2.2

none                                           [this spec]   3.2.2

pgp                                            [this spec]   3.2.2

smime                                          [this spec]   3.2.2

xmldsig                                        [this spec]   3.2.2

document-format-details-supported (1setOf type2 keyword)

[this spec]   7.6

document-source-application-name               [this spec]   7.6

document-source-application-version            [this spec]   7.6

document-source-os-name                        [this spec]   7.6

document-source-os-version                     [this spec]   7.6

document-format                                [this spec]   7.6

document-format-device-id                      [this spec]   7.6

document-format-version                        [this spec]   7.6

document-natural-language                      [this spec]   7.6

job-cover-back-supported (1setOf type2 keyword)  [this spec]   4.1.2.2

<same as “cover-back-supported”                [pwg5100.3]   3.1.4

job-cover-front-supported (1setOf type2 keyword) [this spec]   4.1.2.2

<same as “cover-front-supported”               [pwg5100.3]   3.1.4

job-mandatory-attributes (1setOf type2 keyword)  [this spec]   3.1.2

<any Job Template attribute keyword>

job-state-reasons (1setOf type2 keyword)         [rfc2911]     4.3.8

digital-signature-did-not-verify               [this spec]   6.1

digital-signature-type-not-supported           [this spec]   6.1

errors-detected                                [this spec]   6.1

job-digital-signature-wait                     [this spec]   6.1

warnings-detected                              [this spec]   6.1

media (type2 keyword | name(MAX))                [rfc2911]     4.2.11

choice_iso_a4_210x297mm_na_letter_8.5x11in     [this spec]   6.2

## Enum Attribute Values

The following table lists all the new enum attribute values defined in this document as additional type2 enum values.  These are to be registered according to the procedures in RFC 2911 [rfc2911] section 6.1.

job-finishings (1setOf type2 enum)               [this spec] 4.1.4

job-finishings-default (1setOf type2 enum)       [this spec] 4.1.4.1

job-finishings-ready (1setOf type2 enum)         [this spec] 4.1.4.2

job-finishings-supported (1setOf type2 enum)     [this spec] 4.1.4.3

<enum values are the same as “finishings”>

job-finishings-col-supported (1setOf type2 keyword)

[this spec] 4.1.5.3

<same as for “finishing-col-supported”         [pwg5100.3] 3.2.5

# Internationalization Considerations

The internationalization considerations for the extensions defined in this specification are the same as for handling the Job object in [rfc2911].

# Security Considerations

This specification adds no more security burden on Printer implementations than does [rfc2911].

# Contributors

Don Fullman, Xerox

Ira McDonald, High North

Dennis Carney, IBM

# Acknowledgments

# Author’s Address

Tom Hastings

Xerox Corporation

737 Hawaii St.

El Segundo, CA 90245

Phone: 310 333-6413

Fax:  310 333-5514

e-mail: hastings@cp10.es.xerox.com

Peter Zehler

Xerox Corporation

MS: 0128-30E

800 Phillips Road

Webster, NY 14580-9701

Phone: 1+585+2658755

Fax:

e-mail: PZehler@crt.xerox.com

# Annex A: Additional requirements for a revision of the PWG IEEE/ISTO 5101.1 Candidate Standard [pwg5101.1] to add the new ‘choice_xxx’

This section also generalizes the choice mechanism for inclusion in a revision to the PWG IEEE/ISTO 5101.1 Candidate Standard [pwg5101.1].  This specification gives the outline of the proposed addition to [pwg5101.1] for information only, in order to agree to the one new value defined for use with IPPFAX, namely, ‘choice_iso_a4_210x297mm_na_letter_8.5x11in’.

The proposed extension to [pwg5101.1] will have the following requirements:

- The ABNF will be extended to define the ‘choice_’ prefix to introduce a list of choices of self describing names.

- Following the ‘choice_’ prefix will be two or more fully self describing names, including all three fields.

- The order of the list of fully self describing names MUST be in alphabetic order, so that there is only one canonical order for each list of choices.

- Printer MAY support dynamic combinations of supported values or MAY support the entire choice value as a conventional keyword which it simply matches for validation purposes.

- Printer MUST map the choice value to a particular supported value when selecting the actual media for marking.

- If the Printer supports a ‘choice_xxx’ “media” attribute value and the “media-ready”  Printer attribute, the Printer MUST include the ‘choice_xxx’ value in its “media-ready” Printer attribute as well as the currently loaded value(s) when at least one of the members of a choice value is ready.  For example, if ‘iso_a4_210x297mm’ is loaded, then the “media-ready” Printer attribute MUST include the ‘iso_a4_210x297mm’ and the ‘choice_iso_a4_210x297mm_na_letter_8.5x11in’ values.

# Annex B: Change Log (informative - to be removed before publication)

The following summaries of the changes in chronological order:

Version 0.1, 18 May 2003:

- Split document out of Document Object specification.

- Moved "document-format-target" and "document-format-version-target" from the Job Template section to the Operation Attributes section so that it can be supplied with Document Creation operations and so that it MUST be honored, even if "job-attribute-fidelity" is 'false' or omitted.

- Removed all Document Description attributes and the concept of Job Level and Document Level.  That terminology is introduced in the Document Object spec [ippdoc].  Now [ippdoc] is only an Informative Reference, not a Normative Reference.

Version 0.2, 19 May 2003:

- Did not include the document-format-target and document-format-version-target attributes.  PSI has their own definitions and we can avoid the discussion as to whether they should be Document Template attributes or Operation attributes on Document Creation operations and whether there should be a "document-format-target-default" and a "document-format-version-target-default".

Version 0.3, 28 May 2003:

Agreements reaches at the May 28, 2003 telecon:

- Moved the Close-Job operation to the futures specification.

- Added the “pages-per-subset” Job Template from [pwg5100.4].  See Appendix B section Error: Reference source not found for the minor differences.

- Added the terms: “Best Effort, Finished Document, Page, Page Subset, and Sheet.

- Clarified the conformance relationships between “xxx” Operation attributes and their corresponding “xxx-default” and “xxx-supported” Printer attributes.

- Added rationale for each missing “xxx-default” and “xxx-supported” attribute.

- Added ‘errors-detected’ and ‘warnings-detected’ “job-state-reasons” values.

- Added the new 'choice_iso_a4_210x297mm_na_letter_8.5x11in' “media” attribute value which is a choice of two approximately equal media.

- Added the requirements for choice media keyword values for inclusion in a revision of the PWG IEEE-ISTO 5101.1.

- Changed the “document-format-implemented” member attribute from 1setOf mimeMediaType to mimeMediaType since it is the key attribute value.

- Added another “document-format-implemented” example.

- Updated the IANA section to agree with the additions.

Version 0.4, June 4, 2003:

- Moved “document-format-details-implemented” Printer Description attribute to Futures.

- Moved “pages-per-subset” (1setOf integer(1:MAX)) Job Template attribute to Futures.

- Removed all CONDITIONALLY REQUIRED and CMUST statements and made them OPTIONAL and MAY.  Added a statement of the intent for these attributes, so avoid the conformance statement problems for this specification which is just a list of OPTIONAL attributes.

- Removed the new terms “Honor” and “Best Effort”, since they are synonym for “Support” and the opposite.  It is confusing to have synonyms.

- Added all of the new Operation/Job Description attributes that describe Documents to be supplied in Job Creation operations and copied to the corresponding Job Description attributes as default values for the Document(s) of the Job.

- Added “output-device-default” and “output-device-supported” Printer attribute to provide the default for both the “output-device-requested” Operation attribute and the “output-device-assigned” Job Description attribute.

- Added “document-message” and “document-name” as Operation/Job Description default Document attributes for the Job.

- Clarified that the ‘none’ value of the “document-digital-signature” must be implemented, but the administrator MAY remove the ‘none’ value in order to require signatures to be used.

- Sated that all supplied member attributes of the “document-format-details” Operation attribute MUST have a value, i.e., MUST NOT be empty.

- Normalized all of the Application Names and versions and Operating System Name values from the IANA Registry to lower case, as is done with most other IPP attributes, in order to facilitate matching.

Moved the requirements for the new ‘choice_xxx’ values to be added to PWG 5101.1.

Version 0.5, June 9, 2003:

- added xxx-supported as Job Description attributes for specific operational attributes that apply to documents within the Job

For Document Description attributes that correspond to the Operation attributes defined for Document Creation operations, see the Document object defined in [ippdoc].

[rfc2911] uses the terms “Printer object” and “Printer” interchangeably to mean the same thing.  For simplicity, this document uses the term “Printer” exclusively, except for definitions copied directly from [rfc2911].  Nonetheless, the  intent is as in [rfc2911]: a Printer is an object that MAY be hosted in the device or in a server.

There is also a "document-format-details-implemented" (1setOf collection), see section Error: Reference source not found.

The corresponding “xxx-default” attribute is “output-device-default” Printer attribute (see section 7.11) which is the default for both the “output-device-requested” Job Description attribute and the “output-device-assigned” Job Description attribute.

The corresponding “xxx-supported” attribute is “output-device-supported” Printer attribute (see section 7.12) which is the list of supported output devices for both the “output-device-requested” Job Description attribute and the “output-device-assigned” Job Description attribute.

The operations defined for Document Creation operations do not have corresponding Document Description attributes defined in this specification, since this specification does not assume that there is a Document object.  See [ippdoc] for the corresponding Document Description attributes.

The value of the “ipp-attribute-fidelity” Operation attribute has no effect on other Operation attributes supplied by the client, only on Job Template attributes supplied by the client.

The Printer uses the "job-mandatory-attributes" Job Description attribute for validating subsequent Document Template attributes supplied in Send-Document and Send-URI operations for this job when also supporting [ippdoc] where the client can supply Document Template attributes.

Companies are urged to register there Printer languages with IANA using the ‘application/vnd.XX-yyy vendor tree.  There has also been a long standing PWG proposal to register all of these as ‘application/vnd.pwg-yyy.

If the client wants to achieve a “manifest”, that is a complete list of each file in a packaging, then the client needs to extract each file from the package and submit each one in a separate Document Creation operation.  Note: the client can re-compress each file before sending, so the compression isn’t lost when sending each document separately.

For the Send-Document and the Send-URI Operation attribute, the Printer copies them to the corresponding Document Description attribute defined in [ippdoc].

The "job-finishings" and "job-finishings-col" Job Template attributes also include corresponding "job-finishings-ready" (1setOf enum) and "job-finishings-col-ready" (1setOf collection).

The [jobx] specification does not define any Document Description attributes.  See the [ippdoc] specification.

The “ipp-attribute-fidelity” attribute is defined only as an Operation attribute in [rfc2911].  This specification is defining it as a Job Description attribute as well which the Printer MAY support.

The "ipp-attribute-fidelity" and "job-mandatory-attributes" Operation attributes do have effect after the Job Creation operation when the Document Object is supported [ippdoc].

[rfc2911] did not define a “document-format” Document Description attribute; it is a new attribute defined in this specification.