IPP Notification New Model

From:	Tom Hastings

Date:	8/07/99

File:	ipp-notification-new-model-990807.doc

This document is a new proposal to simplify the IPP Notification Model.  It uses Harry Lewis's suggested terminology of Per-Job subscriptions and Per-Printer subscriptions.  The Subscription object is defined only for use as Per-Printer subscriptions.  Per-Job subscriptions are done entirely with Job attributes and use an exclude event mask (like SNMPv3 trap subscriptions) in order to allow different recipients to get different events.  I've also added conformance requirements for operations and attributes, so that we can more properly reason about the model, before updating the specification document.

This document also includes the results of the 8/4/99 IPP telecon.

There are 17 issues highlighted like this that need to be resolved.

This paper uses object modeling techniques to describe the current IPP Notification Specification object model (July 22, 1999):

ftp://ftp.pwg.org/pub/pwg/ipp/new_NOT/ipp-notification-990722.pdf

Review of this object model before writing the complete spec will help speed up the production of the spec and will also help with the understanding of the semantics.  I suggest that this modeling be kept in the final spec in some form similar to this paper.

This paper also gives an outline of how the final spec might be presented, including being divided into two parts.  After describing the additional object attributes for the Job, Printer, and Subscription objects, the order of the spec is suggested to be:

- the REQUIRED first part (sections 5, 6, and 7) defines the extensions to the Job and Printer object and its operations for Per-Job subscriptions of Job and Printer events and their notification content.

- the OPTIONAL second part (section 8) defines the Per-Printer subscription mechanism and the Subscription object and its operations and attributes.

NOTE - Throughout this model document, the notation [ ] indicates that the sender (client in a request, IPP object in a response, notification sender in a notification) MAY omit the attribute inside the square brackets if there is not data to send.  In addition, IPP Printer conformance requirements for operation and attribute support are indicated in this document with the words and codes:

REQUIRED (R) to support notification at all

CONDITIONALLY REQUIRED (C) - REQUIRED if Per-Printer subscription is supported

OPTIONAL (O) - not required for either Per-Job or Per-Printer Subscrpitions.

Therefore, the [ ] do NOT mean OPTIONAL support and should not be confused with support requirements.

Table of Contents

1	Current IPP Notification Operations	3

1.1	Per-Job Subscriptions - previous model	3

1.2	Per-Printer Subscriptions - previous model	3

2	New IPP Notification Operations	3

2.1	Per-Job Subscriptions - new model	3

2.2	Per-Printer Subscriptions - new model	4

3	Printer, Job, and Subscription object relationships	4

4	Object attributes defined for Notification	6

4.1.1	Job object attributes related to notification	6

4.1.2	Printer object attributes related to notification	7

4.1.3	Subscription object attributes (new and OPTIONAL)	8

5	Events	9

5.1	Job events	9

5.2	Printer events	9

6	Notification Content	10

Job Event Notification content	10

Printer Event Notification content:	11

7	REQUIRED Per-Job Subscription mechanism	12

7.1	Job Creation Operations (Create-Job, Print-Job, Print-URI) and Validate-Job	13

7.2	Get-Printer-Attributes operation	13

7.3	Get-Job-Attributes operation	13

8	OPTIONAL Per-Printer Subscriptions and the Subscription object	13

8.1	Create-Printer-Subscription operation	13

8.2	Get-Printer-Subscription-Attributes operation	14

8.3	Get-Printer-Subscriptions operation	15

8.4	Renew-Printer-Subscription operation	17

8.5	Cancel-Printer-Subscription operation	18

9	References	18

# Current IPP Notification Operations

The current IPP Notification specification (July 22, 1999) consists of the Job object and the Printer object.  A Job object contains zero or one Per-Job Subscription which specifies Job and/or Printer events. The Job events apply only to this Job.  The Printer events apply no matter which job.  A Printer object contains zero or more Per-Printer Subscriptions which specify Job and/or Printer events.  The Job and Printer events apply no matter which job.

The following operations are defined for Per-Job Subscriptions and Per-Printer Subscriptions:

## Per-Job Subscriptions - previous model

| Job object related: | salient inputs besides printer-uri: |
| --- | --- |
| job creation operations | recipients, [events,] [user-data] |
| Subscribe-Job | job-id, recipients, [events] |
| Get-Job-Attributes | job-id, [requested-attributes] |
| Unsubscribe-Job | job-id, subscription-id |

## Per-Printer Subscriptions - previous model

| Printer object related: | salient inputs beside printer-uri: |
| --- | --- |
| Get-Printer-Attributes | [requested-attributes] |
| Renew-Printer-Subscription | [lease time] |
| Get-Printer-Subscriptions | [my-subscriptions] |

# New IPP Notification Operations

This new proposed IPP object model add the Subscription object (to the Job and Printer objects), but only for the Per-Printer subscriptions.

The following operations are defined for Per-Job Subscriptions and Per-Printer Subscriptions:

## Per-Job Subscriptions - new model

| Job object related: | salient inputs besides printer-uri |
| --- | --- |
| job creation operations | recipients, [events,] [user-data,] [exclude-event-mask] |
| Get-Job-Attributes | job-id, [requested-attributes] |
| Replace-Job-Subscription | job-id, [recipients,] [events,] [user-data,] [exclude-event-mask] |

## Per-Printer Subscriptions - new model

| Printer object related: | salient inputs beside printer-uri: |
| --- | --- |
| Get-Printer-Attributes | [requested-attributes] |
| Subscription object related: | salient inputs besides printer-uri: |
| Create-Printer-Subscription | recipients, [events,] [user-data,] [exclude-event-mask,] [lease-time] |
| Get-Printer-Subscription-Attributes | subscription-id, [requested-attributes] |
| Get-Printer-Subscriptions | [my-printer-subscriptions,] [requested-attributes] |
| Renew-Printer-Subscription | subscription-id, [lease time] |
| Cancel-Printer-Subscription | subscription-id |

# Job, Printer, and Subscription object relationships

This section describes the new notification object model that adds a Subscription object which together with the Job and Printer object provide the complete notification semantics.  The Subscription object is OPTIONAL.  The Subscription object MUST be supported if Per-Printer subscriptions are supported and vice-versa.

Per-Job subscriptions do not use the Subscription object.  Instead, a Job can have one Per-Job subscription which is represented entirely as Job attributes.

The object relationships can be seen pictorially as:

Subscription objects (Per-Printer Subscriptions)     Printer object

+----+                                                +------------+

| s1 |<---------------------------------------------->|            |

+----++                                               |            |

| s2 |<--------------------------------------------->|     p1     |

+----++                                              |            |

| s3 |<-------------------------------------------->|            |

+----+                                              +------------+

Job objects

+---------+

|         |

|   j1    |

|         |

+----+    |

| s4 |    |      s4 is a Per-Job subscription

++---+----++

|         |

|   j2    |

|         |

+----+    |

| s5 |    |     s5 is a Per-Job subscription

++---+----++

|         |

|   j3    |

|         |

|         |

+---------+

s1, s2, and s3 are Per-Printer Subscription objects and can identify Printer and/or Job events.

s4 and s5 are Per-Job subscriptions (not objects) and can identify Printer and/or Job events.

The object relationships can be stated as follows:

- The Printer object contains zero or more Subscription objects (p1 contains s1-s3 Subscription objects).

- Each Subscription object (s1, s2, and s3) is contained in one Printer object (p1).

- Each Subscription object (s1, s2, and s3) represents one Per-Printer subscription.

- Each "per-Printer" Subscription object identifies one or more Job and/or Printer events.  Such Job events are for all jobs on the Printer.  Such Printer events are for any Printer event, no matter which job is processing.

- The "per-Job" subscription (not an object) identifies one or more Job and/or Printer events.  Such Job events are only for this job (different than "per-Printer" Subscriptions).  Such Printer events are for any Printer event, no matter which job (same as for "per-Printer" Subscriptions.

- A Job object contains zero or one Per-Job subscriptions (not an object)(Job j1 contains Per-Job subscription s4 Job j2 contains Per-Job subscription s5, Job j3 does not contain a Per-Job subscription.

- A Subscription object cannot be contained in or associated with more than one Printer object

# Object attributes defined for Notification

The following notification attributes are defined for each of the Printer, Job, and (new, but OPTIONAL) Subscription objects.  The explanations about some of the object attributes are collected here so that they can be referred to from the operations that set them.  All Job, Printer, and Subscription object attributes are READ-ONLY, i.e., they MUST NOT be settable with the Set-Job-Attributes or Set-Printer-Attributes operations.  Instead, all of them are set as a side effect of another operation or when an event notification is delivered.  Attributes and objects not in the current, July 22, spec are labeled as "new":

### Job object attributes related to notification

| Job object attributes: | Printer support | READ-ONLY, set by: |
| --- | --- | --- |
| notify-recipients (1setOf uri) | R | job creation operation |
| notify-events (1setOf type2 keyword) | R | job creation operation |
| notify-user-data (octetString(255)) | R | job creation operation |
| notify-exclude-event-masks (1setOf octetString(8)) | O | job creation operation |

| job-trigger-event (type2 keyword) | Printer support | Printer - event occurring |
| --- | --- | --- |
| job-trigger-message (text(255)) | R | Printer - event occurring |
| job-trigger-time (integer(MIN:MAX)) | R | Printer - event occurring |
| job-trigger-date-time (dateTime) | O | Printer - event occurring |
| previous-job-state (type1 enum) | R | Printer - event occurring |
| job-state-reasons-added (1setOf type2 keyword) | R | Printer - event occurring |
| job-state-reasons-deleted (1setOf type2 keyword) | R | Printer - event occurring |

Attribute Notes:

notify-user-data (octetString(255)) - information being sent from the Subscriber to the Notification Recipient(s).  This information could identify the real recipient, in case the Notification Recipient is a general recipient tool, such as Tool Talk or an Instant Message Service.  
ISSUE 1 - is 255 octets too long for "notify-user-data (octetString(255))", since it also has to be sent in the notification content?  How about 64 octets?

notify-exclude-event-masks (1setOf octetString(8)) - an OPTIONAL per-recipient event mask whose values are parallel to the "notify-recipients" attribute.  Each 1 bit indicates that the corresponding recipient is not to receive the 'i'-th event specified in the "notify-events" attribute.  The first event in "notify-events" is numbered '1' which corresponds to the first bit (Big Endian network order) of the event mask octetString.  If an implementation only supports n events, then the maximum number of octets that an implementation need support for this attribute is not really 8 octets, but is n mod 8.  
ISSUE 2 - Should "notify-exclude-event-masks (1setOf octetString(8))" be REQUIRED if the number of Notification Recipients supported in a Per-Job (or Per-Printer) Subscription in greater than 1 (as specified in the Printer's "max-notification-recipients-supported" attribute)?

job-state-reasons-added (1setOf type2 keyword) and job-state-reasons-deleted (1setOf type2 keyword) - ISSUE 3 - why not just "previous-job-state-reasons" instead of "job-state-reasons-added" and "job-state-reasons-deleted" attributes as Job Description attributes?

### Printer object attributes related to notification

| Printer object attributes: | Printer support | READ-ONLY, set by: |
| --- | --- | --- |
| notify-schemes-supported (1setOf uriScheme) | R | Administrator/implementation |
| max-notification-recipients-supported (integer(1:MAX)) | R | Administrator/implementation |
| notify-events-default (1setOf type2 keyword) | R | Administrator/implementation |
| notify-events-supported (1setOf type2 keyword) | R | Administrator/implementation |
| max-events-supported (integer(5:MAX)) | R | Administrator/implementation |
| notify-lease-time-supported (rangeOfInteger(0:MAX)) | C | Administrator/implementation |
| notify-lease-time-default (integer(0:MAX)) | C | Administrator/implementation |
| max-printer-subscriptions-supported (integer(0:MAX)) | C | Administrator/implementation |

Attribute Notes:

notify-schemes-supported (1setOf uriScheme) - the schemes that are supported in Per-Job and Per-Printer "notify-recipients" operation attributes.

max-notification-recipients-supported (integer(1:MAX)) - the maximum number of Notification Recipients that are supported in a Per-Job Subscription.  If Per-Printer subscriptions are supported this same number applies.  An implementation MUST support at least 1.

max-events-supported (integer(5:MAX)) - the maximum number of events that are supported in a Per-Job Subscription.  If Per-Printer subscriptions are supported this same number applies. An implementation MUST support at least 5.

notify-lease-time-supported (rangeOfInteger(0:MAX)) - the minimum and maximum lease times supported.  When the lease time expires for a Per-Printer Subscription without renewing, the Printer MUST delete the Subscription object.  A value of 0 indicates an infinite amount of time, i.e., no lease.  No lease MAY require authorization.

max-printer-subscriptions-supported (integer(0:MAX)) - the maximum number of Per-Printer subscription supported.  0 means there is no limit.

### Subscription object attributes (new and OPTIONAL)

| Subscription object attributes: | Printer support | READ-ONLY, set by: |
| --- | --- | --- |
| notify-recipients (1setOf uri) | C | client - Create-Printer-Subscription |
| notify-events (1setOf type2 keyword) | C | client - Create-Printer-Subscription |
| notify-user-data (octetString(255)) | C | client - Create-Printer-Subscription |
| notify-exclude-event-mask (1setOf octetString(8)) | O | client - Create-Printer-Subscription |
| notify-lease-time (integer(0:MAX)) | C | client - Create-Printer-Subscription |

| notify-lease-time-remaining (integer(0:MAX)) | C | Printer - Create-Printer-Subscription |
| --- | --- | --- |
| subscriber-user-name (name(MAX)) | C | Printer - Create-Printer-Subscription |

| most-recent-request-id (1setOf integer(0:MAX)) | C | Printer - event occurring |
| --- | --- | --- |
| delivery-failure-count (integer(0:MAX)) | C | Printer - event occurring |

Attribute Notes:

"notify-user-data" (octetString(255)) - same as the "notify-user-data" Job Description attribute.

"notify-exclude-event-mask" (1setOf octetString(8)) - same as the "notify-exclude-event-mask" Job Description attribute.

"notify-lease-time" - the number of seconds granted for the subscription.  It is used to restore the subscription on the Restart-Job and Reprocess-Job operations.  A 0 value indicates infinite time.

"notify-lease-time-remaining" - the number of seconds remaining in the lease.  When the Printer counts the value down to 0, the Printer MUST delete the Per-Printer Subscription object.  A client is able to extend a lease using the Renew-Printer-Subscription operation (see section 8.4).  Internally, the implementation may have a more absolute representation of when the least expires, but this relative value is returned to help the client know how much more time is remaining in the lease.  A value of 0 indicates an infinite name, if such a policy is supported as indicated in the "notify-lease-time-supported (integer(0:MAX)) Printer Description attribute.

"subscriber-user-name (name(MAX)) - the printable name of the most authenticated user that created the Per-Printer Subscription (analogous to "job-originating-user-name" for Job objects).

"most-recent-request-id" (1setOf integer(0:MAX))  - the request-id that was delivered in the most recent notification for the subscription for the recipient in the corresponding parallel "notify-recipients" (1setOf uri) attribute.  The reason to have a separate request-id for each recipient is so that when more than one Notification Recipient is specified in a subscription, each recipient will have its own monotonically increasing set of request-ids, i.e., no gaps, in order to be able to detect a missing notification.  If an implementation doesn't support more than one Notification Recipient then this attribute only has one value as well.

"delivery-failure-count" (integer(0:MAX)) - the number of transport delivery failures in delivering a notification to any of the "notify-recipients" using the transport indicated.  Note: for the current delivery methods envisioned (see Notification Requirements [ipp-req], Notification Recipients do not respond to the Notification Source, so this count is not the number of unacknowledged notifications.

# Events

Events are defined to be non-overlapping.  So for example, 'job-state-changed' doesn't include creating the job or completing the job.  This reduces the need to batch events together, since events tends to happen at different times by definition.

Both Job and/or Printer events may be specified for Per-Job or Per-Printer subscriptions:

Job events in a Per-Job subscription indicate Job events for this job only

Job events in a Per-Printer subscription indicates Job events for all jobs

Printer events in a Per-Job subscription indicate Printer events for any job

Printer events in a Per-Job subscription indicate Printer events for any job

ISSUE 4 - Should we disambiguate the 'job-xxx' events and double the number by renaming to 'my-job-xxx' and 'all-job-xxx' events for use in both Per-Job and Per-Printer subscriptions?

## Job events

'none':  REQUIRED - no notifications of any events

ISSUE 5 - Add a 'job-announce' event which is sent to the Notification Recipient(s) when the subscription is accepted to prepare/warn them?

'job-created':  REQUIRED - the Printer object has accepted Create-Job, Print-Job, or Print-URI.

'job-completed':  REQUIRED - the job has reached  'completed', 'aborted', or 'canceled'.

'job-state-changed':  REQUIRED - any job state or state-reason changed, except when the job is created or when the job moves to 'completed', 'aborted', or 'canceled'.

'job-config-changed' – "job-message-from-operator" or any of the non-READ-ONLY Job attributes changed, typically through the Set-Job-Attributes operation.

'job-purged' - Job-Purged operation on a 'non-completed' job.

'job-progress' - sheet or copy completed.  See separate [ipp-prog] spec.

## Printer events

'none' - REQUIRED - no notification of any events

ISSUE 6 - Add a 'printer-announce' event which is sent to the Notification Recipient(s) when the subscription is accepted to prepare/warn them?

'printer-restarted' - device powered up or Restart-Printer operation is performed (see [ipp-adm]).

'printer-shutdown' - device is being powered down or the Shutdown-Printer operation [ipp-adm]).

'printer-state-changed' - REQUIRED - "printer-state", "printer-state-reasons", and/or "printer-is-accepting-jobs" attributes changed, except when the Printer starts up or is shutdown.

'printer-media-changed' – when the media loaded ("media-ready") on a printer has been changed.

'printer-config-changed' – "printer-message-from-operator" or any non-READ-ONLY Printer attributes changed, except for "media-ready" (which has its own event), whether through the Set-Printer-Attributes operation or by other means and whether initiated by a human or not.

'printer-no-longer-full' – Printer may still be printing but has acquired more buffer space to accept jobs.  This event only occurs when the Printer did not have room to accept jobs previously and rejected a Print-Job, Print-URI, Create-Job, Send-Document, or Send-URI operation from any client (desktop or Server).

'printer-almost-idle' – Printer needs another Job in order to stay busy.  Typically sent to a Server that has subscribed.

# Notification Content

ISSUE 7 - How is the User Friendly notification represented, such as is used with the 'mailto:' notification delivery scheme?  What parts gets translated into the Natural Language of the subscriber (presumed to be the same as the recipient), besides the "job-trigger-message" and "printer-trigger-message"?  The idea of using multipart/report has problems going through firewalls because of attachments, so maybe we need a single text message that can be read by humans, and still be parsed by programs for some of the (un-translated) keywords attribute names.  Perhaps, the trigger-message contains user friendly stuff like job-name, printer-name, and the translated job-state and state-reasons values and the rest of the message is a series of lines of the form:

keyword: text

where 'keyword' is the un-translated (i.e, U.S. English) attribute keyword name and the text is the representation of the attribute values.  Which attributes would be required beside the "job-trigger-message" and "printer-trigger-message"?

The notification content is the same for the current and new models.  All attributes indicated are REQUIRED for all events, unless inside [ ] or the comment indicates otherwise:

## Job Event Notification content

The following notification content is sent for both Per-Job and Per-Printer Subscriptions when a Job event occurs (see section 5.1 for Job events):

ISSUE 8 - Most transports only allow 500 to 1000 octets in a packet we have too much stuff.

version-number (integer (0:32767))

status-code (integer (0:32767)) - with the value: basic-job-event(600)

request-id (integer (1:MAX)) - used as notification delivery sequence number for each recipient

job-printer-uri (uri)

job-id (integer(1:MAX))

job-trigger-event (type2 keyword)

[job-trigger-message (text(255))]

ISSUE 9 - is 255 too long for "job-trigger-message (text(255))", since it also has to be sent in the notification content?

ISSUE 10 - How to include user friendly data, like job-name, job-owner, printer-name, date, time, and event name in a Job event notification?

ISSUE 11 - Should we REQUIRE the "printer-trigger-message" to contain the user friendly data localized information about this event, namely:  like job-name, job-owner, printer-name, date, time, and event name and REQUIRE "printer-trigger-message"?

job-trigger-time (integer(1:MAX))

[job-trigger-date-time (dateTime)]

job-state (type1 enum)

previous-job-state (type1 enum) 	not for 'job-completed' event

job-state-reasons (1setOf type2 keyword)

job-state-reasons-added (1setOf type2 keyword) 	not for 'job-completed' event

job-state-reasons-deleted (1setOf type2 keyword) 	not for 'job-completed' event

ISSUE 12 - in a Job notification why not just "previous-job-state-reasons" instead of "job-state-reasons-added" and "job-state-reasons-deleted"?

[job-impressions-completed (integer(0:MAX))]	only for 'job-completed' event

[job-media-sheets-completed (integer(0:MAX))]	only for 'job-completed' event

job-collation-type (type2 enum)    	only for 'job-progress' event

sheet-completed-copy-number (integer(-2:MAX))    	only for 'job-progress' event

sheet-completed-document-number(integer(-2:MAX))    	only for 'job-progress' event

impressions-interpreted (integer(-2:MAX))    	only for 'job-progress' event

impressions-completed-current-copy (integer(-2:MAX))    	only for 'job-progress' event

[subscription-id (integer(1:MAX))] 	MUST if from a Per-Printer subscription

## Printer Event Notification content:

The following notification content is sent for both Per-Job and Per-Printer Subscriptions when a Job event occurs (see section 5.2 for Printer events):

ISSUE 13 - Most transports only allow 500 to 1000 octets in a packet so we have too much stuff here.

version-number (integer (0:32767))

status-code (integer (0:32767)) - with the value: basic-job-event(601)

request-id (integer (1:MAX)) - used as notification delivery sequence number for each recipient

printer-uri (uri)

printer-trigger-event (type2 keyword)

[printer-trigger-message (text(255))]

ISSUE 14 - is 255 too long for "printer-trigger-message (text(255))", since it also has to be sent in the notification content?

ISSUE 15 - How to include user friendly data, like printer-name, date, time, and event name in a Printer event notification?

ISSUE 16 - Should we REQUIRE the "printer-trigger-message" to contain the user friendly data localized information about this event, namely: printer-name, date, time, and event name and REQUIRE "printer-trigger-message"?

printer-trigger-time (integer(1:MAX))

[printer-trigger-date-time (dateTime)]

printer-state (type1 enum)

previous-printer-state (type1 enum)

printer-state-reasons (1setOf type2 keyword)

printer-state-reasons-added (1setOf type2 keyword)

printer-state-reasons-deleted (1setOf type2 keyword)

ISSUE 17 - in a Printer notification why not just "previous-printer-state-reasons" instead of "printer-state-reasons-added" and "printer-state-reasons-deleted"?

[subscription-id (integer(1:MAX))]	MUST if from a Per-Printer subscription

# REQUIRED Per-Job Subscription mechanism

This section lists the REQUIRED operation requests and response that have some relation to notification, i.e., that are related to Per-Job subscription.   Section 8 lists the OPTIONAL operation requests and responses associated with the OPTIONAL Per-Printer subscription and its Subscription object.

## Job Creation Operations (Create-Job, Print-Job, Print-URI) and Validate-Job

As in the current notification spec (July 22, 1999), the usual method for a client to associate one subscription with a Job is to specify the subscription when the job is created.  For a Per-Job Subscription, the client supplies the following notification operation attributes with any of the job creation operations (Create-Job, Print-Job, Print-URI), plus Validate-Job (which doesn't create a job or subscription):

notify-recipients (1setOf uri)	REQUIRED to support

[notify-events (1setOf type2 keyword)]	REQUIRED to support

[notify-user-data (octetString(255))]	REQUIRED to support

[notify-exclude-event-mask (octetString(8))]	OPTIONAL to support

## Get-Printer-Attributes operation

The Get-Printer-Attributes can be used to request the additional Printer Description attributes that indicate the notification supported.  See section 4.1.2.

## Get-Job-Attributes operation

The Get-Job-Attributes can be used to request the additional Job Description attributes that relate to notification.  See section 4.1.1.

# OPTIONAL Per-Printer Subscriptions and the Subscription object

This section lists the operation requests and responses that MUST be supported if the OPTIONAL Per-Printer Subscriptions are supported which REQUIRES that the Subscription object be supported and vice versa.

## Create-Printer-Subscription operation

The Create-Printer-Subscription operation creates a Per-Printer subscription by creating a Per-Printer Subscription object which is added to the Printer object.

Request:

| Group 1: Operation Attributes |  | Printer support |
| --- | --- | --- |
|  | "attributes-charset" (charset) | R |
|  | "attributes-natural-language" (naturalLanguage) | R |
|  | "printer-uri" (uri) | R |
|  | ["requesting-user-name" (name(MAX))] | RECOMMENDED |
|  | "notify-recipients" (1setOf uri) | R |
|  | ["notify-events" (1setOf type2 keyword)] | R |
|  | ["notify-user-data" (octetString(255))] | R |
|  | ["notify-exclude-event-mask" (1setOf octetString(8))] | O |
|  | ["notify-lease-time" (integer(0:MAX))] | R |

Response:

| Group 1: Operation Attributes |  | Printer Support |
| --- | --- | --- |
|  | "status-code" (type2 enum) | R |
|  | "attributes-charset" (charset) | R |
|  | "attributes-natural-language" (naturalLanguage | R |
|  | ["status-message" (text(255))] | O |
|  | ["detailed-status-message" (text(MAX)) | O |
|  | "subscription-id" (integer(1:MAX)) | R |
|  | "notify-lease-time" (integer(0:MAX)) | R |
|  | "notify-lease-time-remaining" (integer(0:MAX)) | R |
| Group 2: Unsupported Attributes |  | R |

Notes:  A "notify-lease-time" and "notify-lease-time-remaining" of 0 means infinite, i.e., the Per-Printer Subscription has no lease.  Supplying a 0 value MAY require authentication in order to be used when supported at all..

The Printer object MUST return the "notify-lease-time" actually granted which may be less than the requester requested if it was greater than the MAX supported or more than the requester requested if it was less than the MIN supported, as indicated in the Printer's "lease-time-supported" (rangeOfInteger(0:MAX)) attribute.

The Printer object MUST return the "notify-lease-time-remaining" which is the number of seconds remaining in the lease.

## Get-Printer-Subscription-Attributes operation

The Get-Printer-Subscription-Attributes returns the requested attributes of the identified Per-Printer Subscription object.  See section 4.1.3.

Request:

| Group 1: Operation Attributes |  | Printer support |
| --- | --- | --- |
|  | "attributes-charset" | R |
|  | "attributes-natural-language" | R |
|  | "printer-uri" (uri) | R |
|  | ["requesting-user-name" (name(MAX))] | RECOMMENDED |
|  | "subscription-id" (integer(1:MAX)) | R |
|  | ["requested-attributes" (1setOf type2 keyword)] | R |

Response:

| Group 1: Operation Attributes |  | Printer support |
| --- | --- | --- |
|  | "status-code" (type2 enum) | R |
|  | ["status-message" (text(255))] | O |
|  | ["detailed-status-message (text(MAX))] | O |
|  | "attributes-charset" | R |
|  | "attributes-natural-language" | R |
| Group 2: Unsupported Attributes |  | R |
| Group 3: <the requested Subscription object attributes> |  | R |

This operation is similar to the Get-Printer-Attributes operation.  If the client omits the "requested-attributes" operation attribute, the Printer MUST respond as if the client had supplied the 'all' value, i.e., return all of the attributes supported for the Subscription object.

## Get-Printer-Subscriptions operation

The Get-Printer-Subscriptions operation returns Per-Printer subscriptions, i.e., Subscription objects.

Request:

| Group 1: Operation Attributes |  | Printer support |
| --- | --- | --- |
|  | "attributes-charset" | R |
|  | "attributes-natural-language" | R |
|  | "printer-uri" (uri) | R |
|  | ["requesting-user-name" (name(MAX))] | RECOMMENDED |
|  | ["limit" (integer(1:MAX))] | R |
|  | ["requested-attributes" (1setOf type2 keyword)] | R |
|  | ["my-subscriptions" (boolean)] | R |

Response:

| Group 1: Operation Attributes |  | Printer support |
| --- | --- | --- |
|  | "status-code" (type2 enum) | R |
|  | "attributes-charset" | R |
|  | "attributes-natural-language" | R |
|  | ["status-message" (text)] | O |
|  | ["detailed-status-message" (text(MAX))] | O |
| Group 2: Unsupported Attributes |  | R |
| Group 3 to N:<the requested Subscription Attributes for each Subscription object in a separate group> |  | R |

Notes:  This operation is similar to the Get-Jobs operation.  If the client wants any attributes returned, including the "subscription-id", it must include the attribute keyword name in the "requested-attributes" operation attribute.  If the "requested-attributes operation attribute is omitted, the Printer MUST respond as if the client supplied the value: 'subscription-id'.

## Renew-Printer-Subscription operation

Request:

| Group 1: Operation Attributes |  | Printer support |
| --- | --- | --- |
|  | "attributes-charset" | R |
|  | "attributes-natural-language" | R |
|  | "printer-uri" (uri) | R |
|  | "requesting-user-name" (name(MAX)) | RECOMMENDED |
|  | "subscription-id" (integer(1:MAX)) | R |
|  | "notify-lease-time" (integer(0:MAX)) | R |

Response:

| Group 1: Operation Attributes |  | Printer support |
| --- | --- | --- |
|  | "status-code" (type2 enum) | R |
|  | "attributes-charset" | R |
|  | "attributes-natural-language" | R |
|  | ["status-message" (text(255))] | O |
|  | ["detailed-status-message" (text(MAX))] | O |
|  | "notify-lease-time" (integer(0:MAX)) | R |
|  | "notify-lease-time-remaining" (integer(0:MAX)) | R |
| Group 2: Unsupported Attributes |  | R |

Notes:  The Printer object MUST return the "notify-lease-time" actually granted which may be less than the requester requested if it was greater than the MAX supported or more than the requester requested if it was less than the MIN supported, as indicated in the Printer's "lease-time-supported" (rangeOfInteger(0:MAX)) attribute.

The Printer object MUST return the "notify-lease-time-remaining" which is the number of seconds remaining in the lease.

Note:  There is no need to renew a Per-Job Subscription, since it is effectively the time that the Job has documents.

## Cancel-Printer-Subscription operation

Request:

| Group 1: Operation Attributes |  | Printer support |
| --- | --- | --- |
|  | "attributes-charset" | R |
|  | "attributes-natural-language" | R |
|  | "printer-uri" (uri) | R |
|  | ["requesting-user-name" (name(MAX))] | RECOMMENDED |
|  | "subscription-id" (integer(1:MAX)) | R |

Response:

| Group 1: Operation Attributes |  | Printer support |
| --- | --- | --- |
|  | "status-code" (type2 enum) | R |
|  | "attributes-charset" | R |
|  | "attributes-natural-language" | R |
|  | ["status-message" (text(255))] | O |
|  | ["detailed-status-message" (text(MAX))] | O |
| Group 2: Unsupported Attributes |  | R |

# References

[ipp-mod]

deBry, R., , Hastings, T., Herriot, R., Isaacson, S., Powell, P., "Internet Printing Protocol/1.1: Model and Semantics", < draft-ietf-ipp-model-v11-04.txt>, work in progress, June 23, 1999.

[ipp-prog]

Hastings, T., Bergman, R., Lewis, H., "Proposed Job Progress Attributes for IPP", <ipp-job-prog-attr-990518.txt>  work in progress, May 18, 1999.

[ipp-req]

Hastings, T., Manros, C., "Internet Printing Protocol/1.0 & 1.1: Requirements for IPP Notification", <draft-ietf-ip-not-02.txt>  work in progress, June 24, 1999.