IPP Bake-Off 2

Results Summary

Table of Contents

1	Revision Log	2

2	Overview	3

3	Summary Tables	4

3.1	Print Submission Interoperability Matrix	4

3.2	Chunking Interoperability Matrix	5

3.3	Security Interoperability Matrix	6

3.4	Create-Job Interoperability Matrix	6

3.5	Print-Uri Interoperability Matrix	7

3.6	Set 1 Interoperability Matrix	7

3.7	Ipp:// scheme support  Matrix	8

3.8	HTTP get on IPP URL support  Matrix	9

3.9	Operation Coverage	9

3.10	Operational Attributes Coverage	10

3.11	Attribute Coverage	12

3.11.1	Printer Description	12

3.11.2	Job Template	13

3.11.3	Job Description	14

3.12	Syntax Coverage	15

4	Issues	15

# Revision Log

3/16/99		pjz	Started with first Bake-Off summary.  Updated issues list.

3/17/99		pjz	Updated tables.

3/18/99		pjz	added additional issues captured by Tom Hastings

3/19/99		pjz	Completed updates

# Overview

The second IPP Bake-Off held March 9th to 12th.  It was hosted by Novel in Orem Utah.  The Bake-Off was an unqualified success.  The 24 participating organizations were:

Apple			AUCO,			Axis, Canon		EFI

Epson			Extended Systems	Fuji Xerox		Genoa

Hewlett Packard		i-data			IBM			JCI

Komatsu		Kyocera			Lexmark		Microsoft

Netsilicon		Novell			Ricoh			Shinesoft

Sun			Tektronix		Xerox

There were 11 IPP Client implementations and 27 IPP Printer implementations.  Out of the 297 possible Client/Printer combinations, 296 were successful in completing a simple "print-job" operation.  That is a success rate of 99.66%.  The previous Bake off had 8 Clients and 16 Printer with a success rate of 96.9%.

Other aspects of IPP were tested for the first time including HTTP chunking and security.  We tested 127 combinations of chunking clients with IPP Printers.  Of those tested 118 worked fine resulting in a success rate of 92.9%.  We knew going into this test that some Printers did not yet support chunking.  The limited security testing that was done was a complete success.  There were 43 combinations supporting Basic authentication.  They all worked.  There were 2 combinations supporting Digest authentication and they worked.  (One printer knew going in its Digest authentication did not work. The printer participated only to prove it was broken.)  Three combinations supported SSL. One combination used Basic authentication over SSL.  These also worked.

All the IPP operations, Printer attributes, Job Description attributes and attribute syntaxes were tested.  The majority of the Job Description attributes and Operational attributes were tested.  We have not strenuously tested the semantics and interactions of these operations and attributes.  There are just too many participants and a lack of automated tools to accomplish such a rigorous test.

Twenty six issues were raised over the four day event.  These issues will be brought to the IPP mailing list and discussed.

The major benefit of the Bake-Off was bringing together the implementers of IPP from across the industry.  The cooperation between the engineers was remarkable.  All were sharing their IPP expertise and working together for the benefit of all.  Every participating vendor will have an improved implementation of IPP as a direct result of this event.

The First day of the Bake-off was for setup.  This was accomplished in less than half a day.  Some time was spent discussing methodology and prioritizing objectives.  A prioritized list and the  IPP Test Plan was used as the basis for Bake-off testing.  The URL for the prioritized list is “ftp://ftp.pwg.org/pub/pwg/ipp/new_TES/Bake-Off2-Official-List.pdf” .  The URL for the Test Plan is “ftp://ftp.pwg.org/pub/pwg/ipp/new_TES/IPP-Test-Plan-990219.pdf”

The remaining time was spent testing basic printing, chunking, security, print by reference, create-job/send-document and the set 1 operations.

# Summary Tables

## Print Submission Interoperability Matrix

This matrix shows the results of submitting a simple print job from IPP Clients to IPP Printerss.  The participants of the Bake-Off agreed on rules of anonymity.  One of the rules was to use designations to identify implementations.  In the following table IPP Clients are on the horizontal and IPP Printers are on the vertical.  A key at the bottom of the table is provided to interpret the results.

|  | C1 | C2 | C3 | C4 | C5 | C6 | C7 | C8 | C9 | C11 | C12 | S/F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P1 | P | P | P | P | P | P | P | P | P | P | P | 11/0 |
| P2 | P | P | P | P | S | S | P | P | P | P | S | 11/0 |
| P3 | S | S | S | S | S | S | S | S | S | S | S | 11/0 |
| P4 | P | P | P | P | S | S | P | P | P | P | S | 11/0 |
| P5 | P | P | P | P | P | P | P | P | P | P | P | 11/0 |
| P6 | S | S | S | S | S | S | S | S | S | S | S | 11/0 |
| P7 | P | P | P | P | P | P | S | P | P | P | S | 11/0 |
| P8 | S | S | S | S | S | S | S | S | S | S | S | 11/0 |
| P9 | S | P | P | P | P | P | P | P | P | P | S | 11/0 |
| P10 | P | P | P | P | S | P | P | P | P | P | P | 11/0 |
| P11 | P | P | P | P | S | P | P | P | P | P | P | 11/0 |
| P12 | S | S | S | S | S | S | S | S | S | S | S | 11/0 |
| P13 | P | P | S | P | P | P | P | P | P | S | P | 11/0 |
| P14 | P | P | P | P | P | P | P | P | P | P | P | 11/0 |
| P15 | S | P | S | S | S | P | S | S | S | S | S | 11/0 |
| P16 | S | S | S | S | S | S | S | S | S | S | S | 11/0 |
| P17 | S | S | P | P | S | S | P | P | P | P | P | 11/0 |
| P18 | S | S | S | S | S | S | S | S | S | S | S | 11/0 |
| P19 | P | P | P | P | P | P | P | P | P | P | S | 11/0 |
| P20 | P | P | P | P | S | P | P | P | P | P | S | 11/0 |
| P21 | S | S | S | S | S | F | S | S | P | S | S | 10/1 |
| P22 | P | P | P | P | P | P | P | P | P | P | P | 11/0 |
| P23 | S | S | S | S | S | S | S | S | S | S | S | 11/0 |
| P24 | P | P | P | P | P | P | P | P | P | P | S | 11/0 |
| P25 | S | S | S | S | S | S | S | S | S | S | S | 11/0 |
| P26 | P | P | P | P | P | P | P | P | P | S | S | 11/0 |
| P27 | S | S | S | S | S | S | S | S | S | S | S | 11/0 |
| S/F | 27/0 | 27/0 | 27/0 | 27/0 | 27/0 | 26/1 | 27/0 | 27/0 | 27/0 | 27/0 | 27/0 | 296/1 |

S = Success

P = Success (Paper output)

F = Failure

## Chunking Interoperability Matrix

This matrix shows the results of submitting a simple print job from IPP Clients to IPP Printers.  The print request is sent using HTTP chunking.   IPP Clients are on the horizontal and IPP Printers are on the vertical.  A key at the bottom of the table is provided to interpret the results.

|  | C1 | C3 | C4 | C7 | C8 | C12 | S/F |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P1 | S | S |  | S |  | S | 4/0 |
| P2 | S | S | S | S | S | S | 6/0 |
| P3 | S | S | S | S | S | S | 6/0 |
| P4 | S | S | S | S | S | S | 6/0 |
| P5 | S | S | S | S | S | S | 6/0 |
| P6 |  |  |  |  |  |  | 0/0 |
| P7 | S | S | S | S | F (500) | S | 5/1 |
| P8 | S | S | S |  | S | S | 5/0 |
| P9 | S | S |  | S | S | S | 5/0 |
| P10 |  | F | F |  | S |  | 1/2 |
| P11 | S | S | S | S | S | S | 6/0 |
| P12 | S | S | S | S | S | S | 6/0 |
| P13 | S | S | S | S | S | S | 6/0 |
| P14 | F (400) | S | F (Inval req) | S | F (no msg) | S | 3/3 |
| P15 | S | S | S | S | S | S | 6/0 |
| P16 |  |  |  |  |  |  | 0/0 |
| P17 | S | S | S | S | S | S | 6/0 |
| P18 | S | S |  |  | S | S | 4/0 |
| P19 |  |  |  |  | F (400) |  | 0/1 |
| P20 | S | S | S | S | S | S | 6/0 |
| P21 | S | S | S | S | S | S | 6/0 |
| P22 |  | S | S |  | S | S | 4/0 |
| P23 | S | S | S | F |  | S | 4/1 |
| P24 |  |  |  |  | F |  | 0/1 |
| P25 | S | S | S |  | S | S | 5/0 |
| P26 | S | S | S | S | S | S | 6/0 |
| P27 | S | S | S | S | S | S | 6/0 |
| S/F | 20/1 | 22/1 | 18/2 | 17/1 | 19/4 | 22/0 | 118/9 |

S = Success

F = Failure

(*) = error code/message

## Security Interoperability Matrix

This matrix shows the results of issuing an IPP request from IPP Clients to IPP Printers.  IPP Clients are on the horizontal and IPP Printers are on the vertical.  The security method is listed with the results.  Note that P1’s Digest implementation was not part of the test.  It was known to be incorrect and tested only to verify it was indeed broken.

All combinations tested succeeded.

|  | C2 | C4 | C5 | C6 | C8 | C9 |
| --- | --- | --- | --- | --- | --- | --- |
| P1 | Basic=S Digest=F | Basic=S Digest=F | Basic=S Digest=F | Basic=S Digest=F | Basic=S Digest=F | Basic=S Digest=F |
| P4 | Basic=S | Basic=S | Basic=S |  | Basic=S | Basic=S |
| P5 | Basic=S | Basic=S Digest=S | Basic=S | Basic=S SSL=S | Basic=S | Basic=S Digest=S |
| P9 | Basic=S | Basic=S |  |  | Basic=S | Basic=S |
| P10 | Basic=S | Basic=S |  |  | Basic=S | Basic=S |
| P12 | Basic=S |  |  |  | Basic=S |  |
| P13 |  |  | Basic=S | Basic=S SSL=S Basic+SSL=S | Basic=S |  |
| P15 |  |  |  |  |  |  |
| P18 | Basic=S | Basic=S | Basic=S |  |  |  |
| P19 | Basic=S | Basic=S |  |  | Basic=S |  |
| P22 | Basic=S | Basic=S |  |  |  | Basic=S |
| P24 | Basic=S | Basic=S |  | Basic=S | Basic=S | Basic=S |
| P27 |  |  |  | SSL=S |  |  |

## Create-Job Interoperability Matrix

This matrix shows the results of issuing of ‘create-job’ and ‘send-document’ requests from IPP Clients to IPP Printers.  IPP Clients are on the horizontal and IPP Printers are on the vertical.

All combinations tested succeeded.

|  | C1 | C2 | C4 | C9 | C11 | S/F |
| --- | --- | --- | --- | --- | --- | --- |
| P5 | S | S | S | S | S | 5/0 |
| P8 |  | S |  |  |  | 1/0 |
| P9 | S | S | S | S |  | 4/0 |
| P12 | S | S | S | S | S | 5/0 |
| P19 | S | S | S |  | S | 4/0 |
| P23 |  | S | S |  |  | 2/0 |
| S/F | 4/0 | 6/0 | 5/0 | 3/0 | 3/0 | 21/0 |

## Print-Uri Interoperability Matrix

This matrix shows the results of issuing of ‘create-job’ and ‘send-document’ requests from IPP Clients to IPP Printers.  IPP Clients are on the horizontal and IPP Printers are on the vertical.

All combinations tested succeeded.

|  | C1 | C2 | C4 | C9 | C11 | S/F |
| --- | --- | --- | --- | --- | --- | --- |
| P5 | S | S | S | S | S | 5/0 |
| P9 | S | S | S |  |  | 3/0 |
| P19 | S | S |  |  |  | 2/0 |
| P23 |  | S |  |  |  | 1/0 |
| P26 |  | S | S | S |  | 3/0 |
| P27 |  | S | S | S |  | 3/0 |
| S/F | 3/0 | 6/0 | 4/0 | 3/0 | 1/0 | 17/0 |

## Set 1 Interoperability Matrix

This matrix shows the results of testing the Set One operations from IPP Clients to IPP Printers.  IPP Clients are on the horizontal and IPP Printers are on the vertical.

It is unclear what level of testing was performed.  The test plan had several steps to perform.  It is not known if these steps were followed.  This was one of the last tests performed and not carefully monitored.  All combinations tested reported success.

NOTE:  The Bake-Off was for IPP v1.0.  The ‘Set One’ operations are not part of the v1.0 specification.

|  | C2 | C5 | C6 | C9 | S/F |
| --- | --- | --- | --- | --- | --- |
| P7 | S | S | S | S | 4/0 |
| P8 |  |  |  |  | 0/0 |
| P9 | S |  |  |  | 1/0 |
| P13 | S |  |  |  | 1/0 |
| P115 | S | S |  | S | 3/0 |
| P18 |  |  |  |  | 0/0 |
| P19 | S |  |  |  | 1/0 |
| S/F | 5/0 | 2/0 | 1/0 | 2/0 | 10/0 |

## Ipp:// scheme support  Matrix

This matrix shows the results of testing the ‘ipp://’ scheme from the C9 IPP Client to the IPP Printers.  This was tested using IPP v1.0 and IPP v1.1 in the IPP header.  The ‘accept’ column indicates that the version and scheme was accepted in a ‘print-job’ operation.  The ‘return’ column indicates the ‘ipp://’ scheme was returned for the job URL in a ‘get-jobs’ operation.  NOTE: I am unsure what ‘Rel’ indicates and counted those as untested.

NOTE:  The Bake-Off was for IPP v1.0.  The ‘ipp://’ scheme is not part of the v1.0 specification.

|  | IPP 1.0 Accept ipp:// | IPP 1.0 Return ipp:// | IPP 1.1 Accept ipp:// | IPP 1.1 Return ipp:// |
| --- | --- | --- | --- | --- |
| P1 | N | N | N | N |
| P2 | Y | N | Y | N |
| P3 |  |  |  |  |
| P4 | Y | Rel | Y | Rel |
| P5 | Y | Y | Y | Y |
| P6 | N | N | N | N |
| P7 | N | N | N | N |
| P8 | Y | N | N | N |
| P9 | Y | Y | N | N |
| P10 | Y | N | Y | N |
| P11 |  |  |  |  |
| P12 | Y | N | Y | N |
| P13 |  |  |  |  |
| P14 | Y | N | Y | N |
| P15 | Y | N | N | N |
| P16 | N | N | N | N |
| P17 | N | N | N | N |
| P18 | N | N | N | N |
| P19 |  |  |  |  |
| P20 | Y | Y | Y | Y |
| P21 |  |  |  |  |
| P22 | Y | N | Y | N |
| P23 | N | N | N | N |
| P24 | Y | N | Y | N |
| P25 | Y | Y | Y | Y |
| P26 | Y | Y | N | N |
| P27 | Y | Y | N | N |
| Y/N | 15/7 | 6/15 | 10/12 | 3/18 |

## HTTP get on IPP URL support  Matrix

This matrix shows the results of issuing an HTTP ‘Get’ on the IPP URL.  This was tested using Netscape Navigator without a proxy and Internet Explorer with a proxy.  The ‘Y’ indicates the printer returned a page of information.  Any other indication indicates a failure to return a page.

| ID | Netscape Navigator No Proxy | Internet Explorer Proxy |
| --- | --- | --- |
| P1 | Y | Y |
| P2 | Doc cont. no data | Time out |
| P3 | Y but redir | Y but redir. |
| P4 | N 404 | N 404 |
| P5 | N 405 | N 405 |
| P6 | N 404 | N 404 |
| P7 | Y CGI | Y CGI |
| P8 | Time out | Time out |
| P9 | Y | Y CGI |
| P10 | Doc cont. no data | Unknown HTTP Error |
| P11 | Y | Y |
| P12 | Y Exe file | Y Exe file |
| P13 | Doc cont. no data | N Invalid. response |
| P14 | Y | Y |
| P15 | Y | Y |
| P16 | N | N |
| P17 | N 405 | Time out |
| P18 | Y | Y |
| P19 | Y | Y |
| P20 | Y for HTTP | Y for HTTP |
| P21 | Y | Time out |
| P22 | Time out | Time out |
| P23 | Time out | Time out |
| P24 | N Object not found | N Object not found |
| P25 | Doc contains no data | Invalid response |
| P26 | N 400 | N 400 |
| P27 | N 400 | N 400 |
| Y/N | 12/15 | 11/16 |

## Operation Coverage

An attribute, group or operation is considered tested when two independent implementations are able to interoperate.  For the purposes of the Bake-off, that means at least two IPP Printers and two IPP Clients understand the attribute, group or operation.  Partial successes are listed giving the number of printers and clients that interoperate.  A value of ‘No’ in the ‘Tested’ column indicates that no printer supported the operation, attribute or group.

| ID | Operation | Mandatory | Tested |
| --- | --- | --- | --- |
| OC01 | print-job | Yes | Yes |
| OC04 | validate-job | Yes | Yes |
| OC05 | get-printer-attributes | Yes | Yes |
| OC06 | get-jobs | Yes | Yes |
| OC07 | get-job-attributes | Yes | Yes |
| OC08 | cancel-job | Yes | Yes |
| OC09 | print-uri | No | Yes |
| OC10 | create-job | No | Yes |
| OC11 | send-document | No | Yes |
| OC12 | send-uri | No | Yes |
| OC13 | hold-job | No | Yes |
| OC14 | release-job | No | Yes |
| OC15 | pause-printer | No | Yes |
| OC16 | resume-printer | No | Yes |
| OC17 | purge-printer | No | Yes |
| OC18 | restart-job | No | Yes |

## Operational Attributes Coverage

An attribute, group or operation is considered tested when two independent implementations are able to interoperate.  For the purposes of the Bake-off, that means at least two IPP Printers and two IPP Clients understand the attribute, group or operation.  Partial successes are listed giving the number of printers and clients that interoperate.  A value of ‘No’ in the ‘Tested’ column indicates that no printer supported the operation, attribute or group.

| Operational Attribute Coverage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ID | Operational Attribute | Group | Comment | Mandatory | Tested |
| OA01 |  | job-attribute | Tested by get-jobs | Yes | Yes |
| OA02 |  | printer-attribute | Tested by get-printer-attributes | Yes | Yes |
| OA03 |  | unsupported-attributes | Tested by print-job with unsupported attributes | Yes | Yes |
| OA04 |  | operational-attribute | See OA08 to OA30 | Yes | Yes |
| OA05 | version-number | preamble | Test with any operation | Yes | Yes |
| OA06 | operation-id | preamble |  | Yes | Yes |
| OA07 | request-id | preamble | Test with any operation | Yes | Yes |
| OA08 | attributes-charset | operational-attribute | Must support utf-8 | Yes | Yes |
| OA09 | attributes-natural-language | operational-attribute | Test imposes en-us requirement | Yes | Yes |
| OA10 | printer-uri | operational-attribute | This or OA11 must be 3rd attribute | Conditional | Yes |
| OA11 | job-uri | operational-attribute | This or OA10 must be 3rd attribute | Conditional | Yes |
| OA12 | job-id | operational-attribute | This must be 4th attribute | Conditional | Yes |
| OA13 | job-name | operational-attribute | Only for print & validate operations | Optional | Yes |
| OA14 | requesting-user-name | operational-attribute |  | Optional | Yes |
| OA15 | document-uri | operational-attribute | Only for print-uri | Conditional | Yes |
| OA16 | last-document | operational-attribute | Only for send-uri and send-document | Conditional | Yes |
| OA17 | status code | preamble | Test with any response | Yes | Yes |
| OA18 | status-message | operational-attribute | Test with any response | Optional | Yes |
| OA19 | compression | operational-attribute | Only for print & validate operations | Optional | Yes |
| OA20 | document-natural-language | operational-attribute | Only for print & validate operations | Optional | No |
| OA21 | ipp-attribute-fidelity | operational-attribute | Only for print & validate operations | Optional | Yes |
| OA22 | job-impressions | operational-attribute | Only for print & validate operations | Optional | No |
| OA23 | job-k-octets | operational-attribute | Only for print & validate operations | Optional | No |
| OA24 | job-media-sheets | operational-attribute | Only for print & validate operations | Optional | 2 Clients 1 Printer |
| OA25 | limit | operational-attribute | Only for get-jobs operations | Optional | Yes |
| OA26 | message | operational-attribute | Only for cancel operations | Optional | Yes |
| OA27 | my-jobs | operational-attribute | Only for get-jobs operations | Optional | Yes |
| OA28 | requested-attributes | operational-attribute | Only for get-* operations | Optional | Yes |
| OA29 | Document-format | operational-attribute | Only for print operations | Optional | Yes |
| OA30 | which-jobs | operational-attribute | Only for get-jobs operations | Optional | Yes |

## Attribute Coverage

An attribute, group or operation is considered tested when two independent implementations are able to interoperate.  For the purposes of the Bake-off, that means at least two IPP Printers and two IPP Clients understand the attribute, group or operation.  Partial successes are listed giving the number of printers and clients that interoperate.  A value of ‘No’ in the ‘Tested’ column indicates that no printer supported the operation, attribute or group.

### Printer Description

| Printer Description |  |  |  |  |
| --- | --- | --- | --- | --- |
| ID | Attribute | Syntax | Mandatory | Tested |
| PD01 | printer-uri-supported | uri | Yes | Yes |
| PD02 | uri-security-supported | 1setOf type2 keyword | Yes | Yes |
| PD03 | printer-name | name | Yes | Yes |
| PD04 | printer-state | type1 enum | Yes | Yes |
| PD05 | operations-supported | 1setOf type2 enum | Yes | Yes |
| PD06 | charset-configured | charset | Yes | Yes |
| PD07 | charset-supported | 1setOf charset | Yes | Yes |
| PD08 | natural-language-configured | naturalLanguage | Yes | Yes |
| PD09 | generated-natural-language-supported | 1setOf naturalLanguage | Yes | Yes |
| PD10 | printer-is-accepting-jobs | boolean | Yes | Yes |
| PD11 | pdl-override-supported | type2 keyword | Yes | Yes |
| PD12 | printer-up-time | integer | Yes | Yes |
| PD13 | printer-location | text | No | Yes |
| PD14 | printer-info | text | No | Yes |
| PD15 | printer-more-info | uri | No | Yes |
| PD16 | printer-driver-installer | uri | No | Yes |
| PD17 | printer-make-and-model | text | No | Yes |
| PD18 | printer-more-info-manufacturer | uri | No | Yes |
| PD19 | printer-state-reasons | 1setOf type2 keyword | No | Yes |
| PD20 | printer-state-message | text | No | Yes |
| PD21 | document-format-default | mimeMediaType | Yes | Yes |
| PD22 | document-format-supported | 1setOf mimeMediaType | Yes | Yes |
| PD23 | queued-job-count | integer | No | Yes |
| PD24 | printer-message-from-operator | text | No | Yes |
| PD25 | color-supported | boolean | No | Yes |
| PD26 | reference-uri-schemes-supported | 1setOf uriScheme | No | Yes |
| PD27 | printer-current-time | dateTime | No | Yes |
| PD28 | multiple-operation-time-out | integer | No | Yes |
| PD29 | compression-supported | 1setOf type3 keyword | No | Yes |
| PD30 | job-k-octets-supported | rangeOfInteger | No | Yes |
| PD31 | job-impressions-supported | rangeOfInteger | No | Yes |
| PD32 | job-media-sheets-supported | rangeOfInteger | No | Yes |

### Job Template

| Job Template |  |  |  |  |
| --- | --- | --- | --- | --- |
| ID | Attribute | Syntax | Mandatory | Tested |
| JT01 | job-priority | integer | No | Yes |
| JT02 | job-priority-default | integer | No | Yes |
| JT03 | job-priority-supported | integer | No | Yes |
| JT04 | job-hold-until | type4 keyword ¦ name | No | Yes |
| JT05 | job-hold-until-default | type4 keyword ¦ name | No | Yes |
| JT06 | job-hold-until-supported | 1setOf type4 keyword ¦ name | No | Yes |
| JT07 | job-sheets | type4 keyword ¦ name | No | Yes |
| JT08 | job-sheets-default | type4 keyword ¦ name | No | Yes |
| JT09 | job-sheets-supported | 1setOf type4 keyword ¦ name | No | Yes |
| JT10 | multiple-document-handling | type2 keyword | No | Yes |
| JT11 | multiple-document-handling-default | type2 keyword | No | Yes |
| JT12 | multiple-document-handling-supported | 1setOf type2 keyword | No | Yes |
| JT13 | copies | integer | No | Yes |
| JT14 | copies-default | integer | No | Yes |
| JT15 | copies-supported | integer | No | Yes |
| JT16 | finishings | 1setOf type2 enum | No | Yes |
| JT17 | finishings-default | 1setOf type2 enum | No | Yes |
| JT18 | finishings-supported | 1setOf type2 enum | No | Yes |
| JT19 | page-ranges | 1setOf rangeOfInteger | No | Yes |
| JT20 | page-ranges-supported | boolean | No | Yes |
| JT21 | sides | type2 keyword | No | Yes |
| JT22 | sides-default | type2 keyword | No | Yes |
| JT23 | sides-supported | 1setOf type2 keyword | No | Yes |
| JT24 | number-up | integer | No | Yes |
| JT25 | number-up-default | integer | No | Yes |
| JT26 | number-up-supported | 1setOf integer ¦ rangeOfInteger | No | Yes |
| JT27 | orientation-requested | type2 | No | Yes |
| JT28 | orientation-requested-default | type2 | No | Yes |
| JT29 | orientation-requested-supported | 1setOf type2 | No | Yes |
| JT30 | media | type4 keyword ¦ name | No | Yes |
| JT31 | media-default | type4 keyword ¦ name | No | Yes |
| JT32 | media-supported | 1setOf type4 keyword ¦ name | No | Yes |
| JT33 | media-ready | 1setOf type4 keyword ¦ name | No | Yes |
| JT34 | printer-resolution | resolution | No | Yes |
| JT35 | printer-resolution-default | resolution | No | Yes |
| JT36 | printer-resolution-supported | 1setOf resolution | No | Yes |
| JT37 | print-quality | type2 enum | No | Yes |
| JT38 | print-quality-default | type2 enum | No | Yes |
| JT39 | print-quality-supported | 1setOf type2 enum | No | Yes |

### Job Description

| Job Description |  |  |  |  |
| --- | --- | --- | --- | --- |
| ID | Attribute | Syntax | Mandatory | Tested |
| JD01 | job-uri | uri | Yes | Yes |
| JD02 | job-id | integer | Yes | Yes |
| JD03 | job-printer-uri | uri | Yes | Yes |
| JD04 | job-name | name | Yes | Yes |
| JD05 | job-originating-user-name | name | Yes | Yes |
| JD06 | job-state | type1 enum | Yes | Yes |
| JD07 | attributes-charset | charset | Yes | Yes |
| JD08 | attributes-natural-language | naturalLanguage | Yes | Yes |
| JD09 | job-more-info | uri | No | No |
| JD10 | job-state-reasons | 1setOf type2 keyword | No | Yes |
| JD11 | job-state-message | text | No | 1 Printer 2 Client |
| JD12 | number-of-documents | integer | No | Yes |
| JD13 | output-device-assigned | name | No | 1 Printer 2 Client |
| JD14 | time-at-creation | integer | No | Yes |
| JD15 | time-at-processing | integer | No | Yes |
| JD16 | time-at-completed | integer | No | Yes |
| JD17 | number-of-intervening-jobs | integer | No | Yes |
| JD18 | job-message-from-operator | text | No | 1 Printer 2 Client |
| JD19 | job-k-octets | integer | No | Yes |
| JD20 | job-impressions | integer | No | No |
| JD21 | job-media-sheets | integer | No | 1 Printer 2 Client |
| JD22 | job-k-octets-processed | integer | No | Yes |
| JD23 | job-impressions-completed | integer | No | 1 Printer 2 Client |
| JD24 | job-media-sheets-completed | integer | No | Yes |

## Syntax Coverage

| Syntax Coverage |  |  |  |  |
| --- | --- | --- | --- | --- |
| ID | Syntax | Attribute | Mandatory | Tested |
| SC01 | text | printer-state-message | No | Yes |
| SC02 | textWithLanguage | Printer-location | No | Yes |
| SC03 | name | printer-name | Yes | Yes |
| SC04 | nameWithLanguage | printer-name | Yes | Yes |
| SC05 | keyword | pdl-override-supported | Yes | Yes |
| SC06 | enum | printer-state | Yes | Yes |
| SC07 | uri | printer-uri-supported | Yes | Yes |
| SC08 | UriScheme (1 setOf) | reference-uri-schemes-supported | No | Yes |
| SC09 | charset | charset-configured | Yes | Yes |
| SC10 | naturalLanguage | natural-language-configured | Yes | Yes |
| SC11 | mimeMediaType | document-format-default | No | Yes |
| SC12 | octetString |  |  | No |
| SC13 | boolean | printer-is-accepting-jobs | Yes | Yes |
| SC14 | integer | printer-up-time | Yes | Yes |
| SC15 | rangeOfInteger | job-k-octets-supported | No | Yes |
| SC16 | dateTime | printer-current-time | No | Yes |
| SC17 | resolution | printer-resolution | No | Yes |
| SC18 | 1setOf X (1setOf type2 enum) | operations-supported | Yes | Yes |

# Issues

1)  PROBLEM:  Is 'application/octet-stream REQUIRED?
Is application/octet-stream REQUIRED.  IPP/1.0 appears not to require it, while IPP/1.1 indicated "REQUIRED".

2)  ADDITION:  We would like to add another operation that forces the server to generate a 401 authentication challenge. 
This is very useful for a client to be able to get into identified mode as soon as possible. Today you have to wait to be challenged by the server, which may never happen – or happens at an unpredictable time.  Unless somebody has a different solution  (Microsoft)

3)  ISSUE:  How reject down stream auto-sensed unsupported PDL?
If auto-sensing happens AFTER the job is accepted (as opposed to auto-sensing at submit time before returning the response), what does the implementation do?
Presumably, it is similar to encountering a mal-formed PDL.  So the implementation aborts the job, puts the job in the 'aborted' state and sets the 'aborted-by-system' value in the job's "job-state-reasons", if supported.

4)  PROBLEM:  Client closes slow channel
Some IPP Printer implementations, such as forwarding servers, want to accept an IPP job, even though the down stream channel is being used at the moment by another job stream that the device supports.  Rejecting the job would mean that an IPP job might never get in, since these other protocols queue the request.
However, some clients close the channel when it is flowed controlled off for too long a time?
Suggested fix:  Clients MUST NOT close the channel when flowed controlled off.  Clients SHOULD do Get-Printer-Attributes and determine state of the device.  Alert user if the printer is stopped.  Let user decide whether to abort the job transmission or not.  Add a new success-ok-but-very-busy status code?

5)  PROBLEM:  Client closes stopped device
When a non-spooling printer is accepting data and putting it on media and runs into a problem, such as paper out or paper jam, what should it do?
Returning an error is not user friendly, if fixing the problem would allow the job to complete normally.
Suggested fix:  clients must not close the channel.  Clients SHOULD do Get-Printer-Attributes and determine state of the device.  Alert user if the printer is stopped.  Let user decide whether to abort the job transmission or not.

6)  PROBLEM:  IPP server supports deflate and gzip.
If client sets "compression attribute = deflate" and sends gziped data, what error does IPP server return to client?

7)  CONVENTION:  Please implement Manufacturer make and model printer attribute and send the .INF file model name of the printer.
If you do this we will automatically install the correct driver (if we have it)  (Microsoft)

8)  ISSUE:  In IPP/1.0 Model and semantics 3.2.6.1, the definition for "limit", "which-jobs" and "my-jobs" is contradicting each other. 
The problem is that the definition for "which-jobs" and "my-jobs" states that a group of job MUST be returned. (Stefan Andersson Axis Communication AB)

9)  PROBLEM:  Customers become very unhappy when they go to the printer to pick up their job and a ream of PostScript source code is sitting in theoutput bin.
Cause:  A PostScript datastream is accidentally sent to a PCL printer.
IPP Issue:  IPP needs to clarify the standard in section 3.2.1.1 of the Model and Semantics document.  Lines 1219-1221 state that:
   If the client does not supply [the document format] attribute,
   the Printer object assumes that the document data is in the
   format defined by the Printer object's "document-format-default"
   attribute.
I would like to see the following clarification:
   If the client does not supply [the document format] attribute
   and the Printer object is not able to auto-sense the document
   format at print-job request time, the Printer object assumes
   that the document data is in the format defined by the Printer
   object's "document-format-default" attribute.
If the Printer object senses that the document format is PostScript, then job should be rejected if it is being sent to a PCL-only printer.  The 'application/octet-stream' mechanism discussed in section 4.1.9 does not seem to be helpful in this case, because it appears to assume that the auto-sensing occurs at document processing time.  Until the document is actually "ripped", the document format remains unknown.  So it seems to me that lines 2453-2476 do not address the problem described above where the wrong document format is submitted. These lines, rather, seem to apply to the case of a printer that handles multiple document formats and assumes that the submitted document is in one of the supported formats.

10)  ISSUE:  How distinguish between submit vs processing auto-sense?
There are two different implementations of auto-sensing:
    at print submit time BEFORE the Print-Job or Send-Document responds
    at document processing (ripping) time AFTER the Print-Job or Send-Document has accepted the job.
The description of 'application/octet-stream' doesn't clarify whether one, the other or both is meant.  How can a client determine which is supported?
Possible solutions:
1. Clarify that 'application/octet-stream' means either depending on implementation
2. Add a new value that means auto-sense at request time and clarify that 'application/octet-stream' means at processing time.
3. Add a new value that means auto-sense at processing time and clarify that 'application/octet-stream' means at request time.
4. Do 1 and add two new values that mean at request time and at processing time.

11)  ISSUE:  If a server receives a request with a document format which is not supported, it returns the client-error-document-format-not-supported  (0x040A) status code. Is it also necessary to include document format in the unsupported attribute group? 
We suggest text which says it need not be supplied in the unsupported group.

12)  ISSUE:  length fields for the "UNSUPPORTED" tag
IPP/1.0: Model and Semantics, 16 Nov 1998, 3.2.1.2, Group 2 (unsupported attributes) -- states that in the case of an unsupported attribute name, the printer object should return a substituted out of band value of "unsupported". This impression is strengthened by the reference to section 4.1, where it gives the legal out of band values, none of which is an empty string.
This appears to conflict with Internet Printing Protocol/1.0: Encoding and Transport, 16 Nov 1998, section 3.10, where it states that the value length must be 0 and the value empty.  (Claudio Cordova, Wade Mergenthal Xerox Corp.)

13)  PROBLEM:  What job-state value should be returned in the Create-Job response?  Pending, pending-held, or either depending on implementation?
The problem with 'pending' is that the job is not a "candidate to start processing" as the definition states.  The 'pending-held' state seems more reasonable.  Its definition is: 'pending-held':  The job is not a candidate for processing for any number of reasons but will return to the 'pending' state as soon as the reasons are no longer present.  The job's "job-state-reason" attribute MUST indicate why the job is no longer a candidate for processing.  Also there is a "job-state-reason" value 'job-incoming' which states: 'job-incoming':  The Create-Job operation has been accepted by the Printer, but the Printer is expecting additional Send-Document and/or Send-URI operations and/or is accessing/accepting document data.  But "job-state-reasons" is OPTIONAL.  Do we mandate it or recommend it if supporting Create-Job?

14)  PROBLEM:  Job-state for a forwarding server?
What job-state value should be returned in the Print-Job response for an IPP object that forwards the data over a one-way interface, such as a parallel port or LPD?  pending, processing, completed, or unknown?  Unknown is the strict interpretation of section 4.3.7 "job-state", but it isn't very user friendly.  The "job-state" SHOULD reflect the actual job state, but these implementations have no idea when the job actually starts or finishes.
How about a new "job-state-reasons" value: 'queued-in-device' (from PWG Job Monitoring MIB)?

15)  ISSUE:  ‘unknown’ and ‘unsupported’ Out of band values.
It is very unclear from the spec as to whether or not you should use the word ‘unknown’ (or unsupported in that case) as the value for attributes that are unknown.
You can read it that you set the length equal to zero and set the type to ‘unknown’. You can also read it as saying you set the value to the string’unknown’.
This is not helped by the spec saying – you must set the length to zero and then telling a client what to do with a non-zero length. (Microsoft)

16)  CONVENTION:  GetPrinterAttributes Polling
Some client polls printer periodically by GetPrinterAttributes without specifying requested^attributes. So printer has to reply all attributes. It consumes printer resouce.  Client should specify requested-attriubtes, if it wants to get printer status.

17)  ISSUE:  What are clients doing with printers that don’t support absolute time? How can client display an absolute time (which is what is useful for a user)?
Possible Solution
Get Uptime from printer 
Get Job(s) 
Calculate Display time = job tick time – uptime + local client absolute time.   The down side is that the client has to get the uptime every time  (Microsoft)

18)  PROBLEM:  Return all errors on Print-Job fidelity=true
If ipp-attributes-fidelity=true, MUST all attributes that are not supported, be returned, or can just the first error be returned?

19)  ISSUE:  User Performing the Operation
The Send-Document and Send-URI commands need the following clarification with 
regard to the user performing the operation user.  In the requesting-user-name section of Send-Document add:
The user performing the Send-Document operation must be the same as for the Create-
Job operation that created the job.  The printer determines the user performing the 
operation from the requesting-user-name or the underlying authentication mechanism as 
described in Section 8.3 of the model document.

The wording in the Send-URI section would imply that the above change applies to Send-URI as well.

20)  PROBLEM:  Non-spooling printers accept/reject additional jobs
Some IPP Printer implementations reject a second Print-Job (or Create-Job) while they are processing a Print-Job.  Other IPP Printer implementations, such as forwarding servers and non-spooling printers, accept some number of subsequent jobs, but flow control them off until the first job is finished.
Suggested fix:  Add a new success-ok-but-very-busy status code so that clients and servers (acting as clients) would know.  Also finish our notification extension so that a device that rejects the submit could subscribe for when the device is ready to accept another job.

21)  ISSUE:  Does 'none' uri-security-supported mean Basic/Digest?
Section 4.4.2 "uri-security-supported" 'none' values says:  'none': There are no secure communication channel protocols in use for the given URI.  Should be clarified that the REQUIRED Basic and Digest are intended for the 'none' value. (Hugo Parra)

22)  ISSUE:  Status code on variable-length attributes that are 'too short'
IPP defines a status code 'client-error-request-value-too-long' for a variable-length attribute that exceeds the maximum length allowed by the attribute.  However, it is not clear what status code to use in the opposite case, i.e. the supplied attribute value is shorter than the requirement.  In the current spec, this problem will arise when a 0-length value is supplied in 'keyword' attributes.  In this case, should the request be rejected with status code 'client-error-request-value-too-long' or 'client-error-bad-request' ?  Furthermore, if ipp-attribute-fidelity is false, should the request be rejected at all ? (Robert's opinion is that, the request should be accepted with the problematic value ignored, even though it violated the 'keyword' syntax)  (Jason Chien-Hung Chen)

23)  ISSUE:  There seems to be some misunderstanding about the unsupported-attributes group.  
Some implementations return the unsupported-attributes group on a get-attributes operation.  The unsupported-attributes presumably contains all the attributes the implementation knows about but does not support.  I do not believe this is the proper use of the unsupported-attributes group.  Do we need a clarification in the specification.

24)  ISSUE    What status does Get-Jobs return when no jobs?
Should Get-Jobs return 'successful-ok' when there are no jobs to be returned?  The client can see that the Jobs group contains no jobs from the response. Returning an error may confuse the client.  Some implementations returned 'client-error-not-found' error code.

25)  ISSUE  - May an IPP object return more Operation attributes?
It is ok for an IPP object to return additional operation attributes in a response (as an extension to the standard)?  If so, then the client MUST ignore or do something with them.  (Hugo Parra)

26)  ISSUE - May an IPP object return additional groups?
It is ok for an IPP object to return additional groups of attributes in a response (as an extension to the standard)?  For example, returning the "job-state" and "job-state-reasons" in a Hold-Job, Release-Job, and/or Cancel-Job operation.  What about newly registered groups of attributes.  If so, then the client MUST ignore or do something with them.  (Hugo Parra)