Subj:  JMP proposal to register mediumTypeConsumed and mediumSizeConsumed

From:  Tom Hastings

Date:  4/29/98

File:  001-medium-xxx-consumed.doc

Status:  APPROVED, Wednesday, April 29, 1998:  No comments were received during the Last Call.  This proposal was reviewed at the PWG meeting, on Friday, April 10, 1998.  It was agreed to send it out for a two-week WG Last Call.  Any comments are due by Monday, April 27, 1998.

Problem:  The Job Monitoring MIB V1.0 does not have a way for an accounting program to charge for media type or media size consumed.  The current mediumConsumed in the name of the medium, which may or may not be known to the accounting program.

Suggested Solution:  This proposal to register two new accounting attributes to be used with the approved PWG Job Monitoring MIB standard.  These new attributes count media by medium type name and medium size name.  The current PWG Job Monitoring MIB specifications for "mediumRequested" (medium type enum, medium name and medium size name) and mediumConsumed" (medium name) are:

mediumRequested(170),	JmMediumTypeTC
	AND/OR
	JmJobStringTC (SIZE(0..63))
INTEGER:  MULTI-ROW:  The type 
AND/OR
OCTETS:  MULTI-ROW:  the name of the medium that is required by the job.

NOTE - The name (JmJobStringTC) values correspond to the prtInputMediaName object in the Printer MIB [print-mib] and the values of the IPP 'media' attribute.

mediumConsumed(171),	Integer32 (-2..2147483647)
	AND
	JmJobStringTC (SIZE(0..63))
INTEGER:  MULTI-ROW:  The number of sheets

AND

OCTETS:  MULTI-ROW:  the name of the medium that has been consumed so far whether those sheets have been processed on one side or on both.

This attribute SHALL have both Integer32 and OCTET STRING (represented as  JmJobStringTC) values.

NOTE - The name (JmJobStringTC) values correspond to the name values of the prtInputMediaName object in the Printer MIB [print-mib].

Suggested text for the registration of the mediumTypeConsumed and mediumSizeConsumed attributes:

mediumTypeConsumed(174),	Integer32 (-2..2147483647)
	AND
	JmJobStringTC (SIZE(0..63))
INTEGER:  MULTI-ROW:  The number of sheets of the indicated medium type that has been consumed so far whether those sheets have been processed on one side or on both

AND

OCTETS:  MULTI-ROW:  the name of that medium type.

This attribute SHALL have both Integer32 and OCTET STRING (represented as JmJobStringTC) values.

NOTE - The type name (JmJobStringTC) values correspond to the type name values of the prtInputMediaType object in the Printer MIB [print-mib].  Values are: 'stationery', 'transparency', 'envelope', etc. These medium type names correspond to the enum values of JmMediumTypeTC used in the mediumRequested attribute.

mediumSizeConsumed(175),	Integer32 (-2..2147483647)
	AND
	JmJobStringTC (SIZE(0..63))
INTEGER:  MULTI-ROW:  The number of sheets of the indicated medium size that has been consumed so far whether those sheets have been processed on one side or on both

AND

OCTETS:  MULTI-ROW:  the name of that medium size.

This attribute SHALL have both Integer32 and OCTET STRING (represented as JmJobStringTC) values.

NOTE - The size name (JmJobStringTC) values correspond to the size name values in the Printer MIB [print-mib] Appendix B.  Values are:  'letter', 'a', 'iso-a4', 'jis-b4', etc.