Subj:	Problems and Solutions to the jmResourceTable continued

From:	Tom Hastings

Date:	3/13/97, version 32

File:	res-type.doc

This version of the JMResourceTC and jmResourceTable is the results of the 2/25/97 JMP teleconm  The revision marks in the explanation up front is with respect to the 2/24/97 version 2 res-type.doc.  The revision marks in the ASN.1 are with respect to the original V0.6 MIB, dated 1/23/97.

There are several problems with the JmResourceTC and jmResourceTable:

- The text appears to say that the same resource is used for both the initial requested value and then reset and used to reflect the actual.  There are no actual resources with such a specification, so the DESCRIPTION should be clarified to indicate that requested and consumed require two separate resources.

- The jmResourceName object cannot be a union of a text string and an integer.

- so we agreed to replace jmResourceName with jmResourceType

- We agreed to have two resource values, one as an integer and one as a text string: jmResourceValueAsText and jmResourceValueAsInteger.  We can even overload the integer to be an index when the value is an index instead of an amount, such as physicalDeviceIndex(11) AND can be an enum when the value is an enum, such as documentFormat(10).

- Instead of having a separate object of the units, we decided to make the jmResourceType enum specify the units as part of the description of the enum.  This may cause us to add a few more enums, when we want to have several different units possible, but the simplicity is an improvement which will contribute to interworking.  Then we are reducing the number of objects in the jmResourceTable by one:

| Current | New |
| --- | --- |
| jmResourceType | renamed to: jmResourceTypeIndex |
| jmResourceIndex | renamed to: jmResourceInstanceIndex |
| jmResourceName | jmResourceValueAsText |
| jmResourceUnits | deleted |
| jmResourceAmount | jmResourceValueAsInteger |

- Clarify for each resource whether jmResourceValueAsText or jmResourceValueAsInteger or both (mediaConsumed) are used.  When jmResourceValueAsText isn’t used, the agent shall return a zero length string.  When the jmResourceValueAsInteger isn’t used, the agent shall return a (-1) signifying other as in the Printer MIB.

- I’ve deleted unknown(2), since an unknown resource wouldn’t be put in the table at all.

- I've added mediumRequested and mediaConsumed.

- I've added outputBin, colorantRequested, and colorantConsumed.

- I’ve added (moved from jmJobTable):  jobSourceChannel, jobSubmissionTime, jobComment, jobTotalKOctets, jobKOctetsCompleted, jobStartedProcessingTime, jobCompletionTime, jobAccountName.

- I’ve deleted faxPhoneNumber(12).

- I’ve renamed existing resources as shown, as per Keith Carter’s e-mail:

| Old | New |
| --- | --- |
| pagesSpooled(15) | impressionsSpooled(15) |
| pagesInterpreted(16) | impressionsInterpreted(16) |
| pagesSentToDevice(17) | impressioinsSentToDeviceCurrentCopy(17) |
| pagesCompleted(18) | impressionsCompletedCurrentCopy(18) |

- I’ve renumberd the enums to fill gaps and overlaps.

- As Harry has pointed out, there are more than just resources in the jmResourceTable such as fileName, documenName, jobSubmissionTime, etc., so I’ve renamed the group and table to jmAttributeTable.  So we have only the following indexes and objects in the jmAttributeTable:

Table 1

| Current | New |
| --- | --- |
| jmJobSetIndex | jmJobSetIndex |
| jmJobIndex | jmJobIndex |
| jmResourceType | renamed to: jmAttributeTypeIndex |
| jmResourceIndex | renamed to: jmAttributeInstanceIndex |
| jmResourceName | jmAttributeValueAsText |
| jmResourceAmount | jmAttributeValueAsInteger |

-- textual-convention : JmAttributeTypeTC

JmAttributeTypeTC ::= TEXTUAL-CONVENTION

STATUS	current

DESCRIPTION

"The type of the attribute.  Attributes may represent information about a job, such as a file-name, or a document-name, or submission-time or completiontime.  Attribute may also represent resources, e.g., medium, ink, staples, processing-time, color-impressions, etc. required to process the job before the job start processing OR  to indicate the amount of the resource that is being consumed while the job is processing.  If both a required and a consumed value of a resource is needed, two separate attribute rows shall be used.

In the following definitions of the enums, each description indicates whether the value of the attribute shall be represented using the  shall uuteValueAsInteger or the jmAttributeResourceext objects by the initial tag: "Integer:" or "Text:", respectively. to indicate the value of theteresources use bots (mediaConsumed)and so have both tags (mediaConsumed).

If the jmAttributeValueAsInteger object is not used (no "Integer:" tag), the agent shall return the value (-1) indicating other.  If the jmAttributeResourceext object is not used (no "Text:" tag), the agent shall return a zero-length string.

The standard attribute types defined so far are:"

-- This is a type 2 enumeration.

SYNTAX	INTEGER {

| --jm --Attribute --TypeIndex |  | Description |
| --- | --- | --- |
| other(1), | -- -- -- | A attribute that is not in the list and/or that has not been registered with IANA. |
| fileName(3), | -- -- -- -- -- -- | Text: The coded character set file name of the document. A row with this attribute item may appear more than once in the jmAttributeTable for a job. |
| documentName(4), | -- -- -- -- -- -- | Text: The coded character set name of the document. A row with this attribute item may appear more than once in the jmAttributeTable for a job. |
| jobCopiesRequested(5), | -- -- -- -- | Integer: The number of copies of the entire job that are to be produce A value of -2 means unknown. |
| jobCopiesCompleted(6), | -- -- -- -- -- | Integer: The number of copies of the entire job that the entire job has completed so far. A value of (-2) means unknown. |
| documentCopiesRequested(7), | -- -- -- -- -- -- -- | Integer: The total count of the number of document copies requested. If there are documents A, B, and C, and document B is specified to produce 4 copies, the number of document copies requested is 6 for the job. |
| documentCopiesCompleted(8), | -- -- -- -- -- -- -- -- | Integer: The total count of the number of document copies completed so far for the job as a whole. If there are documents A, B, and C, and document B is specified to produce 4 copies, the number of document copies starts a 0 and runs up to 6 for the job as the job processes. |
| sides(9), | -- -- -- | Integer: The number of sides that any document in this job will require or did use. |
| documentFormat(10), | -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- | Integer: The interpreter language family corresponding to the Printer MIB prtInterpreterLangFamily object, that this job requires and uses. A document or a job may use more than one PDL. As with all intensive attribute items where multiple rows are allowed, there shall be only one distinct row for each distinct PDL; there shall be no duplicates. A row with this attribute item may appear more than once in the jmAttributeTable for a job, but the jmAttributeValueAsText shall be different for each such row. This enum is a type 2 enum. NOTE: This textual convention is imported from the draft Printer MIB, but is not in RFC 1759. documentFormat(10) TC in this module as well? |
| physicalDeviceIndex(11), | -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- | Integer: The index of the physical device MIB instance requested/used. This value is an hrDeviceIndex value. See the Host Resource MIB. A row with this attribute item may appear more than once in the jmAttributeTable for a job that is using more than one physical device, but the jmAttributeValueAsInteger shall be different for each such row. If there is no physical device MIB instance for this job, this row shall not be present in the jmAttributeTable. |
| physicalDeviceName(12) | -- -- -- | Text: The name of the physical device to which the job is assigned. A row with this attribute item may appear more than once in the jmAttributeTable for a job that is using more than one physical device, but the jmAttributeValueAsText shall be different for each such row. |
| impressionsCompleted(12), | -- -- -- -- -- -- -- -- -- -- | Integer: The number of impressions completed by this job. For a print job, an impression is the marking of the entire side of a sheet. Two-sided processing involves two impressions per sheet. The value of this attribute shall be 0 if processing has not started for this job. |
| sheetsCompleted(13), | -- -- -- -- -- -- -- | Integer: The number of medium sheets that have been completed whether those sheets have been processed on one side or on both. The value of this attribute shall be 0 if processing has not started for this job. |
| mediumRequested(14), | -- -- -- -- -- -- -- -- | Text: The name of the medium that are required by the job. A row with this attribute item may appear more than once in the jmAttributeTable for a job, but the jmAttributeResourceext shall be different for each such row. |
| mediaConsumed(15), | -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- | Text: The name of the medium and Integer: the number of sheets that have been consumed whether those sheets have been processed on one side or on both. A row with this attribute item may appear more than once in the jmAttributeTable for a job, but the jmAttributeResourceext shall be different for each such row. The value of this attribute shall be 0 if processing has not started for this job. |
| impressionsSpooled(16), | -- -- -- | Integer: The number of logical pages spooled to the server or device for the job. |
| impressionsInterpreted(17), | -- -- | Integer: The number of logical pages interpreted for the job. |
| impressionsSentToDevice(18), | -- -- | Integer: The number of logical pages sent to the device for the job. |
| impressionsCompletedCurrentCopy(19), | -- -- -- -- -- -- | Integer: The number of logical pages completed for the job. The value of this attribute shall be 0 if processing has not started for this job. |
| pagesCompletedCurrentCopy(20), | -- -- -- -- -- | Integer: The number of logical pages completed for the current copy of the document. This value is reset to 0 for each document and for each document copy. |
| processingCPUTime(21), | -- -- -- -- -- -- | Integer: The amount of time that the job has been processing in seconds. The value of this attribute shall be 0 if processing has not started for this job. |
| processingMessage(22), | -- -- -- -- -- -- -- -- -- | Text: A coded character set message that is generated during the processing of the job as a simple form of processing log to show progress and any problems. A row with this attribute item may appear more than once in the jmAttributeTable for a job. |
| outputBin(23) | -- -- -- -- -- -- -- -- -- | Text: The name of the output bin to which all or part of the job is placed in. A row with this attribute item may appear more than once in the jmAttributeTable for a job, but the jmAttributeResourceext shall be different for each such row. |
| colorantRequested | -- -- | Text: The name of the colorant requested. A row with this attribute item may appear more than once in the jmAttributeTable for a job, but the jmAttributeResourceext shall be different for each such row. |
| colorantConsumed(24) | -- -- -- -- -- -- -- -- | Text: The name of the colorant consumed. A row with this attribute item may appear more than once in the jmAttributeTable for a job, but the jmAttributeResourceext shall be different for each such row. |
| jobSourceChannelIndex(25) | -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- | Integer: The index of the row in the associated Printer MIB of the channel which is the source of the print job. See RFC 1759. Must be 1 or greater. NOTE - the Job Monitoring MIB points to the Channel row in the Printer MIB, so there is no need for a port object in the Job Monitoring MIB, since the PWG is adding a prtChannelInformation object to the Channel table of the draft Printer MIB. |
| jobSubmissionDateAndTime(26) | -- -- -- -- -- -- -- -- | Text: The date and time that the job was submitted. The value shall be specified using the DateAndTime textual convention from SMIv2-TC (see page ?). NOTE: DateAndTime is not printable characters. |
| jobSubmissionDateAndTimeStamp(26) | -- -- -- -- -- | Number: The time that the job was submitted. The value shall be specified using the TimeStamp textual convention from SMIv2-TC (see page ?). |
| jobStartedProcessingDateAndTime(27) | -- -- -- -- -- | Text: The date and time that the job started processing. The value shall be specified using the DateAndTime textual convention from SMIv2-TC (see page ?). |
| jobStartedPrintingTimeStamp(27) | -- -- -- -- -- | Number: The time that the job started processing. The value shall be specified using the TimeStamp textual convention from SMIv2-TC (see page ?). |
| jobCompletedTime(28) | -- -- -- -- -- -- | Text: The date and time that the job completed processing and the medium is completely in the output bin. The value shall be specified using the DateAndTime textual convention from SMIv2-TC (see page ?). |
| jobCompletedTime(28) | -- -- -- -- -- -- | Number: The time that the job completed processing and the medium is completely in the output bin. The value shall be specified using the TimeStamp textual convention from SMIv2-TC (see page ?). |
| jobComment(29) | -- -- -- -- -- -- -- -- -- -- | Text: An arbitrary human-readable coded character text string supplied by the submitting user or the job submitting application program for any purpose. For example, a user might indicate what he/she is going to do with the printed output or the job submitting application program might indicate how the document was produced. |
| jobTotalKOctets(30) | -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- | Number: The total number of K (1024) octets to be processed in the job, including document and job copies. The agent shall round the actual number of octets up to the next highest K. Thus 0 octets shall be represented as 0, 1-1024 octets shall be represented as 1, 1025-2048 shall be represented as 2, etc.. The server/device may update the value of this object after each document has been transferred to the server/device or the server/device may provide this value after all documents have been transferred to the server/device, depending on implementation. In other words, while the job is in the preProcessing state and when the job is in the held state with the jmJobStateReasons object containing a documentsNeeded value, the value of the jobTotalKOctets object depends on implementation and may not correctly reflect the size of the job. In computing this value, the server/device shall include the multiplicative factors contributed by (1) the number of document copies, and (2) the number of job copies, independent of whether the device can process multiple copies of the job or document without making multiple passes over the job or document data and independent of whether the output is collated or not. Thus the server/device computation is independent of the implementation and shall be: Document contribution: Multiply the size of each document in octets by the number of document copies of that document. Add each document contribution together. Job copy contribution: Multiply the job size by the number of job copies. Round up the result to the next higher K (1024 multiple). The total K octets to be processed can be used in the denominator with the jmJobKOctetsCompleted object in the numerator in order to produce a "thermometer" that indicates the progress of the job. The value (-2) means unknown. |
| jobKOctetsCompleted(31) | -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- | The number of K (1024) octets currently processed by the device, including document and job copies. For printing, the completed count includes processing (interpreting) and marking. For scanning, the completed count include scanning. It is recommended that the agent shall round the actual number of octets completes down to the next lowest K, until the job is completed. Thus 0-1023 octets is represented as 0, 1024-2048 is represented as 1, etc. When the job completes, the agent shall round up, using the same algorithm as for the jobTotalKOctets attribute. Thus when the job completes, the values of the jobTotalKOctets and the jmJobKOctetsCompleted attributes shall be equal. For multiple copies generated from a single data stream, the value shall be incremented as if each copy was printed from a new data stream without resetting the count between copies. See the pagesCompletedCurrentCopy attribute defined on page 1 attribute that is reset on each document copy. The total K octets completed can be used in the numerator with the jobTotalKOctets attribute in the denominator in order to produce a "thermometer" that indicates the progress of the job. The value of this object shall be 0 if processing has not started for this job. |
| jobAccountName(32) | -- -- -- -- -- -- -- -- -- -- | Text: Arbitrary binary information which may be coded character set data or encrypted data supplied by the submitting user for use by accounting services to allocate or categorize charges for services provided, such as a customer account name. NOTE: This attribute need not be printable characters. |

}

shResourceAmount ob these units.

SYNTAX	INTEGER {

|  |  | not one of th |
| --- | --- | --- |
| , | -- | jmResourceValueAsIntege -1 (ot |
|  |  | sysUptTime in the host reso |

| -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- | The Attribute Group The jmAttributeGroup consists attributes of the job and document(s). Attribute may represent information about the job and document(s), such as file-names, document-names, submission-time, completion-time, size. Attributes may also represent requested and/or consumed resources for each job. Instead of allocating distinct objects for each attribute, each attributeresource represented as a separate row in the jmAttributeTable. Each column in the row describes the attributeresources its type represented as an enum, its namevalue represented as text or an integer (or both)amount. rces are allocated to the jmAttributeTable (instead of distinct objects in the jmJobTable) that monitoring applications are unlikely to need to access multiple times, since they have to scan the entire jmAttributeResource find a particular object. Most attribute items shall have only one row per job. However, a few attribute items can have multiple values per job or even per document, where each value is a separate row in the jmAttributeTable. Unless indicated otherwise, an agent shall ensure that each attribute item occurs only once in the jmAttributeTable. Resource items that may appear multiple times in the jmAttributeTable are indicated in their specification in the JmAttributeTypeTC (see page 1). However, such attributeresourcehall not contain duplicates for "intensive" (as opposed to "extensive") attributeresourceexample, each dococumentFormat(11) shall appear in the jmAttributeResourcely once for a job since the interpreter language is an intensive attributeresourceven though the job has a number of documents that all use the same PDL. As another example of an intensive attributeresourcen have multiple entries, if a document or job uses multiple types of media, there shall be only one row in the jmAttributeResourcer each media type, not one row for each document that uses that medium type. On the other hand, if a job contains two documents of the same name, there can be separate rows for the documentName(4) attributeresourceth the same name, since a document name is an extensive attributeresource The jmAttributeGroup consists entirely of the jmAttributeTable which is indexed by (from most significant to least significant): - jmJobSetIndex - an instance index to distinguish separate sets of tables when a server supports more than one device. - jmJobIndex - the job identifier that was generated by the server or device that accepted the job. - jmAttributeTypeIndex - the enum that indicates the type of attributeresourcemAttributeResourcen page 1. - jmAttributeInstanceIndex - a running index of attributes of the same type for each job. For those attributes with only a single instance per job, this index value shall be 1. For those attributes that are a single value per document, the index value shall be the document number, starting with 1 for the first document in the job. Jobs with only a single document shall use the index value of 1. For those attributes that can have multiple values per job and per document, such as documentFormat, the index shall be a running index for the job as a whole, starting at 1. The jmAttributeTable is a per job table with an extra index for each type of attributeresourceibuteTypeIndex) that a job can have and an additional index (jmAttributeInstanceIndex)for those attributes that can have multiple instances per job.require mAttributeResourcex object shall contain an enum type that indicates the type of attributeresource. Sibuteresource typsed to represent a resources that is both for requested consumed as a single values, dependingon the point in time, while other attributes have distinct types for requested versus consumed values. The agent is able to discover the attributes resources rethe job submission protocol itself or from the document PDL. As the documents are interpreted, the interpreter may discover additional attributeresources theqadditional rows to this table. As the resources are actually consumed, the usage counter contained in the jmAttributeResourceValuer object is incremented according to the units indicated in the description of the enum. See JmAttributeTypeTC on page 1by the jmResourc Some attributes are mandatory for conformance, and the rest are optional. The mandatory attributes are: sheetsCompleted(14) she tho shethe Implementation of every object in this group is mandatory. See Section Error: Reference source not found entitled "Error: Reference source not found" on page Error: Reference source not found. |
| --- | --- |

jmAttribute  OBJECT IDENTIFIER ::= { jobmonmib 9 }

jmAttributeTable  OBJECT-TYPE

SYNTAX	SEQUENCE OF JmAttributeEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"A table of attributes  for eac job in each job set.  Attributes may represent information about the job and document(s) or resources required and/or consumed."

::= { jmAttribute 1 }

jmAttributeEntry  OBJECT-TYPE

SYNTAX	JmAttributeEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"Zero or more entries exist in this table for each job in each job set.  Each job shall appear in one and only one job set."

INDEX  { jmJobSetIndex, jmJobIndex, jmAttributeTypeIndex, jmAttributeInstanceResource

::= { jmAttributeTable 1 }

JmAttributeEntry ::= SEQUENCE {

| jmAttributeTypeIndex | JmAttributeTypeTC, |
| --- | --- |
| jmAttributeInstanceIndex | Integer32(1..32767), |
| jmAttributeValuesAtInteger | Integer32(0..2147483647) |
| jmAttributeValueAsText | OCTET STRING(SIZE(0..63)) |

}

jmAttributeTypeIndex OBJECT-TYPE

SYNTAX	JmAttributeTypeTC   -- See page 1

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"The type of attribute.  The type may identify information about the job or document(s) or may identifiy a resource required to process the job before the job start processing and/or consumed by the job as the job is processed.

Examples include jobCopiesRequested, documentCopiesRequested, jobCopiesCompleted, documentCopiesCompleted, fileName, and documentName.  See the JmAttributeTypeTC textual convention on page 1.

This value is a type 2 enum."

::= { jmAttributeEntry 1 }

jmAttributeInstanceIndex OBJECT-TYPE

SYNTAX	Integer32(1..32767)

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"A running index of the attributes of the same type for each job.  For those attributes with only a single instance per job, this index value shall be 1.  For those attributes that are a single value per document, the index value shall be the document number, starting with 1 for the first document in the job.  Jobs with only a single document shall use the index value of 1.  For those attributes that can have multiple values per job and per document, such as documentFormat, the index shall be a running index for the job as a whole, starting at 1. by the

Each job shall be identified by jmJobIndex value and each job shall be in one job set identified by jmJobSetIndex."

::= { jmAttributeEntry 2 }

smResourceAmount on these units.

jmAttributeValueAsInteger OBJECT-TYPE

SYNTAX	Integer32(0..2147483647)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"The value of the attribute, including resource requested or consumedused so far, in the units specified in the enum description (see JmAttributeTypeTC on page 1) o  This value may be a integer, a counter, an index, or an enum, depending on the jmAttributeRypeIndex value.  After the job completes processing for those attributesrthat indicate resource consumption, this value shall indicates the totl usage of this resource made by the job.

A monitoring application is able to copy this value to a suitable longer term storage for later processing as part of an accounting system.

Since the agent may add attributes representing resources to this table while the job is waiting to be processed or being processed, which can be a long time before any of the resources are actually used, the agent shall set the value of the jmAttributeValueAsInteger object to 0 for resources that the job has not yet consumed.

Attributes for which the concept of an integer value is meaningless, such as fileName, interpreter, and physicalDevice, shall return a value of (-1) to indicate other for jmAttributeValueAsInteger as indicated in the JmAttributeTypeTC textual convention description on page 4.

Attributes for which the concept of a text value is meaningless, such as pagesCompleted, shall return a value of a zero length string for jmAttributeValueAsText as specified in the JmAttributeTypeTC textual convention on page 4."

::= { jmAttributeEntry 3 }

jmAttributeValueAsText OBJECT-TYPE

SYNTAX	OCTET STRING(SIZE(0..63))

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"The coded character set text value of the attribute., e.g., iso-a4-white, red-ink, number 2 staples, or the string"

::= { jmAttributeEntry 4 }