-- Module:  GENERAL-MIB

-- Editor:  Tom Hastings

-- File:    gen.doc, .psr, ps., .txt

-- Date:    October 24, 1995

-- Version: 0.1

-- The General MIB for use with other MIBs

-- that use the host resources MIB (RFC 1514)

GENERAL-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY, OBJECT-TYPE, experimental

FROM SNMPv2-SMI                    -- RFC 1442

RowStatus, DisplayString

FROM SNMPv2-TC                     -- RFC 1443

MODULE-COMPLIANCE, OBJECT-GROUP

FROM SNMPv2-CONF                   -- RFC 1444

hrDeviceIndex, InternationalDisplayString

FROM HOST-RESOURCES-MIB            -- RFC 1514

CodedCharSet

FROM PRINTER-TC                   -- file: prttc.txt

CodedLanguage, CodedTerritory,

CodeIndexedStringIndex, Ordinal16

FROM GENERAL-TC;             -- file: gentc.txt

-- Upon publication as RFC, delete this comment and the line following

-- this comment and change the reference of { printmib 101 }

-- (below) to { mib-2 X }.

-- This will result in changing:

-- 1 3 6 1 3 54 generalMIB(101)    to:

-- 1 3 6 1 2  1 generalMIB(X)

-- This will make it easier to translate prototypes to

-- the standard namespace because the lengths of the OID's won't

-- change.

printmib OBJECT IDENTIFIER ::= { experimental 54 }

generalMIB MODULE-IDENTITY

LAST-UPDATED "9510250000Z"

ORGANIZATION "IETF/DMTF PrinterWorking Group"

CONTACT-INFO

"       Thomas N. Hastings

Xerox Corporation, MS ES-AE 242

701 S. Aviation Blvd.

El Segundo, CA 90245

Phone:        1+ (310)333-6413

FAX:          1+ (310)333-6342

E-Mail:       hastings@cp10.es.xerox.com"

DESCRIPTION

"File: gen.doc, .psr, .ps, .txt

Version: 0.1

This MIB module specifies objects of a general nature

for use with MIBs that use the host resources MIB

(see RFC 1514).

See the companion general textual-conventions module:

GENERAL-TC for the textual conventions and for an

explanation of this use of this MIB."

::= { printmib 101 }

--

-- General Current Localization Group

--

-- This is a group to control the localization of static string objects

-- that use the InternationalDisplayString types in existing and new

-- MIBs.

--

-- Implementation of every object in this group is optional.

-- However, MIBs that contain objects of type InternationalDisplayString

-- shall implement this group.

--

-- Contains the scalar localization objects that are needed per device.

--

genCurrentLocalization OBJECT IDENTIFIER ::= { generalMIB 3 }

genCurrentLocalizationTable OBJECT-TYPE

SYNTAX     SEQUENCE OF GenCurrentLocalizationEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A table of general localization information per device.  It is

put in a table so it can be per device using the hrDeviceIndex."

::= { genCurrentLocalization 1 }

genCurrentLocalizationEntry OBJECT-TYPE

SYNTAX      GenCurrentLocalizationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Contains the scalar objects that we want to be per device."

INDEX  { hrDeviceIndex }

::= { genCurrentLocalizationTable 1 }

GenCurrentLocalizationEntry ::= SEQUENCE {

genCurrentLocalizationIndex    Ordinal16

}

genCurrentLocalizationIndex OBJECT-TYPE

SYNTAX     Ordinal16

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The value of the genLocalizationIndex corresponding to the

current language, country, and character set which shall be

used for all objects of type InternationalDisplayString in

all legacy and future MIBs.

Note the Printer MIB has a similar mechanism which controls only

objects (of type OCTET STRING) which are in the Printer MIB."

::= { genCurrentLocalizationEntry 1 }

--

-- General Localization Group

--

-- This is a group to control the localization of static string objects

-- that use the InternationalDisplayString types in legacy and new MIBs.

--

-- Implementation of every object in this group is optional.

-- However, MIBs that contain objects of type InternationDisplayString

-- shall implement this group.

--

-- Contains the localizations supported by this device.

--

genLocalization OBJECT IDENTIFIER ::= { generalMIB 4 }

genLocalizationTable OBJECT-TYPE

SYNTAX     SEQUENCE OF GenLocalizationEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"The available localizations in this device."

::= { genLocalization 1 }

genLocalizationEntry OBJECT-TYPE

SYNTAX      GenLocalizationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"An entry exists in this table for each Localizaion, e.g. for

each combination of Language, Territory and Coded Character

Set supported in some objects of InternationalDisplayString

type and for each device."

INDEX  { hrDeviceIndex, genLocalizationIndex }

::= { genLocalizationTable 1 }

GenLocalizationEntry ::= SEQUENCE {

genLocalizationIndex            Ordinal16,

genLocalizationRowStatus        RowStatus,

genLocalizationASCIIName        DisplayString,

genLocalizationName             InternationalDisplayString,

genLocalizationLanguage         CodedLanguage,

genLocalizationTerritory        CodedTerritory,

genLocalizationCharSet          CodedCharSet

}

genLocalizationIndex OBJECT-TYPE

SYNTAX     Ordinal16

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"A unique value used by the device to identify this

localization entry.  Although these values may change due to a

major reconfiguration of the device (e.g., the addition of new

Cover sub-units to a printer), values are expected to remain

stable across successive device power cycles."

::= { genLocalizationEntry 1 }

genLocalizationRowStatus OBJECT-TYPE

SYNTAX     RowStatus

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"This object manages the row status of this conceptual row in

the 'genLocalizationTable'."

::= { genLocalizationEntry 2 }

genLocalizationASCIIName OBJECT-TYPE

SYNTAX     DisplayString (SIZE (0..255))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The unlocalized human readable name in NVT ASCII of the

localization with the language, followed by the territory,

followed by the character set.

Example: English US ASCII"

::= { genLocalizationEntry 3 }

genLocalizationName OBJECT-TYPE

SYNTAX     InternationalDisplayString (SIZE (0..255))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The localized human readable name of the localization with

the language, followed by the territory, followed by the

character set.

Example: Japanese Japan Kanji (Represented with Kanji chars.)"

::= { genLocalizationEntry 4 }

genLocalizationLanguage OBJECT-TYPE

SYNTAX     CodedLanguage

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"A two character language code from ISO 639.

A blank string (two space characters) shall indicate that the

territory is not defined.

Examples EN, GB, CA, FR, DE."

::= { genLocalizationEntry 5 }

genLocalizationTerritory OBJECT-TYPE

SYNTAX     CodedTerritory

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"A two character country code from ISO 3166.

A blank string (two space characters) shall indicate that the

territory is not defined.

Examples: US, FR, DE, ..."

::= { genLocalizationEntry 6 }

genLocalizationCharSet OBJECT-TYPE

SYNTAX     CodedCharSet

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The coded character set used for this localization.

The value is the enum for the coded character set registered

with IANA.  See the CodedCharSet textual-convention in the

Printer MIB (RFC 1759)."

::= { genLocalizationEntry 7 }

--

-- General Code Indexed String Group

--

-- This is a group to control the coded character set representation

-- of codeset indexed string objects that use the CodeIndexedStringIndex

-- types in new MIBs.

--

-- Implementation of every object in this group is optional.

-- However, MIBs that contain objects of type CodeIndexedStringIndex

-- shall implement this group.

--

-- Contains all the current codeset indexed strings for this device.

--

genCodeIndexedString  OBJECT IDENTIFIER ::= { generalMIB 5 }

genCodeIndexedStringTable OBJECT-TYPE

SYNTAX     SEQUENCE OF GenCodeIndexedStringEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"The current codeset indexed strings for this device."

::= { genCodeIndexedString 1 }

genCodeIndexedStringEntry OBJECT-TYPE

SYNTAX     GenCodeIndexedStringEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"An entry exists in this table containing a code-indexed string

in a particular coded character set.  There are separate sets of

code-indexed strings for each device.

If an agent supports multiple coded character representations

of a codeset indexed string, including code conversion between,

there will be a separate entry for each such coded

representation.

The genCodeIndexedStringCharSet selects which coded

character representation is to be returned.  The value of

genCodeIndexedStringCharSet is the coded character set enum

registered with IANA. See the CodedCharSet textual-convention

in the Printer MIB (RFC 1759)."

INDEX {

hrDeviceIndex,

genCodeIndexedStringIndex,

genCodeIndexedStringCharSet

}

::= { genCodeIndexedStringTable 1 }

GenCodeIndexedStringEntry ::= SEQUENCE {

genCodeIndexedStringIndex         CodeIndexedStringIndex,

genCodeIndexedStringCharSet       CodedCharSet,  -- an index

genCodeIndexedStringRowStat       RowStatus,

genCodeIndexedStringData          OCTET STRING

}

genCodeIndexedStringIndex OBJECT-TYPE

SYNTAX     CodeIndexedStringIndex (1..2147483647)

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A unique value used by the device, along with the following

genCodeIndexedStringCharSet, to identify this Coded String

entry.

A value of 0 is not valid for this object."

::= { genCodeIndexedStringEntry 1 }

genCodeIndexedStringCharSet OBJECT-TYPE

SYNTAX     CodedCharSet

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The coded character set used for this code-indexed string.

The value is the enum for the coded character set registered

with IANA.  See the CodedCharSet textual-convention in the

Printer MIB (RFC 1759)."

::= { genCodeIndexedStringEntry 2 }

genCodeIndexedStringRowStat OBJECT-TYPE

SYNTAX     RowStatus

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"This object manages the row status of this conceptual row

in the 'genCodeIndexedStringTable'."

::= { genCodeIndexedStringEntry 3 }

genCodeIndexedStringData OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..255))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"This is the actual code-indexed string data, represented in the

coded character set identified by genCodeIndexedStringCharSet."

::= { genCodeIndexedStringEntry 4 }

-- The General Coded Char Set Group

--

-- This is a group to control the coded character set representation

-- of code-indexed string objects that use the CodeIndexedStringIndex

-- types in new MIBs.

--

-- Implementation of every object in this group is optional.

-- However, MIBs that contain objects of type CodeIndexedStringIndex

-- shall implement this group.

--

-- Contains the coded character set identifications of the coded

-- character sets supported by this device for representing code-indexed

-- strings.

genCodedCharSet  OBJECT IDENTIFIER ::= { generalMIB 6 }

genCodedCharSetTable OBJECT-TYPE

SYNTAX     SEQUENCE OF GenCodedCharSetEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"The supported coded character sets for this device."

::= { genCodedCharSet 1 }

genCodedCharSetEntry OBJECT-TYPE

SYNTAX     GenCodedCharSetEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"An entry exists in this table for each coded character set

supported in some objects of CodeIndexedStringIndex type for

each device."

INDEX  { hrDeviceIndex, genCodedCharSetCharSet }

::= { genCodedCharSetTable 1 }

GenCodedCharSetEntry ::= SEQUENCE {

genCodedCharSetCharSet        CodedCharSet,

genCodedCharSetRowStatus      RowStatus,

genCodedCharSetASCIIName      DisplayString

}

genCodedCharSetCharSet OBJECT-TYPE

SYNTAX     CodedCharSet

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The supported coded character set enum identifier, used an

index for this table.  The value is the enum for the coded

character set registered with IANA.  See the CodedCharSet

textual-convention in the Printer MIB (RFC 1759)."

::= { genCodedCharSetEntry 1 }

genCodedCharSetRowStatus OBJECT-TYPE

SYNTAX     RowStatus

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"This object manages the row status of this conceptual row in

the 'genCodedCharSetTable'."

::= { genCodedCharSetEntry 2 }

genCodedCharSetASCIIName OBJECT-TYPE

SYNTAX     DisplayString (SIZE (0..255))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The unlocalized human readable name in ASCII of the

coded character set.

Example: 'ASCII'"

::= { genCodedCharSetEntry 3 }

--

-- General MIB Conformance Information

--

-- With only Optional Conformance Groups

--

genMIBConformance

OBJECT IDENTIFIER ::= { generalMIB 2 }

genMIBGroups

OBJECT IDENTIFIER ::= { genMIBConformance 2 }

--

-- Compliance Statements for the General Localization Group

--

genMIBLocalizationComp MODULE-COMPLIANCE

STATUS     current

DESCRIPTION

"The compliance statement for the agent that implements the

General MIB Current Localization Group.

This group is optional."

MODULE -- this module

MANDATORY-GROUPS {

genCurrentLocalizationGroup,

genLocalizationGroup

}

OBJECT      genCurrentLocalizationIndex

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT      genLocalizationIndex

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only"

OBJECT      genLocalizationRowStatus

SYNTAX      INTEGER { active(1) }  -- subset of RowStatus

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only, and

only one of the enumerated values for the 'RowStatus' textual

convention need be supported, specifically:  active(1)."

::= { genMIBConformance 3 }

genMIBCodeIndexedStringComp MODULE-COMPLIANCE

STATUS       current

DESCRIPTION

"The compliance statement for the agent that implements the

General MIB Coded String Group.

This group is optional."

MODULE -- this module

MANDATORY-GROUPS {

genCodeIndexedStringGroup,

genCodedCharSetGroup

}

OBJECT      genCodeIndexedStringRowStat

SYNTAX      INTEGER { active(1) }  -- subset of RowStatus

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only, and

only one of the enumerated values for the 'RowStatus' textual

convention need be supported, specifically:  active(1)."

OBJECT      genCodedCharSetRowStatus

SYNTAX      INTEGER { active(1) }  -- subset of RowStatus

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only, and

only one of the enumerated values for the 'RowStatus' textual

convention need be supported, specifically:  active(1)."

::= { genMIBConformance 5 }

--

--  Conformance Groups

--

genCurrentLocalizationGroup OBJECT-GROUP

OBJECTS {

genCurrentLocalizationIndex

}

STATUS  current

DESCRIPTION

"The General MIB Current Localization Group."

::= { genMIBGroups 3 }

genLocalizationGroup OBJECT-GROUP

OBJECTS {

genLocalizationIndex,

genLocalizationRowStatus,

genLocalizationASCIIName,

genLocalizationName,

genLocalizationLanguage,

genLocalizationTerritory,

genLocalizationCharSet

}

STATUS  current

DESCRIPTION

"The General MIB Localization Group."

::= { genMIBGroups 4 }

genCodeIndexedStringGroup OBJECT-GROUP

OBJECTS {

genCodeIndexedStringIndex,

genCodeIndexedStringCharSet,

genCodeIndexedStringRowStat,

genCodeIndexedStringData

}

STATUS  current

DESCRIPTION

"The General MIB Coded String Group."

::= { genMIBGroups 5 }

genCodedCharSetGroup OBJECT-GROUP

OBJECTS {

genCodedCharSetCharSet,

genCodedCharSetRowStatus,

genCodedCharSetASCIIName

}

STATUS  current

DESCRIPTION

"The General MIB String Support Group."

::= { genMIBGroups 6 }

END