Comparison of Job Monitoring MIB implementations
with the Job Monitoring MIB objects
Xerox Printxchange -DPA implementation

From:	Tom Hastings

Date:	01/09/97

Version:	0.6

File:	ftp://ftp.pwg.org/pub/snmpmib/jobs-mib/mono-map/mono-map.doc  .ps

Please fill in the comparison with your job monitoring implementations, whether using a MIB or not.  This list reflects that changes agreed at the 8-Jan-1997 JMP meeting.

Please fill out each of the three columns provided:

- obj/attr name - the name of the MIB object or attribute that has the same semantics as the JMP MIB object.

- data type - the data type of your implementation

- any notes on differences, etc.

Then rename your file to something to reflect your implementation and upload into the following directory:

ftp://ftp.pwg.org/pub/snmpmib/jobs-mib/mono-map/

# List of objects for the Job Monitoring MIB

## The Job Set Group

The JobSetGroup consists of objects that are for all Job Set instances, not just a single instance.  The jmJobSetGroup consists entirely of the jmJobSetEntry which is indexed by:

- jmJobSetIndex -  a running index of Job Set instances supported by this printer or server.

| jmJobSetGroup (M) | DataType | Obj/attr name | Data type | Notess |
| --- | --- | --- | --- | --- |
| - jmJobSetIndex - a running index of Job Set instances supported by this printer or server. | Integer32(1..2^31) |  |  |  |

## The General Group

The jmGeneralGroup consists of objects of a general nature that are not per-job.  The jmGeneralGroup consists entirely of the jmGeneralEntry which is indexed by:

- jmJobSetIndex -  a running index of Job Set instances supported by this printer or server.

| jmGeneralGroup (G) | DataType | Obj/attr name | Data type | Notess |
| --- | --- | --- | --- | --- |
| - jmJobSetIndex - a running index of Job Set instances supported by this printer or server. | Integer32(1..2^15-1) |  |  |  |
| - jmGeneralJobCompletedPolicy - the time in seconds that jobs are kept in the jmJobTable and the jmCompletedTable after processing. | Integer32(0..2^31-1) |  |  |  |
| - jmGeneralMaxNumberOfJobs - the maximum number of job; (-1) means no limit. | Integer32(0..2^31-1) |  |  |  |
| - jmGeneralCurrentNumberOfJobs - the total number of jobs currently in the Job Table (pending and completed). | Integer32(0..2^31-1) |  |  |  |
| - jmGeneralQueuingAlgorithm - the current scheduling algorithm being used or none (no queuing is possible). | JMQueuingAlgorigthm |  |  |  |

## The Queue Group

The jmQueueGroup is made up entirely of the jmQueueTable which is an ordered list of jobs that have not completed processing.  The jmQueueGroup consists of objects that are not needed after the job has completed processing.  The jmQueueGroup is conditionally mandatory and shall be implemented by a server or print that performs queuing (or spooling).  The jmQueueGroup shall not be implemented if the value of jmGeneralQueuingAlgorithm is none.  The jmQueueTable is indexed by:

- jmJobSetIndex -  a running index of Job Set instances supported by this printer or server.

- jmQueueIndex - a running index of the jobs that have not finished processing.

| jmQueueGroup (Q) | DataType | Obj/attr name | Data type | Notes |
| --- | --- | --- | --- | --- |
| - jmJobSetIndex - a running index of Job Set instances supported by this printer or server. | Integer32(1..2^15) |  |  |  |
| - jmQueueIndex - a running index of the jobs that have not finished processing. | Integer32(1..2^31-1) |  |  |  |
| - jmQueueIndex - the job’s identifier generated by the printer or server implementing this JM MIB | Integer32(0..2^31-1) |  |  |  |
| - jmQueueNumberOfInterveningJobs - the number of jobs in front of this job | Integer32(0..2^31-1) | intervenint-jobs | Integer32 |  |
| - jmJobPriority - Job priority | Integer32(0..100) |  |  |  |
| - jmJobProcessAfterTime - process-after-time | GeneralizedTime | job-print-after | GeneralizedTime |  |
| - jmJobMessageToOperator - job-message-to-operator from submitting user or device | OCTET STRING(SIZE((63)) |  |  |  |

## The Completed Group

The jmCompletedGroup consists entirely of the jmCompletedTable which is an ordered list of the job that have completed processing.  The jmCompletedTable is indexed by:

- jmJobSetIndex -  a running index of Job Set instances supported by this printer or server.

- jmCompletedIndex - a running index of the jobs that have finished processing.

| jmCompletedGroup (C) | DataType | Obj/attr name | Data type | Notess |
| --- | --- | --- | --- | --- |
| - jmJobSetIndex - a running index of Job Set instances supported by this printer or server. | Integer32(1..2^15-1) |  |  |  |
| - jmCompletedIndex - a running index of the jobs that have finished processing. | Integer32(1..2^31) |  |  |  |
| - jmJobIndex - the job’s identifier generated by the printer or server implementing this JM MIB | Integer32(1..) | job-identifier | string | string instead of an integer, but contents are digits |

## The Job Group

The jmJobGroup consists of (1) job identification, (2) job parameters, and (3) job status and accounting objects that have a single value per job.  The jmJobGroup consists entirely of the jmJobTable which is indexed by:

- jmJobSetIndex -  an instance index to distinguish separate sets of tables when a server supports more than one printer.

- jmJobIndex -  the job identifier that was generated by the server or printer that accepted the job.

| jmJobGroup - Identification (I) | DataType | Obj/attr name | Data type | Notess |
| --- | --- | --- | --- | --- |
| - jmJobSetIndex - a running index of Job Set instances supported by this printer or server. | Integer32(1..2^15-1) |  |  |  |
| - jmJobIndex - the job’s identifier generated by the server or printer implementing this JM MIB | Integer32(1..2^31-1) |  |  |  |
| - jmJobName - Job name assigned by job owner which is not necessarily unique. | OCTET STRING(SIZE(63)) | job-name | string |  |
| - jmJobNameId - the job’s identifier name generated by the job submitting software using the job submission protocol. This name can be anything that helps identifer the job to the job submitter, including the name of the queue from which the job was submitted. | OCTET STRING(SIZE(63)) | job-identifier-on-client | string |  |
| - jmJobNumberId - the job’s identifier number generated by the job submitting software using the job submission protocol. A (-2) value shall indicate that the submitter did not supply a job identifier number. | Integer32(0..2^31-1) |  |  |  |
| - jmJobTypes - Job types (print, fax, scan, etc.) - bit vector to get multiple values in a single object | JMJobType - enum encoded as bits |  |  |  |
| - jmJobOwner - Job owner (User name of the user that originally submitted print job) | OCTET STRING(SIZE(63)) | job-owner | string |  |
| - jmJobDeviceNameRequested - Device name (Device-specific name of device) requested by the submitting user. | OCTET STRING(SIZE((63)) | printer-name-requested | string |  |
| - jmDeviceIndex - the host resources index of the corresponding Printer MIB that the job was submitted to or has been assigned to be printed on by the server. 0 indicates if the server has not assigned a printer to the job. | Integer32(0..2^31-1) | printers-assigned | distinguished-name | not an index to a MIB |
| - jmJobSourceChannel - Source channel on which the job was submitted (index of channel row in the Printer MIB) | PrtChannelIndex |  |  |  |
| - jmJobSubmissionTime - Date/Time of job submission by job owner | DateAndTime | job-submission-time | GeneralizedTime |  |
| - jmJobComment - Job comment | OCTET STRING(SIZE(63)) | job-comment | string |  |

| jmJobGroup - Parameters (J) | DataType | Obj/attr name | Data type | Notes |
| --- | --- | --- | --- | --- |
| - jmJobTotalKOctets - total K octets to be processed in the job - rounded up to next higher K | Integer32(0..2^31-1) | total-job-octets | Integer64 | In octets, not K. |

| jmJobGroup - Status and Accounting (S) | DataType | Obj/attr name | Data type | Notess |
| --- | --- | --- | --- | --- |
| - jmJobCurrentState - Job state (pending, processing, completed, etc.) | JMJobState | job-current-stae | OID | same states as Job Monitoring MIB |
| - jmJobStateReasons - Job state reasons - additional information about the job state: reasons being held, additional completed information such as successful, warnings, or errors. | OCTET STRING(SIZE(0..63)) -bit vector | job-state-reasons | OIDs | same reasons as Job Monitoring MIB |
| - jmJobKOctetsCompleted - K Octets completed - should be rounded down to lower K until completed. | Integer32(0..2^31-1) |  |  |  |
| - jmJobStartedProcessingTime - Date/Time of day job started processing on device | DateAndTime | started-printing-time | GeneralizedTime |  |
| - jmJobCompletionTime - Date/Time of day job finished using the device | DateAndTime | completion-time | GeneralizedTime |  |
| - jmJobAccountName - Account Name | OCTET STRING(SIZE(63)) | user-name | string |  |

## The Resource Group

The jmResourceGroup consists of requested and used resources objects that can have multiple values per job.  The jmResourceGroup consists entirely of the jmResourceTable which is indexed by:

- jmJobSetIndex -  an instance index to distinguish separate sets of tables when a server supports more than one printer.

- jmJobIndex -  the job identifier that was generated by the server or printer that accepted the job.

- jmResourceIndex -  a running index of resources for each job

| jmResourceGroup (R) | DataType | Obj/attr name | Data type | Notes |
| --- | --- | --- | --- | --- |
| - jmJobSetIndex - a running index of Job Set instances supported by this printer or server. | Integer32 |  |  |  |
| - jmJobIndex - the job’s current identifier generated by the server or printer implementing this JM MIB | Integer32(0..) |  |  |  |
| - jmResourceIndex - a running index of the resources requested and/or used by the job. | Integer32 |  |  |  |
| - jmResourceType - Resources required/used (table): | JMResourceType |  |  |  |
| - documentName(3) - Document name(s) (or file-names) | OCTET STRING(63) | document-name | string |  |
| - jobCopiesRequested(4) - Number of job copies requested | Integer32(0..2^31-1) | job-copies | Integer32 |  |
| - jobCopiesProduced(5) - Number of job copies produced | Integer32(0..2^31-1) | job-copies-completed | Integer32 |  |
| - documentCopiesRequested(6) - Number of document copies requested | Integer32(0..2^31-1) | copy-count | Integer32 |  |
| - documentCopiesProduced(7) - Number of document copies produced | Integer32(0..2^31-1) |  |  |  |
| - sides(8) - Number of sides requested/used (one-sided, two-sided) | Integer32(1..2) | sides | Integer32 |  |
| - interpreters(9) - PDLs requested/used | PrtInterpreterFamily | document-format | 3-element structure | 1st element is OID that maps directly to enum |
| - physicalDevices(10) - physical devices requested/used | hrDeviceIndex | physical-printers-requested | string | string instead of a MIB index |
| - faxPhoneNumber(10) - FAX phone number requested/used | OCTET STRING(255) |  |  |  |
| - impressionsCompleted(11) - Impressions (sides) completed | Counter32(0..2^31-1) |  |  |  |
| - sheetsCompleted(12) - Sheets completed for the job. | Counter32(0..2^31-1) |  |  |  |
| - pagesSpooled(13) - logical pages spooled for the job. | Counter32(0..2^31-1) |  |  |  |
| - pagesInterpreted(14) - logical pages intepreted for the job. | Counter32(0..2^31-1) |  |  |  |
| - pagesSentToDevice(15) - logical pages sent to the device for the job. | Counter32(0..2^31-1) |  |  |  |
| - pagesCompleted(16) - logical pages completed for the job. | Counter32(0..2^31-1) |  |  |  |
| - pagesCompletedCurrentCopy(17) - logical pages completed on the current copy. | Integer32(0..2^31-1) |  |  |  |
| - processingTime(18) - Processing time so far | Integer32(0..2^31-1) |  |  |  |
| - processingMessage(19) - Processing Messages | OCTET STRING(63) |  |  |  |
| - jmResourceName - resource required/usage name | OCTET STRING(63) or Integer32 |  |  |  |
| - jmResourceUnits - resource required/used usage-unit | JMResourceUnits |  |  |  |
| - jmResourceAmount - resource amount requested/used; -2 - unknown | Integer32 |  |  |  |

The following job attributes are supported by Printxchange, but are not currently in our Job Monitoring MIB list:

- initial-value-job

- notification-profile

- job-message-from-administrator

- previous-job-state

- job-submission-complete

- number-of-documents

- job-sheets

- output-bin

- job-discard-time

- job-retention-period

- job-hold

- job-identifier-on-printer [listed as an issue to be added to the Job Monitoring MIB for configuration 2b]

The following document attributes are supported by Printxchange, but are not currently in our Job Monitoring MIB list:

- default-character-set

- default-font

- default-input-tray

- default-medium

- content-orientation

- number-up

- page-select

- plex

- binding-edge

- x-image-shift

- y-image-shift

- reset-printer

- bottom-margin

- left-margin

- right-margin

- top-margin

- length

- width

- header-text

- footer-text

- number-pages

- repeated-tab-stops

- finishing

- document-file-name (added to resource list)

- document-revision-date

- document-state

- page-order-received

- octet-count

- document-sheets