Name of Job Submission Protocol:  ISO DPA

Version of Job Submission Protocol:  Final Draft, March 1996

File name:  iso-dpa.doc

Assigned person:  Tom Hastings

| Job Identification | Job Submission Protocol Attribute |
| --- | --- |
| . Job id on client | job-client-id |
| . Job id on printer | job-identifier-on-printer |
| . Job id on intermediate server(s) - focus on main scheduling server | job-identifier-on-client |
| . Job type (print, fax, scan, etc.) | - |
| . Name of client node | - |
| . Name of submitting end user | job-originator |
| . Job owner | job-owner |
| . Name of source port/channel | - |
| . Domain of name of source port/channel | - |
| . Source port/channel type | - |
| . Job name assigned by end-user | job-name |
| . Submitting application name | - |
| . Document name(s) | document-name |
| . Document file name(s) | document-file-name |
| . Time of job submission | submission-time |
| . Job comment | job-comment |
| . Printer-specific id of printer | printer-name |
| Job Status |  |
| . Job state (held, pending, executing, completed, etc.) | current-job-state |
| . Physical device(s) being used | printers-assigned |
| . Job state reasons - additional information about the job state: reasons being held, additional executing information such as device(s) needs attention, additional completed information such as successful, warnings, or errors. | job-state-reasons |
| . Percent complete | compute from octets-completed and total-job-octets |
| . (Logical) Pages executed so far | pages-completed |
| . Impressions (sides) executed so far | impressions-completed |
| . Sheets executed so far | media-sheets-competed |
| . Procesing time used so far | processing-time |
| . Time of day received by intermediate | - |
| . Time of day sent from intermediate | - |
| . Time of day received by printer |  |
| . Time of day job started processing on printer | started-printing-time |
| Job Parameters |  |
| . Number of copies requested | job-copies |
| . Upper limit on processing time | job-abort-criteria=processing-time=threshold |
| . Upper limit on number of pages | job-abort-criteria=page-completed=threshold |
| . Media requested | page-medium-select?, default-medium? media-used? |
| . Media path requested (one-sided, two-sided, LEF, SEF) | sides |
| . PDLs requested | document-format |
| . Job priority | job-priority |
| . Deadline time | deadline-time |
| . Discard (expiration) time | discard-time |
| . Color requested | - |
| . Stapling requested | job-finishing |
| . Finishing requested | job-finishing |
| . Resolution requested | default-printer-resolution? |
| . Fonts required/requested | default-font?, fonts-used? |
| . Forms required/requested | default-resources?, resources-used? |
| . Destination (including phone numbers for FAX, logical printers for printing) | printer-name-requested |
| . Class requested (administrator-defined) | ?? |
| Accounting |  |
| . Accounting User Name | user-name |
| . Account Name | accounting-information |
| . (Logical) pages completed | pages-completed |
| . Impressions completed | impressions-completed |
| . Sheets completed | media-sheets-completed |
| . Simplex sheets completed | compute: 2*media-sheets-completed - pages-completed |
| . Duplex sheets completed | compute: pages-completed - media-sheets-completed |
| . Octets completed | octets-completed |
| . Processing time completed | processing-time |
| . Consumable consumed | - |
| . Media types consumed (paper trays selected) | - |
| . Devices used | devices-assigned |
| . Finishing used | - |
| . PDLs used | document-format for each document |
| . Job Submission Channels used | - |
| . Interfaces used | - |
| . Human operator labor used | - |
| . Setup time used | - |
| . Wall clock time used executing the job | compute: completion-time - started-printing-time |
| . Fonts used | fonts-used |
| . Phone charges | - |
| . Wasted sheets | - |
| . Number-up (N-up) used | compute: pages-comleted / impressions-completed |