Name of Job Submission Protocol:	PJL

Version of Job Submission Protocol: PJL-DateCode

File name:	pjl.doc

Assigned person:	Binnur Al-Kazily/Bob Pentecost

| Job Identification | Job Submission Protocol Attribute |
| --- | --- |
| - . Job id on client | - |
| - . Job id on printer | See Job ID Info |
| - . Job id on intermediate server(s) - focus on main scheduling server | See Job ID Info |
| - . Job type (print, fax, scan, etc.) | - |
| - . Name of client node | - |
| - . Name of submitting end user | - |
| - . Job owner | - |
| - . Name of source port/channel | - |
| - . Domain of name of source port/channel | - |
| - . Source port/channel type | - |
| - . Job name assigned by end-user | @PJL JOB NAME=”job name” |
| - . Submitting application name | - |
| - . Document name(s) | - |
| - . Document file name(s) | - |
| - . Time of job submission | - |
| - . Job comment | @PJL COMMENT remark |
| - . Printer-specific id of printer | - |
| Job Status |  |
| - . Job state (held, pending, executing, completed, etc.) | Status on EOJ == # Pages completed. |
| - . Physical device(s) being used | - |
| - . Job state reasons - additional information about the job state: reasons being held, additional executing information such as device(s) needs attention, additional completed information such as successful, warnings, or errors. | @PJL USTATUS JOB=ON - When EOJ, it returns a RESULT field: OK : Last page of the job was printed USER_CANCELLED: control panel HOST_CANCELLED : host system RESOURCE_CANCELLED : resource wasn’t available (ex. media not loaded). |
| - . Percent complete | - |
| - . (Logical) Pages executed so far | - |
| - . Impressions (sides) executed so far | @PJL USTATUS PAGE = ON |
| - . Sheets executed so far | - |
| - . Processing time used so far | - |
| - . Time of day received by intermediate | - |
| - . Time of day sent from intermediate | - |
| - . Time of day received by printer | - |
| - . Time of day job started processing on printer | - |
| Job Parameters | @PJL SET |
| - . Number of copies requested | @PJL SET COPIES ={ 1¦...¦999 } |
| - . Upper limit on processing time | - |
| - . Upper limit on number of pages | @PJL JOB START=first page END=last page |
| - . Media requested | @PJL SET MEDIATYPE= @PJL SET PAPER= { LETTER ¦ LEGAL ¦ A4 ¦…¦ CUSTOM } |
| - . Media path requested (one-sided, two-sided, LEF, SEF) | @PJL SET DUPLEX={ON ¦ OFF} @PJL SET MANUALFEED={ ON ¦ OFF } @PJL SET ORIENTATION={PORTRAIT ¦ LANDSCAPE } @PJL SET BINDING={ LONGEDGE ¦ SHORTEDGE } |
| - . PDLs requested | @PJL SET PERSONALITY= { AUTO ¦ PCL ¦ POSTSCRIPT ¦ …} |
| - . Job priority | - |
| - . Deadline time | - |
| - . Discard (expiration) time | - |
| - . Color requested | @PJL SET RENDERMODE |
| - . Stapling requested | - |
| - . Finishing requested | - |
| - . Resolution requested | @PJL SET RESOLUTION= { 300 ¦ 600 } |
| - . Fonts required/requested | - |
| - . Forms required/requested | - |
| - . Destination (including phone numbers for FAX, logical printers for printing) | @PJL SET OUTBIN= { UPPER ¦ LOWER ¦ OPTIONALOUTPUTBIN1..62 } |
| - . Class requested (administrator-defined) | - |
| Accounting |  |
| - . Accounting User Name | - |
| - . Account Name | - |
| - . (Logical) pages completed | - |
| - . Impressions completed | See Job Pages Info |
| - . Sheets completed | - |
| - . Simplex sheets completed | - |
| - . Duplex sheets completed | - |
| - . Octets completed | - |
| - . Processing time completed | - |
| - . Consumable consumed | - |
| - . Media types consumed (paper trays selected) | - |
| - . Devices used | - |
| - . Finishing used | - |
| - . PDLs used | - |
| - . Job Submission Channels used | - |
| - . Interfaces used | - |
| - . Human operator labor used | - |
| - . Setup time used | - |
| - . Wall clock time used executing the job | - |
| - . Fonts used | - |
| - . Phone charges | - |
| - . Wasted sheets | - |
| - . Number-up (N-up) used | - |

JOB ID Info:

- If unsolicited job status is enabled when the printer begins processing the PJL job, the printer responds to the PJL JOB command with @PJL USTATUS JOB START response. The printer response includes a job ID number that is associated with the job name supplied in the JOB command.

// printer receives the PJL JOB command

@PJL JOB NAME = “job name”

// If unsolicited job status is enabled, printer would generate an ID number and respond with:

@PJL USTATUS JOB<CR><LF>

START<CR><LF>

NAME=”job name”<CR><LF>

ID=346<CR><LF>

<FF>

JOB Page Info:

- When the job is completely finished, the printer sends a END of job command:

// If unsolicited job status is enabled, printer would respond at the end of a job:

@PJL USTATUS JOB<CR><LF>

END<CR><LF>

NAME=”job name”<CR><LF>		// from the EOJ command

PAGES=page count<CR><LF>

<FF>

- @PJL USTATUS PAGE=ON command is used to notify the application that a particular page finished printing. The number of pages incremented by one for each simplex page, and by two for each duplex page.

@PJL USTATUS PAGE <CR><LF>

1<CR><LF>

ID=#<CR><LF>

<FF>

@PJL USTATUS PAGE <CR><LF>

2<CR><LF>

ID=#<CR><LF>

<FF>