Subj:  Template for Mapping Job Identification attributes to JMP objects/attributes

From:	Tom Hastings

Date:	02/28/97

File:	ftp://ftp.pwg.org/pub/pwg/jmp/protomap/jmtmplt1.doc

At the Feb 19 meeting we agreed to the following action item:

ACTION ITEM (responsible persons indicated below): Map the job identification objects of the current Job Monitoring MIB to their assigned job submission protocol, so that we can make sure that the specification of each object is clear.  The resulting mapping will be incorporated into Appendix A of the Job Monitoring MIB.  As a second step we will add all of the rest of the Job Monitoring MIB objects.

.	Copy jmtmplt1.doc from ftp://ftp.pwg.org/pub/pwg/jmp/protomap/jmtmplt1.doc.

.	Fill in the first five lines of information about your job submission protocol immediately after the "cut here" line.

.	Delete the template before and including the "cut here".

.	Edit in the name of each of the job submission protocol attributes that corresponds to each of the object/attribute from the brainstorm list in the table below.

.	For objects/attributes that do not have a corresponding attribute in your job submission protocol, please enter just a - (hyphen).

.	If you aren't sure whether the job attribute you found is correct, put a question mark after the name of your job submission protocol attribute.

.	Add any notes to the last column labeled NOTES.

.	Rename your file to the xxx.doc file name indicated below for your job submission protocol.

.	Post your file in:
    ftp://ftp.external.hp.com/snmpmib/jobs-mib/protomap/xxx.doc

.	Send mail to the jmp list indicating that you have posted your mapping.

I've already copied all of the original files into the historic sub-directory under protomap/, so you can use the same file name as before.

| Job Submission Protocol | file name | Responsible person |
| --- | --- | --- |
| . AppleTalk PAP | applepap.doc | Ron Bergman |
| . IPDS | ipds.doc | Harry Lewis |
| . ISO DPA | iso-dpa.doc | Tom Hastings |
| . LPR/LPD - RFC 1179, PSIS extensions | lpr-lpd.doc | Ron Bergman |
| . NDPS | ndps.doc | Craig Whittle |
| . PJL | pjl.doc | Bob Pennacost |
| . PostScript | ps.doc | Bob Setterbo |
| . PSERVER | pserver.doc | Craig Whittle |
| . RPRINTER | rprinter.doc | Craig Whittle |
| . SMB | smb.doc | Bob Setterbo |
| . TIPSI/NPAP | tipsi.doc | Don Wright |

For convenience, I have copied the full description of each of the objects that we are mapping to job submission protocols.  In the table is a short description of each object, but the full text is what you should use to pick the curresponding attribute in your job submission protocol.

jmJobIndexxe "jmJobIndex"§ OBJECT-TYPE

SYNTAX	Integer32(10..2147483647)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"The identifier of the job on the device or server.    The job's identifier is generated by the server or device when that server or device accepted the job.  However, if the device does not generate a job identifier for each job, then the Job Monitoring MIB agent shall generate the job identifier for the job.

The value 0 shallneed not be generated."

::= { jmJobEntry 1 }

jmJobNamexe "jmJobName"§ OBJECT-TYPE

SYNTAX	OCTET STRING(SIZE(0..63))

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"This object is the human readable string name of the job as assigned by the submitting user to help the user distinguish between his/her various jobs.  This name does not need to be unique.

This attribute is intended for enabling a user or the user's application to convey a job name that may be printed on a start sheet, returned in a query result, or used in notification or logging messages.

If this attribute is not specified, no job name is assumed, but implementation specific defaults are allowed, such as the value of the documentName(4) resource item of the first document in the job or the fileName(3) resource item of the first document in the job."

::= { jmJobEntry 2 }

jmJobIdNamexe "jmJobIdName"§ OBJECT-TYPE

SYNTAX	OCTET STRING(SIZE(0..63))

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Identifies the job on the "client-side" of the printing process as coded character set data in combination with the jmJobIdNumber object.

The jmJobIdName and the jmJobIdNumber objects are referred to as the "client-side" identifiers because they allow the user, operator, or the system administrator to uniquely identify the print jobs of interest from all the jobs currently "known" by the server or device.

The client-side identifiers can be assigned by either the job submission client's local system or a downstream server, depending on implementation and the job submission protocol.  The format of the coded character set data and point of assignment of the client-side identifiers depend upon the job submission protocol in use.  See Appendix A on page Error: Reference source not found for the mapping from selected job submission protocols to these client-side job identifiers.

Unlike jmJobName, which is assigned by the submitting user, the jmJobIdName and jmJobIdNumber client-side identifiers provide for unique identification of jobs.

The jmJobIdName object may be used alone or in conjunction with the jmJobIdNumber object, depending upon the format of the job submission protocol client side identifier.  For example, the LPD job identifier normally contains three alpha characters followed by a three digit number.  The agent may represent the alpha portion by jmJobIdName and the numeric portion by jmJobIdNumber.  Alternatively, the agent may represent the LPD  client-side id entirely in the jmJobIdName object."

::= { jmJobEntry 3 }

jmJobIdNumberxe "jmJobIdNumber"§ OBJECT-TYPE

SYNTAX	Integer32(0..2147483647)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Identifies the job on the "client-side" of the printing process in combination with the jmJobIdName object. This object may be used alone or in conjunction with the jmJobIdName  object, depending upon the format of the job submission protocol client-side identifier.  Refer to the jmJobIdName object specification."

::= { jmJobEntry 4 }

jmJobOwnerxe "jmJobOwner"§ OBJECT-TYPE

SYNTAX	OCTET STRING(SIZE(0..63))

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"The coded character set name of the user that submitted the job.  The method of assigning this user name will be system and/or site specific but the method must insure that the name is unique to the network that is visible to the client and target device.

This value should be the authenticated name of the user submitting the job."

::= { jmJobEntry 6 }

jmJobDeviceNameRequestedxe "jmJobDeviceNameRequested"§ OBJECT-TYPE

SYNTAX	OCTET STRING(SIZE(0..63))

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"The administratively defined coded character set name of the target device.  Its value corresponds to the Printer MIB: prtGeneralAdminName object (added to the draft Printer MIB).

NOTE - while this object could be considered a resource and could be allocated in the jmResourceTable, it has been allocated as a separate object, since management applications are likely to want to get this value each time they access a job, rather than have to copy the entire jmResourceTable to search for it."

::= { jmJobEntry 7 }

jmJobCommentxe "jmJobComment"§ OBJECT-TYPE

SYNTAX	OCTET STRING(SIZE(0..63))

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"This object contains the arbitrary human-readable coded character text string supplied by the submitting user or the job submitting application program for any purpose.  For example, a user might indicate what he/she is going to do with the printed output or the job submitting application program might indicate how the document was produced."

::= { jmJobEntry 11 }

| fileName(3), | -- -- -- -- -- -- -- -- -- -- -- -- -- | OCTET STRING(SIZE(0..63)) | The coded character set file name of the document. A row with this resource item may appear more than once in the jmResourceTable for a job. Issue 1 - OK to have added fileName(3) to jmResourceType? OK to have added fileName(3) to jmResourceType? |
| --- | --- | --- | --- |
| documentName(4), | -- -- -- -- -- -- -- -- -- -- | OCTET STRING(SIZE(0..63)) | The coded character set name of the document. If the document does not have a name, the file name containing the document may be used as a default. A row with this resource item may appear more than once in the jmResourceTable for a job. |

Please fill in the following information followed by the attributes from your job submission protocol in the following table:

<--------------------------------- cut here -------------------------------->

Name of Job Submission Protocol:

Versoin of Job Submission Protocol:

Is your MIB mappings for a MIB in the Printer or a Server or both:

File name:

Assigned person:

| Job Monitoring MIB object | Corresponding job submission protocol attribute | NOTES |
| --- | --- | --- |
| jmJobIndex - the job's identifier generated by the server or device implementing this Job Monitoring MIB | - |  |
| jmJobName - Job name (assigned by job owner) which is not necessarily unique. | - |  |
| jmJobIdName - the job's identifier name generated by the job submitting software using the job submission protocol. This name can be anything that helps identifier the job to the job submitter, including the name of the queue from which the job was submitted. | - |  |
| jmJobIdNumber - the job's identifier number generated by the job submitting software using the job submission protocol. A (-2) value shall indicate that the submitter did not supply a job identifier number. | - |  |
| jmJobOwner - Job owner (User name of the user that originally submitted the job) | - |  |
| jmJobDeviceNameRequested - Device name (or queue nameDevice-specific name of device) requested by the submitting user. | - |  |
| jmJobComment - Job comment supplied by the submitting user. | - |  |
| fileName(3) - file names of the document. | - |  |
| documentName(4) - Document name(s) (defaults from file-name) | - |  |