INTERNET-DRAFT                                              Ron Bergman

Dataproducts Corp.

October 15, 1997

Job Submission Protocol Mapping Recommendations

for the Job Monitoring MIB

<draft-ietf-printmib-job-protomap-00.txt>

Expires Apr 15, 1997

Status of this Memo

This document is an Internet-Draft.  Internet-Drafts are working

documents of the Internet Engineering Task Force (IETF), its areas,

and its working groups.  Note that other groups may also distribute

working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six

months and may be updated, replaced, or obsoleted by other

documents at any time.  It is inappropriate to use Internet-Drafts

as reference material or to cite them other than as "work in

progress".

To learn the current status of any Internet-Draft, please check the

"1id-abstracts.txt" listing contained in the Internet-Drafts Shadow

Directories on ftp.is.co.za (Africa), nic.nordu.net (Europe),

munnari.oz.au (Pacific Rim), ds.internic.net (US East Coast), or

ftp.isi.edu (US West Coast).

Abstract

This Internet-Draft defines the recommended mapping for many

currently popular Job submission protocols to objects and

attributes the Job Monitoring MIB.

TABLE OF CONTENTS

1.0  INTRODUCTION

2.0  LINE PRINTER DAEMON (LPR/LPD) PROTOCOL

2.1  jmJobSubmissionId Mapped to LPR/LPD

2.2  jmJobIndex Mapped to LPR/LPD

2.3  Other MIB Objects Mapped to LPR/LPD

2.4  The Attribute Group Mapped to LPD

3.0  APPLETALK PROTOCOL

3.1  jmJobSubmissionId Mapped to AppleTalk

3.2  Other AppleTalk Mappings

4.0  INTERNET PRINTING PROTOCOL (IPP)

4.1  jmJobSubmissionId Mapped to IPP

4.2  jmJobIndex Mapped to IPP

4.3  Other MIB Objects Mapped to IPP

4.4  The Attribute Group Mapped to IPP

5.0  INTELLIGENT PRINTER DATA STREAM (IPDS)

6.0  DOCUMENT PRINTING APPLICATION (DPA)

7.0  NOVELL DISTRIBUTED PRINT SERVICE (NDPS)

7.1  jmJobSubmissionId Mapped to NDPS

7.2  jmJobIndex Mapped to NDPS

7.3  Other MIB Objects Mapped to NDPS

7.4  The Attribute Group Mapped to NDPS

8.0  PRINTER JOB LANGUAGE (PJL)

8.1  jmJobSubmissionId Mapped to PJL

8.2  jmJobIndex Mapped to PJL

8.3  The Attribute Group Mapped to PJL

9.0  POSTSCRIPT

10.0  NETWARE PSERVER

10.1  jmJobSubmissionId Mapped to PServer

10.2  jmJobIndex Mapped to PServer

10.3  The Attribute Group Mapped to PServer

11.0  NETWARE NPRINTER or RPRINTER

12.0  SERVER MESSAGE BLOCK (SMB) PROTOCOL

13.0  TRANSPORT INDEPENDENT PRINTER/SYSTEM INTERFACE (TIP/SI)

14.0  REFERENCES

1.0  INTRODUCTION

The Job Monitoring MIB [JobMIB] is functional with any job submission protocol.  However, the information available and the method of presentation varies significantly by job submission protocol.  A common method of mapping job submission information to the Job Monitoring MIB is essential for interoperability of Job MIB agents and monitoring applications.  This document defines recommended mappings for most popular job submission protocols to insure this compatibility.

All mappings are unidirectional from the job submission protocol to the MIB.  It is assumed that support of the job submission protocol in the printer implies that the reverse information flow is presently defined and does not require interaction from the MIB.  This mapping is not defined in this document as it should be obvious.

This document refers to system configurations that are defined in the Job Monitoring MIB [JobMIB].  For those readers that are familiar with the configuration descriptions, a short summary appears here.  Please see the Job MIB document for further details.

Configuration 1:  This is a simple peer-to-peer system which contains

only a client and a printer.  The Job MIB agent is resident in

the printer.

Configuration 2:  This system contains a client, server, and a

printer.  The Jib MIB agent is resident in the server.

Configuration 3:  This system, as in configuration 2, contains a

client, server, and a printer.  In this case the Job MIB agent is

implemented within the printer.

The most important object to be mapped is jmJobSubmissionId, since this is the key for the user to locate a submitted job.  Therefore, jmJobSubmissionId is specified for all job submission protocols defined in this document.  The remaining objects mapped include only those items that have the equivalent information presented to the printer by the job submission protocol.

2.0  LINE PRINTER DAEMON (LPR/LPD) PROTOCOL

The LPR/LPD printing protocol [LPD] is used with BSD Unix systems in the client-server-printer configuration.  Usage of the Job Monitoring MIB with LPR/LPD will most likely conform to Configuration 3, where the monitor application or the server uses SNMP to obtain job information from the printer.  The client communicates with the Unix server using the existing LPD protocol to obtain job information.

The LPR/LPD protocol is also used in the Windows environment to implement peer-to-peer printing, as shown in configuration 1.  In this case, SNMP is used by the client and/or the monitor application to obtain the job information.

One of the major problems of LPR/LPD is the large number of vendor unique extensions currently used with the protocol and the resulting compatibility issues between available implementations.  To avoid these issues, this mapping of LPR/LPD is restricted to the protocol as defined by RFC 1179.

The LPR/LPD protocol transfers print job data and control information in separate files, known as the Data File and Control File respectively.  Most of the information concerning the print job is contained in the Control File.  In many LPD implementations, the Control File is transferred following the Data File.  Thus much of the information concerning the job may not be available until the completion of the data transmission.

2.1  jmJobSubmissionId Mapped to LPR/LPD

The LPR/LPD Receive Data File command contains a parameter which defines the name of the data file.  This name field is structured as follows:

dfaXXX<host-name>  or  daXXXX<host-name>

Where XXX or XXXX is the numeric job number assigned by the LPR/LPD client submitting the print job.  The recommended mapping of this name field to jmJobSubmissionId is:

octet 1:   '9'

octets 2-40:  Contains the <host-name> portion of the name field.  If

the <host-name> portion is less than 40 octets, the

left-most character in the string shall appear in octet

position 2.  Otherwise, only the last 39 bytes shall be

included.

octets 41-48:  ‘00000XXX’  or  ‘0000XXXX’.

2.2  jmJobIndex Mapped to LPR/LPD

The job index (jmJobIndex) is assigned by the SNMP job monitoring agent and is independent of the XXX (or XXXX) index assigned by the LPR/LPD client.  This will allow the SNMP agent to track jobs received from multiple sources.

2.3  Other MIB Objects Mapped to LPR/LPD

MIB Object             | LPR/LPD Parameter

-----------------------+------------------------------------------------

jmJobKOctetsRequested  | Number of bytes as defined in the Data File

jmJobOwner             | User Identification string in the Control File

2.4  The Attribute Group Mapped to LPD

Other attributes that are applicable, but not defined in this section such as attributes that map to a vendor unique extension, may also be included.

MIB attribute         | LPR/LPD information             | Data type

----------------------+---------------------------------+--------------

serverAssignedJobName | Name of the data file (note 1)  | Octet String

jobSourceChannelIndex | prtChannelIndex (note 2)        | Integer

queueNameRequested    | Queue name from the Data File   | Octet String

fileName              | Name of the source file (note 3)| Octet String

documentName          | The document title name (note 3)| Octet String

Notes:

------

1. See section 2.1 (jmJobSubmissionId).

2. Included if the Printer MIB is also supported by the agent.

3. The information is optional in the Control File.  The attribute

should be included if present in the Control File.

3.0  APPLETALK PROTOCOL

AppleTalk was originally developed as a peer-to-peer network protocol, as described in configuration 1, for use with Apple Macintosh computers.  Today, print spoolers are also available for use with Macintosh computer networks that conform to configurations 2/3.  In addition, printing with the AppleTalk protocol is supported from both Windows NT servers and Novell servers also per configurations 2/3.

The AppleTalk protocol provides very little information that can be used with the Job Monitoring MIB.  The Macintosh print drivers are able to provide information concerning the user and document name but imbed this information in the PDL, which is typically PostScript.  The preferred jmJobSubmissionId is constructed from the information in the PostScript file, as defined in section 9.0.

3.1  jmJobSubmissionId Mapped to AppleTalk

An alternative jmJobSubmissionId may be constructed from the Connection Identifier contained in the AppleTalk Printer Access Protocol (PAP) header.  Since the Connection Id is not readily available in any of the defined AppleTalk implementations, this approach may be of little utility.

octet 1:   '?'   ***** NEW TYPE CODE ???? *****

octets 2-40:  Contains the AppleTalk printer name, with the first

character of the name in octet 2.  (AppleTalk printer

names are a maximum of 31 characters.)

octets 41-48:  ‘00000XXX’, where ‘XXX’ is the decimal representation

of the Connection Id.

3.2  Other AppleTalk Mappings

No other Job MIB objects or parameters can be derived from information available in the AppleTalk headers

4.0  INTERNET PRINTING PROTOCOL (IPP)

The Internet Printing Protocol [IPP] supports printing using any one of the three possible configurations.  For configuration 2, the mapping defined herein is performed on a server.  Otherwise, the mapping is performed on an agent within the printer.

4.1  jmJobSubmissionId Mapped to IPP

IPP contains a rich set of parameters which allow several methods of creating the jmJobSubmissionId object.  The preferred method is to use the IPP job-uri attribute as follows:

octet 1:   '4'

octets 2-40:  Contains the IPP job-uri job template attribute.  If

the job-uri is less than 40 octets, the left-most

character in the string shall appear in octet position

2.  Otherwise, only the last 39 bytes shall be included.

octets 41-48:  Contains the job-id job template attribute.

If the job-uri is not available to the agent, the job-name job template attribute shall be used.

octet 1:   '?'  ***** New format required ? (1 for clients) *****

octets 2-40:  Contains the IPP job-name job template attribute.  If

the job-name is less than 40 octets, the left-most

character in the string shall appear in octet position

2.  Otherwise, only the last 39 bytes shall be included.

octets 41-48:  Contains the job-id job template attribute.

If both the job-uri and the job-name are not available, the job-originating-user job template attribute shall be used.

octet 1:   '4'

octets 2-40:  Contains the IPP job-originating-user job template

attribute.  If the job-originating-user name is less

than 40 octets, the left-most character in the string

shall appear in octet position 2.  Otherwise, only the

last 39 bytes shall be included.

octets 41-48:  Contains the job-id job template attribute.

4.2  jmJobIndex Mapped to IPP

The job index (jmJobIndex) assigned by the SNMP job monitoring agent may be identical to the IPP job-id job template attribute if the agent is receiving jobs only from a single server or client.  If jobs are to be received from multiple sources, jmJobIndex and job-id must be independent.

4.3  Other MIB Objects Mapped to IPP

MIB Object                | IPP Job template attribute

--------------------------+---------------------------------------------

jmJobOwner                | job-originating-user

jmJobKOctetsRequested     | job-k-octets

jmJobKOctetsProcessed     | job-k-octets-processed

jmJobImpressionsRequested | job-impressions

jmJobImpressionsProcessed | job-impressions-completed

jmJobStateReasons1        | job-state-reasons (note 1)

jmNumberOfInterveningJobs | number-of-intervening-jobs

Notes:

------

1. JobStateReasons is a bit map described in one object and three

attributes.  The IPP condition may change one or more of the bits

in one or more of these Job MIB items.

4.4  The Attribute Group Mapped to IPP

The following mappings are required if the listed IPP job template attribute is provided.

MIB attribute              | IPP job template attribute   | Data type

---------------------------+------------------------------+-------------

jobName                    | job-name                     | Octet String

documentFormat             | document-format              | Octet String

jobPriority                | job-priority                 | Integer

jobHoldUntil               | job-hold-until               | Octet String

sides                      | sides                        | Integer

finishing                  | finishings                   | Integer

printQualityRequested      | print-quality                | Integer

printerResolutionRequested | printer-resolution           | Integer

jobCopiesRequested         | copies                       | Integer

mediumRequested            | media                        | Octet String

jobSubmissionTime          | time-at-pending              | Integer

jobStartedProcessingTime   | time-at-processing           | Integer

jobCompletionTime          | time-at-completed            | Integer

sheetsRequested            | job-media-sheets             | Integer

jobURI                     | job-uri                      | Octet String

jobStateReasonsN           | job-state-reasons (note 1)   | Integer

physicalDevice             | output-device-assigned       | Octet String

sheetsCompleted            | job-media-sheets-completed   | Integer

Notes:

------

1. JobStateReasons is a bit map described in one object and three

attributes.  The IPP condition may change one or more of the bits

in one or more of these Job MIB items.

5.0  INTELLIGENT PRINTER DATA STREAM (IPDS)

6.0  DOCUMENT PRINTING APPLICATION (DPA)

7.0  NOVELL DISTRIBUTED PRINT SERVICE (NDPS)

Novell Distributed Print Services is a DPA based job submission protocol that conforms to configuration 3.

7.1  jmJobSubmissionId Mapped to NDPS

NDPS supports the generation of a properly formatted jmJobSubmissionId for use in the Job MIB.

7.2  jmJobIndex Mapped to NDPS

NDPS does not provide a value that can be mapped to jmJobIndex.

7.3  Other MIB Objects Mapped to NDPS

MIB Object             | NDPS Parameter

-----------------------+------------------------------------------------

jmJobOwner             |

7.4  The Attribute Group Mapped to NDPS

The following mappings are required if the listed PJL attribute or command option is provided.

MIB attribute              | NDPS parameter               | Data type

---------------------------+------------------------------+-------------

jobAccountName             |

serverAssignedJobName      |

jobName                    |

jobServiceTypes            |

numberOfDocuments          |

fileName                   |

documentName               |

jobComment                 |

documentFormatIndex        |

documentFormat             |

jobPriority                |

jobProcessAfterDateAndTime |

jobHoldUntil               |

outputBin                  |

sides                      |

finishing                  |

printQualityRequested      |

printerResolutionRequested |

jobCopiesRequested         |

mediumRequested            |

jobSubmissionToServerTime  |

jobSubmissionTime          |

8.0  PRINTER JOB LANGUAGE (PJL)

PJL [PJL] has been developed by Hewlett-Packard to provide job control information to the printer and status information to applications, independent of the PDL.  PJL is normally transferred using a traditional job submission language such as LPR/LPD or NetWare PServer.

8.1  jmJobSubmissionId Mapped to PJL

PJL has defined the SUBMISSIONID option for the JOB command which indicates a properly formatted jmJobSubmissionId for use in the Job MIB.  The PJL JOB command is presented at the start of a print job with options that apply only the attached job.  The syntax for this command option is:

@PJL JOB SUBMISSIONID = “id string”

Driver software that implements this PJL command option must provide the “id string” in one of the client version formats specified in the Job MIB for jmJobSubmissionId.

For drivers that are not able to create the SUBMISSIONID option, it is recommended that jmJobSubmissionId format 0 be created by the agent using the PJL attribute DocOwner or DocOwnerId.

octet 1:   '0'

octets 2-40:  Contains the string associated with DocOwner or

DocOwnerId.  If the string is less than 40 octets, the

left-most character in the string shall appear in octet

position 2.  Otherwise, only the last 39 bytes shall be

included.  If DocOwner or DocOwnerId cannot be obtained,

this field shall be blank.

octets 41-48:  Contains the value of jmJobIndex associated with the

job.

8.2  jmJobIndex Mapped to PJL

PJL does not provide a value that can be mapped to jmJobIndex.

8.3  The Attribute Group Mapped to PJL

The following mappings are required if the listed PJL attribute or command option is provided.

MIB attribute         | PJL attribute or command option  | Data type

----------------------+----------------------------------+--------------

jobOwner              | DocOwner or DocOwnerId attribute | Octet String

serverAssignedJobName | DocName attribute or the command | Octet String

|  @PJL JOB Name = “string”        | Octet String

submittingServerName  | SrcServerName attribute          | Octet String

jobOriginatingHost    | SrcPort attribute                | Octet String

queueNameRequested    | SrcQ attribute                   | Octet String

fileName              | JobFName attribute               | Octet String

jobComment            | JobDesc attribute                | Octet String

jobSubmissionTime     | TimeSubmit attribute             | Octet String

9.0  POSTSCRIPT

10.0  NETWARE PSERVER

The NetWare PServer job submission protocol is implemented in a client-server-printer system on the server to printer link as defined in configuration 3.

10.1  jmJobSubmissionId Mapped to PServer

octet 1:   '?'   ***** New format req’d ??? *****

octets 2-40:  Contains the Directory Path Name as recorded by the

Novell File Server in the queue directory.  If the

string is less than 40 octets, the left-most character

in the string shall appear in octet position 2.

Otherwise, only the last 39 bytes shall be included.

octets 41-48:  ‘000XXXXX’  The ASCII representation of the Job Number

as per the queue directory.

10.2  jmJobIndex Mapped to PServer

The job index (jmJobIndex) is assigned by the SNMP job monitoring agent and is independent of the Job Number assigned by the NetWare File Server.  This will allow the SNMP agent to track jobs received from multiple sources.

10.3  The Attribute Group Mapped to PServer

The following mappings are required if the listed PServer parameter is provided in the Novell File Server queue directory.

MIB attribute              | PServer parameter           | Data type

---------------------------+-----------------------------+--------------

jobOwner                   | Client Id Number            | Integer

serverAssignedJobName      | Job File Name               | Octet String

queueNameRequested         | Queue Id                    | Integer

physicalDevice             | Server Id Number            | Integer

jobComment                 | Job Description             | Octet String

jobPriority                |

jobProcessAfterDateAndTime | Target Execution Time       | Octet String

jobHoldUntil               |

jobCopiesRequested         | Number of Copies            | Integer

mediumRequested            | Form Name                   | Octet String

jobSubmittedToServerTime   | Job Entry Time              | Octet String

11.0  NETWARE NPRINTER or RPRINTER

The NetWare NPrinter/RPrinter protocol was designed to transfer print data from a Novell File Server to a printer attached directly to a local port (e.g. parallel or serial) on a PC.  NPrinter/RPrinter is an extremely lightweight printing protocol.  Consequently, no information required by the Job Monitoring MIB is provided and a meaningful jmJobSubmissionId cannot be generated.

It is recommended that an additional job submission layer, such as PJL or another vendor private protocol,  be included on top of NPrinter/RPrinter to provide the required information.  The mapping should then be performed according to the recommendations of the higher layer submission protocol.

12.0  SERVER MESSAGE BLOCK (SMB) PROTOCOL

13.0  TRANSPORT INDEPENDENT PRINTER/SYSTEM INTERFACE (TIP/SI)

14.0  REFERENCES

[IPP]  The Internet Printing Protocol RFC XXXX, Model RFC XXXX

[JobMIB]  The Job Monitoring MIB, RFC XXXX, IETF informational document.

[LPD]  Line Printer Daemon Protocol, RFC 1179, IETF informational document.

[PJL]  Printer Job Language Technical Reference Manual, Hewlett-Packard part number 5021-0328.

[PrtMIB]  The Printer MIB, RFC 1759, IETF standards track document.