Subj:	Groups for Printer Job Monitoring and Spooling Job Monitoring objects/attributes

From:	Tom Hastings

Date:	4/15/96

File:	job-grps.doc

Version:	0.5

# Criteria for dividing objects/attributes into groups

This document is intended to help devise groups for the Printer Job Monitoring MIB/MIF and the Spooling Job Monitoring MIB/MIF.

The criteria for dividing information objects/attributes into groups includes:

- Separate job vs. document, since document needs another index

- Separate general vs. print-specific

- Separate spooling vs. printer

- Separate accounting

- Separate media supported table, but don’t use index

- Separate multi-values objects/attributes into separate groups/tables

# Legend

The following legend is used in the following tables in this document:

| Column Heading | Description |
| --- | --- |
| Section | The section number in the ISO DPA standard that describes the attribute that corresponds to the object. A blank section number indicates an object that doesn’t have a corresponding ISO DPA attribute. PSIS indicates that the attribute is specified by the X/Open Print Systems Interoperability Specification (PSIS) draft standard. |
| G/P | Indicates General (G) or print-specific (P) attribute. Some print-specific names are being changed to make the attribute apply to scan and FAX jobs as well. |
| MIB | Indicates which MIF/MIB the attribute/object is intended: N: non-spooling (printer) job monitoring MIF/MIB S: spooling job monitoring MIF/MIB (future) S&N: appears in both the spooling and non-spooling job monitoring MIB/MIF with different values <blank>: not proposed for either MIF/MIB |
| M/O | Proposed conformance for the job monitoring MIB: M: mandatory CM: conditional mandatory - mandatory if doing accounting O: optional <blank>: proposed not to be included in the either the printer or the spooling job monitoring MIF/MIB |
| MIB Datatype | Proposed datatype for the MIB object: A(255) - ASCII string, upto 255 octets I(255) - InternationalDisplayString 255 octets - for identifying server objects T(255) - CodeIndexedStringIndex - any coded character set from the client |
| Attribute | Name of the corresponding ISO DPA attribute |
| Ron | Ron Bergman’s proposed names |
| page no | Page number of the ISO DPA attribute specification. A blank page number indicates that this object does not have a corresponding ISO DPA attribute |

There are 3 objects on Ron Bergman’s list that are not in my list, because they are per-printer object/attributes, not per-job (I believe):  jobsCurrentlyInProcess, jobLastActiveNumber, jobPrinterComment,

# Job General Non-Spooling Mandatory group

The following table lists just the job objects/attributes that are being proposed to be in the Job General Non-spooling mandatory group.

| Section | G/P | S/N | M/O | MIB/MIF Datatype | Attribute | Ron’s name | Prob | page no |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G | N | M | Ordinal16 | GeneralCurrentLocalization |  |  |  |
| 9.2.1.1 | G | N | M | A(255) | Job-identifier-on-server |  |  | 104 |
| 9.2.1.2 | G | N | M | A(255) | Job-identifier-downstream | jobIdentifierOnPrinter |  | 104 |
| 9.2.1.3 | G | N | M | A(255) | Job-identifier-upstream | jobIdentifierOnClient |  | 104 |
| 9.2.1.4 | G | N | M | T(255) | Job-client-id |  |  | 105 |
|  | G | N | M | A(255) | Job-parent-identifier |  |  |  |
|  | G | N | M | Oid | Job-type |  | O3 |  |
| 9.2.1.5 | G | N | M | T(255) | Job-name | jobName | U2 | 105 |
| 9.2.1.7 | G | N | M | T(255) | Job-owner |  | U2 | 106 |
|  | G | N | M | PrtChannelType | job-source-channel-type (from rfc 1759) |  | 21 |  |
|  | G | N | M | CodedCharSet | Submitted-coded-char-set (that the submitting client used to encode text attributes -IANA MIBenum character set values) |  | U2 |  |
| 9.2.8.1 | G | N | M | JobState | Current-job-state | jobCurrentState | U2 | 124 |
| 9.2.8.2 | G | N | M | bit-vector | Job-state-reasons |  | U2 | 127 |

# Job General Non-Spooling Optional group

The following table lists just the job objects/attributes that are being proposed to be in the Job General Non-spooling optional group.

| Section | G/P | S/N | M/O | MIB/MIF Datatype | Attribute | Ron’s name | Prob | page no |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9.2.1.6 | G | N | O | T(255) | Job-originator | jobOriginator | U2 | 106 |
| 9.2.1.8 | G | N | O | T(255) | Job-comment | jobComment | U2 | 106 |
| 9.2.2.1 | G | N | O | Integer32 | job-copies |  |  | 108 |
| 9.2.2.1 | G | N | O | index32 | output-bin |  |  | 108 |
| 9.2.8.6 | G | S&N | O | DateAndTime | Estimated-completion-time |  |  | 129 |
| 9.2.8.7 | G | S&N | O | DateAndTime | Submission-time |  |  | 130 |
| 9.2.8.11 | G | N | O | Counter32 | Pages-completed | jobLogicalPageCount |  | 130 |
| 9.2.8.14 | G | N | O | Counter32 | Octets-completed | jobSizeInBytes |  | 132 |
| 9.2.8.23 | G | N | O | Cardinal32 | Error-count |  |  | 135 |
| 9.2.8.24 | G | N | O | Cardinal32 | Warning-count |  |  | 135 |
| 9.2.8.25 | G | N | O | Cardinal32 | Processing-time | jobProcessingTime |  | 135 |
| 9.2.8.27 | G | N | O | Cardinal32 | Number-of-documents |  |  | 135 |

# Document General Non-spooling Mandatory group

The following table lists the document objects/attributes being proposed for the Document General Non-spooling Mandatory objects/attributes group.  The document table is a single table for all jobs on the device, so that each row is indexed by the hrDeviceIndex from the host resources MIB, the job-identifier generated by the server when the server accepted the job, and the document-sequence-number (starts at 1 for each job).

| Section | G/P | S/N | M/O | MIB/MIF Datatype | Attribute | Ron’s name | Prob | page no |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9.3.1.1 | G | N | M | T(255) | Document-name |  |  | 139 |
| 9.3.1.2 | G | N | M | T(255) | Document-file-name |  |  | 139 |
| 9.3.3.1 | G | S | M | enum | Document-type |  |  | 166 |
| 9.3.3.3 | G | N | M | enum | Document-format |  |  | 167 |
| 9.3.3.3 | G | N | M | T(255) | Document-format-variants |  |  | 167 |
| 9.3.3.3 | G | N | M | T(255) | Document-format-version |  |  | 167 |
| 9.3.4.16.1 | G | N | M | Integer32 | Octet-count |  |  | 175 |
| 9.3.5.1 | G | N | M | Integer32 | Document-sequence-number |  |  | 177 |
| 9.3.5.2 | G | N | M | enum | Document-state |  |  | 177 |

# Document Printing Non-spooling Optional group

The following table lists the document objects/attributes being proposed for the Document Printing Non-Spooling Optional group.  The document table is a single table for all jobs on the device, so that each row is indexed by the hrDeviceIndex from the host resources MIB, the job-identifier generated by the server when the server accepted the job, and the document-sequence-number (starts at 1 for each job).

| Section | G/P | S/N | M/O | MIB/MIF Datatype | Attribute | Ron’s name | Prob | page no |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9.3.2.1 | P | N | O | I(255) | Default-medium |  |  | 140 |
| 9.3.2.2 | P | N | O | index16 | Default-input-tray |  |  | 141 |
| 9.3.2.13 | P | N? | O | I(255) | Finishing [future?] |  |  | 153 |
| 9.3.2.15 | P | N | O | enum | Output [method] |  |  | 154 |
| 9.3.2.16.1 | P | S | O | Cardinal32 | Number-up |  |  | 155 |
| 9.3.2.19 | P | N | O | Int(1..2) | Sides to Print |  |  | 163 |
| 9.3.2.22 | P | N | O | Cardinal32 | Copy-count |  |  | 165 |
| 9.3.5.3 | P | N | O | Cardinal32 | Copies-completed |  |  | 178 |

# Job General Accounting Conditional Mandatory group

The following table lists only the accounting objects/attributes being proposed for the Job General Conditional Mandatory Accounting group.  The objects in this group are mandatory, if the device does accounting.

| Section | G/P | S/N | M/O | MIB/MIF Datatype |  | Attribute | Ron’s names | Prob | page no |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9.2.6.1 | G | N | CM | T(255) | User-name |  |  |  | 120 |
| 9.2.6.2 | G | N | CM | O(255) | Accounting-information |  |  |  | 120 |
| 9.2.8.9 | G | N | CM | DateAndTime | Started-processing[printing]-time |  | jobStartTime |  | 130 |
| 9.2.8.12 | G | N | CM | Counter32 | Impressions-completed |  | jobImpressionCount |  | 131 |
| 9.2.8.13 | G | N | CM | Counter32 | Media-sheets-completed |  |  |  | 131 |
| 9.2.8.26 | G | N | CM | DateAndTime | Completion-time |  | jobEndTime |  | 135 |

# Job Media Consumed Optional Accounting group

The following table lists only the accounting objects/attributes being proposed for the Job Media Consumed Optional accounting group.

| Section | G/P | S/N | M/O | MIB/MIF Datatype | Attribute | Ron’s names | Prob | page no |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSIS | P | N | O | index16 | Media-consumed-type-name - table | jobPaperTraysSelected, jobFormSize |  |  |
| PSIS | P | N | O | Counter32 | Media-consumed-sheet-count - table | jobMediaSheetCount |  |  |

# Job Color Impressions Consumed Optional Accounting group

The following table lists only the accounting objects/attributes being proposed for the Job Color Impressions Consumed Optional accounting group.

| Section | G/P | S/N | M/O | MIB/MIF Datatype | Attribute | Ron’s names | Prob | page no |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSIS? | P | N | O | index16 | Color-impressions-consumed-name - table |  |  |  |
| PSIS? | P | N | O | Counter32 | Color-impressions-consumed - table |  |  |  |

# Job General Spooling Mandatory group

The following table lists all the job objects/attributes being proposed for the Job General Spooling Mandatory group.

| Section | G/P | S/N | M/O | MIB/MIF Datatype | Attribute | Ron’s names | Prob | page no |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G | S | M | PrtChannelType | job-destination-channel-type (from rfc 1759) |  |  |  |
| 9.2.2.1 | G | S | M | Ordinal32 | number-of-job-sets |  |  | 108 |
| 9.2.5.1 | G | S | M | T(255) | Device[Printer]-name-requested |  |  | 118 |
| 9.2.8.3 | G | S | M | JobState | Previous-job-state |  |  | 128 |
| 9.2.8.4 | G | S | M | hrDeviceIndex | Devices-assigned |  |  | 128 |
| 9.2.8.5 | G | S | M | enum | Devices-state-of-devices-assigned |  |  | 129 |
| 9.2.8.15 | G | S | M | Cardinal32 | Total-job-octets |  |  | 136 |
| 9.2.8.16 | G | S | M | Cardinal32 | Intervening-jobs |  |  | 132 |

# Job General Spooling Optional group

The following table lists all the job objects/attributes being proposed for the Job General Spooling Optional group.

| Section | G/P | S/N | M/O | MIB/MIF Datatype | Attribute | Ron’s names | Prob | page no |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9.2.4.5 | G | S | M | DateAndTime | Job-process[print]-after |  |  | 116 |
| 9.2.4.6 | G | S | O | Int(1..100) | Job-priority |  |  | 116 |
| 9.2.4.7 | G | S | O | DateAndTime | Job-deadline-time |  |  | 116 |
| 9.2.4.8 | G | S | O | DateAndTime | Job-discard-time |  |  | 117 |
| 9.2.4.9 | G | S | M | DeltaTime | Job-retention-period |  |  | 117 |
| 9.2.4.10 | G | S | O | T(255) | Job-message-to-operator |  |  | 117 |
| 9.2.8.10 | GP | N | O | Counter32 | Job-copies-completed |  |  | 130 |
| 9.2.8.20 | G | S | O | T(255) | Job-message-from-administrator |  |  | 134 |
| 9.2.9.1 | G | S | O | Cardinal32 | Job-page-count |  |  | 137 |

# Job Media Supported group

The following optoinal table contains the media supported by this system.

| Section | G/P | S/N | M/O | MIB Datatype | Attribute | Ron’s names | Prob | page no |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G | N | O | A(255) | ASCII-name |  |  |  |
|  | G | N | O | T(255) | Name |  |  |  |
|  | G | N | O | T(255) | Description |  |  |  |
|  | G | N | O | enum | MediaSizeUnit (.0001 inches or micrometers) |  |  |  |
|  | G | N | O | Ordinal32 | X-dimension |  |  |  |
|  | G | N | O | Ordinal32 | Y-dimension |  |  |  |
|  | G | N | O | A(255) | Type |  |  |  |
|  | G | N | O | Cardinal32 | Weight |  |  |  |
|  | G | N | O | A(255) | Color |  |  |  |