|                                               |                               | Group Status

|           General Printer Status              |         prtAlertTable         | (PrtSubUnitStatusTC)

|-----------------------------------------------+-------------------------------+---------------------

|             |                 |               |               |               | prtInputStatus

|             |                 |               |               |               | (per input tray)

|             |                 |               |               |               | prtOutputStatus

|             |                 |               |               |               | (per output bin)

|             |                 |               |               |               | prtMarkerStatus

|             |                 |               |               |               | (per marker)

|             |                 |               |               |               | prtMediaPathStatus

|             |                 | hrPrinter     |               |               | (per path)

Printer            | hrDevice    | hrPrinter       | Detected      |               |               | prtChannelStatus

State              | Status      | Status          | ErrorState    | prtAlertGroup | prtAlertCode  | (per channel)

-------------------+-------------+-----------------+---------------+---------------+---------------+---------------------

Normal or idle     |             |                 |               |               |               | all groups are:

(no binary alerts) | running (2) | idle (3)        | (0x0)         | (no binary    | (no binary    |   Idle (0)

|             |                 |               |  alerts)      |               |

Printing           |             |                 |               |               |               | Availability may be:

(no binary alerts) | running (2) | printing (4)    | (0x0)         | (no binary    | (no binary    |   Idle (0) or

|             |                 |               |  alerts)      |  alerts)      |   Active (4) or

|             |                 |               |               |               |   Busy (6)

|             |                 |               |               |               | depending on

|             |                 |               |               |               | subsystem state

|             |                 |               |               |               |

Off-line           |             |                 |               |               |               | prtChannelStatus(32)

(no binary alerts) | down (5)    | other (1)       | Offline (2)   | (no binary    | (no binary    | other groups are:

|             |                 |               | alerts)       |  alerts)      |   (indeterminate)

|             |                 |               |               |               |

Standby or Power   |             |                 |               |               |               | all groups are:

Saver Mode         | running (2) | other (3)       | (0x0)         | (no binary    | (no binary    |   Standby (2)

(no binary alerts) |             |                 |               |               |  alerts)      |

|             |                 |               |               |               |

Initial Power Up   |             |                 |               |               |               | all groups are:

(no binary alerts  | down (5)    | warmup (5)      | Offline (2)   | (no binary    | (no binary    |   Availability=

(while warming up) |             |                 |               |  alerts)      |  alerts)      |   Unknown+Off-line+

|             |                 |               |               |               |   Transitioning

|             |                 |               |               |               |   (5+32+64)

Warming Up         | running (2) | warmup (5)      | (0x0)         | (no binary    | (no binary    | all groups are:

(no binary alerts) |             |                 |               |  alerts)      |  alerts)      |   Availability=

|             |                 |               |               |               |   Standby+

|             |                 |               |               |               |   Transitioning

|             |                 |               |               |               |   (2+64)

Critical Errors: the printer can not print

(the error may be current while printing)

Jam                | down (5)    | other (1)       | Jammed(8)     | appropriate   | jammed(8)     | PrtSubUnitStatusTC

|             |                 |               | group         |               | (3+16+

|             |                 |               |               |               |  On-Line+

|             |                 |               |               |               |  Transitioning)

|             |                 |               |               |               |

Cover/Door         | down (5)    | other (1)       | Door Open(16) | cover(6)      | coverOpened(3)| prtCoverStatus

Open (listed       |             |                 |               |               | coverOpened   |    coverOpen (3)

in cover table)    |             |                 |               |               | (501)#        |

|             |                 |               |               |               |

Input Tray         | down (5)    | other (1)       | No Paper(64)  | input(8)      | subunitMissing| prtInputStatus

Missing            |             |                 |               |               | (9)           | (3+16+

|             |                 |               |               | inputMediaTray|  On-Line+

|             |                 |               |               | Missing(801)# |  Transitioning)

|             |                 |               |               |               |

Input Tray         | down (5)    | other (1)       | No Paper(64)  | input(8)      | subunitEmpty  | prtInputStatus

Empty              |             |                 |               |               | (13)          | (3+16+

|             |                 |               |               | inputMediaSup-|  On-Line+

|             |                 |               |               | plyEmpty(808)#|  Transitioning)

|             |                 |               |               |               |

Output Tray        | down (5)    | other (1)       | Service       | output(9)     | subunitMissing| prtOutputStatus

Missing            |             |                 | Requested(1)  |               | (9)           | (3+16+

|             |                 | + Offline (2) |               | outputMedia   |  On-Line+

|             |                 |               |               | TrayMissing   | Transitioning)

|             |                 |               |               | (901)#        |

|             |                 |               |               |               |

Output Tray        | down (5)    | other (1)       | Service       | output(9)     | subunitFull   | prtOutputStatus

Full               |             |                 | Requested(1)  |               | (15)          | (3+16+

|             |                 | + Offline (2) |               | outputMedia   |  On-Line+

|             |                 |               |               | TrayFull(903)#|  Transitioning)

|             |                 |               |               |               |

Marker Supply      | down (5)    | other (1)       | No Toner(16)  | markerSupplies| subunitMissing| prtMarkerStatus

Missing            |             |                 |               | (11)          | (9)           | (3+16+

( toner missing,   |             |                 |               |               | markerToner   |  On-Line+

ink missing )    |             |                 |               |               | Cartridge     |  Transitioning)

|             |                 |               |               | Missing(1115)#|

|             |                 |               |               |               |

Marker Supply      | down (5)    | other (1)       | No Toner(16)  | markerSupplies| subunitEmpty  | prtMarkerStatus

Empty (i.e.,       |             |                 |               | (11)          | (13)          | (3+16+

toner empty,      |             |                 |               |               | markerToner   |  On-Line+

ink low)          |             |                 |               |               | Empty (1101)# |  Transitioning)

|             |                 |               |               | markerInk     |

|             |                 |               |               | Empty (1102)# |

Non-Critical Errors: the printer can still print while condition exists

(the warning may be current while printing)

Input Tray         | warning (3) | idle (3) or     | Low           | input(8)      | subunitAlmost | prtInputStatus

Low                |             | printing (4) or | Paper(128)    |               | Empty(12)     | (8+Availability+

|             | warmup (5)      |               |               |               |  On-Line+

|             |                 |               |               |               |  Transitioning)

|             |                 |               |               |               |

Output Tray        | warning (3) | idle (3) or     | Service       | output(9)     | subunitAlmost | prtOutputStatus

Almost Full        |             | printing (4) or | Requested(1)  |               | Full(14)      | (8+Availability+

|             | warmup (5)      |               |               | outputTray    |  On-Line+

|             |                 |               |               | AlmostFull    |  Transitioning)

|             |                 |               |               | (902)#        |

|             |                 |               |               |               |

Marker Supply      | warning (3) | idle (3) or     | Low           | markerSupplies| subunitAlmost | prtMarkerStatus

Almost Empty       |             | printing (4) or | Toner (32)    | (11)          | Empty(12)     | (8+Availability+

(i.e., toner low,  |             | warmup (5)      |               |               | markerToner   |  On-Line+

ink low)          |             |                 |               |               | AlmostEmpty   |  Transitioning)

|             |                 |               |               | (1104)#       |

|             |                 |               |               | markerInk     |

|             |                 |               |               | AlmostEmpty   |

|             |                 |               |               | (1105)#       |

|             |                 |               |               |               |

Input Tray         | warning (3) | idle (3) or     | Low           | input(8)      | subunitMissing| prtInputStatus

Missing (when n-1  |             | printing (4) or | Paper(128)    |               | (9)           | (8+Availability+

trays are missing  |             | warmup (5)      |               |               | inputMediaTray|  On-Line+

with linking)      |             |                 |               |               | Missing(801)# |  Transitioning)

|             |                 |               |               |               |

Input Tray         | warning (3) | idle (3) or     | Low           | input(8)      | subunitEmpty  | prtInputStatus

Empty (when n-1    |             | printing (4) or | Paper(128)    |               | (13)          | (8+Availability+

trays are empty    |             | warmup (5)      |               |               | inputMediaSup-|  On-Line+

with linking)      |             |                 |               |               | plyEmpty(808)#|  Transitioning)

|             |                 |               |               |               |

Output Tray        | warning (3) | idle (3) or     | Service       | output(9)     | subunitMissing| prtOutputStatus

Missing (when n-1  |             | printing (4) or | Requested(1)  |               | (9)           | (8+Availability+

trays are missing  |             | warmup (5)      |               |               | outputMedia   |  On-Line+

with linking)      |             |                 |               |               | TrayMissing   | Transitioning)

|             |                 |               |               | (901)#        |

|             |                 |               |               |               |

Output Tray        | warning (3) | idle (3) or     | Service       | output(9)     | subunitFull   | prtOutputStatus

Full (when n-1     |             | printing (4) or | Requested(1)  |               | (15)          | (8+Availability+

trays are full     |             | warmup (5)      |               |               | outputMedia   |  On-Line+

with linking)      |             |                 |               |               | TrayFull(903)#|  Transitioning)

|             |                 |               |               |               |

The definition of PrtSubUnitStatusTC specifies that SubUnitStatus is an integer that is the

sum of 5 distinct values, Availability, Critical, Non-Critical, On-line and Transitioning.

These values are...

# The generic codes are recommended for use. These type specific codes should not be used

in newer implementations of the printer MIB.