Printer Management Interface

Management Object Summary

Version 0.3

By

Ronald L. Smith

Texas Instruments

email:	rlsmith@nb.ppd.ti.com
phone:	(817) 774-6151

F.D. Wright

Lexmark International

email:   don@lexmark.com

phone:    606-232-4808

Printer Component

To be supplied by DMTF to correspond to MIB II

DMTF MIF "Component ID: (From DMI Specification - Draft 4.2 - 12 January 1994)

1.	Manufacturer	String64	Name of the organization that produces this component.

2.	Product	String64	Name of this Printer

3.	Version	String64

4.	Serial Number	String64

5.	Installation	Date	Time and Date of the Last Installation

6.	Verify	Integer	Verification Level for this printer

"Asking for the 'Verify' attribute causes the component instrumentation code to perform checks to verify that the component is still in the system and working properly. It should return on of the following values:

Value	Meaning

0	An error occurred; Check status code

1	Component does not exist

2	Verify not supported

4	Component exists, functionally untested

5	Component exists, functionally unknown

6	Component exists, functionally no good

7	Component exists, functionally good

General Printer Group Objects

| prtGeneral Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Status | Read-Only | Octet | PrintStatus | Overall Printer Status |
| ConfigChange | Read-only | Counter32 |  | Counts configuration changes |

Notes:

| prtGeneral Mandatory prtSkinTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | Index of Skin component |
| Description | Read-Only | String |  | Manufacturer provided skin component name |
| Status | Read-Onl;y | Enum | EntStatus |  |

Notes:

| prtGeneral - Extended Optional |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Reset | Write-Only | Integer |  | Writing a 1 simulates a power cycle |
| ResetSpecial | Write-Only | Enum (3) | ResetType | Writing a 1 causes a special reset |

Notes:

Input Group Objects - Mandatory

| prtInput Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer |  | ID of the Default Input Component |

Notes:

| prtInput Mandatory prtInputTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | Value used by the printer to identify associated input device |
| Type | Read-Only | Enum (2) | InputType |  |
| DimUnits | Read-Only | Enum (1) | PhyMediaUnit |  |
| CurDimAcrossFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| CurDimFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| CapacityUnit | Read-Only | Enum (1) | PhyMediaUnit |  |
| MaxCapacity | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| CurrentCapacity | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| Status | Read-Only | Enum | EntStatus | Current Status of Input Device |
| MediaName | Read-Write | String |  | "Hillary's Pink Presidential Letterhead" |
| MediaSize | Read-Write | Enum (1) | MediaSize | ISO, JIS & North American media size identifiers |

Notes:

Input Group Objects - Optional

| prtInput - TypeExtended Optional prtInputTabelAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Name | Read-Writer | String |  | Name assigned to this device |
| VendorName | Read-Only | String |  |  |
| Model | Read-Only | String |  |  |
| Version | Read-Only | String |  |  |
| SerialNumber | Read-Only | String |  |  |
| Description | Read-Only | String |  |  |
| Security | Read-Write | Enum (2) | YesNo-OnOff | Indication if the Device has some security associated with it. Note 1 |

Notes:

1.	Does there need to be an optional object that elaborates on the type of input security available?

| prtInput - DimExtended Optional prtInputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| PrintAreaOffsetAcrossFeed | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| PrintAreaExtentAcross Feed | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| PrintAreaOffset Feed | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| PrintAreaExtent Feed | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |

Note:

| prtInput - MediaExtended Optional prtInputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| MediaDimUnits | Read-Only | Enum (2) | PhyMediaUnits | Units used in the dimensions of media |
| MediaDimAcrossFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| MediaDimFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| MediaWeight | Read-Write | Integer32 | grams/meter2 | -1 = Other, -2 = Unknown |
| MediaType | Read-Write | String |  | ISO10175 (DPA) & ISO10180 (SPDL) Definitions |
| MediaColor | Read-Write | String |  | ISO10175 (DPA) & ISO10180 (SPDL) Definitions |
| MediaFormParts | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |

Notes:

Output Group Objects - Mandatory

| prtOutput Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer |  | ID of Default Output Device |

Notes:

| prtOutput Mandatory prtOutputTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | Value used by the printer to identify associated output device. |
| Type | Read-Only | Enum (2) | OutputType |  |
| CapacityUnit | Read-Only | Enum (1) | PhyMediaUnit |  |
| MaxCapacity | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| CurrentCapacity | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| Status | Read-Only | Enum | EntStatus |  |

Notes:

Output Group Objects - Optional

| prtOutput - TypeExtended Optional prtOutputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Name | Read-Writer | String |  | Name assigned to this Device |
| VendorName | Read-Only | String |  |  |
| Model | Read-Only | String |  |  |
| Version | Read-Only | String |  |  |
| SerialNumber | Read-Only | String |  |  |
| Description | Read-Only | String |  |  |
| Security | Read-Write | Enum (2) | YesNo-OnOff | Note 1, Note 2 |

Notes:

1.	I Propose that Device Security be moved from the Output Type Features subclass to be consistent with Input Device

2.	Does there need to be an optional object that elaborates on the type of output security available?

| prtOutput - DimExtended Optional prtOutputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DimUnit | Read-Only | Enum (1) | PhyMediaUnit |  |
| CurDimAcrossFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| CurDimFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| MaxDimAcrossFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| MaxDimFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| MinDimAcrossFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| MinDimFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |

Notes:

| prtOutput - Features Optional prtOutputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| StackingOrder | Read-Only | Enum (1) | StackingOrder |  |
| Bursting | Read-Write | Enum (1) | YesNo-OnOff |  |
| Decollating | Read-Write | Enum (1) | YesNo-OnOff |  |
| PageCollated | Read-Write | Enum (1) | YesNo-OnOff |  |
| Job Collated | Read-Write | Enum (1) | YesNo-OnOff |  |
| OffsetStacking | Read-Write | Enum (1) | YesNo-OnOff |  |
| Slipsheet | Read-Write | Enum (1) | YesNo-OnOff |  |
| DeviceSecurity | Read-Only | Enum (1) | YesNo-OnOff | Note1, Note 2 |

Notes

1.	I Propose that Device Security be moved to the Output Type Extened subclass to be consistent with Input Device

2.	Does there need to be an optional object that elaborates on the type of output security available?

| prtOutput - Associated Finisher Optional ?? |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Stitching | Read-Only | String |  |  |
| Folding | Read-Only | String |  |  |
| Binding | Read-Only | String |  |  |
| Punching | Read-Only | String |  |  |
| Trimming | Read-Only | String |  |  |
| Insertion | Read-Only | String |  |  |

Notes:

Finisher Group Objects - Optional

| prtFinisher Optional |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer32 |  | 0 = Finisher Not Used in Default Configuration |

Notes:

| prtFinisher Optional prtFinisherTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | String |  | Note 1 Value used by the printer to identify associated finisher device. |
| Type | Read-Only | String |  | Note 2 |
| NamedFinishing | Read-Only | String |  | Note 3 |
| ConsumableUnit | Read-Only | String |  | Note 4 |
| MaxConsumableCapacity | Read-Write | Integer32 |  | Note 4 -1 = Other, -2 = Unknown |
| CurrentConsumableCapacity | Read-Write | Integer32 |  | Note 4 -1 = Other, -2 = Unknown |
| Status | Read-Only | Enum | EntStatus | Current Status of Device |

Notes: Above definition came from Printer Management Objects

1.	The ID object being a string is inconsistent with the rest of the objects. The ID should be an integer.

2.	The Type object being a string is inconsistent with the rest of the objects. The Type should be an enumeration with the enumeration's being of the value defined in FinisherType Enumeration Table.

3.	The NamedFinishing object being a string is inconsistent with other like objects. Should it be an Enumeration or a string?

4.	This approach limits each finisher to one "Consumable" and no provision has not been made for "receptacles" or "containers" that contain waste materials such as produced by a hole punch. Also it is possible that a particular finisher may have "supplies" and "waste".

Marker Group Objects - Mandatory

| prtMarker Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer32 |  | ID of Default Marker |

Notes:

| prtMarker Mandatory prtMarkerTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | Value used by the printer to identify associated marker device. |
| MarkTech | Read-Only | Enum (1) | MarkingTech | Type of Marking Technology used for this making device |
| CounterUnits | Read-Only | Enum (1) | CounterUnits | Units that will be used to report counter values |
| LifeCount | Read-Only | Counter32 |  | Life of printer counter in CounterUnits |
| PowerOnCount | Read-Only | Counter32 |  | Number of units counted since power on in CounterUnits |
| ProcessColorant | Read-Only | Integer32 |  | Number of Process Colors (-1 = Other, -2 = Unknown) |
| SpotColorant | Read-Only | Integer32 |  | Number of Spot Colors (-1 = Other, -2 = Unknown) |
| AddressabilityUnit | Read-Only | Enum (1) | AddrUnits | Resolution Unit |
| AddressabilityFeedDir | Read-Only | Integer32 |  | Feed Resolution (-1 = Other, -2 = Unknown) |
| AddressabilityCrossFeedDir | Read-Only | Integer32 |  | CrossFeed Resolution (-1 = Other, -2 = Unknown) |
| Status | Read-Only | Enum | EntStatus | Current Status of Marker Component |

Notes:

| prtMarkerSupplies Mandatory prtMarkerSuppliesTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| MarkerID | Read-Only | Integer32 |  | ID of this supply entity |
| ColorantID | Read-Only | Integer32 |  | Associate Colorant ID (0 if none) |
| Class | Read-Only | Enum (1) | MkrSupClass | Class (Supply or receptacle) |
| Type | Read-Only | Enum(3) | MkrSupType | Type of supply |
| Description | Read-Only | String |  | Description of the supply or receptacle |
| SupplyUnit | Read-Only | Enum(3) | MkrSupUnit | Units of the marker supply container/receptacle |
| MaxCapacity | Read-Only | Integer32 |  | Maximum capacity expressed in SupplyUnit |
| CurrentLevel | Read-Only | Integer32 |  | Current level expressed in SupplyUnit |

Notes:

| prtMarkerColorant Optional prtMarkerColorantTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ColorantID | Read-Only | Integer32 |  | ID of this colorant |
| MarkerID | Read-Only | Integer32 |  | ID of Marker that this colorant entry is associated with. |
| Role | Read-Only | Enum (1) | ColoranRole | Role played by this colorant |
| Order | Read-Only | Integer |  | Ordinal number of this colorant in the order in which colorants are imaged onto the medium within its role. |
| Value | Read-Only | String | ColorantValue | Name of the color. |
| Tonality | Read-Only | Integer32 |  | Number of levels of tonality. Must be at least 2. |

Notes:

Media Path Group Objects - Mandatory

| prtMediaPath Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer |  | ID number of the default Media Path |
| MaxSpeedPrintUnit | Read-Only | Enum (1) | SpeedPrintUnit | Numerator units of print speed |
| MaxSpeedTimeUnit | Read-Only | Enum (1) | SpeedTimeUnit | Denominator units of print speed |
| PhyMediaUnit | Read-Only | Enum (1) | PhyMediaUnit | Units of media size |

Notes:

| prtMediaPath Mandatory prtMediaPathTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | Value used by printer to identify Medai Path |
| MaxSpeed | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| MaxMediaFeedDir | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| MaxMediaAcrossFeedDir | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| MinMediaFeedDir | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| MinMediaAcrossFeedDir | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| MediaPathType | Read-Only | Enum() | MediaPathType |  |
| MediaPathDescription | Read-Only | String |  | Manufacturer provided description of this Media Path |
| Status | Read-Only | Enum | EntStatus | Status of this Media Path |

Notes:

Media Path Device Group Objects - Optional   (DELETED)

| prtMediaPathDevice Optional |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |

Notes:

| prtMediaPathDevice Optional prtMediaPathDeviceTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |

Notes:

Channel Group Objects

| prtChannel Mandatory prtChannelTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ChannelIndex | Read-Only | Integer32 |  | Index of channels |
| CurrentJobCntlLang | R/W | Integer |  | Pointer to Interpreter Table |
| DefaultPageDescLang | R/W | Integer |  | Pointer to Interpreter Table |
| ChannelActive | R/W | Enum(1) |  | State of the Channel |
| ChannelType | Read-Only | Enum(2) |  | Type of Channel |

Notes:

Operator Console Group Objects

| prtDisplayConsole |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Localization Index | R/W | Integer32 |  | Index into localization table |
| NumberOf DisplayLines | Read-Only | Integer32 |  | Number of lines on the display |
| NumberOfDisplayChars | Read-Only | Ineger32 |  | Number of Characters per Lines |

| prtOperatorConsole Mandatory prtConsoleDisplayTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| LineIndex | Read-Only | Integer32 |  | Index into display |
| LineText | R/W | String |  | Text on or to be written to the display |

| prtOperatorConsole Mandatory prtConsoleLightsTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| LightID | Read-Only | Integer32 |  | Index of lights |
| BlinkOn | Read-Only | Integer32 |  | On time in milliseconds of blinking (0=on always) |
| BlinkOff | Read-Only | Integer32 |  | Off time in milliseconds of blinking (0=off always) |
| OffColor | Read-Only | Enum(?) | Light Color | Color of the light when off |
| OnColor | Read-Only | Enum(?) | Light Color | Color of the light when on |
| Description | Read-Only | String |  | Vendor Description of what the light does |

Interpreter Group Objects - Mandatory

| prtInterpreter Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer | IntpLangFam |  |

Notes:

| prtInterpreter Mandatory prtInterpreterTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| LangFamily | Read-Only | Integer | IntpLangFam |  |
| LangLevel | Read-Only | String |  |  |
| LangVersion | Read-Only | String |  |  |
| Description | Read-Only | String |  |  |
| Version | Read-Only | String |  |  |
| FeedAddressability | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| CrossFeedAddressability | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| DefaultOrientation | Read-Write | Enum (2) | MediaOrientation |  |

Notes:

Alerts Group Object - Mandatory

| prtAlert Mandatory prtAlertTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Index | Read-Only | Integer32 |  |  |
| SeverityLevel | Read-Only | Enum ( ) |  |  |
| InfoClass | Read-Only | Enum ( ) | Inform Class |  |
| Group | Read-Only | Enum ( ) |  |  |
| Group Index | Read-Only | Integer |  |  |
| Location | Read-Only | Integer |  |  |
| Alert Code | Read-Only | Enum ( ) |  |  |
| Description | Read-Only | String |  |  |

Notes:

Group Status

| Printer Status Bit Meaning 0 Non-critical State exists 1 Critical State exists 2 On Request 3 Unavailable 4 Busy / Temporarily Unavailable 5 Line Status Bit 0 6 Line Status Bit 1 Bit 6 Bit 7 Meaning 0 0 On Line 0 1 Pending On Line 1 0 Pending Off Line 1 1 Off Line 7 Not Used | EntStatus 1 Other 2 Unknown 3 Ready/Available 4 Critical State 5 Non-critical state 6 Both Critical and Non-critical state 7 On-request 8 Unavailable 9 Busy/Temp Unavailable |
| --- | --- |

PMI Enumerations - Units

| Addressabililty Unit 1 Other 2 Unknown 3 Inches 4 Centimeters | Physical Media Unit 1 Other 2 Unknown 3 Inches 4 Millimeters 5 Sheets | Counter Unit 1 Other 2 Unknown 3 Characters 4 Lines 5 Impressions 6 Inches 7 Feet 8 Millimeters 9 Sheets 10 Seconds 11 Minutes 12 Hours |
| --- | --- | --- |
| Speed Print Unit 1 Other 2 Unknown 3 Characters 4 Lines 5 Impressions 6 Inches 7 Feet 8 Millimeters 9 Meters 10 Sheets | Speed Time Unit 1 Other 2 Unknown 3 Seconds 4 Minutes 5 Hours | Finisher Consumable Unit 1 Other 2 Unknown |
| Marker Supply Units 1 Other 2 Unknown 3 Ounces 4 Grams 5 Fluid Ounce 6 Milliliter 7 Sheets 8 Inches 9 Millimeters |  |  |

PMI Enumerations - Device Types

| Input Type 1 Other 2 Unknown 3 Sheet-feed-auto-removable-tray 4 sheet-feed-auto-non removable tray 5 Sheet Feed Manual 6 continuous-roll 7 continuous-fan-fold | Output Type 1 Other 2 Unknown 3 Removable Bin 4 UnRemovable Bin 5 Continuous Roll Device 6 Mail Box | Media Path Type 1 Other 2 Unknown 3 Long Edge Binding Duplex 4 Short Edge Binding Duplex |
| --- | --- | --- |
| Finisher Type 1 Other 2 Unknown 3 Stitching 4 Binding 5 Trimming 6 Die-cutting 7 Punching 8 Perforating 9 Slitting 10 Inserting 11 Covers 12 Folding | Channel Type 1. Other 2. Unknown 3. Serial Port 4. Parallel Port 5. IEEE1284 Port 6. SCSI Port 7. Apple PAP 8. LPD Server 9. Novell RPrinter 10. Novell Pserver 11. port 9100 12. FTP/TFTP 13. Adobe AppSocket (9101) 14. DLC/LLC Port 15. Coax 16. Twinax 17. Fax 18. IEEE 1394 | Interpreter Language Family 1 Other 2 Unknown 3 PCL 4 HPGL 5 PJL 6 PostScript 7 PPDS 8 EscapeP 9 Epson 10 DDIF 11 Interpress 12 ISO6429 13 LinePrinter 14 MODCA 15 ReGIS 16 SCS 17 SPDL 18 Tek4014 19 PDS 20 IGP 21 Code-V 22 DSC-DSE 23 WPS 24 LN03 25 CCITT 26 QUIC |
| Marking Technology 1 Other 2 Unknown 3 Electophotographic:LED 4 Electrophotographic:Laser 5 Electrophotographic:Other 6 Impact:Moving_Head:Dot_Matrix:9pin 7 Impact:Moving_Head:Dot_Matrix:24pin 8 Impact:Moving_Head:Dot_Matrix:other 9 Impact:Moving_Head:Fully_Formed 10 Impact:band 11 Impact:other 12 Inkjet:Aqueous 13 Inkjet:Solid 14 Inkjet:Other 15 Pen 16 Thermal:Transfer 17 Thermal:Sensitive 18 Thermal:Diffusion 19 Thermal:Other 20 Electroerosion 21 Electrostatic 22 Photographic:Microfiche 23 Photographic:Imagesetter 24 Photographic:Other 25 Ion-deposition 26 E-beam 27 Typesetter | Marker Supply Type 1 Other 2 Unknown 3 Toner 4 Waste Toner 5 Ink 6 Ink Cartridge 7 Ink Ribbon 8 Waste Ink 9 OPC (Organic Photo Contuctor) 10 Developer 11 Fuser Oil | Marker Supply Class 1 Other 2 Unknown 3 Supply that is consumed 4 Receptacle that is filled |

PMI Enumerations - Misc

| Media Orientation 1 Other 2 Unknown 3 Portrait 4 Reverse Portrait 5 Landscape 6 Reverse Landscape | Media Color 1 Other 2 Unknown 3 White 4 Pink 5 Yellow 6 Buff 7 Goldenrod 8 Blue 9 Green 10 Transparent | Line Status 1 Other 2 Unknown 3 On Line 4 Pending On Line 5 Off Line 6 Pending Off Line |
| --- | --- | --- |
| Named Finshing 1 Other 2 Unknown 3 Staple 4 Staple Top Left 5 Staple Bottom Left 6 Stable Top Right 7 Staple Bottom Right 8 Saddle Stitch 9 Edge Stitch 10 Punch 11 Cover 12 Bind | ColorantValue - String ASCII representation of the integer asociated with the folloing names. 1. Other 2. Unknown 3. White 4. Red 5. Green 6. Blue 7. Cyan 8. Magenta 9. Yellow 10. Black Additional names using ISO9070 naming convention. | Light Color 1. Other 2. Unknown 3. White 4. Red 5. Green 6. Blue 7. Cyan 8. Magenta 9. Yellow 10. Black |
| YesNo -OnOff 1. Other 2. Unknown 3. Yes - On 4. No - Off | ResetType 1. Other 2. Unknown 3. Reset NVRAM 4. Reset to Factory Defaults | ColorantRole 1. Other 2. Unknown 3. Process 4. Spot |
| Inform Class 1 Other 2 Unknown 3 Untrained 4 Trained 5 Field Serve 6 Management | Channel Active 1. Other 2. Unknown 3. Only Print Data Accepted 4. Only Control Data Accepted 5. Print and Control Data Accepted 6. No Data Accepted | StackingOrder 1. Other 2. Unknown 3. Face-Up 4. Face-Down |

PMI Enumerations - Misc

Media Size 1 other 2 unknown 3 na-letter -- North American letter size: 8.5 by 11 inches 4 na-legal -- North American legal size: 8.5 by 14 inches 5 na-10x13-envelope -- North American 10x13 size: 10 by 13 inches 6 na-9x12-envelope -- North American 9x12 size: 9 by 12 inches 7 na-number-10-envelope -- North American number 10 business envelope 8 na-7x9-envelope -- North American 7x9 size: 7 by 9 inches 9 na-9x11-envelope -- North American 9x11 size: 9 by 11 inches 10 na-10x14-envelope -- North American 10x14 size: 10 by 14 inches 11 na-number-9-envelope -- North American number 9 business envelope 12 na-6x9-envelope -- North American 6x9 size: 6 by 9 inches 13 na-10x15-envelope -- North American 10x15 size: 10 by 15 inches 14 a -- engineering A size 15 b -- engineering B size 16 c -- engineering C size 17 d -- engineering D size 18 e -- engineering E size 23 iso-a0 -- ISO A0 size: 841 mm by 1189 mm 24 iso-a1 -- ISO A1 size: 594 mm by 841 mm 25 iso-a2 -- ISO A2 size: 420 mm by 594 mm 26 iso-a3 -- ISO A3 size: 297 mm by 420 mm 27 iso-a4 -- ISO A4 size: 210 mm by 297 mm 28 iso-a5 -- ISO A5 size: 148 mm by 210 mm 29 iso-a6 -- ISO A6 size: 105 mm by 148 mm 30 iso-a7 -- ISO A7 size: 74 mm by 105 mm 31 iso-a8 -- ISO A8 size: 52 mm by 74 mm 32 iso-a9 -- ISO A9 size: 37 mm by 52 mm 33 iso-a10 -- ISO A10 size: 26 mm by 37 mm 43 iso-b0 -- ISO B0 size: 1000 mm by 1414 mm 44 iso-b1 -- ISO B1 size: 707 mm by 1000 mm 45 iso-b2 -- ISO B2 size: 500 mm by 707 mm 46 iso-b3 -- ISO B3 size: 353 mm by 500 mm 47 iso-b4 -- ISO B4 size: 250 mm by 353 mm 48 iso-b5 -- ISO B5 size: 176 mm by 250 mm 49 iso-b6 -- ISO B6 size: 125 mm by 176 mm 50 iso-b7 -- ISO B7 size: 88 mm by 125 mm 51 iso-b8 -- ISO B8 size: 62 mm by 88 mm 52 iso-b9 -- ISO B9 size: 44 mm by 62 mm 53 iso-b10 -- ISO B10 size: 31 mm by 44 mm 66 iso-c3 -- ISO C3 size: 229 mm by 324 mm 67 iso-c4 -- ISO C4 size: 229 mm by 324 mm 68 iso-c5 -- ISO C5 size: 162 mm by 229 mm 69 iso-c6 -- ISO C6 size: 162 mm by 229 mm 70 iso-designated -- ISO Designated Long size: 110 mm by 220 mm 83 jis-b0 -- JIS B0 size 84 jis-b1 -- JIS B1 size 85 jis-b2 -- JIS B2 size 86 jis-b3 -- JIS B3 size 87 jis-b4 -- JIS B4 size 88 jis-b5 -- JIS B5 size 89 jis-b6 -- JIS B6 size 90 jis-b7 -- JIS B7 size 91 jis-b8 -- JIS B8 size 92 jis-b9 -- JIS B9 size 93 jis-b10 -- JIS B10 size

PMI Enumerations - Alert Codes

| Severity Level Alert Code 1 Other 2 Unknown 3 Critical 4 Warning | Alert Class Code 1 Other 2 Unknown 3 Management 4 Technical 5 Operator | Alert Group Code 1 Other 2 Unknown 3 Input 4 Output 5 Finisher 6 Media Path 7 General Printer 8 Marker 9 System Controller 10 Interface 11 Interpreter 12 Operator Console |
| --- | --- | --- |
| Input Alert Code 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam 9 Media Tray Missing 10 Media Size Change 11 Media Weight Change 12 Media Type Change 13 Media Color Change 14 Media Form Parts Change 15 Media Supply Low 16 Media Supply Empty | Output Alert Code 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam 9 Media Tray Missing 10 Media Tray Almost Full 11 Media Tray Full | Marker Alert Code 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam 9 Toner Empty 10 Ink Empty 11 Print Ribbon Empty 12 Toner Almost Empty 13 Ink Almost Empty 14 Print Ribbon Almost Empty 15 Waste Toner Receptical Almost Full 16 Waste Ink Receptical Almost Full 17 Waste Toner Receptical Full 18 Waste Ink Receptical Full 19 Fuser Under Temperature 20 Fuser Over Temperature 21 OPC LIfe Almost Over 22 OPC Life Over 23 Developer Almost Empty 24 Developer Empty |
| General Printer Alert Code 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Door Open 8 Door Closed | Media Path Device Alert Code 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam 9 Media Tray Missing 10 Media Tray Almost Full 11 Media Tray Full | Finisher Alert Code 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam 9 Media Tray Missing 10 Supply Empty 11 Supply Low 12 Receptical Almost Full 13 Receptical Full |
| Interpreter Alert Code 1 Other 2 Unknown 3 Configuration Change 4 Memory Increase 5 Memory Decrease 6 Cartridge Added 7 Cartridge Deleted 8 Resource Added 9 Resource Deleted 10 Resource Unavailable | Interface Alert Code 1 Other 2 Unknown 3 Configuration Change |  |