//////////////////////////////////////////////////////////////////////////////

//                                                                          //

//                 DMTF PRINTER MIF DEFINITION V.8.2                        //

//                                                                          //

//////////////////////////////////////////////////////////////////////////////

//                                                                          //

// This document specifies the mapping of semantic definitions for common   //

// class objects defined by the IETF-DMTF Printer Working Group to the      //

// concrete syntax of the Management Information Format language supported  //

// by the DMI Desktop Management Interface.                                 //

//                                                                          //

// This specification is based on the "Printer Management Interface Common  //

// MIB/MIF Specification, Version 0.4" dated August 14, 1994 which is a     //

// working draft in progress, the "Desktop Management Interface             //

// Specification Version 1.0", dated April 29, 1994 which is a published    //

// specification produced by the Desktop Management Task Force, and the     //

// IETF Printer MIB Working Group Printer MIB dated Nov. 25, 1994.          //

//                                                                          //

// Questions or comments on this draft MIF specification should be made to  //

// Harry Lewis, IBM Printing Systems, via the PMI mailing list.             //

//////////////////////////////////////////////////////////////////////////////

//                                                                          //

//                   EXPLANATION OF ENUM REGISTRATION                       //

//                                                                          //

//////////////////////////////////////////////////////////////////////////////

//Enumerations (enums) are sets of symbolic values defined for use with     //

//one or more objects.  Some common enumeration sets are assigned a         //

//symbolic data type name (textual convention).  These enumerations are     //

//listed at the beginning of this specification.                            //

//                                                                          //

//                  Registering Additional Enumerated Values                //

//                                                                          //

//This working group has defined several type of enumerations which differ  //

//in the method employed to control the addition of new enumerations.       //

//Throughout this document, references to "enumeration (n)", where n can    //

//be 1, 2 or 3 are found in the various tables. The definition of these     //

//enumerations classes follows:                                             //

//                                                                          //

//enumeration (1)  All the values are defined in the Printer MIF.           //

//  Additional enumerated values require a new RFC.                         //

//                                                                          //

//enumeration (2)  An initial set of values are defined in the Printer      //

//  MIF.  Additional enumerated values are registered following review by   //

//  the Printer Working Group. Send requests to pmi@hpbs987.boi.hp.com.     //

//                                                                          //

//enumeration (3)  Same as enum (2) for the Printer MIF. Enumeration (3)    //

//  exists due to a dual registration process in the SNMP MIB.              //

//////////////////////////////////////////////////////////////////////////////

//

start component

name        = "Your Printer Component Name"

description = "Your Component Description goes here"

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                      GLOBAL ENUMERATIONS                            //

//                                                                     //

// Enumerations are MIF language block definitions that allow strings  //

// to be associated with signed 32-bit integers. They are primarily    //

// used by component instrumentation to pass integers through the DMI  //

// so that management applications can display the corresponding text  //

// string in the users native language. This section defines Named     //

// enumerations specified at the component level. These definitions    //

// are global to the component and may be referenced by name in        //

// attribute definitions that use them.                                //

/////////////////////////////////////////////////////////////////////////

START ENUM

name = "AddrUnit"     //Enumeration type 1

type = INTEGER

1 = "Reserved"

2 = "Reserved"

3 = "Ten Thousandths of Inches"

4 = "Micrometers"

END ENUM

START ENUM

name = "MediaUnit"     //Enumeration type 1

type = INTEGER

1 = "Reserved"

2 = "Reserved"

3 = "Ten Thousandths of Inches"

4 = "Micrometers"

8 = "Sheets"

END ENUM

START ENUM

name = "CapacityUnit"   //Enumeration type 1

type = INTEGER

1 = "Reserved"

2 = "Reserved"

3 = "Ten Thousandths of Inches"

4 = "Micrometers"

8 = "Sheets"

16 = "Feet"

17 = "Meters"

END ENUM

START ENUM

name = "PresentOnOff"  //Enumeration type 1

type = INTEGER

1 = "Other"

2 = "Reserved"

3 = "On"

4 = "Off"

5 = "Not Present"

END ENUM

START ENUM

name = "CodedCharSet"  //Enumeration type 3

type = INTEGER

1 = "Other"

2 = "Reserved"

//STANDARD CHARACTER SETS

//

//Standard coded character sets (in the range 3-999) that do not have

//subset implementations

3 = "ASCII"

//ASCII

//ISO 646-1991 IRV with the dollar sign as the

//currency symbol. Note that this set is 7-bit ASCII

//with either the 8th bit omitted, or zero; this

//set is not 8-bit ASCII which is the same as ISO

//8859-1(ISO Latin-1)

4 = "ISOLatin1"

//Latin Alphabet No. 1

//8-bit Latin for Western Europe and Western

//Hemisphere. See ISO 8859-1

5 = "ISOLatin2"

//ISO Latin Alphabet No. 2

//8-bit Latin for Eastern Europe See ISO 8859-2

6 = "ISOLatin3"

//ISO Latin Alphabet No. 3

//8-bit Latin for Southern Europe and Western

//Hemisphere. See ISO 8859-3

7 = "ISOLatin4"

//ISO Latin Alphabet No. 4

//8-bit Latin for Northern Europe See ISO 8859-4

8 = "ISOLatinCyrillic"

//ISO Latin/Cyrillic Alphabet

//8-bit ASCII plus Cyrillic. See ISO 8859-5

9 = "ISOLatinArabic"

//ISO Latin/Arabic Alphabet

//8-bit ASCII plus Arabic. See ISO 8859-6

10 = "ISOLatinGreek"

//ISO Latin/Greek Alphabet

//8-bit ASCII plus Greek. See ISO 8859-7

11 = "ISOLatinHebrew"

//ISO Latin/Hebrew Alphabet

//8-bit ASCII plus Hebrew. See ISO 8859-8

12 = "ISOLatin5"

//ISO Latin Alphabet No. 5

//8-bit Latin for EU Member countries. See ISO

//8859-9

13 = "ISOLatin6"

//ISO Latin Alphabet No. 6

// 8-bit Latin for Scandinavian countries better

// than Latin4 for those countries. See ISO 8859-10

14 = "ISOTextComm"

//ISO Latin 8-bit Text Communication

//Used in ISO ASN.1 T61String. See ISO 6937

15 = "JISX0201"

//Half Width Katakana

//One byte only, this is equivalent to JIS/Roman

//(similar to ASCII) plus eight-bit half-width

//Katakana. See JIS X 0201-1976

16 = "JISEncoding"

//JIS Encoding

//Uses ISO 2022 escape sequences to shift code

//sets as documented in JIS X 0202-1991

//PCL Symbol Set id: 17K

17 = "ShiftJIS"

//Shift JIS

//Also known as MS Kanji. A Microsoft code that

//extends csHalfWidthKatakana(15) to include Kanji

//by adding a second byte when the value of the

//first byte is in the ranges 81-9F or

//E0-EF.

18 = "EUCPkdFmtJapanese"

//Extended UNIX Code Packed Format for Japanese

//Standardized by OSF, UNIX International, and

//UNIX Systems Laboratories Pacific.

//Uses ISO 2022 rules to select

//code set 0: JIS Roman (a single 7-bit byte set)

//code set 1: JIS X0208-1990 (a double 8-bit byte

//set) restricted to A0-FF in both bytes

//code set 2: Half Width Katakana (a single 7-bit

//byte set) requiring SS2 as the character prefix

//code set 3: JIS X0212-1990 (a double 7-bit byte

//set) restricted to A0-FF in both bytes

//requiring SS3 as the character prefix

19 = "EUCFixWidJapanese"

//Extended UNIX Code Fixed Width for Japanese

//Used in Japan.

//Each character is 2 octets.

//code set 0: JIS Roman (a single 7-bit byte set)

// 1st byte = 00

// 2nd byte = 20-7E

//code set 1: JIS X 0208-1990 (a double 7-bit byte

//set) restricted to A0-FF in both bytes

//code set 2: Half Width Katakana (a single 7-bit

//byte set)

// 1st byte = 00

// 2nd byte = A0-FF

//code set 3: JIS X0212-1990 (a double 7-bit byte

//set) restricted to A0-FF in the first byte

//and 21-7E in the second byte

20 = "ISO4UnitedKingdom"

//United Kingdom national std set

//See registration 4 -  ISO 2375.

//Registration Authority: ECMA

21 = "ISO11SwedishforNames"

//Swedish for Names national std set

//See registration 11 - ISO 2375

//Registration Authority: ECMA

22 = "ISO15Italian"

//Italian national set

//See registration 15 - ISO 2375

//Registration Authority: ECMA

23 = "ISO17Spanish"

//Spanish national set

//See registration 17 - ISO 2375

//Registration Authority: ECMA

24 = "ISO21German"

//German national std set

//See registration 21 - ISO 2375

//Registration Authority: ECMA

25 = "ISO60DanishNorwegian"

//Danish Norwegian national std set

//See registration 60 - ISO 2375

// Registration Authority: ECMA

26 = "ISO69French"

//French - see ISO Reg No 69

//See registration 69 - ISO 2375

//Registration Authority: ECMA.

//UNICODE CHARACTER SETS

//Unicode and vendor-supplied combinations of ISO collections (in the

//range 1000-1999):

//NOTE: Unicode and ISO 10646 character coded data may be processed and

//stored in either Big Endian (most significant octet first) or Little

//Endian (least significant octet first) order.  Intel x86, VAX, and

//Alpha/AXP architectures are examples of Little Endian processor

//architectures. Furthermore, in environments where either order may

//occur, so-called Unicode BYTE ORDER MARK (BOM) character (which is ISO

//10646 ZERO WIDTH NO BREAK SPACE), coded as FEFF in two octets and

//0000FEFF in four octets is used at he beginning of the data as a

//signature to indicate the order of the following data (See ISO 10646

//Annex F).  Thus either ordering and BOM may occur in print data streams

//sent to the interpreter.

//However, ISO 8824/8825 (ASN.1/BER) used by SNMP is quite clear that Big

//Endian order shall be used and BOM shall NOT be used in transmission in

//the protocol. Transmitting Unicode in Big Endian order in SNMP should

//not prove to be a hardship for Little Endian machines, since SNMP

//ASN.1/BER requires integers to be transmitted in Big Endian order as

//well. So SNMP implementations on Little Endian machines are already

//reversing the order of integers to make them Big Endian for

//transmission via SNMP. Also Unicode characters are usually treated as

//two-octet integers, not short text strings, so that it will be

//straightforward for Little Endian machines to reverse the order of

//Unicode character octets as well before transmitting them and after

//receiving them via the SNMP protocol.

1000 = "Unicode"

//Unicode

//Universal Coded Character set.

//Any character. See ISO 10646

//two-octet representation,

//implementation level 3

1001 = "UCS4"

//Four-octet Universal Coded Character Set

//Any character. See ISO 10646

//four-octet representation,

//implementation level 3

1002 = "UnicodeASCII"

//ASCII subset of Unicode

//Basic Latin = collection 1

//See ISO 10646, Appendix A

1003 = "UnicodeLatin1"

//ISO Latin-1 subset of Unicode

//Basic Latin and Latin-1 Supplement

//collections 1 and 2

//See ISO 10646, Appendix A

1004 = "UnicodeIBM2039"

//IBM Latin-1 SAA Core Coded Character Set Extended

//ISO 8859-1

//Presentation Set, GCSGID:2039

1005 = "UnicodeIBM1261"

//IBM Latin-2, -3, -5, Extended

//Presentation Set, GCSGID:1261

1006 = "UnicodeIBM1268"

//IBM Latin-4 Extended

//Presentation Set, GCSGID:1268

1007 = "UnicodeIBM1276"

//IBM Cyrillic Greek Extended

//Presentation Set, GCSGID:1276

1008 = "UnicodeIBM1264"

//IBM Arabic

//Presentation Set, GCSGID:1264

1009 = "UnicodeIBM1265"

//IBM Hebrew

//Presentation Set, GCSGID: 1265

//VENDOR CHARACTER SETS

//Vendor developed coded character sets (in the range 2000-xxx)

2000 = "Windows30Latin1"

//Windows Latin-1

//Extended ISO 8859-1 for Windows 3.0

//PCL Symbol Set id:9U

2001 = "Windows31Latin1"

//Windows Latin-1

//Extended ISO 8859-1 for Windows 3.1

//PCL Symbol Set id:19U

2002 = "Windows31Latin2"

//Windows Latin-2

//Extended ISO 8859-2 for Windows 3.1

//PCL Symbol Set id:9E

2003 = "Windows31Latin5"

//Windows Latin-5

//Extended ISO 8859-5 for Windows 3.1

//PCL Symbol Set id:5T

2004 = "HPRoman8"

//HP 8-bit Latin set, ASCII in LH

//PCL Symbol Set id:8U

2005 = "AdobeStandardEncoding"

//Adobe Standard Encoding

//PCL Symbol Set id: 10J

2006 = "VenturaUS"

//Ventura US

//ASCII plus characters typically used in

//publishing, like pilcrow, copyright, registered,

//trade mark, section, dagger, and double dagger

//in the range A0 (hex) to FF (hex).

//PCL Symbol Set id:14J

2007 = "VenturaInternational"

//Ventura International

//ASCII plus coded characters similar to Roman8

//PCL Symbol Set id:13J

2008 = "DECMCS"

//DEC Multinational

//Similar to ISO 8859-1

//Digital Equipment Corp

2009 = "PC850Multilingual"

//PC Multilingual code page 850

//ASCII in the LH

//Other Latin characters in the RH

//PCL Symbol Set id: 12U

2010 = "PCp852"

//PC Latin-2 code page 852

//PCL Symbol Set id: 17U

2011 = "PC8CodePage437"

//PC Code page 437

//US PC code page

//PCL Symbol Set id:10U

2012 = "PC8DanishNorwegian"

//PC Danish Norwegian

//8-bit PC set for Danish Norwegian

//PCL Symbol Set id:11U

2013 = "PC862LatinHebrew"

//PC Latin Hebrew code page 862

//PCL Symbol Set id:15H

2014 = "PC8Turkish"

//PC Latin Turkish

//PCL Symbol Set id:9T

2015 = "IBMSymbols"

//IBM Symbols

//Presentation Set, CPGID:259

2016 = "IBMThai"

//IBM Thai

//Presentation Set, CPGID:838

2017 = "HPLegal"

//HP Legal

//PCL Symbol Set id:1U

2018 = "HPPIFont"

//HP PI font

//PCL Symbol Set id:15U

2019 = "HPMath8"

//HP Mathematics set

//HP 8-bit math set

//PCL Symbol Set id:8M

2020 = "HPPSMath"

//HP PostScript Math

//Similar to AdobeSymbolEncoding

//PCL Symbol Set id:5M

2021 = "HPDesktop"

//HP Desktop

//PCL Symbol Set id:7J

2022 = "VenturaMath"

//Ventura Math

//PCL Symbol Set id:6M

2023 = "MicrosoftPublishing"

//Microsoft Publishing

//PCL Symbol Set id:6J

2024 = "Windows31J"

//Windows Japanese

//A further extension of csShiftJIS to include

//several OEM-specific Kanji extensions. Like

//csShiftJIS, it adds a second byte when the value

//of the first byte is in the ranges 81-9F or

//E0-EF.

//PCL Symbol Set id:19K

2025 = "GB2312"

//Chinese for People's Republic of China (PRC)

//mixed one byte, two byte set:

//20-7E = one byte ASCII

//A1-FE = two byte PRC Kanji

//See GB 2312-80

//PCL Symbol Set Id:18C

2026 = "Big5"

//Chinese for Taiwan

// Multi-byte set.

// PCL Symbol Set Id:18T

END ENUM

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                  COMPONENT ID GROUP -Required                       //

//                                                                     //

// Every MIF file must contain a Component ID group with an ID = 1.    //

// This group offers base-level identification of the component and    //

// represents the minimum amount of information that a component       //

// provider should provide when meaningful. An attribute that is not   //

// supported or that has no meaning for a given component should be    //

// given the key word UNSUPPORTED as its value.                        //

/////////////////////////////////////////////////////////////////////////

start group

name    = "ComponentId"

id      = 1

class   = "DMTF|ComponentID|001"

description = "This group defines attributes common to all components. "

"The group is required in all DMTF components and must "

"have Group ID = 1. "

start attribute

name    = "Manufacturer"

id      = 1

description = "The organization or manufacturer that produced this "

"component."

access  = READ-ONLY

storage = COMMON

type    = STRING(64)

value   = ""

end attribute

start attribute

name    = "Product"

id      = 2

description = "The name of this component or product."

access  = READ-ONLY

storage = COMMON

type    = STRING(64)

value   = ""

end attribute

start attribute

name    = "Version"

id      = 3

description = "The version string for this component."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(64)

value   = ""

end attribute

start attribute

name    = "Serial Number"

id      = 4

description = "The serial number for this component."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(64)

value   = ""

end attribute

start attribute

name    = "Installation"

id      = 5

description = "The time and date of the last install of this "

"component on this system."

access  = READ-ONLY

storage = SPECIFIC

type    = DATE

value   = ""  // yyyymmddHHMMSS.uuuuuu+000 (Supplied by DMI SL)

end attribute

start attribute

name    = "Verify"

id      = 6

description = "Verification level for this printer."

access  = READ-ONLY

storage = COMMON

type    = START ENUM

name = "VerifyType"  //Enumeration type 1

//Note: MIB enum = ptrVerifyType

type = INTEGER

0 = "an error occurred; check status code"

1 = "component does not exist"

3 = "reserved"

4 = "component exists; functionality untested"

5 = "component exists; functionality unknown"

6 = "component exists; functionality no good"

7 = "component exists; functionality good"

END ENUM

value  = 7

end attribute

end group

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                  PRINTER INTERFACES TABLE -required                 //

//                                                                     //

// This table attempts to mimic the attributes in the Interfaces Table //

// of SNMP MIB-II, for use in the Printer MIF.                         //

//                                                                     //

// Compatibility with SNMP is favored even where printer related       //

// information could have been better optimized in the MIF.            //

//                                                                     //

// Note that ENUMs may not follow the standard DMTF convention for     //

// 0=Other and 1=Unknown. This is because SNMP had previously defined  //

// these enums as Integers and assigned the values.                    //

//                                                                     //

/////////////////////////////////////////////////////////////////////////

start group

name    = "Printer Interfaces Table"

id      = 2

class   = "DMTF|Printer Interfaces Table|001"

description = "This group specifies general attributes that apply "

"to the interface characteristics of the printer."

key     = 1

start attribute

name    = "Index"

id      = 1

description = "A unique value for each interface supported by "

"printer.  The value for each device must remain "

"constant at least from one initialization of the "

"device to the next initialization."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0

end attribute

start attribute

name    = "Description"

id      = 2

description = "A textual string containing information about the "

"interface.  This string should include the name of "

"the manufacturer, the product name and the version "

"of the hardware interface."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(256)

value   = ""  //Interface description goes here

end attribute

start attribute

name    = "Interface Type"

id      = 3

description = "Type of Interface"

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "InterfaceType"  //Enumeration type 2

//Note: MIB enum = ifType

type = INTEGER

0 = "Reserved"

1 = "Other"

2 = "Regular1822"

3 = "HDH1822"

4 = "DDN-x25"

5 = "RFC877-x25"

6 = "Ethernet-csmacd"

7 = "ISO88023-csmacd"

8 = "ISO88024-tokenBus"

9 = "ISO88025-tokenRing"

10 = "ISO88026-man"

11 = "StarLan"

12 = "Proteon-10Mbit"

13 = "Proteon-80Mbit"

14 = "Hyperchannel"

15 = "FDDI"

16 = "LAPB"

17 = "SDLC"

18 = "DS1"                // T-1

19 = "E1"                 // European equiv. of T-1

20 = "BasicISDN"

21 = "PrimaryISDN"

22 = "PropPointToPointSerial" // proprietary serial

23 = "PPP"

24 = "SoftwareLoopback"

25 = "Eon"                // CLNP over IP

26 = "Ethernet-3Mbit"

27 = "NSIP"               // XNS over IP

28 = "SLIP"               // generic SLIP

29 = "Ultra"              // ULTRA technologies

30 = "DS3"                // T-3

31 = "SIP"                // SMDS

32 = "Frame-relay"

END ENUM

value  = 1     // Other if not specified

end attribute

start attribute

name    = "MTU"

id      = 4

description = "The size of the largest datagram which can be "

"sent or received on the interface, specified in "

"octets.  For interfaces that are used for "

"transmitting network datagrams, this is the size "

"of the largest network datagram that can be sent "

"on the interface."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0

end attribute

start attribute

name    = "Speed"

id      = 5

description = "An estimate of the interface current bandwidth "

"in bits per second. For interfaces which do not "

"vary in bandwidth or for those where no accurate "

"estimation can be made, this object should contain "

"the nominal bandwidth."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0

end attribute

start attribute

name    = "Physical Address"

id      = 6

description = "The interface address at the protocol layer "

"immediately below (in OSI terms) the network layer in"

"the protocol stack.  For interfaces which do not have "

"such an address (e.g., a serial line), this object "

"should contain an octet string of zero length."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(64)

value   = ""

end attribute

start attribute

name    = "Administrative Status"

id      = 7

description = "The desired state of the interface. The "

"testing(3) state indicates that no operational "

"packets can be passed."

access  = READ-WRITE

storage = SPECIFIC

type    = START ENUM

name = "AdminStatus"  //Enumeration type 1

//Note: MIB enum = ifAdminStatus

type = INTEGER

0 = "Other"

1 = "Up"

2 = "Down"

3 = "Testing"

END ENUM

value  = 0     // Other if not specified

end attribute

start attribute

name    = "Operational Status"

id      = 8

description = "The current operational state of the interface. The "

"testing(3) state indicates that no operational "

"packets can be passed."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "OperStatus"  //Enumeration type 1

//Note: MIB enum = ifOperStatus

type = INTEGER

0 = "Other"

1 = "Up"

2 = "Down"

3 = "Testing"

END ENUM

value  = 0     // Other if not specified

end attribute

end group

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//               PRINTER STORAGE TABLE -required                       //

//                                                                     //

// This table attempts to mimic attributes in the SNMP                 //

// hrStorage Table for use in the MIF.                                 //

//                                                                     //

// Compatibility with SNMP is favored even where printer related       //

// information could have been better optimized in the MIF.            //

//                                                                     //

// Note that ENUMs may not follow the standard DMTF convention for     //

// 0=Other and 1=Unknown. This is because SNMP had previously defined  //

// these enums as Integers and assigned the values.                    //

//                                                                     //

/////////////////////////////////////////////////////////////////////////

start group

name    = "Printer Storage Table"

id      = 3

class   = "DMTF|Storage Table|001"

description = "This group specifies general attributes that apply "

"to the storage characteristics of the printer and "

"forms a table of logical storage areas on the printer."

""

"An entry shall be placed in the storage table for "

"each logical area of storage that is allocated and "

"has fixed resource limits.  The amount of storage "

"represented in an entity is the amount actually "

"usable by the requesting entity, and excludes loss "

"due to formatting or file system reference "

"information."

""

"This table is intended to be a useful diagnostic "

"for `out of memory' and `out of buffers' types of "

"failures.  In addition, it can be a useful "

"performance monitoring tool for tracking memory, "

"disk, or buffer usage."

key     = 1

start attribute

name    = "Index"

id      = 1

description = "A unique value for each storage area in the "

"printer.  The value for each device must remain "

"constant at least from one re-initialization of the "

"agent to the next re-initialization."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0

end attribute

start attribute

name    = "Storage Type"

id      = 2

description = "Type of Storage"

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "StorageType"   //Enumeration type 1

//Note: MIB enum = hrStorageType

type = INTEGER

0 = "Reserved"

1 = "Other"

2 = "Ram"

3 = "VirtualMemory" //Temporary paged or swapped memory

4 = "FixedDisk"

5 = "RemovableDisk"

6 = "FloppyDisk"

7 = "CompactDisc"

8 = "RamDisk"

END ENUM

value  = 1     // Other if not specified

end attribute

start attribute

name    = "Description"

id      = 3

description = "A description of the type and instance of the "

"storage described by this entry in the localization "

"specified by General Printer Current Localization."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(256)

value   = ""      //Storage description goes here.

end attribute

start attribute

name    = "Allocation Units"

id      = 4

description = "The size, in bytes, of the data objects allocated "

"from this pool.  If this entry is monitoring "

"sectors, blocks, buffers, or packets, for example, "

"this number will commonly be greater than one. "

"Otherwise this number will typically be one."

access  = READ-ONLY

storage = SPECIFIC

type    = GAUGE

value   = ""  //allocation unit goes here

end attribute

start attribute

name    = "Size"

id      = 5

description = "The size of the storage represented by this entry, "

"expressed in Allocation Units."

access  = READ-WRITE

storage = SPECIFIC

type    = GAUGE

value   = 0   //0 if unspecified

end attribute

start attribute

name    = "Used"

id      = 6

description = "The amount of the storage represented by this entry, "

"that is allocated, expressed in Allocation Units."

access  = READ-ONLY

storage = SPECIFIC

type    = GAUGE

value   = 0   //0 if unspecified

end attribute

end group

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                PRINTER DEVICE TABLE -required                       //

//                                                                     //

// This table attempts to mimic objects in the Device Group of the     //

// SNMP Host MIB.                                                      //

//                                                                     //

// Compatibility with SNMP is favored even where printer related       //

// information could have been better optimized in the MIF.            //

//                                                                     //

// Note that ENUMs may not follow the standard DMTF convention for     //

// 0=Other and 1=Unknown. This is because SNMP had previously defined  //

// these enums as Integers and assigned the values.                    //

//                                                                     //

/////////////////////////////////////////////////////////////////////////

start group

name    = "Printer Device Table"

id      = 4

class   = "DMTF|Device Table|001"

description = "This group specifies general attributes that apply "

"to the device characteristics of the printer."

key     = 1

start attribute

name    = "Index"

id      = 1

description = "A unique value for each device contained by the "

"printer.  The value for each device must remain "

"constant at least from one re-initialization of the "

"agent to the next re-initialization."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0

end attribute

start attribute

name    = "Device Type"

id      = 2

description = "Type of Device "

"If this value is Processor (3) then an entry exists "

"in the Processor Table which corresponds to this device."

"If this value is Printer (5) then an entry exists "

"in the Printer Table which corresponds to this device."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "DeviceType"   //Enumeration type 1

//Note: MIB enum = hrDeviceType

type = INTEGER

0 = "Reserved"

1 = "Other"

2 = "Unknown"

3 = "Processor"

5 = "Printer"

7 = "Reserved"

8 = "Reserved"

9 = "Reserved"

10 = "Video"

11 = "Audio"

12 = "Coprocessor"

13 = "Keyboard"

14 = "Modem"

16 = "Pointing"

18 = "Tape"

19 = "Clock"

END ENUM

value  = 2     // Unknown if not specified

end attribute

start attribute

name    = "Description"

id      = 3

description = "A description of this device, including the "

"the manufacturer, revision, and serial number "

"in the localization  specified by General Printer "

"Localization."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(64)

value   = ""      //Device description goes here.

end attribute

start attribute

name    = "ID"

id      = 4

description = "The Product ID for this device in the "

"localization  specified by General Printer "

"Localization."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(64)

value   = ""      //Product ID goes here.

end attribute

start attribute

name    = "Status"

id      = 5

description = "The current operational state of the device "

"described by this row of the table. A value "

"unknown(1) indicates that the current state of the "

"device is unknown. Running(2) indicates that the "

"device is up and running and that no unusual error "

"conditions are known. The warning(3) state "

"indicates an "

"unusual error condition

but the device is "

"still 'operational'.

A value of "

"testing(4), indicates that the device is not "

"available for use because it is in the testing "

"state. The state of down(5) is used only when the "

" device is not "

"available for any use."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "DeviceStatus"   //Enumeration type 1

//Note: MIB enum = hrDeviceStatus

type = INTEGER

0 = "Reserved"

1 = "Unknown"

2 = "Running"

3 = "Warning"

4 = "Testing"

5 = "Down"

6 = "Standby"        //Note: Should be added to MIB.

END ENUM

value  = 2     // Unknown if not specified

end attribute

start attribute

name    = "Processor Firmware ID"

id      = 6

description = "The Product ID of the firmware associated with the "

"Processor, in the Localization specified by General "

"Printer Localization"

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(64)

value   = ""      //Product ID goes here if the corresponding value

//of Device Table, Device Type = "Processor"

end attribute

start attribute

name    = "Processor Load"

id      = 7

description = "The average, over the last minute, of the "

"percentage of time that this processor was not "

"idle."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0   //Value is ignored unless Device Type = "Processor"

end attribute

start attribute

name    = "Printer Status"

id      = 8

description = "The current status of this printer device. When "

"in the idle(3), printing(4), or warmup(5) state, "

"the corresponding Device Status should be "

"running(2) or warning(3).  When in the unknown(2) "

"state, the corresponding Device Status should be "

"unknown(1)."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "PrinterStatus"   //Enumeration type 1

//Note: MIB enum = hrPrinterStatus

type = INTEGER

0 = "Reserved"

1 = "Other"

2 = "Unknown"

3 = "Idle"

4 = "Printing"

5 = "Warmup"

6 = "Offline"         //Note: Should be added to MIB

7 = "NotApplicable"   //Note: Should be added to MIB

END ENUM

value  = 2     // Unknown if not specified

end attribute

start attribute

name    = "Printer Detected Error State"

id      = 9

description = "This object represents any error conditions "

"detected by the printer.  The error conditions are "

"encoded as bits in an octet string, with the "

"following definitions:"

""

"Condition         Bit #    Device Status "

""

"lowPaper          0        warning(3) "

"noPaper           1        down(5) "

"lowToner          2        warning(3) "

"noToner           3        down(5) "

"doorOpen          4        down(5) "

"jammed            5        down(5) "

"offline           6        down(5) "

"serviceRequested  7        warning(3) "

""

"If multiple conditions are concurrently detected and "

"Device Status would not otherwise be unknown(1) or "

"testing(4), then Device Status shall correspond to the "

"worst state of those indicated, where down(5) is worse "

"than warning(3) which is which is worse than running(2)."

""

"Bits are numbered starting with the most significant "

"bit of the first byte being bit 0, the least significant "

"bit of the first byte being bit 7, the most significant "

"bit of the second byte being bit 8, and so on. A one bit "

"encodes that the condition was detected, while a zero bit "

"encodes that the condition was not detected."

""

"This object is useful for alerting an operator to "

"specific warning or error conditions that may "

"occur, especially those requiring human "

"intervention."

access  = READ-ONLY

storage = SPECIFIC

type    = OCTETSTRING

value   = ""  //Value is ignored unless Device Type = "Printer"

end attribute

end group

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                  PRINTER GENERAL GROUP - required                   //

//                                                                     //

// This group specifies general attributes that apply to the overall   //

// printer. In the common IETF-DMTF printer model, this is one of the  //

// four required attribute groups comprising the General Printer       //

// sub-unit. In the DMTF MIF, it is simply a required group.           //

//                                                                     //

/////////////////////////////////////////////////////////////////////////

start group

name    = "Printer General"

id      = 5

class   = "DMTF|General|001"

description = "This group specifies general attributes that "

"apply to the overall printer and its configuration."

start attribute

name    = "Configuration Changes"

id      = 1

description = "A count of the configuration changes for this "

"printer.  This value is incremented each time a "

"configuration change is made to this printer "

"component. This value may be compared against a "

"previous read value to determine if any configuration "

"changes have been made to the printer."

""

"Management applications may cache infrequently "

"changed configuration information about sub-units "

"on the printer. This object should be incremented "

"whenever the agent wishes such applications to "

"invalidate that cache and re-download all of this "

"configuration information, thereby signaling a "

"change of the printer configuration. "

""

"Configuration changes represent changes in "

"capabilities. These changes involve modifications "

"which may affect the set of jobs that be printed "

"by a system, increasing or decreasing the likelihood "

"of successful printing. Examples might include "

"the addition or removal of input devices, output "

"devices, media paths and interpreters among other "

"items"

""

"For example, if an input tray that contained paper of "

"different dimensions was added, this counter would be "

"incremented."

""

"As an additional example, this counter would not be "

"incremented when an input tray is removed or the "

"level of an input device is changed."

access  = READ-ONLY

storage = SPECIFIC

type    = COUNTER

value   = 0   //Initial count = 0.

end attribute

start attribute

name    = "Current Localization"

id      = 2

description = "This value is an index into the printer localization "

"table specifying the current language, country code "

"and character set to be used in the DESCRIPTION "

"string values for components, groups, and attributes. "

"If there are no strings under the control of this "

"object that must be localized, the value should be "

"set to zero."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = 0

end attribute

start attribute

name    = "Reset"

id      = 3

description = "Setting this value to Power Cycle Reset, Reset to "

"NVRAM or Reset To Factory Defaults will result in "

"the resetting of the printer. When read, this "

"object will always have the value Not Resetting. "

"Setting the value to Not Resetting will have no "

"effect on the printer."

access  = READ-WRITE

storage = SPECIFIC

type    = START ENUM

name = "ResetType" //Enumeration type 3

type = INTEGER

3 = "Not Resetting"

4 = "Power Cycle Reset - cold start"

5 = "Reset to NVRM - warm start"

6 = "Reset to Factory Defaults - resets NVRAM"

END ENUM

value   = 3      //Not Resetting

end attribute

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                    PRINTER RESPONSIBLE PARTY - optional             //

//                                                                     //

// These attributes are optional. They extend the general printer      //

// to include information about current operator and service person.   //

// An attribute that is not supported or that has no meaning for this  //

// input device, should give the key word UNSUPPORTED as its value.    //

//                                                                     //

// Optional group interleaved with Mandatory attributes for            //

// consistency with SNMP Printer MIB                                   //

/////////////////////////////////////////////////////////////////////////

start attribute

name    = "Printer Current Operator"

id      = 4

description = "The name of the operator or person responsible for "

"this printer. This string may include additional "

"information such as phone number."

access  = READ-WRITE

storage = SPECIFIC

type    = STRING(127)

value   = ""     //Operator Name goes here

end attribute

start attribute

name    = "Printer Service Person"

id      = 5

description = "The name of the last person responsible for "

"servicing this printer. This string may include "

"additional information such as phone number."

access  = READ-WRITE

storage = SPECIFIC

type    = STRING(127)

value   = ""     //Operator Name goes here

end attribute

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//              PRINTER DEVICE DEFAULT INDEXES - required              //

//                                                                     //

// These attributes specify the default Input, Output, Marker, and     //

// Media Paths that will be used in the printer.                       //

// The value of the Index parameter specified in this object is        //

// an index to a specific Input, Output, Marker or Media Path entry in //

// the appropriate table.                                              //

/////////////////////////////////////////////////////////////////////////

start attribute

name    = "Input Default Index"

id      = 6

description = "The value of the Index corresponding to the default "

"Input device in the Input Table. This value selects "

"the default source of input media."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = 0

end attribute

start attribute

name    = "Output Default Index"

id      = 7

description = "The value of the Index corresponding to the default "

"Output device in the Output Table. This value "

"selects the default destination of print media."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = 0

end attribute

start attribute

name    = "Marker Default Index"

id      = 8

description = "The value of the Index corresponding to the default "

"Marker device in the Marker Table. This value "

"selects the default marker device."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = 0

end attribute

start attribute

name    = "Media Path Default Index"

id      = 9

description = "The value of the Media Path Index corresponding to "

"the default media path; that is, the selection of "

"the default media path."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = 1   //Value must be 1 or greater

end attribute

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                        PRINTER CONSOLE - required                   //

//                                                                     //

// The MIF only allows 1 Console per printer.                          //

/////////////////////////////////////////////////////////////////////////

start attribute

name    = "Console Localization"

id      = 10

description = "An index into the localization table giving the "

"language, country, and character set to be used "

"for the console."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = 1   //Value must be 1 or greater

end attribute

start attribute

name    = "Console Number Of Display Lines"

id      = 11

description = "The number of lines on the printer's physical "

"display. This value is zero if there are no lines "

"on the physical display or if there is no physical "

"display."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0   //No lines if not specified

end attribute

start attribute

name    = "Console Number Of Display Characters"

id      = 12

description = "The number of characters per line displayed on the "

"physical display."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0   //No characters if not specified

end attribute

start attribute

name    = "Console Disable"

id      = 13

description = "A lock to provide one-at-a-time access:"

"If the value is (3 - operatorConsoleEnabled) then"

"input is accepted from the operator console. If"

"the value is (4 - operatorConsoleDisabled) then"

"input is not accepted from the operator console."

access  = READ-WRITE

storage = SPECIFIC

type    = START ENUM

name = "DisableState"   //Enumeration type 2

type = INTEGER

1 = "Other"

3 = "operatorConsoleEnabled"

4 = "operatorConsoleDisabled"

END ENUM

value   = 3      //Operator Console Enabled if not specified

end attribute

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                 PRINTER INTERFACE DEFAULTS - optional               //

//                                                                     //

// These attributes are optional. They extend the general printer      //

// to include information about the number of interfaces and the       //

// default interface for this device.                                  //

/////////////////////////////////////////////////////////////////////////

start attribute

name    = "Number of Interfaces"

id      = 14

description = "The number of network interfaces (regardless of "

"their current state) present on this Printer."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0

end attribute

start attribute

name    = "Interface Default Index"

id      = 15

description = "The value of the Index corresponding to the default "

"Interface in the Interface Table. This value selects "

"the default Interface for the printer."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = 0

end attribute

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                 PRINTER STORAGE DEFAULTS - optional                 //

//                                                                     //

// These attributes are optional. They extend the general printer      //

// to include information about the primary storage in the printer.    //

/////////////////////////////////////////////////////////////////////////

start attribute

name    = "Memory Size"

id      = 16

description = "The amount of physical main memory contained by the "

"printer in K-Bytes."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0

end attribute

end group

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                      PRINTER COVER TABLE -required                  //

//                                                                     //

// This table specifies general attributes that apply to the doors     //

// and interlocks of the printer. There is a group entry in the cover  //

// table for each door or interlock. Although these values may change  //

// due to a major reconfiguration of the device (e.g. the addition of  //

// of equipment with additional doors or interlocks), the values are   //

// expected to remain stable across successive power cycles.           //

//                                                                     //

// In the common IETF-DMTF printer model, the cover table is one of    //

// the four required attribute groups comprising the General Printer   //

// sub-unit. In the DMTF MIF, it is simply a required table.           //

/////////////////////////////////////////////////////////////////////////

start group

name    = "Printer Cover Table"

id      = 6

class   = "DMTF|Cover Table|001"

description = "This group specifies general attributes that apply "

"to the doors and interlocks of the printer."

key     = 1

start attribute

name    = "Cover Index"

id      = 1

description = "A unique value used by the printer to identify this "

"cover. Although these values may change due to a "

"major reconfiguration of the device ( e.g. the "

"addition of new covers to the printer), values are "

"expected to remain stable across successive printer "

"printer power cycles."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0

end attribute

start attribute

name    = "Description"

id      = 2

description = "The manufacturer provided cover name in the "

"localization specified by General Printer Current "

"Localization."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(256)

value   = ""      //Cover description goes here.

end attribute

start attribute

name    = "Status"

id      = 3

description = "The current status of this cover."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "CoverStatus" //Enumeration type 2

type = INTEGER

1 = "Other"

2 = "Unknown"

3 = "Cover Open"

4 = "Cover Closed"

5 = "Interlock Open"

6 = "Interlock Closed"

END ENUM

value   = 2      //Initial status = Unknown.

end attribute

end group

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                 PRINTER LOCALIZATION TABLE - required               //

//                                                                     //

// The localization table identifies the natural language, country and //

// character set in which character strings are expressed. One or more //

// localizations may be supported by a printer. The available          //

// localizations are defined by entries in this table.                 //

//                                                                     //

// In the common IETF-DMTF printer model, this table is one of         //

// the four required attribute groups comprising the General Printer   //

// sub-unit. In the DMTF MIF, it is simply a required table.           //

/////////////////////////////////////////////////////////////////////////

start group

name    = "Printer Localization Table"

id      = 7

class   = "DMTF|Localization Table|001"

description = "This table identifies the natural language, country "

"and character sets supported by the printer."

key     = 1

start attribute

name    = "Index"

id      = 1

description = "A unique value used to identify a specific entry "

"in the localization table. Although these values "

"may change due to a major reconfiguration of the "

"device (e. g. the addition of new Character Sets "

"to the printer), values are expected to remain "

"stable across successive printer power cycles."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0

end attribute

start attribute

name    = "Language"

id      = 2

description = "A two character language code defined by ISO 639. "

"Examples are EN, FR and DE."

access  = READ-ONLY

storage = SPECIFIC

type    = OCTETSTRING(2)

value  = ""      //Language code goes here.

end attribute

start attribute

name    = "Country"

id      = 3

description = "A two character country code defined by ISO 639. "

"Examples are US, GB, CA, and DE."

access  = READ-ONLY

storage = SPECIFIC

type    = OCTETSTRING(2)

value   = ""    //Blank value means that country is undefined.

end attribute

start attribute

name    = "Localization Character Set"

id      = 4

description = "The coded character set used for this localization "

"Must be 1 or greater."

access  = READ-ONLY

storage = SPECIFIC

type    = "CodedCharSet"

value   = 2       //Unknown if not specified.

end attribute

end group

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                       PRINTER INPUT TABLE                           //

//                                                                     //

// A printer may have multiple input devices associated with it. For   //

// example, a printer can have two input source trays. The input       //

// devices associated with a printer are identified in this table.     //

// Each entry in the table is an input device definition with an       //

// ordered set of attributes. Input devices typically have a location, //

// a type, an identifier, a set of constraints on possible media sizes //

// and potentially other media characteristics, and may be capable of  //

// indicating current status or capacity. Only those attributes        //

// applicable to the input device need be specified.                   //

/////////////////////////////////////////////////////////////////////////

start group

name    = "Printer Input Table"

id      = 8

class   = "DMTF|Input Table|001"

description = "A list of input devices capable of providing "

"printing media resources to this printer including "

"information regarding their identity, possible and "

"current configuration, and status."

key     = 1

start attribute

name    = "Index"

id      = 1

description = "A unique value used by this printer to identify an  "

"associated input device. Although these values may  "

"change due to a major reconfiguration of the "

"printer (e.g. the addition of new input devices to "

"the printer), values are expected to remain stable "

"across successive printer power cycles."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0

end attribute

start attribute

name    = "Type"

id      = 2

description = "The type of technology (discriminated primarily "

"according to feeder mechanism type) employed by "

"input device."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "InputType"   //Enumeration type 2

type = INTEGER

1 = "Other"

2 = "Unknown"

3 = "Sheet Feed Auto Removable Tray"

4 = "Sheet Feed Auto Non-Removable Tray"

5 = "Sheet Feed Manual"

6 = "Continuous Roll"

7 = "Continuous Fan Fold"

END ENUM

value  =  2      //Unknown if current input type is not specified.

end attribute

start attribute

name    = "Dimension Unit"

id      = 3

description = "The unit of measure for specifying dimensional "

"values for this input device."

access  = READ-ONLY

storage = SPECIFIC

type    = "MediaUnit"

value   = 2       //Unknown if not specified.

end attribute

start attribute

name    = "Declared Media Dimension Feed Direction"

id      = 4

description = "This object provides the value of the declared  "

"dimension in the feed direction of the media that "

"is placed or will be placed in this input device. "

"The feed direction is the direction in which the "

"media is fed on this device. The dimension is "

"measured in Input Dim Units. If this device can "

"reliably sense this value, the value is sensed and "

"is read-only access. Otherwise the value is read-"

"write access and may be written by management or "

"control panel applications. The value (-1) means "

"other and specifically indicates that this device "

"places no restrictions on this parameter. The value "

"(-2) indicates unknown."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = -2       //Unknown if not specified

end attribute

start attribute

name    = "Declared Media Dimension Cross Feed Direction"

id      = 5

description = "This object provides the value of the declared  "

"dimension in the cross feed direction of the media "

"that is placed or will be placed in this device. "

"The cross feed direction is ninety degrees relative "

"to the feed direction on this device. The dimension "

"is measured in Input Dimension Units. If this device "

"can reliably sense this value, the value is sensed "

"and is read-only access. Otherwise the value is read "

"-write access and may be written by management or "

"control panel applications. The value (-1) means "

"other and specifically indicates that this device "

"places no restrictions on this parameter. The value "

"(-2) indicates unknown."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = -2     //Unknown if not specified

end attribute

start attribute

name    = "Chosen Media Dimension Feed Direction"

id      = 6

description = "This object provides the value of the granted "

"dimension in the feed direction of the media that "

"is placed or will be placed in this input device. "

"Feed dimension measurements are taken parallel "

"relative to the feed direction of the device and "

"measured in Input Dimension Units. If the printer "

"supports the declared dimension, the granted "

"dimension is the same as the declared dimension. If "

"not, the granted dimension is set to the closest "

"dimension that the printer supports when the "

"declared dimension is set. The value (-1) means "

"other and specifically indicates that this device "

"places no restrictions on this parameter. The value "

"(-2) indicates unknown."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = -2       //Unknown if not specified

end attribute

start attribute

name    = "Chosen Media Dimension Cross Feed Direction"

id      = 7

description = "This object provides the value of the granted "

"dimension in the cross feed direction of the media "

"that is placed or will be placed in this device. "

"The cross feed direction is ninety degrees relative "

"to the feed direction on this device. The dimension "

"is measured in Input Dimension Units. If the printer "

"supports the declared dimension the granted dimension "

"is the same as the declared dimension. If not the "

"granted dimension is set to the closet dimension "

"that the printer supports when the declared dimension "

"is set. The value (-1) means other and specifically "

"indicates that this device places no restrictions "

"on this parameter. The value (-2) indicates unknown."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = -2     //Unknown if not specified

end attribute

start attribute

name    = "Capacity Units"

id      = 8

description = "The unit of measure used for measuring capacity "

"values for this device."

access  = READ-ONLY

storage = SPECIFIC

type    = "CapacityUnit"

value   = 2        //Unknown if not specified.

end attribute

start attribute

name    = "Maximum Capacity"

id      = 9

description = "The maximum capacity of the input device in Input "

"Capacity Units. There is no convention "

"associated with the media itself so this value is "

"claimed capacity. If the input device can reliably "

"sense this value, the value is sensed by the printer "

"and is read-only: otherwise the value may be written "

"by a management or control console application.  The "

"value (-1) means other and specifically indicates "

"that the input device places no restrictions on this "

"parameter. The value (-2) means unknown."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = -2    //Unknown if not explicitly specified

end attribute

start attribute

name    = "Current Level"

id      = 10

description = "The current capacity of the input device in Input "

"Capacity Units. If the input device can "

"reliably sense this value, the value is sensed by "

"the printer and is read-only; otherwise the value "

"may be written by a management or control console "

"application.  The value (-1) means other and "

"specifically indicates that this devices places no "

"restrictions on this parameter. The value (-2) "

"means unknown."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = -2    //Unknown unless explicitly specified

end attribute

// Capacity Attribute Relationships //

//

//               INPUT MEASUREMENT

//

//     _______ |                     |

//      ^      |                     |

//      |      |                     |                    |

//      |      |_ _ _ _ _ _ _ _ _ _ _| _________________  |direction

//      |      |                     |      ^             v

// MaxCapacity |                     |      |

//      |      | Sheets left in tray | CurrentLevel

//      |      |                     |      |

//      v      |                     |      v

//     _______ +_____________________+ _______

start attribute

name    = "Status"

id      = 11

description = "This value indicates the current status of this "

"input device."

""

"SubUnitStatus is an integer sum of 5 "

"distinct values: Availability, Non-Critical, "

"Critical, On-line, and Transitioning. These "

"values are:"

""

"Availability                   value "

""

" Available and Idle              0       000'b' "

" Available and Standby           2       010'b' "

" Available and Active            4       100'b' "

" Available and Busy              6       110'b' "

" Unavailable and OnRequest       1       001'b' "

" Unavailable because Broken      3       011'b' "

" Unknown                         5       101'b' "

""

"Non-Critical "

""

" No Non-Critical Alerts          0 "

" Non-Critical Alerts             8 "

""

"Critical "

""

" No Critical Alerts              0 "

" Critical Alerts                 16 "

""

"On-Line "

""

" Intended state is On-Line       0 "

" Intended state is Off-Line      32 "

""

"Transitioning "

""

" At intended state               0 "

" Transitioning to intended state 64 "

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0       //Initial status = Ready Available

end attribute

start attribute

name    = "Media Name"

id      = 12

description = "The name of the current media contained in this "

"input device. Examples are ISO-A4-White or Legal "

"Tender Bond Paper. See ISO standard name "

"references for recommended values."

access  = READ-WRITE

storage = SPECIFIC

type    = STRING(63)

value   = ""      //Media name goes here

end attribute

//          INPUT TYPE EXTENDED  -optional attributes

//

// The following attributes are optional parameters that augment the

// the type attribute for this input device. An attribute that is not

// supported or that has no meaning for this input device, should give

// the key word UNSUPPORTED as its value.

start attribute

name    = "Name"

id      = 13

description = "The name assigned to this input device."

access  = READ-WRITE

storage = SPECIFIC

type    = STRING(63)

value   = ""      //Input device name goes here

end attribute

start attribute

name    = "Vendor Name"

id      = 14

description = "The vendor name of this input device."

"component."

access  = READ-ONLY

storage = COMMON

type    = STRING(63)

value   = ""      //Vendor name goes here

end attribute

start attribute

name    = "Model"

id      = 15

description = "The model name of this input device."

access  = READ-ONLY

storage = COMMON

type    = STRING(63)

value   = ""

end attribute

start attribute

name    = "Version"

id      = 16

description = "The version string for this input device."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(63)

value   = ""

end attribute

start attribute

name    = "Serial Number"

id      = 17

description = "The serial number assigned to this input device."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(63)

value   = ""

end attribute

start attribute

name    = "Description"

id      = 18

description = "A free form text description of this input device "

"in the localization specified by General Current "

"Localization."

access  = READ-ONLY

storage = SPECIFIC

type    = String (255)

value   = ""       //Free form text goes here

end attribute

start attribute

name    = "Security"

id      = 19

description = "Indicates if this device has some security "

"associated with it."

access  = READ-WRITE

storage = COMMON

type    = "PresentOnOff"

value   = 5      //Assumed not present if not specified

end attribute

//          INPUT MEDIA EXTENDED  -optional attributes

//

// The following attributes are optional parameters that augment the

// the media attribute for this input device.

// The input media extended attributes support identification of media

// installed or available for use on a printing device. Medium resources

// are identified by name. and include a collection of characteristic

// attributes that may further be used for selection or management of

// them.

// An attribute that is not supported or that has no meaning for this

// input media, should give the key word UNSUPPORTED as its value.

start attribute

name    = "Media Weight"

id      = 20

description = "The weight of the media associated with this Input "

"device in grams / per meter squared. The value (-1) "

"means other and specifically indicates that the "

"device places no restriction on this parameter. The "

"value (-2) means unknown."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = -2    //Unknown unless explicitly specified

end attribute

start attribute

name    = "Media Type"

id      = 21

description = "The name of the type of the type of medium "

"associated with this input device. Valid values are "

"standardized strings from ISO 10175 (DPA) and ISO "

"10180  (SPDL) which are: stationary, transparency, "

"envelope, envelope-plain, envelope window, "

"continuous-long, continuous-short, tab-stock, "

"labels, multi-layer."

access  = READ-WRITE

storage = SPECIFIC

type    = STRING(63)

value   = ""    //Media type name goes here

end attribute

start attribute

name    = "Media Color"

id      = 22

description = "The name of color of the medium associated with "

"this input device. Valid values are standardized "

"strings from ISO 10175 (DPA) and ISO 10180 (SPDL) "

"which are: other, unknown, white, pink, yellow, "

"buff, goldenrod, blue, green, transparent."

access  = READ-WRITE

storage = SPECIFIC

type    = STRING(63)

value   = ""    //Name of the color of medium goes here

end attribute

start attribute

name    = "Media Form Parts"

id      = 23

description = "The number of parts associated with the media "

"associated with this input device if the media "

"is a multi-part form. The value (-1) means other "

"and specifically indicates the device places "

"no restrictions on this parameter. The value (-2) "

"means unknown."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = -2    //Unknown unless explicitly specified

end attribute

end group

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                      PRINTER OUTPUT TABLE                           //

//                                                                     //

// A printer may have multiple output devices associated with it. For  //

// example, a printer can have two output trays. The output            //

// devices associated with a printer are identified in this table.     //

// Each entry in the table is an output device definition with an      //

// ordered set of attributes. Output devices typically have a location,//

// a type, an identifier, a set of constraints on possible media sizes //

// and potentially other media characteristics, and may be capable of  //

// indicating current status or capacity. Only those attributes        //

// applicable to the output device need be specified.                  //

/////////////////////////////////////////////////////////////////////////

start group

name    = "Printer Output Table"

id      = 9

class   = "DMTF|Output Table|001"

description = "A list of output devices capable of receiving "

"media delivered from the printing process including "

"information regarding their identity, possible and "

"current configuration, and status."

key     = 1

start attribute

name    = "Index"

id      = 1

description = "A unique value used by this printer to identify an  "

"associated output device. Although these values may "

"change due to a major reconfiguration of the "

"printer (e.g. the addition of new output devices to "

"the printer), values are expected to remain stable "

"across successive printer power cycles."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0

end attribute

start attribute

name    = "Type"

id      = 2

description = "The type of technology employed by this output "

"device."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "OutputType"   //Enumeration type 2

type = INTEGER

1 = "Other"

2 = "Unknown"

3 = "Removable Bin"

4 = "Unremovable Bin"

5 = "Continuous Roll Device"

6 = "Mailbox"

7 = "Continuous Fan Fold"

END ENUM

value   = 2      //Unknown if current output type is not specified.

end attribute

start attribute

name    = "Capacity Unit"

id      = 3

description = "The unit of measure for specifying capacity "

"values for this device."

access  = READ-ONLY

storage = SPECIFIC

type    = "CapacityUnit"

value   = 2      //Unknown if not specified.

end attribute

start attribute

name    = "Maximum Capacity"

id      = 4

description = "The maximum capacity of this device in Output "

"Capacity Units. There is no convention "

"associated with the media itself so this value is "

"claimed capacity. If the device can reliably "

"sense this value, the value is sensed by the printer "

"and is read-only: otherwise the value may be written "

"by a management or control console application.  The "

"value (-1) means other and specifically indicates "

"that the device places no restrictions on this "

"parameter. The value (-2) means unknown."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = -2    //Unknown if not explicitly specified

end attribute

start attribute

name    = "Remaining Capacity"

id      = 5

description = "The remaining capacity of the possible capacity "

"in Capacity Units. If the output device can "

"reliably sense this value, the value is sensed by "

"the printer and is read-only; otherwise the value "

"may be written by a management or control console "

"application.  The value (-1) means other and "

"specifically indicates that this devices places no "

"restrictions on this parameter. The value (-2) "

"means unknown."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = -2    //Unknown unless explicitly specified

end attribute

// Capacity Attribute Relationships //

//

//

//               OUTPUT MEASUREMENT

//

//     _______ |                     | _______

//      ^      |                     |      ^

//      |      |                     |      |

//      |      |                     | RemainingCapacity

// MaxCapacity |                     |      |

//      |      |                     |      v               ^

//      |      |_ _ _ _ _ _ _ _ _ _ _| ___________________  |direction

//      |      |                     |                      |

//      |      |  Sheets in output   |

//      v      |                     |

//     _______ +_____________________+

//

start attribute

name    = "Status"

id      = 6

description = "This value indicates the current status of this "

"output device."

""

"SubUnitStatus is an integer sum of 5 "

"distinct values: Availability, Non-Critical, "

"Critical, On-line, and Transitioning. These "

"values are:"

""

"Availability                   value "

""

" Available and Idle              0       000'b' "

" Available and Standby           2       010'b' "

" Available and Active            4       100'b' "

" Available and Busy              6       110'b' "

" Unavailable and OnRequest       1       001'b' "

" Unavailable because Broken      3       011'b' "

" Unknown                         5       101'b' "

""

"Non-Critical "

""

" No Non-Critical Alerts          0 "

" Non-Critical Alerts             8 "

""

"Critical "

""

" No Critical Alerts              0 "

" Critical Alerts                 16 "

""

"On-Line "

""

" Intended state is On-Line       0 "

" Intended state is Off-Line      32 "

""

"Transitioning "

""

" At intended state               0 "

" Transitioning to intended state 64 "

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0      //Initial status = Ready Available

end attribute

//          OUTPUT TYPE EXTENDED  -optional attributes

// The following attributes are optional parameters that augment the

// the type attribute for this output device. An attribute that is not

// supported or that has no meaning for this output device, should give

// the key word UNSUPPORTED as its value.

start attribute

name    = "Name"

id      = 7

description = "The name assigned to this output device."

access  = READ-WRITE

storage = SPECIFIC

type    = STRING(63)

value   = ""      //Output device name goes here

end attribute

start attribute

name    = "Vendor Name"

id      = 8

description = "The vendor name of this output device."

"component."

access  = READ-ONLY

storage = COMMON

type    = STRING(63)

value   = ""      //Vendor name goes here

end attribute

start attribute

name    = "Model"

id      = 9

description = "The model name of this output device."

access  = READ-ONLY

storage = COMMON

type    = STRING(63)

value   = ""

end attribute

start attribute

name    = "Version"

id      = 10

description = "The version string for this output device."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(63)

value   = ""

end attribute

start attribute

name    = "Serial Number"

id      = 11

description = "The serial number assigned to this output device."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(63)

value   = ""

end attribute

start attribute

name    = "Description"

id      = 12

description = "A free form text description of this output device "

"in the localization specified by General Current "

"Localization."

access  = READ-ONLY

storage = SPECIFIC

type    = String (255)

value   = ""       //Free form text goes here

end attribute

start attribute

name    = "Security"

id      = 13

description = "Indicates if this device has some security "

"associated with it."

access  = READ-WRITE

storage = COMMON

type    = "PresentOnOff"

value   = 5     //Assumed not present if not specified

end attribute

//          OUTPUT DIMENSIONS EXTENDED  -optional attributes

// The following attributes are optional parameters that augment the

// the dimensions for this output device.

start attribute

name    = "Dimension Unit"

id      = 14

description = "The unit of measure for specifying dimensional "

"values for this output device."

access  = READ-ONLY

storage = SPECIFIC

type    = "MediaUnit"

value   = 2      //Unknown if not specified.

end attribute

start attribute

name    = "Maximum Dimension Feed Direction"

id      = 15

description = "This object provides the value of the maximum "

"dimension in the feed direction of the media that "

"is placed or will be placed in this output device. "

"Feed dimension measurements are taken parallel "

"relative to the feed direction of the device and "

"measured in Output Dim Units. If the printer "

"can reliably sense this value, the value is sensed "

"by the printer and is read-only; otherwise the "

"value may be written (set) by a management "

"application."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = -2       //Unknown if not specified

end attribute

start attribute

name    = "Maximum Dimension Cross Feed Direction"

id      = 16

description = "The maximum dimensions supported by this output "

"device for measurements taken ninety degrees "

"relative to the feed direction associated with "

"this device in Output Dim Units. If this value "

"can be reliably sensed by this device, the value "

"is sensed by the printer and is read-only; "

"otherwise the value may be written (set) by a "

"management application."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = -2     //Unknown if not specified

end attribute

start attribute

name    = "Minimum Dimension Feed Direction"

id      = 17

description = "This object provides the value of the minimum "

"dimension in the feed direction of the media that "

"is placed or will be placed in this output device. "

"Feed dimension measurements are taken parallel "

"relative to the feed direction of the device and "

"measured in Output Dim Units. If the printer "

"can reliably sense this value, the value is sensed "

"by the printer and is read-only; otherwise the "

"value may be written (set) by a management "

"application."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = -2       //Unknown if not specified

end attribute

start attribute

name    = "Minimum Dimension Cross Feed Direction"

id      = 18

description = "The minimum dimensions supported by this output "

"device for measurements taken ninety degrees "

"relative to the feed direction associated with "

"this device in Output Dim Units. If this value "

"can be reliably sensed by this device, the value "

"is sensed by the printer and is read-only; "

"otherwise the value may be written (set) by a "

"management application."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = -2     //Unknown if not specified

end attribute

//              OUTPUT FEATURES  -optional attributes

// The following attributes are optional parameters that augment the

// the output device table.

start attribute

name    = "Stacking Order"

id      = 19

description = "The current state of the stacking order for this "

"output device. First To Last means that as pages are "

"output, the front of the next page is placed  "

"against the back of the previous page. Last to "

"First means as pages are output, the back of the "

"next page is placed against the front of the "

"previous page."

access  = READ-WRITE

storage = SPECIFIC

type    = START ENUM

name = "StackingOrder"  //Enumeration type 1

type = INTEGER

1 = "Other"

2 = "Unknown"

3 = "First To Last"

4 = "Last to First"

END ENUM

value   = 2      //Unknown if not specified

end attribute

start attribute

name    = "Page Delivery Orientation"

id      = 20

description = "The reading surface that will be up when pages are "

"delivered to the output device. Values are Face Up "

"and Face Down."

access  = READ-WRITE

storage = SPECIFIC

type    = START ENUM

name = "PageDeliveryOrientation"  //Enumeration type 1

type = INTEGER

1 = "Other"

2 = "Unknown"

3 = "Face Up"

4 = "Face Down"

END ENUM

value   = 2    //Unknown if not specified

end attribute

start attribute

name    = "Bursting"

id      = 21

description = "This object indicates that the output unit supports "

"bursting, and if so, whether the feature is "

"enabled. Bursting is the process by which "

"continuous media is separated into individual "

"sheets, typically by bursting along perforations."

access  = READ-WRITE

storage = COMMON

type    = "PresentOnOff"

value   = 5     //Assumed not present if not specified

end attribute

start attribute

name    = "Decollating"

id      = 22

description = "This object indicates that the output device "

"supports decollating, and if so, whether the "

"feature is enabled. Decollating is the process "

"by which the parts of a multi-part form are "

"separated and sorted into separate stacks for "

"each part."

access  = READ-WRITE

storage = COMMON

type    = "PresentOnOff"

value   = 5     //Assumed not present if not specified

end attribute

start attribute

name    = "Page Collated"

id      = 23

description = "This object indicates that the output device "

"supports page collation, and if so, whether the "

"feature is enabled."

access  = READ-WRITE

storage = COMMON

type    = "PresentOnOff"

value   = 5     //Assumed not present if not specified

end attribute

start attribute

name    = "Offset Stacking"

id      = 24

description = "This object indicates that the output device "

"supports offset stacking and if so "

"whether the feature is enabled."

access  = READ-WRITE

storage = COMMON

type    = "PresentOnOff"

value   = 5     //Assumed not present if not specified

end attribute

end group

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                         PRINTER MARKER TABLE                        //

//                                                                     //

// Printer Marker is the mechanism that produces marks on the print    //

// media. A printer can contain one or more marking mechanisms. Some   //

// examples of multiple marker devices are: a printer with separate    //

// markers for normal and magnetic ink, or an imagesetter that can     //

// output to both a proofing device and final film. Each marker device //

// is represented by and entry in the Printer Marker Table. Each       //

// marker can have its own associated characteristics such as marking  //

// technology and resolution.                                          //

/////////////////////////////////////////////////////////////////////////

start group

name    = "Printer Marker Table"

id      = 10

class   = "DMTF|Marker Table|001"

description = "A list of marker devices associated with this "

"printer capable of producing marks on the print "

"media."

key     = 1

start attribute

name    = "Index"

id      = 1

description = "A unique value used by this printer to identify an  "

"associated marker device. Although these values may "

"change due to a major reconfiguration of the "

"printer (e.g. the addition of new marker devices to "

"the printer), values are expected to remain stable "

"across successive printer power cycles."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 1   //Value must be 1 or greater

end attribute

start attribute

name    = "Marking Technology"

id      = 2

description = "The type of technology employed by this marker "

"device."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "MarkingTech"   //Enumeration type 2

type = INTEGER

1 = "Other"

2 = "Unknown"

3 = "Electrophotographic LED"

4 = "Electrophotographic Laser"

5 = "Electrophotographic Other"

6 = "Impact Moving Head Dot Matrix 9 Pin"

7 = "Impact Moving Head Dot Matrix 24 Pin"

8 = "Impact Moving Head Dot Matrix Other"

9 = "Impact Moving Head Fully Formed"

10 = "Impact Band"

11 = "Impact Other"

12 = "Inkjet Aqueous"

13 = "Inkjet Solid"

14 = "Inkjet Other"

15 = "Pen"

16 = "Thermal Transfer"

17 = "Thermal Sensitive"

18 = "Thermal Diffusion"

19 = "Thermal Other"

20 = "Electroerosion"

21 = "Electrostatic"

22 = "Photographic Microfiche"

23 = "Photographic Image Setter"

24 = "Photographic Other"

25 = "Ion Deposition"

26 = "E Beam"

27 = "Typesetter"

END ENUM

value   = 2    //Unknown if marker technology is not specified.

end attribute

start attribute

name    = "Counter Unit"

id      = 3

description = "The unit that will be used by the printer when "

"reporting counter values for this marker device."

"The time units of measure are provided for a device "

"like a strip recorder that does not or can not "

"track the physical dimension of the media and does "

"not use characters, lines or sheets."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "CounterUnit"  //Enumeration type 1

type = INTEGER

1 = "Reserved"

2 = "Reserved"

3 = "Ten Thousandths Of Inches"

4 = "Micrometers"

5 = "Characters"

6 = "Lines"

7 = "Impressions"

8 = "Sheets"

9 = "Dot Rows"

11 = "Hours"

16 = "Feet"

17 = "Meters"

END ENUM

value   = 2    //Unknown if not specified.

end attribute

start attribute

name    = "Life Count"

id      = 4

description = "The number of units counted "

"during the life of the printer using the units "

"specified by Counter Unit."

access  = READ-ONLY

storage = SPECIFIC

type    = COUNTER

value   = 0   //INITIAL COUNT = 0.

end attribute

start attribute

name    = "Power On Count"

id      = 5

description = "The number of units counted "

"since the printer was most recently powered on using "

"the units of measure specified by Counter Unit."

access  = READ-ONLY

storage = SPECIFIC

type    = COUNTER

value   = 0   //Initial count = 0.

end attribute

start attribute

name    = "Process Colorants"

id      = 6

description = "The number of process colors supported by this "

"marker. A process color of 1 implies monochrome. "

"The value of this object and the value of the "

"Spot Colorant object can not both be 0."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER    //Must be 0 or greater

value   = 1   //Monochrome

end attribute

start attribute

name    = "Spot Colorants"

id      = 7

description = "The number of spot colors supported by this marker. "

"The value of this object and the value of the "

"Process colorant object can not both be 0."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER    //Must be 0 or greater

value   = 0   //No spot color if not specified

end attribute

start attribute

name    = "Addressability Unit"

id      = 8

description = "The unit of measure of distance."

access  = READ-ONLY

storage = SPECIFIC

type    = "AddrUnit"

value   = 2    //Unknown if not specified.

end attribute

start attribute

name    = "Addressability Feed Direction"

id      = 9

description = "The number of addressable marking positions in the "

"feed direction per 1000 units of measure specified "

"by Addressable Unit. A value of (-1) implies "

" 'other' or 'infinite' while a value of (-2) "

"implies 'unknown'."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = -2  //Unknown if not specified

end attribute

start attribute

name    = "Addressability Cross Feed Direction"

id      = 10

description = "The number of addressable marking positions in the "

"cross feed direction per 1000 units of measure "

"specified by Addressable Unit. A value of (-1)  "

"implies 'other' or 'infinite' while a value of (-2) "

"implies 'unknown'."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = -2  //Unknown if not specified

end attribute

start attribute

name    = "North Margin"

id      = 11

description = "The margin, in units identified by Addressable "

"Unit, from the leading edge of the medium as the "

"medium flows through the marking engine with the "

"side to be image facing the observer. The leading "

"edge is the North edge and the other edges are "

"defined by the normal compass layout of directions "

"with the compass facing the observer. Printing "

"within the area bounded by all four margins is "

"guaranteed for all interpreters. The value (-2) "

"means unknown."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = -2  //Unknown if not specified

end attribute

start attribute

name    = "South Margin"

id      = 12

description = "The margin from the South edge (see North Margin) "

"of the medium in units identified by Addressable "

"Unit. Printing within the area bounded by all four "

"margins is guaranteed for all interpreters. The "

"value (-2) means unknown."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = -2  //Unknown if not specified

end attribute

start attribute

name    = "West Margin"

id      = 13

description = "The margin from the West edge (see North Margin) "

"of the medium in units identified by Addressable "

"Unit. Printing within the area bounded by all four "

"margins is guaranteed for all interpreters. The "

"value (-2) means unknown."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = -2  //Unknown if not specified

end attribute

start attribute

name    = "East Margin"

id      = 14

description = "The margin from the East edge (see North Margin) "

"of the medium in units identified by Addressable "

"Unit. Printing within the area bounded by all four "

"margins is guaranteed for all interpreters. The "

"value (-2) means unknown."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = -2  //Unknown if not specified

end attribute

start attribute

name    = "Status"

id      = 15

description = "This value indicates the current status of this "

"marker device. "

""

"SubUnitStatus is an integer sum of 5 "

"distinct values: Availability, Non-Critical, "

"Critical, On-line, and Transitioning. These "

"values are: "

""

"Availability                   value "

""

" Available and Idle              0       000'b' "

" Available and Standby           2       010'b' "

" Available and Active            4       100'b' "

" Available and Busy              6       110'b' "

" Unavailable and OnRequest       1       001'b' "

" Unavailable because Broken      3       011'b' "

" Unknown                         5       101'b' "

""

"Non-Critical "

""

" No Non-Critical Alerts          0 "

" Non-Critical Alerts             8 "

""

"Critical "

""

" No Critical Alerts              0 "

" Critical Alerts                 16 "

""

"On-Line "

""

" Intended state is On-Line       0 "

" Intended state is Off-Line      32 "

""

"Transitioning "

""

" At intended state               0 "

" Transitioning to intended state 64"

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   =  0      //Initial status = Ready Available

end attribute

end group

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                   PRINTER MARKER SUPPLIES TABLE                     //

//                                                                     //

// A marker can have one or more supplies associated with it. For      //

// example a marker may use both toner and fuser oil supplies.         //

// A marker may also have more then one supply source for a given type //

// of supply e.g. multiple supply sources of toner. Each unique source //

// of supply for a marker device is an entry in the marker supply table//

// Each supply entry can have its own characteristics associated with  //

// it such as colorant and current supply level.                       //

/////////////////////////////////////////////////////////////////////////

start group

name    = "Printer Marker Supplies Table"

id      = 11

class   = "DMTF|Marker Supplies Table|001"

description = "A list of marker devices, with their associated "

"supplies and supplies characteristics."

key     = 1

start attribute

name    = "Index"

id      = 1

description = "A unique value used by the printer to identify this "

"marker supply container/receptacle. Although these "

"values may change due to a major reconfiguration of "

"the printer (e.g. the addition of new marker "

"supplies to the printer), values are expected to "

"to remain stable across successive printer power "

"cycles."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 1   //Value must be 1 or greater

end attribute

start attribute

name    = "Marker Index"

id      = 2

description = "The value of the Marker Index corresponding to the "

"marker device with which this marker supply entity "

"is associated."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 1   //Value must be 1 or greater

end attribute

start attribute

name    = "Colorant Index"

id      = 3

description = "The value of the Colorant Index corresponding to "

"the colorant with which this marker supply entity "

"is associated. This value shall be 0 if there is no "

"Colorant Table."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0   //Value is 0 if there is no Colorant Table

end attribute

start attribute

name    = "Class"

id      = 4

description = "This value indicates whether this supply entity "

"represents a supply container that is consumed or a "

"receptacle that is filled."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "MkrSupplyClass" //Enumeration type 1

type = INTEGER

1 = "Other"

2 = "Reserved"

3 = "Supply That is Consumed"

4 = "Receptacle That is Filled"

END ENUM

value   = 2      //"Unknown if not specified

end attribute

start attribute

name    = "Type"

id      = 5

description = "The type of this supply."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "MarkingTech"   //Enumeration type 3

type = INTEGER

1 = "Other"

2 = "Unknown"

3 = "Toner"

4 = "Waste Toner"

5 = "Ink"

6 = "Ink Cartridge"

7 = "Ink Ribbon"

8 = "Waste Ink"

9 = "OPC"

10 = "Developer"

11 = "Fuser Oil"

12 = "Solid Wax"

13 = "Ribbon Wax"

14 = "Waste Wax"

END ENUM

value   = 2    //Unknown if not specified.

end attribute

start attribute

name    = "Description"

id      = 6

description = "The description of this supply/receptacle in the "

"localization specified by General Current "

"Localization."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(256)

value   = ""    //Free form text description goes here

end attribute

start attribute

name    = "Supply Unit"

id      = 7

description = "Unit of measure of this marker supply container "

"or receptacle."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "MkrSupplyUnit"      //Enumeration type 1

type = INTEGER

1 = "Reserved"

2 = "Reserved"

3 = "Ten Thousandths Of Inches"

4 = "Micrometers"

7 = "Impressions"

8 = "Sheets"

11 = "Hours"

12 = "Thousandths Of Ounces"

13 = "Tenths Of Grams"

14 = "Hundredths of Fluid Ounces"

15 = "Tenths Of Milliliters"

16 = "Feet"

17 = "Meters"

END ENUM

value   = 2    //Unknown if not specified.

end attribute

start attribute

name    = "Maximum Capacity"

id      = 8

description = "The maximum capacity of this supply container/ "

"receptacle expressed in Supply Units. If this "

"supply container/receptacle can reliably sense this "

"value, the value is sensed by the printer and is "

"read-only; otherwise the value may be written by "

"by a control panel or management application. The "

"value (-1) means other and places no restrictions "

"on this parameter. The value (-2) means unknown."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = -2          // Unknown if not specified

end attribute

start attribute

name    = "Current Level"

id      = 9

description = "The current level if this supply is a container; "

"the remaining space if this supply is a receptacle. "

"If this supply container receptacle can reliably "

"sense this value, the value is sensed by the "

"printer and is read-only; otherwise the value may "

"be written by a control panel or management "

"application. The value (-1) means other and "

"places no restrictions on this parameter. The "

"value (-2) means unknown."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = -2          // Unknown if not specified

end attribute

end group

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                 PRINTER MARKER COLORANT TABLE                       //

//                                                                     //

/////////////////////////////////////////////////////////////////////////

start group

name    = "Printer Marker Colorant Table"

id      = 12

class   = "DMTF|Marker Colorant Table|001"

description = "A table of all the colorants available on the "

"printer."

key     = 1

start attribute

name    = "Index"

id      = 1

description = "A unique value used by the printer to identify this "

"colorant. Although these values may change due to a "

"major reconfiguration of the device (e.g. the "

"addition of new colorants to the printer), values "

"are expected to remain stable across successive "

"printer power cycles."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 1   //Value must be 1 or greater

end attribute

start attribute

name    = "Marker Index"

id      = 2

description = "The value of the Marker Index corresponding to the "

"marker device with which this colorant entry "

"is associated."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 1   //Value must be 1 or greater

end attribute

start attribute

name    = "Role"

id      = 3

description = "The role played by this colorant."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "ColorantRole"       //Enumeration type 1

type = INTEGER

1 = "Other"

2 = "Unknown"

3 = "Process"

4 = "Spot"

END ENUM

value  = 2    //Unknown if not specified.

end attribute

start attribute

name    = "Value"

id      = 4

description = "The name of the color of this colorant using "

"standardized string names from ISO 10175 (DPA) "

"and ISO 10180 (SPDL) which are:                "

"  other                                        "

"  unknown                                      "

"  white                                        "

"  red                                          "

"  green                                        "

"  blue                                         "

"  cyan                                         "

"  magenta                                      "

"  yellow                                       "

"  black                                        "

"Implementers may add additional string values. "

"The ISO 9070 naming conventions are recommended "

"in order to avoid potential name clashes."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(63)

value  = ""  //Colorant string

end attribute

start attribute

name    = "Tonality"

id      = 5

description = "The distinct levels of tonality realizable by a "

"marking device when using this colorant. This value "

"does not include the number of levels of tonal "

"difference that an interpreter can obtain by "

"techniques such as half toning. This value must be "

"at least 2."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value  = 2    //Minimum value if not specified

end attribute

end group

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                     PRINTER MEDIA PATH GROUP                        //

//                                                                     //

// The media paths encompass the mechanisms in the printer that move   //

// the media through the printer and connect all other media related   //

// devices: input, outputs, markers and finishers. A printer contains  //

// one or more media paths defined in the Media Path Table. This group //

// defines the attributes that apply to all paths in the table.        //

/////////////////////////////////////////////////////////////////////////

start group

name    = "Printer Media Path Table"

id      = 13

class   = "DMTF|Media Path Table|001"

description = "This object defines attributes that apply to all of "

"the media paths defined in the Media Path Table. "

Key     = 1

start attribute

name    = "Index"

id      = 1

description = "A unique value used by the printer to identify this "

"media path. Although these values may change due to "

"a major reconfiguration of the device (e.g. the "

"addition of new media paths to the printer), values "

"are expected to remain stable across successive "

"printer power cycles."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 1   //Value must be 1 or greater

end attribute

start attribute

name    = "Maximum Speed Print Unit"

id      = 2

description = "The unit of measure used in specifying the speed "

"of all Media Paths in the printer, i.e., the "

"numerator of the units of measure of the media path "

"speed."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "SpeedPrintUnit"     //Enumeration type 1

type = INTEGER

1 = "Reserved"

2 = "Reserved"

3 = "Ten Thousandths of Inches per Hour"

4 = "Micrometers per Hour"

5 = "Characters per Hour"

6 = "Lines per Hour"

7 = "Impressions per Hour"

8 = "Sheets per Hour"

9 = "Dot Rows per Hour"

16 = "Feet per Hour"

17 = "Meters per Hour"

END ENUM

value   = 2        //Unknown if not specified.

end attribute

start attribute

name    = "Media Size Unit"

id      = 3

description = "The unit of measure of media size for use in "

"in specifying dimensional values for all media "

"paths in the printer."

access  = READ-ONLY

storage = SPECIFIC

type    = "MediaUnit"

value   = 2        //Unknown if not specified.

end attribute

start attribute

name    = "Maximum Speed"

id      = 4

description = "The maximum printing speed of this media path "

"expressed in media speed units (units of Max Speed "

"Print Unit per Max Speed Time Unit). A value (-1) "

"implies Other."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = -1  //Unlimited

value   = -2  //Unknown if not specified

end attribute

start attribute

name    = "Maximum Media Feed Direction"

id      = 5

description = "The maximum physical media size in the feed "

"direction of this media path expressed in units of "

"measure specified by Media Size Unit. A value (-1) "

"implies 'unlimited'. A value (-2) implies 'unknown'."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = -1  //Unknown if not specified

end attribute

start attribute

name    = "Maximum Media Cross Feed Direction"

id      = 6

description = "The maximum physical media size across the feed "

"direction of this media path expressed in units of "

"measure specified by Media Size Unit. A value (-1) "

"implies other."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = -1  //Unknown if not specified

end attribute

start attribute

name    = "Minimum Media Feed Direction"

id      = 7

description = "The minimum physical media size in the feed "

"direction of this media path expressed in units of "

"measure specified by Media Size Unit. A value (-1) "

"implies other."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = -1  //Unknown if not specified

end attribute

start attribute

name    = "Minimum Media Cross Feed Direction"

id      = 8

description = "The minimum physical media size across the feed "

"direction of this media path expressed in units of "

"measure specified by Media Size Unit. A value (-1) "

"implies other."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = -1  //Unknown if not specified

end attribute

start attribute

name    = "Type"

id      = 9

description = "The type of media path for this media path."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "MediaPathType"        //Enumeration type 2

type = INTEGER

1 = "Other"

2 = "Unknown"

3 = "Long Edge Binding Duplex"

4 = "Short Edge Binding Duplex"

5 = "Simplex"

END ENUM

value   = 2      //Unknown if not specified.

end attribute

start attribute

name    = "Description"

id      = 10

description = "The manufacturer provided description of this media "

"path in the localization specified by General "

"Current Localization."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(256)

value   = ""   //Description string goes here

end attribute

start attribute

name    = "Status"

id      = 11

description = "This value indicates the current status of this "

"media path. "

""

"SubUnitStatus is an integer sum of 5 "

"distinct values: Availability, Non-Critical, "

"Critical, On-line, and Transitioning. These "

"values are: "

""

"Availability                   value "

""

" Available and Idle              0       000'b' "

" Available and Standby           2       010'b' "

" Available and Active            4       100'b' "

" Available and Busy              6       110'b' "

" Unavailable and OnRequest       1       001'b' "

" Unavailable because Broken      3       011'b' "

" Unknown                         5       101'b' "

""

"Non-Critical "

""

" No Non-Critical Alerts          0 "

" Non-Critical Alerts             8 "

""

"Critical "

""

" No Critical Alerts              0 "

" Critical Alerts                 16 "

""

"On-Line "

""

" Intended state is On-Line       0 "

" Intended state is Off-Line      32 "

""

"Transitioning "

""

" At intended state               0 "

" Transitioning to intended state 64 "

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0      //Initial status = Ready Available

end attribute

end group

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                      PRINTER CHANNEL TABLE                          //

//                                                                     //

// Channels are independent sources of print data. Here, print data is //

// the term used for information that is to be used to construct print //

// and may have both data and control aspects. The output of a channel //

// is in a form suitable for input to one of the interpreters as a     //

// stream that may be independently enabled (allowing print data to    //

// flow) or disabled (stopping the flow of print data). A printer may  //

// have one or more channels.                                          //

/////////////////////////////////////////////////////////////////////////

start group

name    = "Printer Channel Table"

id      = 14

class   = "DMTF|Channel Table|001"

description = "The Channel Table represents the set of input data "

"sources which can provide print data to one or "

"of the interpreters available on the printer."

Key     = 1

start attribute

name    = "Index"

id      = 1

description = "A unique value used by the printer to identify this "

"data channel. Although these values may change due "

"to a major reconfiguration of the device (e.g. the "

"addition of new data channels to the printer), "

"values are expected to remain stable across "

"successive printer power cycles."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 1   //Value must be 1 or greater

end attribute

start attribute

name    = "Type"

id      = 2

description = "The type of print data channel. This object "

"provides the linkage to Channel-Type specific "

"groups that may (conceptually) extend the Channel "

"Table with additional details about that channel."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "ChannelType"   //Enumeration type 2

type = INTEGER

1 = "Other"

3 = "Serial Port"

4 = "Parallel Port"

5 = "IEEE1248 Port"

6 = "SCSI Port"

7 = "AppletalkPAP"

8 = "LPD Server"

9 = "Netware RPrinter"

10 = "Netware PServer"

11 = "Port9100"

12 = "App Socket"

13 = "FTP"

14 = "TFTP"

15 = "DLC LLC Port"

16 = "IBM 3270"

17 = "IBM 5250"

18 = "FAX"

19 = "IEEE 1394"

20 = "Transport 1"

21 = "CPAP"

22 = "DCE Remote Proc Call"

23 = "ONC Remote Proc Call"

24 = "OLE"

25 = "Named Pipe"

26 = "PC Print"

27 = "Server Message Block"

28 = "DPMF"

29 = "DLL Protocol Call"

30 = "VxD Protocol Call"

31 = "System Object Manager"

32 = "DEC LAT"

33 = "NPAP"

END ENUM

value   = 1        //Other if not specified.

end attribute

start attribute

name    = "Protocol Version"

id      = 3

description = "The version of the protocol used on this channel. "

"The format used for version numbering depends on "

"Channel Type."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(63)

value   = ""  //Version string goes here

end attribute

start attribute

name    = "Current Job Control Language Index"

id      = 4

description = "The value of Interpreter Index corresponding to the "

"Control Language interpreter for this channel. "

"This interpreter defines the syntax used for "

"control functions, such as querying or changing "

"environment variables and identifying job "

"boundaries (e.g. PJL, PostScript, NPAP, IPDS)."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = 1   //Value must be 1 or greater

end attribute

start attribute

name    = "Default Page Description Language Index"

id      = 5

description = "The value of Interpreter Index corresponding to the "

"Page Description Language interpreter for this "

"channel. This interpreter defines the default "

"page description language interpreter to be used "

"for the print data unless the Control language is "

"used to select a specific interpreter (e.g., PCL, "

"PostScript, IPDS, auto-sense)."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = 1   //Value must be 1 or greater

end attribute

start attribute

name    = "State"

id      = 6

description = "The state of this print channel. The value "

"determines whether control information "

"and print data is allowed through this channel or "

"not."

access  = READ-WRITE

storage = SPECIFIC

type    = START ENUM

name = "ChannelState"         //Enumeration type 2

type = INTEGER

1 = "Other"

3 = "No Data Accepted"

END ENUM

value   = 1       //Other if not specified.

end attribute

start attribute

name    = "Interface Index"

id      = 7

description = "The value of Interface Index (in the Interface Table) "

"When one or more row of the table is relevant, "

"this is the index of the row representing the top "

"layer in the interface hierarchy. A value of zero "

"indicates that no interface is associated with this "

"channel."

access  = READ-WRITE

storage = SPECIFIC

type    = INTEGER

value   = 0

end attribute

start attribute

name    = "Status"

id      = 8

description = "This value indicates the current status of this "

"channel. "

""

"SubUnitStatus is an integer sum of 5 "

"distinct values: Availability, Non-Critical, "

"Critical, On-line, and Transitioning. These "

"values are: "

""

"Availability                   value "

""

" Available and Idle              0       000'b' "

" Available and Standby           2       010'b' "

" Available and Active            4       100'b' "

" Available and Busy              6       110'b' "

" Unavailable and OnRequest       1       001'b' "

" Unavailable because Broken      3       011'b' "

" Unknown                         5       101'b' "

""

"Non-Critical "

""

" No Non-Critical Alerts          0 "

" Non-Critical Alerts             8 "

""

"Critical "

""

" No Critical Alerts              0 "

" Critical Alerts                 16 "

""

"On-Line "

""

" Intended state is On-Line       0 "

" Intended state is Off-Line      32 "

""

"Transitioning "

""

" At intended state               0 "

" Transitioning to intended state 64"

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0      //Initial status = Ready Available

end attribute

end group

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                     PRINTER INTERPRETER TABLE                       //

//                                                                     //

// Interpreters are responsible for the conversion of a description    //

// of intended print instances into images that are to be marked on    //

// the media. A printer may have one or more interpreters. The         //

// interpreter table has one entry per interpreter where interpreters  //

// include both Page Description and Control Language interpreters.    //

//                                                                     //

/////////////////////////////////////////////////////////////////////////

start group

name    = "Printer Interpreter Table"

id      = 15

class   = "DMTF|Interpreter Table|001"

description = "The Interpreter Table represents the set of "

"interpreters available on the printer."

Key     = 1

start attribute

name    = "Index"

id      = 1

description = "A unique value for each PDL or control language for "

"which there exists an interpreter or emulator in "

"the printer. The value is used to identify this "

"interpreter. Although these values may change due "

"to a major reconfiguration of the device (e.g. the "

"addition of new interpreters to the printer), "

"values are expected to remain stable across "

"successive printer power cycles."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 1   //Value must be 1 or greater

end attribute

start attribute

name    = "Language Family"

id      = 2

description = "The family name of a Page Description Language "

"(PDL) or control language which this interpreter in "

"in the printer can interpret or emulate."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "IntLangFam"    //Enumeration type 2

type = INTEGER

1 = "Other"

3 = "PCL"

4 = "HPGL"

5 = "PJL"

6 = "PS"

7 = "IPDS"

8 = "PPDS"

9 = "Escape P"

10 = "Epson"

11 = "DDIF"

12 = "Interpress"

13 = "ISO 6429"

14 = "Line Data"

15 = "MODCA"

16 = "ReGIS"

17 = "SCS"

18 = "SPDL"

19 = "Tek4014"

20 = "PDS"

21 = "IGP"

22 = "Code V"

23 = "DSC DSE"

24 = "WPS"

25 = "Ln03"

26 = "CCITT"

27 = "QUIC"

28 = "CPAP"

29 = "DEC PPL"

30 = "simple text"

31 = "NPAP"

32 = "DOC"

33 = "imPRESS"

34 = "PIN WRITER"

35 = "NPDL"

36 = "NEC201PL"

37 = "automatic"

38 = "PAGES"

39 = "LIPS"

40 = "TIFF"

41 = "Diagnostic"

42 = "PSprinter"

43 = "CaPSL"

44 = "EXCL"

45 = "LCDS"

46 = "XES"

END ENUM

value   = 1       //Other if not specified.

end attribute

start attribute

name    = "Language Level"

id      = 3

description = "The level of the language which this interpreter "

"is interpreting or emulating. This might contain a "

"value like 5e for an interpreter which is "

"interpreting level 5e of the PCL language. It might "

"contain 2 for an interpreter which is emulating "

"level 2 of the PostScript language. Similarly it "

"might contain 2 for an interpreter which is "

"emulating level 2 of the HPGL language."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(63)

value   = ""  //Level string goes here

end attribute

start attribute

name    = "Language Version"

id      = 4

description = "The date or version of the language which this "

"interpreter is interpreting or emulating. "

"Channel Type."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(63)

value   = ""  //Version string goes here

end attribute

start attribute

name    = "Description"

id      = 5

description = "A string to identify this interpreter in the "

"localization specified by General Current "

"Localization as opposed to the language which is "

"being interpreted. It is anticipated that this "

"string will allow manufacturers to unambiguously "

"identify their interpreters."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(256)

value   = ""  //Description string goes here

end attribute

start attribute

name    = "Version"

id      = 6

description = "The date code, version number or other product "

"specific information tied to this interpreter. This "

"value is associated with the interpreter, rather "

"than the version of the language which is being "

"interpreted or emulated."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING(63)

value   = ""  //Version string goes here

end attribute

start attribute

name    = "Default Orientation"

id      = 7

description = "The current default media orientation for this "

"interpreter. May be overridden for a particular job "

"by a command in the input datastream."

access  = READ-WRITE

storage = SPECIFIC

type    = START ENUM

name = "MediaOrientation" //Enumeration type 1

type = INTEGER

1 = "Other"

2 = "Unknown"

3 = "Portrait"

4 = "Landscape"

END ENUM

value   = 2       //Unknown if current default is not specified.

end attribute

start attribute

name    = "Addressability Feed Direction"

id      = 8

description = "The maximum interpreter addressability in the feed "

"direction in  10000 Marker Addressability Units "

"(see Marker Addressability Feed Dir) for this "

"interpreter. The value (-1) means other and "

"specifically indicates that this interpreter "

"places no restrictions on this parameter."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = -1  //Other if not specified

end attribute

start attribute

name    = "Addressability Cross Feed Direction"

id      = 9

description = "The maximum interpreter addressability in the cross "

"feed direction in 10000 Marker Addressability Units "

"(see Marker Addressability XFeed Dir) for this "

"interpreter. The value (-1) means other and "

"specifically indicates that this interpreter "

"places no restrictions on this parameter."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = -1  //Other if not specified

end attribute

start attribute

name    = "Default Character Set In"

id      = 10

description = "The default Character Set for octets encountered "

"outside a context in which the PDL establishes the "

"interpretation of octets."

access  = READ-WRITE

storage = SPECIFIC

type    = "CodedCharSet"

value   = 2       //Unknown if current default is not specified.

end attribute

start attribute

name    = "Default Character Set Out"

id      = 11

description = "The default Character Set for data coming from "

"this interpreter through the printer's output channel."

access  = READ-WRITE

storage = SPECIFIC

type    = "CodedCharSet"

value   = 2       //Unknown if current default is not specified.

end attribute

start attribute

name    = "Two Way"

id      = 12

description = "This value indicates whether or not this "

"interpreter returns information back across the "

"data channel to the host."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "TwoWay"      //Enumeration type 1

type = INTEGER

3 = "Yes"

4 = "No"

END ENUM

value   = 4      //No if not specified

end attribute

end group

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                PRINTER CONSOLE DISPLAY BUFFER TABLE                 //

//                                                                     //

// Many printers have a console on the printer, the operator console,  //

// that is used to display and modify the state of the printer. The    //

// console can be as simple as a few indicators and switches or as     //

// or as complicated as full screen displays and keyboards . There     //

// can be at most one console.                                         //

/////////////////////////////////////////////////////////////////////////

start group

name    = "Printer Console Display Buffer Table"

id      = 16

class   = "DMTF|Display Buffer Table|001"

description = "This table contains one entry for each physical "

"line on the display. Lines cannot be added or "

"deleted."

key     = 1

start attribute

name    = "Index"

id      = 1

description = "A unique value for each console line in the "

"printer. The value is used to identify the console "

"line. Although these values may change due to a "

"major reconfiguration of the device (e.g. the "

"addition of new console lines to the printer). "

"values are expected to remain stable across "

"successive printer power cycles."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 1   //Value must be 1 or greater

end attribute

start attribute

name    = "Line Text"

id      = 2

description = "The content of a line in the logical display buffer "

"of the operator's console of the printer. When a "

"write operation occurs, normally a critical "

"message, to one of the Line Text strings, the agent "

"should make that line displayable if a physical "

"display is present. The printer should not "

"overwrite this line once written until a zero "

"length string is written to unlock that line. "

"Printer generated strings shall be in the "

"localization specified in the Console Localization. "

"Management Application generated strings should be "

"localized by the Management Application."

access  = READ-WRITE

storage = SPECIFIC

type    = STRING(256)

value   = ""  //Line text goes here

end attribute

end group

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                   PRINTER CONSOLE LIGHTS TABLE                      //

//                                                                     //

/////////////////////////////////////////////////////////////////////////

start group

name    = "Printer Console Lights Table"

id      = 17

class   = "DMTF|Console Lights Table|001"

description = "This table contains one entry for each console "

"light."

key     = 1

start attribute

name    = "Index"

id      = 1

description = "A unique value used by the printer to identify this "

"light. Although these values may change due to a "

"major reconfiguration of the device (e.g. the "

"addition of new console lights to the printer). "

"values are expected to remain stable across "

"successive printer power cycles."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 1   //Value must be 1 or greater

end attribute

start attribute

name    = "On Time"

id      = 2

description = "The on time in milliseconds of blinking of this "

"light; 0 indicates off always. If both Console On "

"Time and Console Off Time are 0, then the light is "

"always off."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0

end attribute

start attribute

name    = "Off Time"

id      = 3

description = "The off time in milliseconds of blinking of this "

"light; 0 indicates on always. If both Console On "

"Time and Console Off Time are 0, then the light is "

"always off."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0

end attribute

start attribute

name    = "Color"

id      = 4

description = "The color of this light."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "LightColor"  //Enumeration type 2

type = INTEGER

1 = "Other"

2 = "Unknown"

3 = "White"

4 = "Red"

5 = "Green"

6 = "Blue"

7 = "Cyan"

8 = "Magenta"

9 = "Yellow"

END ENUM

value   = 2    //Unknown if not specified

end attribute

start attribute

name    = "Description"

id      = 5

description = "The vendor description or label of this light in "

"the localization specified by Console Localization."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING (255)

value   = ""  //Description goes here

end attribute

end group

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//                        PRINTER ALERT TABLE                          //

//                                                                     //

// The alert table lists all the critical and non-critical alerts      //

// currently active in the printer. A critical alert is one that       //

// stops the printer from printing immediately and can not continue    //

// until the critical alert condition is eliminated. Non-critical      //

// alerts are those items that do not stop printing but may at some    //

// future time. The table contains information on the severity,        //

// component, detail location within the component, alert code and     //

// description of each critical alert that is currently active within  //

// the printer.                                                        //

/////////////////////////////////////////////////////////////////////////

start group

name    = "Printer Alert Table"

id      = 18

class   = "DMTF|Alert Table|001"

description = "This table lists all of the critical and "

"non-critical alerts currently active in the "

"printer."

key     = 1

start attribute

name    = "Index"

id      = 1

description = "The index value used to determine which alerts have "

"been added or removed from the alert table.  This "

"in an incrementing integer starting from zero every "

"time the printer is powered on. When the printer "

"adds an alert to the table, that alert is assigned "

"the next higher integer value from the last item "

"entered into the table. If the index value reaches "

"its maximum value, the next item entered will cause "

"the index value to roll over and start at zero "

"again. The first event placed in the alert table "

"after power up initialization of the printer shall "

"have an index value of 1.  NOTE: The management "

"application will read the alert table when a trap "

"or event notification occurs or at a periodic rate "

"and then parse the table to determine if any new "

"entries were added by comparing the last known "

"index value with the current highest index value. "

"The management application will then update its "

"copy of the alert table.  When the printer "

"discovers that an alert is no longer active, the "

"printer shall remove the row for that alert from "

"the table and shall reduce the number of rows in "

"the table. The printer may add or delete any number "

"of rows from the table at any time."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 0

end attribute

start attribute

name    = "Severity Level"

id      = 2

description = "The level of severity of this alert table entry. "

"The printer determines the severity level assigned "

"to each entry in the table."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "SeverityLevelAlertCode"  //Enumeration type 1

type = INTEGER

1 = "Other"

3 = "Critical"

4 = "Warning"

END ENUM

value   = 1    //Other if not specified

end attribute

start attribute

name    = "Training Level"

id      = 3

description = "The level of training required to handle this alert. "

"The training level is an enumeration that is "

"determined and assigned by the printer manufacturer "

"based on the information or training required to "

"handle this alert. The printer will break alerts "

"into these different training levels. It is the "

"responsibility of the management application in the "

"system to determine how a particular alert is "

"handled and how and to whom that alert is routed. "

"The following are the four training levels of "

"alerts: (1) FIELD SERVICE -alerts that typically "

"require training and technical knowledge of the "

"printer and its devices. An example of a technical "

"person would be a manufacturer's Field Service "

"representative, or other person formally trained by "

"the manufacturer or similar representative; (2) "

"TRAINED -alerts that require an intermediate or "

"moderate level of knowledge of the printer and its "

"devices. A typical example of an alert that a "

"trained operator can handle is replacing toner "

"cartridges; (3) UNTRAINED -alerts that can be fixed "

"without prior knowledge of the printer and its "

"devices either because the action to correct the "

"alert is obvious or the printer can help the "

"untrained person to fix the problem. Typical "

"examples of such alerts is reloading paper trays "

"and emptying output bins on a low end printer; (4) "

"MANAGEMENT -Alerts that have to do with the overall "

"operation and configuration of the printer. "

"Examples of management events are configuration "

"changes of devices."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "TrainingLevel"        //Enumeration type 2

type = INTEGER

1 = "Other"

2 = "Unknown"

3 = "Untrained"

4 = "Trained"

5 = "Field Service"

6 = "Management"

END ENUM

value   = 2    //Unknown if not specified

end attribute

start attribute

name    = "Group"

id      = 4

description = "The type of device within the printer model that "

"this alert is related. Input, Output, and Markers "

"are examples of printer model groups."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "AlertGroup"             //Enumeration type 1

type = INTEGER

1 = "Other"

3 = "Storage Reference"

4 = "Device Reference"

5 = "General Printer"

6 = "Cover"

7 = "Localization"

8 = "Input"

9 = "Output"

10 = "Marker"

11 = "Marker Supplies"

12 = "Marker Colorant"

13 = "Media Path"

14 = "Channel"

15 = "Interpreter"

16 = "Console"

END ENUM

value   = 1      //Other if not specified

end attribute

start attribute

name    = "Group Index"

id      = 5

description = "An index of the row within the principle table in "

"the group identified by Alert Group that "

"represents the device of the printer that caused "

"this alert. The combination of the Group and Group "

"Index defines exactly which printer sub-unit caused "

"the alert; for example Input #1, Output #2, and "

"Marker # 1."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = 1   // Must be 1 or greater

end attribute

start attribute

name    = "Location"

id      = 6

description = "The device location that is defined by the printer "

"manufacturer to further refine the location of this "

"alert within the designated device. The location is "

"used in conjunction with the Group and Group Index "

"values: for example, there is an alert at Input #2 "

"at location number 7."

access  = READ-ONLY

storage = SPECIFIC

type    = INTEGER

value   = -1 // -1 if Unknown

end attribute

start attribute

name    = "Alert Code"

id      = 7

description = "The code that describes the type of alert for this "

"entry. There are different codes for each sub-unit "

"type; for example, Media Supply Low and Media "

"Supply Empty are alert codes for input devices."

access  = READ-ONLY

storage = SPECIFIC

type    = START ENUM

name = "AlertCode"             //Enumeration type 2

type = INTEGER

1 = "Other"

2 = "Unknown"

3 = "Cover Open"            // Common Codes

4 = "Cover Closed"

5 = "Interlock Open"

6 = "Interlock Closed"

7 = "Configuration Change"

8 = "Jam"

501 = "Door Open"             // General Printer Codes

502 = "Door Closed"

503 = "Power Up"

504 = "Power Down"

801 = "Input Media Tray Missing" // Input Codes

802 = "Input Media Size Change"

803 = "Input Media Weight Change"

804 = "Input Media Type Change"

805 = "Input Media Color Change"

806 = "Input Media Forms Part Change"

807 = "Input Media Supply Low"

808 = "Input Media Supply Empty"

901 = "Output Media Tray Missing" // Output Codes

902 = "Output Media Tray Almost Full"

903 = "Output Media Tray Full"

1001 = "Marker Fuser Under Temperature" // Marker Codes

1002 = "Marker Fuser Over Temperature"

1101 = "Marker Toner Empty" // Marker Supplies Codes

1102 = "Marker Ink Empty"

1103 = "Marker Print Ribbon Empty"

1104 = "Marker Toner Almost Empty"

1105 = "Marker Ink Almost Empty"

1106 = "Marker Print Almost Ribbon Empty"

1107 = "Marker Waste Toner Receptacle Almost Full"

1108 = "Marker Waste Ink Receptacle Almost Full"

1109 = "Marker Waste Toner Receptacle Full"

1110 = "Marker Waste Ink Receptacle Full"

1111 = "Marker OPC Life Almost Over"

1112 = "Marker OPC Life Over"

1113 = "Marker Developer Almost Empty"

1114 = "Marker Developer Empty"

1301 = "Media Path Media Tray Missing" // Media Path Codes

1302 = "Media Path Media Tray Almost Full"

1303 = "Media Path Media Tray Full"

1501 = "Interpreter Memory Increase" // Interpreter Codes

1502 = "Interpreter Memory Decrease"

1503 = "Interpreter Cartridge Added"

1504 = "Interpreter Cartridge Deleted"

1505 = "Interpreter Resource Added"

1506 = "Interpreter Resource Deleted"

1507 = "Interpreter Resource Unavailable"

END ENUM

value   = 1      //Other if not specified

end attribute

start attribute

name    = "Description"

id      = 8

description = "A description of this alert entry in the "

"localization specified by General Current "

"Localization. The description is provided by "

"the printer to further elaborate on the enumerated "

"alert or provide information in the case where the "

"code is classified other or unknown. The printer "

"is required to return a description string but the "

"the string may be a null string."

access  = READ-ONLY

storage = SPECIFIC

type    = STRING (255)

value   = ""  //Description goes here

end attribute

/////////////////////////////////////////////////////////////////////////

//                                                                     //

//          ALERT TIME STAMP  -optional                                //

//                                                                     //

// The following attributes are optional parameters that augment the   //

// alert table.                                                        //

//                                                                     //

/////////////////////////////////////////////////////////////////////////

start attribute

name    = "Time"

id      = 9

description = "The time when this alert was generated."

access  = READ-ONLY

storage = SPECIFIC

type    = DATE

value   = ""  // yyyymmddHHMMSS.uuuuuu+000 (Supplied by DMI SL)

end attribute

end group

end component